Input paper:   VTS60-8.3.4

Input paper for the following Committee(s): 		Purpose of paper:

(Select as appropriate)

☐ ARM	☐  ENG		☐ PAP			☒ Input

☐ ENAV	☒ VTS					☐ Information

Agenda item 			8.3

Technical domain/ Task number 2	1.3.1

Author(s)/Submitter(s): Daewon Kim (KMOU), Sewoong Oh (KRISO), Jungjin Lee (GC), KCG

# Proposal on Use Cases of VTS Traffic Functions

# Summary

This document proposes a description of the Traffic Functions and related use cases in order to assist discussion on the operational aspects of VTS digital communications.

The Traffic Functions are intended to assist VTS operators and vessels in forming a common situational awareness of the current traffic situation, a vessel's navigational intentions, and navigational hazards, by digitally exchanging traffic-related information between the VTS and the vessel.

The Traffic Functions consist of the following three digital VTS services.

- Digital Traffic Image ServiceV

- Digital Intended Track Exchange Service

- Digital Navigation Support Service

This document describes the pre-defined conditions for defining the use cases of each service, and the Annex presents the use cases related to each service.

## Purpose of the document

The purpose of this document is to elaborate the scope and operational concept of the Traffic Functions and to present the related use cases, in order to assist further discussion on the operational aspects of VTS digital communications.

It also aims to clarify how the three services that make up the Traffic Functions can assist VTS operations by digitally exchanging traffic situation, a vessel's navigational intentions, and navigation support information between the VTS and the vessel.

## Related documents

IALA Draft Guideline on VTS Digital Communications

Use cases for VTS digital communications

# Background

In defining digital VTS services, the VTS Committee is working on use cases separately. Use cases are being developed and reviewed sequentially by service, and it is necessary to develop use cases for the services related to the Traffic Functions.

# Discussion

## Need for Defining Use Cases Applying SECOM

Unlike conventional VHF voice communication, digital VTS services presuppose the digital exchange of structured data between a vessel and the VTS. Therefore, it is necessary not only to define the operational purpose and business procedures of a service, but also to specifically define the procedures by which service requests, data transmission, acknowledgement, and error handling are carried out in an actual digital communication environment.

In particular, since SECOM (IEC 63173-2) can be used as a communication interface to assist service discovery and data exchange in maritime digital services, defining use cases that apply SECOM is necessary to elaborate the use cases of digital VTS services into actual system-to-system communication procedures.

The currently defined use cases for the Traffic Functions mainly describe the operational interaction among the vessel, the VTSO, and the VTS. However, in order to implement the actual service, it is necessary to additionally define through which digital communication procedures these operational procedures are carried out between the vessel-side system and the VTS-side system.

Defining use cases applying SECOM makes it possible to specify the following matters.

- Service discovery and connection: the procedure by which a vessel and the VTS identify the required digital VTS service and access that service

- Service request and response: the data exchange procedure required in the process of requesting a service, retrieving information, and providing results

- Data transmission/reception: the procedure for delivering service messages between the vessel and the VTS through the SECOM interface

- Acknowledgement: the procedure for confirming whether a service message has been successfully delivered to and processed by the counterpart system

- Service status management: the communication procedures associated with the start, modification, cancellation, and termination of a service

- Communication exception handling: the response procedures required in situations such as message transmission failure or service unavailability

Accordingly, for a single digital VTS service, it is necessary to define the Operational Use Case and the SECOM-based Communication Use Case in an interlinked manner.

## Definition of Relevant Users for Defining Use Cases

The use cases of digital VTS services need to be defined to include not only the operational interaction between the vessel and the VTS, but also the data exchange procedures between the vessel- and shore-side systems and the SECOM service that occur during the actual provision of the service.

Accordingly, this document divides the relevant Users participating in the use cases into the Ship Side and the Shore Side, and separately defines the roles of the person (User), the business system (System), and the SECOM-based communication service (SECOM Service).

This is intended to distinguish and express the operational actions of the service, the internal processing of the system, and the system-to-system data exchange using SECOM.

### Ship-Side Users

The Ship-Side Users consist of the Master, the Vessel System, and the Vessel SECOM Service.

#### Master

The Master is the Operational User who uses digital VTS services on the ship side.

The Master determines the need to use a digital VTS service based on navigation and the surrounding traffic situation, and requests the service through the Vessel System or checks the service results provided by the VTS.

In the case of the Digital Traffic Image Service, when the Master determines that traffic image information is needed before entering the VTS area or during navigation, the Master submits a service request through the Vessel System, and checks the traffic image information provided by the VTS to use it for voyage planning or decision-making.

#### Vessel System

The Vessel System is the ship-side business system that processes requests for and results of digital VTS services between the Master and the Vessel SECOM Service.

The Vessel System generates the Master's service request into the data format required by the digital VTS service and delivers it to the Vessel SECOM Service; conversely, it processes the service results received through the Vessel SECOM Service and provides them so that the Master can review them.

Accordingly, the Vessel System performs the role of connecting the Master's operational actions with SECOM-based data exchange.

#### Vessel SECOM Service

The Vessel SECOM Service is a communication service on the ship side for transmitting and receiving digital data to and from the shore-side service based on SECOM.

It delivers the service request message generated by the Vessel System to the shore-side SECOM Service, and receives the service message transmitted from the shore side to deliver it to the Vessel System.

It also assist with the digital communication procedures between the vessel and the shore, such as data transmission and acknowledgement, as required by SECOM.

### Shore-Side Users

Based on the Digital Traffic Image Service, the Shore-Side Users consist of the Shore SECOM Service, the VTS System, and the VTSO.

#### Shore SECOM Service

The Shore SECOM Service is the service on the shore side responsible for the SECOM-based data exchange of digital VTS services.

It receives the Digital Traffic Image Service request delivered from the Vessel SECOM Service and forwards it to the VTS System, and transmits the traffic image information generated by the VTS System to the Vessel SECOM Service.

Accordingly, the Shore SECOM Service performs the role of connecting the digital message exchange required for the Digital Traffic Image Service between the Vessel SECOM Service and the VTS System.

The Shore SECOM Services in the Traffic Functions are the Traffic Image SECOM Service, the Intended Track Exchange SECOM Service, and the Navigation Assistance SECOM Service.

#### VTS System

The VTS System is the shore-side business system that assist with the VTS's traffic monitoring and control operations and processes the information required for digital VTS services.

In the Digital Traffic Image Service, the VTS System processes the vessel's request received through the Shore SECOM Service, and generates or extracts the traffic image information to be provided to that vessel based on the traffic information held by the VTS.

The generated service result is delivered to the Shore SECOM Service and transmitted to the ship side through SECOM.

#### VTSO

The VTSO is the shore-side Operational User that provides digital VTS services.

The VTSO monitors vessel traffic within its area of responsibility and provides digital VTS services using the VTS System. Depending on the nature of the service, the VTSO determines whether to provide the service or checks the service results, and, if necessary, can input or modify the information to be provided to the vessel through the VTS System.

### Relationships among Users

In this use case, the relevant Users are organized into the following hierarchy.

Ship Side

Master ↔ Vessel System ↔ Vessel SECOM Service

↕ SECOM-based data exchange

Traffic Image SECOM Service ↔ VTS System ↔ VTS

Shore Side

The roles of each User are distinguished as follows.

| Category | User | Role |
| --- | --- | --- |
| Ship | Master | Determines service requests and checks/utilizes results |
| Ship | Vessel System | Generates and processes service request/response data |
| Ship | Vessel SECOM Service | SECOM-based data transmission/reception |
| Shore | Shore SECOM Service | SECOM data exchange for the VTS System |
| Shore | VTS System | Processes service requests and generates/provides traffic image information |
| Shore | VTSO | Provides services and performs traffic monitoring and control duties |

Applying this User definition makes it possible to express the use cases of digital VTS services by distinguishing between operational interaction and technical data exchange procedures.

For example, the overall flow for a Digital Traffic Image Service request can be defined as follows.

The Master determines the service request

→ The Vessel System generates the request message

→ The Vessel SECOM Service transmits the request data

→ The Traffic Image SECOM Service receives the request

→ The VTS System processes the request and generates traffic image information

→ The Shore SECOM Service transmits the service result

→ The Vessel SECOM Service receives the result

→ The Vessel System processes the result

→ The Master checks and utilizes the traffic image information

This distinction clarifies the relationship between the operational procedures of digital VTS services and the SECOM-based communication procedures, and can be used as a common reference for defining detailed use cases and sequences for each service going forward.

# Action requested of the Committee

The Committee is requested to consider the contents of this document and take appropriate action.

Use CASE of DIGITAL TRAFFIC IMAGE SERVICE

USE CASE 1 - VTS sends traffic image information to vessel

SEQUENCE

Description: A case where the VTS transmits traffic image information to a vessel.

Typical sequence:

1. A vessel navigating far from the VTS area requests the Digital Traffic Image Service from the VTS

2. The VTS provides the traffic image (of the whole or a part of the area) to the vessel

3. The vessel confirms receipt of the traffic image (of the whole or a part of the area) information

SEQUENCE DIAGRAM

Use CASE of DIGITAL INTENDED TRACK EXCHANGE SERVICE

USE CASE 1 - Vessel requests intended track exchange service via VTS

SEQUENCE

Description: A case where a vessel requests the exchange of an intended track via the VTS

Typical sequence:

1. Vessel A determines the risk of encounter with Vessel B and transmits its intended track to the VTS

2. The VTS relays Vessel A's intended track to Vessel B

3. Vessel B reviews Vessel A's intended manoeuvre and transmits an 'Approve (agreed with the intended track)' or 'Reject (disagreed, including reason/counter-proposal)' response to the VTS

4. The VTS relays Vessel B's response to Vessel A

5-1. If Vessel B 'Rejects': Vessel A reviews the reason for rejection and retransmits a revised track to the VTS, repeating steps 1–4

5-2. If Vessel B 'Approves': the agreement is completed, and the VTS sends a confirmation of the final agreed content to both vessels

SEQUENCE DIAGRAM

USE CASE 2 - VTS requests intended track exchange service

SEQUENCE

Description: A case where the VTS requests the exchange of an intended track from a vessel

Typical sequence:

1. The VTS requests Vessel A to submit its intended track

2. Vessel A transmits its intended track to the VTS

3. The VTS relays Vessel A's intended track to Vessel B

4. Vessel B transmits an 'Approve' or 'Reject (including reason for rejection)' response to the VTS

5. The VTS relays Vessel B's response to Vessel A

6-1. If Vessel B 'Rejects': Vessel A reviews the reason for rejection and retransmits a revised track to the VTS, repeating steps 1–5

6-2. If Vessel B 'Approves': the agreement is completed, and the VTS sends a confirmation of the final agreed content to both vessels

SEQUENCE DIAGRAM

Use CASE of DIGITAL NAVIGATION Support SERVICE

USE CASE 1 - VTSO provides navigation SUPPORT service for individual vessels

SEQUENCE

Description: A case where the VTSO provides navigation assistance to an individual vessel

Typical sequence:

1. The vessel subscribes to the Navigation Support Service(NSS)

2. The VTSO transmits a message for hazard-situation support (including location of occurrence, message type (navigational information, advice, warning, instruction), and risk level (Low, Medium, High)) to the NSS (this may be assisted by the VTS System)

3. The NSS Service transmits the message to the vessel

4. The vessel transmits acknowledgement that it has recognized the hazardous situation

5. The VTSO continues monitoring and control until the vessel's hazardous situation is resolved

6. The VTSO notifies that the vessel's hazardous situation has ended

SEQUENCE DIAGRAM

USE CASE 2 - VTSO provides navigation assistance service for all vessels

SEQUENCE

Description: A case where the VTSO provides navigation assistance to all vessels

Typical sequence:

1. The vessel subscribes to the Navigation Support Service

2. The VTSO transmits a message for hazard-situation support (including location of occurrence, message type (navigational information, advice, warning, instruction), and risk level (Low, Medium, High)) to the NSS Service (this may be assisted by the VTS System)

3. The NSS Service transmits the message to all vessels

4. The vessel transmits acknowledgement that it has recognized the hazardous situation

5. The VTSO continues monitoring and control until the vessel's hazardous situation is resolved

6. The VTSO notifies that the vessel's hazardous situation has ended

SEQUENCE DIAGRAM

Input document number, to be assigned by the Committee Secretary

Leave open if uncertain