Input paper:   VTS60-9.1.2

Input paper for the following Committee(s): 		Purpose of paper:

(Select as appropriate)

☐ ARM	☐  ENG		☐ PAP			☒ Input

☐ ENAV	☒ VTS					☐ Information

Agenda item 			9.1

Technical domain/ Task number 2	2.5.2.d

Author(s)/Submitter(s)			Agency for Maritime Services and Coast (Belgium)

# Proposal for a Draft Service Design for VTS Under Keel Clearance Service

## 1. Summary

Technical Working Group 2 (WG2) has been working on the Service Specification for the VTS Under Keel Clearance Service. During VTS 59, version 1.0 was finalised. The next step in the development process is the Service Design.

This input paper describes the work carried out by the Agency for Maritime Services and Coast (Belgium) to develop a draft Service Design for the VTS Under Keel Clearance Service. The current draft is shared with the Committee as Annex A.

#### 1.1. Purpose of the Document

The Agency for Maritime Services and Coast (Belgium) has prepared a candidate draft for a SECOM-based Service Design for the VTS Under Keel Clearance Service. If the Committee finds it useful, this draft could be used as a basis for developing a version for testing, as set out in Chapter 4.

## Related documents

IMO Resolution MSC.530(106)/REV.1 "Performance standards for electronic chart display and information systems (ECDIS)"

IALA G1128 “The Specification of e-Navigation Technical Services”

IALA G1199 “VTS Digital Communications” version 1.0

Service Specification for VTS Under Keel Clearance Service, version 1.0

# Background

The development of the S-129 Under Keel Clearance Management product specification has provided an important standardised data model for the exchange and presentation of UKC-related information. The product specification defines how UKC plans and associated information can be structured in an S-100 environment and is therefore an essential building block for harmonised implementation by shore-based services and shipborne systems.

However, the product specification itself does not define the operational interaction required to request, deliver, update, acknowledge, cancel or otherwise manage UKC information as part of a live service between ship and shore. The Committee has noted on several occasions that a product specification is necessary, but not sufficient, for implementing a complete digital service.

In accordance with IALA G1128, a technical service specification and subsequent service design are needed to describe the service context, operational sequences, information exchange requirements, dynamic behaviour, service interfaces, security provisions and implementation choices. At present, no dedicated technical service exists for S-129. This creates a gap between the availability of the S-129 data model and the practical ability to implement a harmonised, secure and interoperable UKC service. The work to define such a technical service should therefore be considered a necessary next step to enable consistent use of S-129 in operational VTS and ship-shore environments.

# Discussion

The Agency for Maritime Services and Coast (Belgium) has participated in the development of several Service Specifications for VTS. Building on this experience, the Agency for Maritime Services and Coast (Belgium) has undertaken action to prepare a candidate draft Service Design for the VTS Under Keel Clearance Service. This Service Design is aligned with the existing Service Designs for the VTS Traffic Clearance Service and the VTS Route Exchange Service. It also takes into account the lessons learned from the development of previous S-100-based Technical Services for VTS.

# Action requested of the Committee

The Committee is requested to:

- Review the proposed Service Design in Annex A and, if deemed appropriate, use Annex A as the working draft for further development of the Service Design for VTS Under Keel Clearance Service within the Technical Working Group.

Input document number, to be assigned by the Committee Secretary

Leave open if uncertain