ialA Technical service

SD0005

Service Design for Under Keel Clearance Digital Service

Edition 0.4

July 2026

urn.mrn.iala:techsvc:sd0005:ed0.4

| Date | Details | Approval |
| --- | --- | --- |
| June 2026 | 0.3 |  |
| July 2026 | 0.4 |  |

1	introduction	5

1.1	Purpose	5

1.2	Intended readership	5

1.3	Input from other sources	5

2	Service design template Identification	6

3	TECHNOLOGY INTRODUCTION	7

3.1	General	7

3.2	Service Technology and Service Transportation protocol	7

3.3	Security	7

3.3.1	Communication Channel Security	7

3.3.2	Data Protection	7

3.3.3	Data Signature	8

3.3.4	Data Encryption	8

4	Service OVERVIEW	9

4.1	General	9

4.2	Data products Exchanged	9

4.3	Service interfaces	9

4.4	Service Discovery	12

5	PHYSICAL DATA MODEL	13

5.1	S-212 underKeelClearanceMessage (vessel → service)	13

5.2	S-129 UnderKeelClearancePlan (service → vessel)	13

5.3	Container Type	13

5.4	Coordinate System	13

5.5	Timestamps	14

5.6	Identifiers	14

6	Service Interface Behaviour	15

6.1	Cross-Cutting Conventions	15

6.1.1	Nullable parameters	15

6.1.2	Common SECOM Parameter values	15

6.1.3	Error codes	16

6.1.4	Maritime Identity Requirements	16

6.2	Submit Pre-Plan Request (UC1, UC2)	17

6.3	Subscribe to Actual-Plan Updates (UC3)	17

6.4	Deliver UKC Plan (UC1, UC2, UC4, UC5, UC6)	19

6.5	End-to-End Acknowledgement (all use cases)	20

6.6	Provide Vessel Stability Parameters (UC6)	20

6.7	Large-Message Handling	21

6.8	Cancel Actual-Plan Subscription (UC7, UC9)	21

6.9	Critical UKC Alert to VTS (UC8)	22

6.10	Discover Service Capabilities (UKCDSF007)	22

6.11	Liveness and Key Distribution	23

6.12	SECOM Interfaces Not Used	23

7	Service Dynamic Behaviour	24

7.1	Generic sequence for signatures	24

7.2	Dynamic behaviour of the service interface	24

7.2.1	Pre-Plan Request (Use cases 1 & 2)	25

7.2.2	Subscription to Actual Plan (Use case 3)	26

7.2.3	Actual Plan Delivery and Updates (Use cases 4 & 5)	27

7.2.4	Detailed Actual Plan with Stability Parameters (Use case 6)	28

7.2.5	Subscription Cancellation (Use cases 7 & 9)	29

7.2.6	Critical UKC Alert to VTS (Use case 8)	30

7.2.7	Large-Message Handling — UploadLink / GetByLink	30

8	Definitions	31

9	Abbreviations	32

10	References	33

11	Appendix 1 – Service Design Template OPENAPI (SWAGGER) in JSON	34

12	Appendix 2 – Guidance on Implementation	35

List of Tables

Table 1 Service Design Identification	6

Table 2 Service interfaces	10

List of Figures

No table of figures entries found.

# introduction

This document specifies the design of the Under Keel Clearance (UKC) service according to the technical service specification [2] using SECOM as the transport mechanism.

This document was produced as part of the work of IALA VTS committee task group on development of technical service specifications for VTS. The document is structured according to the IALA Guideline G1128 The Specification of e-Navigation Technical Services [1] and is based on the SECOM template found therein.

## Purpose

The purpose of this document is to define a service design that realises the UKC Digital Service [2] over SECOM. It defines, in implementation terms:

how the logical operations of SS0005 (pre-plan request, actual-plan delivery, subscription, cancellation, VTS alert) are realised over a concrete set of SECOM REST interfaces

how the payload data is encoded — using the IHO S-129 Under Keel Clearance Management product specification [13] for UKC plans, and the S-212 VTS Digital Information Service for the underKeelClearanceMessage [14] carrying pre-plan requests and vessel stability parameters

how route information consumed by the service is not carried through this service but is obtained by re-using S-421 routes already exchanged via the VTS Route Exchange Service [9]

how the requirements for maritime identity, authentication, integrity and end-to-end acknowledgement defined in SS0005 are implemented.

## Intended readership

This Service Design is intended to be read by service architects, system engineers and developers in charge of designing and developing an instance of the UKC Digital Service, as well as by manufacturers of vessel-side systems that consume the service.

## Input from other sources

This service design follows the requirements, use cases and dynamic behaviour outlined in SS0005 [2]. The interfaces, parameters and logic of the API is compliant with IEC 63173-2 SECOM [4] and borrows much of its content from there.

The data models used are:

- IHO S-129 Under Keel Clearance Management Product Specification, Edition 2.0.0 [13]

- IALA S-212 VTS Digital Information Service (for underKeelClearanceMessage)

- IEC 63173-1 S-421 Route Plan based on S-100 [9] (only as referenced via the Route Exchange Service [12]

“The author thanks the International Electrotechnical Commission (IEC) for permission to reproduce Information from its International Standards. All such extracts are copyright of IEC, Geneva, Switzerland. All rights reserved. Further information on the IEC is available from www.iec.ch. IEC has no responsibility for the placement and context in which the extracts and contents are reproduced by the author, nor is IEC in any way responsible for the other content or accuracy therein.”.

# Service design Identification

This section provides a unique identification of the service and describes where the service is in the engineering lifecycle.

Table 1 Service Design Identification

| - Name | VTS — Under Keel Clearance Digital Service using SECOM, Technical Service Design |
| --- | --- |
| - Implements | VTS – UKC Digital Service Technical Service Specification 1.0 urn:mrn:iala:techsvc:ss0005:1.0 |
| - ID | urn.mrn.iala:techsvc:sd0005:ed0.3 |
| - Version | 0.4 |
| - Description | Defines how to implement an instance of the UKC Digital Service using SECOM as the secure transport between vessels and the shore-side service for the exchange of underKeelClearanceMessage (S-212) requests and UnderKeelClearancePlan (S-129) responses. Routes are not carried by this service; they are re-used from the VTS Route Exchange Service [12]. |
| - Keywords | SECOM, VTS, REST, UKC Digital Service, Ship Traffic Management, S-129, S-212, S-421 |
| - Architect(s) | Sil Janssens, Peter Coenen |
| - Status | Draft |

# TECHNOLOGY INTRODUCTION

## General

This design realises SS0005 [2] using SECOM Edition 2 as defined in IEC 63173-2 ED2 [4]. Services and consumers conforming to this design must be implemented using REST APIs over HTTPS with TLS protection of all communication in transit.

The service is shore-side; the consumer is the vessel systemor its shore based proxy. Both must be able to act as HTTP client and HTTP server because some interactions are bidirectional (e.g. acknowledgements, subscription notifications, push of updated plans).

## Service Technology and Service Transportation protocol

Reference: IEC 63173-2 SECOM ed2 Clause 5.3 Service Technology

The technology (architectural style) chosen is REST (REpresentational State Transfer) upon HTTP/1.1 (RFC 7231).

## Security

### Communication Channel Security

Reference: IEC 63173-2 SECOM ed2 Clause 6 SECOM communication channel security

The channel security between the SECOM REST service and a consumer are

HTTP/1.1 according to RFC-7231

HTTPS over TLS according to RFC-2818

Valid versions of TLS for this version of service design template are

TLS version 1.3 (RFC-8446)

X.509 Certificates are used in the TLS according to RFC 5280 and RFC 2459.

Certificates shall be verified with OCSP and/or CRL methods.

### Data Protection

Reference: IEC 63173-2 SECOM ed2 Clause 4.3.4 SECOM data protection

Reference: IHO S-100 ed5.2.0 Part 15 Data Protection Scheme

The data is mandatory to be signed by the sender to enable data authentication and integrity check by the receiver.

The data can optionally be encrypted by the sender, and the sender is responsible for exchanging the encryption key with the receiver.

While sending standalone S-129 and S-212 documents without the additional metadata of datasets or exchange sets is preferred, if datasets or exchange sets are used the data (one or more data files) can optionally be compressed before being signed and sent.

### Data Signature

Reference: IEC 63173-2 SECOM ed2 Clause 7.3 Data authentication and signing

Reference: IHO S-100 ed5.2.0 Part 15-8 Data Authentication

Reference: NIST Digital Signature Standard (DSS–FIPS Publication 186)

The algorithm for signing data must be ECDSA-384-SHA2.

The signature is transported in HEX.

### Data Encryption

Reference: IEC 63173-2 SECOM ed2 Clause 7.4 Data encryption

Reference: IHO S-100 ed5.2.0 Part 15-6 Data Encryption

The encryption algorithm for encryption is AES and CBC mode.

The symmetric encryption key can be exchanged by different means, including using the SECOM REST API to exchange the encryption key.

# Service OVERVIEW

## General

The design uses SECOM defined APIs. As such, it is important to understand that both the UKC Digital service and its consumers must be able to function as client and server as understood in the traditional HTTP world. As such from here on when the term service is used, it applies to the under keel service and the consumer is the ship system that is communicating with VTS.

This design does not concern itself with how the service will communicate with the VTS system. The instances of this design may be developed as components directly integrated with the VTS system or as independent microservices that communicate with the VTS system via different integration mechanisms e.g., APIs or by emitting and consuming events.

## Data products Exchanged

| Direction | Payload | Purpose | Used in use cases |
| --- | --- | --- | --- |
| Vessel Service | S-212 underKeelClearanceMessage containing a basic route (waypoints + schedule, encoded as embedded S-421 fragment) | Trigger a pre-plan calculation (max draught or tidal window). | UC1, UC2 |
| Vessel Service | S-212 underKeelClearanceMessage containing vessel stability parameters (lengthOverall, breadth and optional fields per UC6) | Provide stability data needed for a detailed actual plan. | UC6 |
| Service Vessel | S-129 UnderKeelClearancePlan (pre-plan or actual plan) | Deliver the calculated UKC plan / update. | UC1, UC2, UC4, UC5, UC6 |
| Service VTS System | Implementation-specific (out of scope) | Internal alerting & integration. | UC8 |

## Service interfaces

Reference: IEC 63173-2 SECOM ed2 Clause 5.7 SECOM service interface definitions

SECOM does not require that all the interfaces defined in the standard must be implemented. Thus, for the purposes of this service and its consumers, only the following interfaces are required:

Table 2 Service interfaces

| - Interface | SECOM Reference | Comment |
| --- | --- | --- |
| Upload | IEC 63173-2 SECOM ed2 Clause 5.7.2 service interface – Upload | This interface is called when the client uploads (pushes) a payload (S-212 request or S-129 plan) to the server. This interface is provided by both the consumer and service. |
| Upload Link | IEC 63173-2 SECOM ed2 Clause 5.7.3 service interface – Upload Link | This interface is called when the client uploads (pushes) a reference pointer to a payload (S-212 request or S-129 plan) that is larger than what the server allows via HTTP POST. The data is downloaded using interface Get By Link. This interface is provided by both the consumer and service. |
| Acknowledgement | IEC 63173-2 SECOM ed2 Clause 5.7.4 service interface – Acknowledgement | This interface is called as response to Acknowledgement request in the previous request. This interface is provided by both the consumer and service. |
| Get | IEC 63173-2 SECOM ed2 Clause 5.7.5 service interface – Get | This interface is called when the client gets (pulls) data from the service. This interface must be provided by the consumer, but supporting get-requests from the consumer is optional for the service. |
| Get Summary | IEC 63173-2 SECOM ed2 Clause 5.7.6 service interface – Get Summary | This interface is called when the client gets a summary of available data from the service. The data is retrieved (pulled) using the interface Get. |
| Get By Link | IEC 63173-2 SECOM ed2 Clause 5.7.7 service interface – Get By Link | This interface is called when the client downloads (pulls) a large payload by reference given from interface Upload Link. This interface is provided by both the consumer and service. |

| Interface | SECOM Reference | Comment |
| --- | --- | --- |
| Subscription | IEC 63173-2 SECOM ed2 Clause 5.7.10 service interface – Subscription | This interface is called when the client or server initiates subscription on data from the service. Response is given with interface Upload and Subscription Notification. This interface is provided by both the consumer and service. |
| Remove Subscription | IEC 63173-2 SECOM ed2 Clause 5.7.11 service interface – Remove Subscription | This interface is called when the client or server removes subscription. Response is given with interface Subscription Notification. This interface is provided by both the consumer and service. |
| Subscription Notification | IEC 63173-2 SECOM ed2 Clause 5.7.12 service interface – Subscription Notification | This interface is called as response from Subscription or Remove Subscription. This interface is provided by both the consumer and service. |
| Capability | IEC 63173-2 SECOM ed2 Clause 5.7.13 service interface – Capability | This interface is called when the client asks for the service capabilities. This interface is provided by both the consumer and service. |
| Ping | IEC 63173-2 SECOM ed2 Clause 5.7.14 service interface – Ping | This interface is called when the client checks the availability of the service. This interface is provided by both the consumer and service. |
| EncryptionKey | IEC 63173-2 SECOM ed2 Clause 5.7.15 service interface – EncryptionKey | This interface is called when sending (pushing) encryption key to a receiver. This interface is provided the service and is optional for the consumer. |
| PublicKey | IEC 63173-2 SECOM ed2 Clause 5.7.16 service interface – PublicKey | This interface is called when the client gets (pulls) the public certificate(s) from the service. This interface is provided by the service and is optional for the consumer. |

All unused interfaces of SECOM should be implemented to return HTTP 501 as specified in [4].

There are three components that are of interest from the perspective of the service design:

The service has a SECOM-component which supports the SECOM REST APIs defined in the table above. All other components of the service are left to the decisions of the implementing party.

The vessel has a SECOM-component which will accepts the incoming connections from the service and store all messages until delivered to the vessel. This interface is typically on a shoreside server as it must be always available and at a static address.

The vessel has an implementation of a SECOM client which allows it to make direct SECOM calls to the service without having to proxy all calls via the SECOM-component on shore.

In this service design we will not define the communication between the service and VTS system or between the vessel and the vessel’s shoreside SECOM interface. These are specific for each implementation and depend on the VTS system and vessel’s system.

## Service Discovery

Instances shall register with a Maritime Service Registry (MSR) conforming to the MSR specification. The instance description shall include:

- the technical service design MRN and version (urn:mrn:iala:techsvc:sd0005:ed1.0)

- the geographic coverage (the VTS area to which the UKC service applies)

- the service-level Capability advertised: pre-plan only / pre-plan + actual plan / pre-plan + detailed actual plan (UKCDSF007)

- the endpoint URL and the X.509 certificate(s) used

Consumers discover compatible UKC service instances by invoking the SECOM Search service interface [4] on a registry endpoint, using:

- the dataProductType (S-129 or S-212)

- the service design MRN (urn:mrn:iala:techsvc:sd0005:ed1.0)

- a geometry describing the planned voyage area WKT, EPSG:4326 per [4])

- optionally an UN/LOCODE of the destination port

The Search service returns the list of matching instances. The consumer then queries each instance's Capability to confirm that the required UKC service levels and SECOM interfaces are implemented before subscribing.

# PHYSICAL DATA MODEL

The physical data model is a combination of:

- JSON for SECOM envelopes (see [4]§5)

- XML for the S-212 and S-129 payloads (validated against their respective S-100 GML schemas).

## S-212 underKeelClearanceMessage (vessel → service)

The S-212 message used by this service shall populate the underKeelClearanceMessage element with:

underKeelClearancePurpose = prePlan

underKeelClearanceCalculationRequested = maxDraught (UC1) or timeWindow (UC2)

a route element (S-421 basic route) carrying at least one destination waypoint and an ETA

for tidal-window pre-plans (UC2), the vessel's freshWater draftOrDraught (underKeelClearanceMessage  draftOrDraught)

lengthOverall and breadth are mandatory in use-cases 6

for UC6, the stability parameters draftOrDraught (forward / amidships / aft), deadweightTonnage, metacentreOrMetacenter, displacementTonnageLoaded, freeFluidSurfaceCorrection, shipRollPeriod, centreOfBuoyancy, centreOfGravity, blockCoefficient are optional but improve calculation accuracy.

## S-129 UnderKeelClearancePlan (service → vessel)

The S-129 payload shall comply with [13] and contain:

- underKeelClearancePurpose = prePlan (UC1, UC2) or actualPlan / actualUpdate (UC4–UC6);

- for UC1 - max-draught responses: maximum allowed draught per affected fairway segment

- for UC2 - tidal-window pre-plan responses: global tidal window with start/end times

- for actual-plan responses: vessel maximum fresh-water draught used in the calculation, global tidal window, and controlPoints with distance above UKC limit, expected passing speed, expected passing time, and local tidal window per control point

- optional geometry for non-navigable and almost-non-navigable areas including distance above UKC limit

- a reference to the originating message:

  - for actualUpdate (UC5), the originalMessageReference shall identify the S-129 plan being updated.

## Container Type

Both standalone (preferred) and S-100 dataset / exchange-set transmissions are supported as required by SS0005 §4.1. Implementations shall:

- prefer standalone documents (containerType = NONE) for typical operation

- support dataset and exchange-set transmissions when the consumer or operating context requires retention with full S-100 metadata

- reject any incoming exchange set containing Lua scripting (per SS0005 §4.1).

## Coordinate System

All geometries shall use EPSG:4326 with axis order latitude, longitude, in line with SS0005. The coordinate system declared on the bounding box of the message is inherited by all child geometries unless explicitly overridden.

## Timestamps

All temporal values (ETAs, tidal-window start/end, control-point passing times, etc.) shall be expressed in UTC, in ISO 8601 form extended format compatible with XML datetime, per UKCDSF013.

## Identifiers

- The transactionIdentifier of every SECOM call shall be a fresh UUID (v4 recommended). It is unique per transaction and is used to correlate acknowledgements (per UKCDSF010 and SS0005 §3.2.3).

- The dataReference in subsequent calls (e.g. subscription, actualUpdate (UC5), link interfaces) shall be the transactionIdentifier of the originating message it refers to. This avoids the S-421 gml:id constraints used in the Route Exchange Service Design [12]; the UKC payloads (S-129 / S-212) carry their own identifiers internally and there is no need to bind the UUID to a gml:id.

# Service Interface Behaviour

This section describes the UKC-specific logical operations defined in SS0005 §3.1 and prescribes how each one is implemented over SECOM. The structure follows the UKC business operations, not the SECOM interface inventory. The SECOM interface(s) used to realise each operation are listed at the end of every sub-clause.

Only those parameters whose value or meaning is specific to this design are described. All other parameters follow IEC 63173-2 [4]. The OpenAPI document is provided in Annex A.

## Cross-Cutting Conventions

The conventions below apply to every operation in this section

### Nullable parameters

All parameters that are not set should be omitted or passed with a null value.

### Common SECOM Parameter values

| SECOM Parameter | Value used by the UKC Digital Service |
| --- | --- |
| containerType | NONE (standalone S-129 / S-212) is the default. S100_DataSet or S100_ExchangeSet are only used when the operating context requires it (e.g. on-board archival with full S-100 metadata). |
| dataProductType | - S-212 for vessel-originated payloads (pre-plan request, stability parameters) - S-129 for service-originated payloads (UKC plan and updates). Values are CharacterString. Note that S-421 is never carried by this service, it is exchanged via the Route Exchange Service [12]. |
| productVersion | The product specification version, e.g. 2.0.0 for S-129 [13] and the version of S-212 mandated by SS0005. |
| exchangeMetadata.protectionScheme | SECOM. |
| envelopeSignature | The envelope signature as a HEX string with no whitespace input object tables). |
| envelopeSignatureCertificate | The sender's public certificate(s), used by the receiver to verify the envelope signature. |
| transactionIdentifier | A fresh UUIDv4 per UKC transaction. The same transactionIdentifier is reused as dataReference in subsequent acknowledgements, link interfaces and update messages that relate to that transaction. |
| callbackEndpoint | Base URL of the consumer's SECOM endpoint (subscription/object subpaths are appended by the service). If not supplied, the service shall resolve it via the SECOM Search service [4] using the vessel MRN extracted from the TLS certificate. |

Note that the UKC Digital Service does not require the transactionIdentifier to be embedded in any gml:id of the payload (contrast with [12] §5.1). S-129 and S-212 are full S-100 product instances with their own internal identifiers; correlation across SECOM calls is done exclusively via the SECOM transactionIdentifier.

### Error codes

| Condition | SECOM_ResponseCode | HTTP status |
| --- | --- | --- |
| S-129 / S-212 schema validation fails | 3 | 400 |
| Business-rule validation fails (e.g. UKCDSF007 service-level not supported, UKCDSF008 route invalid for UKC, mismatch between vessel identity and route) | 0 | 400 |
| Invalid TLS certificate | 2 | 400 |
| Payload signature verification fails | 1 | 400 |
| Payload too large to be carried in a single body | n/a | 413 (triggers the large-message flow, §6.7) |
| Rate / throttling limit exceeded (UKCDSNF004) | n/a | 429 (with Retry-After) |
| Unsolicited UKC plan received by vessel | n/a | 403 (vessel side only) |

### Maritime Identity Requirements

Reference: UKCDSF009; G1183 [3].

- Both parties shall present a valid X.509 maritime identity certificate issued under the MIR.

- The vessel certificate shall carry the vessel's MMSI and, where applicable, IMO number.

- For actual-plan calculations (UC4 - UC6), the service shall verify that the MMSI/IMO from the TLS certificate matches the MMSI/IMO of the route consumed from the Route Exchange Service [12]. A mismatch shall be rejected (SECOM_ResponseCode = 0, HTTP 400) and no UKC plan shall be calculated or delivered.

- The service shall identify the vessel for subscription, plan delivery and acknowledgement purposes by the MRN extracted from the TLS certificate, never from the request body.

## Submit Pre-Plan Request (UC1, UC2)

Purpose

The vessel asks the service for a pre-plan UKC computation: a maximum-allowed draught (UC1) or a tidal window (UC2). The request carries a basic route (waypoints + ETA) embedded in the S-212 message. No S-421 route is sent through this service.

Payload (vessel → service)

S-212 underKeelClearanceMessage with:

- underKeelClearancePurpose = prePlan;

- underKeelClearanceCalculationRequested = maxDraught (UC1) or timeWindow (UC2);

- a basic route element with at least one destination waypoint and an ETA;

- for UC2: the vessel's freshWater draftOrDraught.

Service requirements

- Verify the TLS certificate and the payload signature.

- Validate the S-212 against schema (responseCode = 3 on failure) and against the business rules of SS0005 (UKCDSF007, UKCDSF008 — responseCode = 0 on failure).

- Confirm that the requested calculation type is advertised by this instance's Capability (§6.10). If not, reject with responseCode = 0.

- Compute the pre-plan from the basic route, astronomical-tide forecasts and the UKC waterway limits.

- Deliver the resulting S-129 to the vessel as described in §6.4 with underKeelClearancePurpose = prePlan.

SECOM realisation

Submission uses the SECOM Upload interface from vessel to service with dataProductType = S-212 and ackRequest = 3 (delivered + opened).

No subscription is created, the response is one-shot.

## Subscribe to Actual-Plan Updates (UC3)

Purpose

The vessel registers an ongoing interest in the actual-plan stream for its current voyage. While the subscription is active, the service is responsible for pushing every relevant update (UC4 – UC6) to the vessel.

Pre-conditions

- The vessel's S-421 route has been shared with the Route Exchange Service [12], including schedule and draught (UKCDSF005).

- The vessel knows the routeId returned by [12], this routeId must be the same as was sent to the service from the vessel.

- The instance Capability (§6.10) advertises actualPlan (and, if the vessel intends to send stability parameters, detailedActualPlan).

Subscription parameters

per [4]SubscriptionRequestObject:

| Parameter | Use in the UKC service |
| --- | --- |
| containerType | NONE. |
| dataProductType | S-129. |
| dataReference | Required. UUID of the vessel's route as registered with the Route Exchange Service [12]. The service uses it to locate the route to be used as input to UKC calculations. |
| productVersion | The S-129 version mandated by SS0005 (e.g. 2.0.0 per [13]). |
| geometry | Optional, WKT per [4] §5.6.11. May be used by the consumer to narrow the area of interest within the VTS area. |
| unlocode | Optional, the UN/LOCODE of the destination port; useful for vessels whose only known reference is the port. |
| subscriptionPeriodStart | Optional. If absent or in the past, the service treats the subscription as starting immediately. |
| subscriptionPeriodEnd | Optional. Should correspond to the planned departure from the VTS area / planned end of voyage. The service may also auto-cancel earlier per UC9 (§6.8). |
| callbackEndpoint | Base URL of the vessel's SECOM endpoint that will receive the pushed S-129 plans. If absent, the service resolves it via the SECOM Search service (§4.4). |
| pushAll | Optional. true instructs the service to send an initial actual plan immediately upon subscription creation, before continuing with cyclic updates. false (the default behaviour for UKC subscriptions) instructs the service to push only changes that occur after subscription creation. For UC4 a vessel typically sets pushAll = true so it receives a baseline plan as soon as it subscribes. |
| envelopeSignature* | Filled per §3.3.3 (envelope signed with ECDSA-384/SHA-384, HEX-encoded). |

The SubscriptionResponseObject returns a subscriptionIdentifier (UUID) used to cancel (§6.8) and to correlate any later asynchronous notifications.

Identity binding

The vessel's MRN is taken from the TLS certificate. The body of the subscription request shall not be used to identify the vessel.

SECOM realisation

Subscription interface for creation, SubscriptionNotification interface for asynchronous state changes (e.g. operator-side maintenance).

## Deliver UKC Plan (UC1, UC2, UC4, UC5, UC6)

Purpose

The service delivers an UnderKeelClearancePlan to the vessel. The same operation realises several logically distinct deliveries:

| Trigger | underKeelClearancePurpose | fromSubscription | originalMessageReference |
| --- | --- | --- | --- |
| Pre-plan response (UC1, UC2) | prePlan | false | absent |
| First actual plan after subscription (UC4) | actualPlan | true | absent |
| Update following route change (UC5) | actualUpdate | true | required |
| Update following environmental change (UC4 cyclic) | actualUpdate | true | required |
| Detailed actual plan after stability params (UC6) | actualPlan (or actualUpdate if a previous plan exists) | true | required if updating |

Payload (service → vessel)

S-129 UnderKeelClearancePlan per [13] and signed per §3.3.3, containing the elements listed in §5.2.

Update sequence

Per UC4/UC5/UC6 the service shall issue an updated plan whenever:

a new route is received from [12] for a subscribed vessel

stability parameters are received under an active subscription (UC6)

the input forecasts (waterlevel, current, weather) changes. Depending on the variability of the met-ocean conditions, the update frequency could vary, for example between 5 and 60 minutes (SS0005).

Acknowledgement

Every plan must request both DELIVERED_ACK and OPENED_ACK (ackRequest = 3). The vessel sends them per §6.5.

Vessel-side acceptance rule

The vessel may reject with HTTP 403 any S-129 upload that does not correspond to a pre-plan it requested (UC1/UC2) or to an active subscription (UC3). Shore systems shall not respond with HTTP 403 to an upload from a vessel presenting a valid maritime identity.

SECOM realisation

Upload from service to vessel for plans that fit the body limit.

UploadLink and GetByLink (§6.7) when the plan is too large.

## End-to-End Acknowledgement (all use cases)

Reference: SS0005 §4.1.5 Send and receive acknowledgement.

UKC information is safety-relevant, the synchronous HTTP-200 returned by an Upload confirms only that the next hop accepted the bytes. SS0005 mandates a separate end-to-end acknowledgement that proves the payload reached the end system and was acted upon by a user.

Two ack types are used (per [4] §5.6.9, Table 11 AckRequestEnum):

| AckRequestEnum | Meaning |
| --- | --- |
| 1 — Delivered ACK requested | The payload has been delivered to the end system (on-board ECDIS or VTS workstation), beyond any intermediate gateway. |
| 2 — Opened ACK requested | A user has interacted with the payload. If the receiving system auto-displays the plan, this ack is sent only after the user has explicitly acknowledged the dialog or opened the ENC layer. |
| 3 — Delivered + Opened ACK requested | Both of the above. This is the value used by every UKC Upload in this design. |
| 0 — No ACK requested | Not used by this design. UKC information is safety-relevant and always requires end-to-end acknowledgement. |

Every acknowledgement carries the transactionIdentifier of the message it refers to. The originating sender can therefore correlate ack timestamps with the original Upload and present them to the operator.

SECOM realisation

Acknowledgement interface in both directions.

## Provide Vessel Stability Parameters (UC6)

Purpose

In service instances that advertise detailedActualPlan capability, the vessel may submit detailed stability data so that the actual plan models squat, heel and dynamic-draught more accurately.

Payload

S-212 underKeelClearanceMessage with stability parameters:

Mandatory: lengthOverall, breadth.

Optional: draftOrDraught (forward / amidships / aft), deadweightTonnage, metacentreOrMetacenter, displacementTonnageLoaded, freeFluidSurfaceCorrection, shipRollPeriod, centreOfBuoyancy, centreOfGravity, blockCoefficient.

Service obligations

Validate identity, signature and schema as in §6.2.

Reject with responseCode = 0 if the instance does not advertise detailedActualPlan (UKCDSF007), or if no active actual-plan subscription (§6.3) exists for this vessel.

Persist the stability profile against the subscription for the duration of the voyage.

Recompute the actual plan and deliver an updated S-129 (§6.4).

SECOM realisation

Upload (vessel  service) with dataProductType = S-212, then a follow-up Upload (service → vessel) of the recomputed S-129.

## Large-Message Handling

Purpose

A pre-plan or, more typically, an actual plan covering a long route with many control points may exceed the body limit accepted by either party's Upload endpoint. To guarantee SS0005's requirement that arbitrarily large plans (and routes of at least 1000 waypoints, 999 legs and 1000 action points) can always be exchanged, this design uses a link-based fall-back.

Rules

The link-based flow shall not be the first interaction in a transaction. It is only valid after an Upload returned HTTP 413, or after a previous interaction in the same conversation has established that the next payload is known to be large. An unsolicited link request shall be rejected with HTTP 403.

The same transactionIdentifier is used across the failed Upload, the subsequent UploadLink, the responding GetByLink, and any Acknowledgement that follows.

The link URL shall be HTTPS, shall require the same mutual TLS as the rest of the service, and should be single-use and expire shortly after retrieval.

The same flow is used in both directions (vessel  service for very large UC1/UC2 requests, service  vessel for very large UC4–UC6 plans).

SECOM realisation

UploadLink (sender announces the link) and GetByLink (receiver fetches the payload).

## Cancel Actual-Plan Subscription (UC7, UC9)

UC7 - Vessel-initiated.

The vessel cancels its UKC subscription (with the subscriptionIdentifier returned by §6.3) when:

the voyage is completed or cancelled

the route has changed such that it no longer enters the VTS area

the vessel no longer requires UKC information

A cancellation of the UKC subscription shall not be interpreted by the service as a cancellation of the voyage. The route remains under the control of the Route Exchange Service [12].

UC9 - Service-initiated.

The service cancels the subscription toward the vessel when:

the route status reported by [12] indicates the vessel has departed the VTS area or completed the voyage;

the subscription has been inactive longer than an operator-configured timeout (avoids producing plans for vessels no longer relevant);

the operator decommissions the service or the affected coverage area;

other conditions for cleaning subscriptions.

In both directions, on receipt of a cancellation the receiving peer shall clear its corresponding subscription state and stop sending UKC plans / updates for that subscription.

SECOM realisation

RemoveSubscription interface with optional asynchronous SubscriptionNotification carrying the REMOVED event.

## Critical UKC Alert to VTS (UC8)

Purpose

When the service produces an actual plan whose UKC clearance does not satisfy the fairway design draught, an alert shall be raised to the VTS operator so that traffic management can react (re-routing, hold, escort, etc.).

Important scope statement

This alert is internal between the UKC Digital Service and the VTS system. It is out of scope of the SECOM contract between vessel and service. The vessel still receives the same plan via the regular UC4/UC5/UC6 flow (§6.4). The alert does not change anything sent to the vessel.

The internal channel between the UKC Digital Service and the VTS system is implementation-specific (proprietary API, event bus, message queue, etc.) and is not standardised by this document.

SECOM realisation

None toward the vessel. The vessel-facing flow is exactly the actual-plan delivery of §6.4.

## Discover Service Capabilities (UKCDSF007)

The service shall advertise both:

- Which SECOM interfaces are implemented, in the standard SECOM Capability response payload (per [4] §5.7.13 and Table 68 CapabilityObject). The values used by a fully-featured UKC instance are:

| Capability flag (per [4] Table 68) | Value for UKC Digital Service |
| --- | --- |
| containerType | NONE (or also S100_DataSet / S100_ExchangeSet if supported) |
| dataProductType | S-129 and/or S-212 |
| productSchemaUrl | URL of the published S-129 / S-212 GML schema |
| upload | true |
| uploadLink | true |
| get / getSummary / getByLink | false for get and getSummary; true for getByLink |
| subscription | true |
| access | false |
| encryptionKey | true if encryption is offered, otherwise false |
| publicKey | true |

- Which UKC service levels are offered by this instance (UKCDSF007), as a service-design-specific extension carried in the same Capability response and mirrored in the MSR instance description:

| UKC service-level flag | Meaning |
| --- | --- |
| prePlan_maxDraught | UC1 supported. |
| prePlan_tidalWindow | UC2 supported. |
| actualPlan | UC3 + UC4 + UC5 supported (subscription-based actual plans without detailed stability data). |
| detailedActualPlan | UC6 supported (the instance accepts and uses S-212 stability parameters). |

A service instance shall reject any operation that targets a service level it does not advertise (e.g. a UC6 stability-parameter submission to an instance that does not support detailedActualPlan) with responseCode = 0 and HTTP 400.

SECOM realisation

Capability ([4] §5.7.13).

## Liveness and Key Distribution

These operations follow IEC 63173-2 [4] without modification:

Ping — liveness check; both parties.

PublicKey — retrieval of the peer's signing certificate(s); used by the verifier to validate payload signatures (§3.3.3).

EncryptionKey — symmetric-key delivery; used only when payload encryption (§3.3.4) has been agreed for the conversation.

## SECOM Interfaces Not Used

The following SECOM interfaces from [4] are not used by this design and shall return HTTP 501 Not Implemented if invoked:

- Get, GetSummary: there is no public library of UKC plans for arbitrary lookup. Plans are only delivered to the vessel they are computed for.

- Access, AccessNotification: the service never requests data held by the vessel (the route is obtained from [12], not directly from the vessel).

# Service Dynamic Behaviour

In the following diagrams and descriptions, the communication between the vessel’s onboard systems and its SECOM service is out of scope for the purposes of this design.

Similarly, the communication and interfaces between the service and the VTS system is out of scope of this design.

## Generic sequence for signatures

This sequence assumes the following actors:

Data Provider ® Data Sender ® Data Receiver ® Data Consumer

The sequence describes the data from its source through transport using SECOM to the end-user.

Provider preparation:

The data provider must have a valid maritime identity according to G1183 [10]

The data provider creates a private-public key pair

The data provider sends the public key to the MIR to get it signed and connected to a specific identity/identifier in the MIR (Certificate Signing Request)

The data sender must have a valid maritime identity according to G1183

The data sender creates a private-public key pair

The data sender sends the public key to the MIR to get it signed and connected to a specific identity/identifier in the MIR (Certificate Signing Request)

Data Provider and Data Sender:

The data provider selects identifier and certificate to use for the data signing

The data provider creates a signature for the data

The data provider transfers the data with its signature to the data sender.

The data sender protects the data transferring by the selected communication mechanism, e.g. SECOM. SECOM Upload wraps the data, the signature and metadata into an envelope. The envelope is signed by the data sender.

The data sends contacts the data receiver and transfers the data

Data Receiver and Data Consumer:

The data receiver receives the transferred object with envelope. The envelope signature is verified.

The data receiver forwards the data and data signature to the data consumer

The data consumer verifies the data signature

## Dynamic behaviour of the service interface

The following sections contain sequence diagrams for the service interfaces. This section expands upon the dynamic behavior described in the service specification.

In the following sequence diagrams the “Shore SECOM interface” may be onboard, onshore or a mixture thereof. This design does not place constraints on the implementation of the ship systems from a deployment perspective.

The dynamic behavior of ping, capability, encryptionKey and publicKey are not described here to avoid unnecessary duplication.

### Pre-Plan Request (Use cases 1 & 2)

UC1 returns a maximum allowed draught.

UC2 returns global tidal windows.

In both cases the response is a single S-129, no subscription is created and no further updates are pushed.

### Subscription to Actual Plan (Use case 3)

Pre-conditions

The vessel has previously shared its route via the Route Exchange Service [12] and possesses the corresponding routeId.

### Actual Plan Delivery and Updates (Use cases 4 & 5)

For UC5, every actualUpdate shall carry a reference to the transactionIdentifier (or S-129 message id) of the plan it supersedes. Update cadence is bounded by SS0005 (typically 5–60 minutes depending on met-ocean variability) and by the throttling policy of the instance (UKCDSNF004).

### Detailed Actual Plan with Stability Parameters (Use case 6)

lengthOverall and breadth are mandatory, the optional stability parameters (e.g. metacentreOrMetacenter, centreOfGravity, blockCoefficient) are persisted in the service's vessel-stability profile for the duration of the subscription and improve the accuracy of squat, heel and dynamic-draught modelling.

### Subscription Cancellation (Use cases 7 & 9)

A RemoveSubscription from the vessel shall not be interpreted as the vessel cancelling its voyagen, it merely ends the UKC information flow. Conversely, a vessel cancelling its route in the Route Exchange Service [12] should trigger an automatic RemoveSubscription from the UKC Digital Service (UC9).

### Critical UKC Alert to VTS (Use case 8)

The integration between the UKC Digital Service and the VTS system for alerting is implementation-specific (internal API, event bus, message queue, etc.) and is out of scope of the SECOM design between vessel and service. The vessel still receives the plan via the normal SECOM Upload of S-129.

### Large-Message Handling — UploadLink / GetByLink

The transactionId T is the same across the failed Upload, the UploadLink and the GetByLink. It also remains the correlation id used in any subsequent Acknowledgement for this payload.

# Definitions

| Term | Definition |
| --- | --- |
| Actual Plan | UKC plan calculated close to arrival/departure (e.g. 48–24 h prior), using up-to-date met-ocean and hydrographic forecasts. Updated frequently. |
| Almost-non-navigable area | An area where, given current/forecast conditions, the UKC limit is approached but not violated. Reported with the distance above the UKC limit. |
| Basic Route | A minimal S-421 route fragment carried inside an S-212 underKeelClearanceMessage, containing at least the destination waypoint and ETA, used to drive a pre-plan calculation. |
| Control Point | A location (typically a waypoint or close to one) along the route for which detailed UKC information is reported in the S-129 plan: distance above UKC limit, expected passing speed and time, local tidal window. |
| Detailed Actual Plan | An actual plan computed using vessel stability parameters (UC6) in addition to the basic geometric/draught data. |
| Global Tidal Window | The most restrictive intersection of all local tidal windows along the planned route, i.e. the time interval during which the vessel can complete the entire transit while respecting the UKC limit. |
| Local Tidal Window | The time interval during which a specific control point along the route can be safely passed at the vessel's planned draught. |
| Non-navigable area | An area where, given current/forecast conditions, the UKC limit cannot be respected. |
| Operational Node | A logical entity that performs activities, independent of any physical realisation. |
| Pre-plan | A high-level UKC computation produced days/weeks in advance, returning either a maximum allowed draught (UC1) or a tidal window (UC2) for the planned voyage. |
| UKC Limit | The minimum vertical distance, defined per fairway / waterway segment, that a vessel's keel must maintain above the seabed for safe passage. |

# Abbreviations

API	Application Programming Interface

MIR	Maritime Identity Registry

MRN	Maritime Resource Name

MSR	Maritime Service Registry

REST	REpresentational State Transfer

SECOM	Secure Communication

UUID	Universally Unique IDentifier

# References

- IALA. 2022. Guideline 1128 on Specification of e-Navigation Technical Services v1.6

- IALA 2026 SS0005 Technical Service Specification for UKC Digital Service v1.0

- IMO 2024. MSC.1CIRC.1610REV.1 INITIAL DESCRIPTIONS OF MARITIME SERVICES IN THE CONTEXT OF E-NAVIGATION https://wwwcdn.imo.org/localresources/en/OurWork/Safety/Documents/enavigation/MSC.1-CIRC.1610%20- %20Initial%20Descriptions%20Of%20Maritime%20ServicesIn%20The%20Context%20Of%20E-Navigation%20(Secretariat)%20(1).pdf

- IEC 63173-2 SECOM  Committee Draft 80/2532/PCC

- SECOM Service Interface OpenAPI

- IHO. 2024. Universal Hydrographic Data Model Standard S-100 5.2.0

- NIST Digital Signature Standard (DSS–FIPS Publication 186)

- RFC 9110 HTTP Semantics

- IEC 63173-1 S-421 Route Plan based on S-100 Committee Draft 80/2542/PCC

- IALA. 2022 Guideline G1183 Provision of Maritime Identities v1.1

IALA Guideline GXXXX Maritime Service Registry Technical Specification v1.0

IALA. 2025. SS0002, Service specification for VTS – Vessel Route Exchange, Edition 1.1

IHO. 2024. Under Keel Clearance Management Product Specification, Edition 2.0.0, Dec 2024

IALA TBD SSXXXX S-212 specifications

# Appendix 1 – Service Design Template OPENAPI (SWAGGER) in JSON

The OpenAPI definition for the SECOM REST interfaces is provided by IEC 63173-2 [5]. Implementations of this design shall publish an OpenAPI document derived from the SECOM OpenAPI in which:

the dataProductType enumeration is restricted to S-129 and S-212;

the unused interfaces (Get, GetSummary, Access, AccessNotification) are either omitted or documented as returning HTTP 501;

the Capability response payload exposes the four UKC service-level flags listed in §6.10;

per-instance constraints (max payload size for Upload before UploadLink is required, throttling limits, supported S-129/S-212 versions) are documented.

# Appendix 2 – Guidance on Implementation

More information can be found in ref [4]

EC 63173-2 SECOM ed2 “Copyright © 2022 IEC Geneva, Switzerland. www.iec.ch”

Author to insert actual service name and version number