Input paper:   VTS60-9.1.3

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

□  ARM	□  ENG		□  PAP					X  Input

□  ENAV	X  VTS							□  Information

Agenda item 			9.1

Technical Domain / Task Number 2	2.5.2

Author(s) / Submitter(s)			China Maritime Safety Administration

# Proposals on the “Service Specification for                         VTS - Vessel Route Exchange Edition 1.1”

# Summary

This proposal proposes revisions to Service Specification for VTS - Vessel Route Exchange Edition 1.1 from the aspects of operational context, use cases, functional and non-functional requirements, and key terms.

## Purpose of the document

The purpose of this document is to provide input document for the VTS committee to update the Service Specification for VTS - Vessel Route Exchange Edition 1.1.

## Related documents

[1]	VTS57-6.1.1 Committees Work programme 2025-2027

[2] 	 VTS53-6.3.2 Development of Technical Service Specifications for Digital Data Exchange between VTS and Other Entities - Primarily Ships

[3] 	 VTS54-9.1.3 Proposals on the New Task of "Developing Technical Service Specifications for Digital Data Exchange between VTS and Other Entities - Primarily Ships"

[4] 	 VTS55-9.1.1 Proposals on the “Service Specification for VTS Traffic Clearance Service Edition 1.0”

[5] 	 VTS56-9.1.1 Proposal on the "Service Specification for VTS Traffic Clearance Service Edition 1.3” and "Service design"

[6] 	VTS57-9.1.2  Proposals on the "Service Specification for VTS Traffic Clearance Edition 1.4"

[7] 	VTS57-9.1.1  Proposals on the "Service Specification for VTS-Vessel Route Exchange Edition 0.3"

[8] 	VTS58-9.1.1  Proposals on the "Service Specification for VTS-Vessel Route Exchange Edition 1.0"

[9] 	VTS58-12.2.3 Service Specification for Route Exchange Edition 1.1

[10] 	C02-10.5.2 Revised G1128 Specification of e-Navigation Technical Services Edition 1.7

[11] 	IMO NCSR 13-9  Operational Guidance for Route Exchange

# Background

2.1	The IALA VTS Committee Work Plan 2023-2027 raises the task” Development of technical service specifications for digital data exchange between VTS and other entities - primarily ships” (task 2.5.2),  Service Specification for VTS -Vessel Route Exchange Edition 1.1 had been finalized at VTS58.

2.2     As the co-sponsor of  the new work task proposal (VTS 53-6.3.2) at  VTS53, China MSA continued to submit " Proposals on the New Work Task of Developing Technical Service Specifications for Digital Data Exchange between VTS and Other Entities (VTS54 9.1.3) "  and " Draft of Service Specification for digital VTS Anchorage Assignment Service Edition 0.5" (VTS54 8.3.2) at VTS54, “Proposals on the Service Specification for VTS Traffic Clearance Service Edition 1.0” (VTS55-9.1.1) at VTS55, and "Proposals on the Service Specification for VTS Traffic Clearance Service Edition 1.3 and Service design" (VTS56-9.1.1) at VTS56 , "Proposals on the Service Specification for VTS Traffic Clearance Edition 1.4" (VTS57-9.1.2), "Proposals on the Service Specification for VTS -Vessel Route Exchange Edition 0.3" (VTS57-9.1.1) at VTS57, "Proposals on the Service Specification for VTS -Vessel Route Exchange Edition 1.0" (VTS58-9.1.1 )at VTS58, “Proposals on the Service Specification for VTS Traffic Clearance Service Edition1.0” (VTS55-9.1.1) and “Proposals on the Service Specification for UKC Edition 0.5” at VTS59.

# Discussion

## Proposals on updating the operational context

G1128 emphasizes that "the operational context shall describe any requirement the service shall fulfil or adhere to. This refers to functional as well as non-functional requirements at a high level".

The Operational Guidance for Route Exchange (NCSR 13/9 Annex 4), approved by NCSR 13 in June 2026, applies to the digital exchange of route information between ships and shore-based authorities or services (including, but not limited to, vessel traffic services), covering both ship-to-shore and shore-to-ship exchanges. The Guidance has been developed to provide safe, effective and harmonized operational instructions for the voluntary exchange of digital route plans between ships and shore-based Maritime Services within the S-100 framework, and highlights its role in supporting the ongoing development of maritime services. Implementation of this Guidance is expected to enhance safety of navigation, improve compliance with environmental regulations and facilitate voyage planning. Consequently, any use of ship-shore digital route exchange should take this Guidance fully into account.

Upon review, it is suggested to update the operational context section from the following aspects:

  - a.	Update the objectives of the route exchange service to enhance situational awareness, support decision-making for both ship and shore, and improve safety and operational efficiency (e.g., optimizing traffic flow, reducing voice communication burden);

  - b.	Incorporate the terminology and definitions for Digital Route exchange ;

  - c.	Update the definitions for Route crosscheck service, Route reference service, and Route monitoring service. It is suggested to add the definition for Route exchange service.

## Proposals on updating  use cases

it is suggested to add the following operational service details:

  - a.	In UC1, Typical sequence 4, "The [service] (checks that the route is valid according to the schema)" should be refined to [route crosscheck service], to correspond with the [route exchange service] in Typical sequences 5 and 6;

  - b. 	In UC3, Typical sequences 1, 2, and 3, "The [service] (sends "received" acknowledgement automatically)" should be refined to [route exchange service];

  - c. 	In UC4, Typical sequence 3, "The [service] (checks that the route is valid according to the schema)" should be refined to [route crosscheck service];

  - d. 	In UC11, Typical sequence 2, both instances of "The [service] (checks that the route is valid according to the schema)" should be refined to [route crosscheck service];

  - e. In UC12, Typical sequence 3, both instances of "The [service] (checks that the route is valid according to the schema)" should be refined to [route crosscheck service]. In Typical sequence 4, "The [service] (delivers the route to VTS system and sends acknowledgment to ship systems that route has been delivered to VTS system)" should be refined to [route exchange service];

  - f. 	UC11 should add a reference to the [route exchange service];

  - g. 	UC13 should add a reference to the [route monitoring service];

  - h. 	UC13 should add a reference to the [route reference service].

## Proposals on updating functional and non-functional requirements

3.3.1 Update functional requirements

With reference to UKCDSF007 and UKCDSF013 from the VTS UKC Service Specification Edition 1.0 adopted at VTS59 in April 2026, two new functional requirements are proposed: "route exchange service details must be published in service registry for service discovery" and "UTC timestamps information exchanged".

3.3.2 Update non-functional requirements

With reference to UKCDSNF004 from the VTS UKC Service Specification Edition 1.0, a new non-functional requirement "Throttling -- rate limitations" is proposed.

Additionally, the Requirement ID  "TCSNF001~004 " should be renamed to RESNF001~004.

## Proposals on changes to key terms

3.4.1 Replace "Mariner" with "OOW"

The "Mariner" node is proposed in Operational nodes, with the definition "means a person who is part of the bridge team [that is responsible for the navigation of the vessel]". "Mariner" appears repeatedly up to 25 times in this specification. Considering that:

  - a.	The IMO Performance Standards for Bridge Navigational Watch Alarm System (BNWAS) (Resolution MSC.128(75)) defines OOW in the appendix ("The Officer of the Watch (OOW) is the officer responsible for the navigational watch on the bridge"), and this definition is cited by the STCW Code (A-VIII/2) and other official documents such as the "Principles and operational guidance for officers in charge of a navigational watch in port" (IMO Assembly Resolution A.337(IX));

  - b. 	Paragraph 14 of the Operational Guidance for Route Exchange approved by NCSR13 clearly states that in the route exchange scenario, the OOW is responsible for receiving, reviewing, and confirming route suggestions sent from shore, while ensuring that the safe conduct of the navigational watch is not compromised ("The OOW should not be expected to validate an incoming route if doing so would detract from the safe conduct of the navigational watch.");

  - c. 	In scenarios involving navigational safety responsibilities, IMO instruments never use the term “mariner” to describe personnel with statutory duties. Replacing it with “OOW” clearly defines the attribution of responsibilities and aligns with the legal context of STCW and IMO resolutions.

In summary, it is suggested to replace the "Mariner" node in 3.4.2 Operational nodes with "Officer of the Watch (OOW)", updating it to the authoritative IMO definition mentioned above, and change all 25 instances of "Mariner" in this specification to "OOW", including updating Figures 1, 2, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 15 and Table 1, among others.

3.4.2 Replace "VTSO / VTS Operator" with "VTS Personnel"

The "VTS operator" node is proposed in Operational nodes. Yet, this specification repeatedly uses "VTS Personnel" up to 27 times, with the definition "The personnel of the VTS centre, means persons performing tasks associated with vessel traffic services, trained in vessel traffic services operations and appropriately qualified". Considering that:

  - a. 	The official definitions in IMO Resolution A.1158(32) and the IALA Dictionary are both "VTS Personnel", not "VTS operator";

  - b. 	"VTSO/VTS Operator" narrowly refers to "VTS personnel performing operational tasks", whereas "VTS Personnel" covers multiple roles such as Operator, Supervisor, Manager, etc. In context, "VTS Personnel" is more appropriate for this specification.

It is suggested to replace the "VTSO/VTS Operator" node in 3.4.2 Operational nodes with "VTS Personnel". The definition can remain unchanged. It is also suggested to delete "VTSO VTS Operator" from Section 8 Abbreviations.

3.4.3 Replace "Ship system(s)", "on-board system(s)", "end user system(s)" with "ECDIS"

This specification contains dozens of instances where terms like "Ship system(s)", "on-board system(s)", "end user system(s)" are used interchangeably with "ECDIS". Considering that:

  - a. 	The ECDIS Performance Standards amendment (MSC.530(106)/Rev.1 11.3.4), approved for release by MSC108 in 2024, already supports ECDIS-based ship-shore route exchange;

  - b. 	The 1.3.1 VTS Digital Communications Task Group intersessional meeting working paper on USE CASES for VTS Digital Communications, updated on 8 July, has updated all 9 use cases for route exchange to use "ECDIS" instead of terms like "Ship system(s)", "on-board system(s)", and "end user system(s)".

It is suggested to replace the dozens of "Ship system(s)", "on-board system(s)", and "end user system(s)" in this specification with "ECDIS", and update Figures 1, 2, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 15, 16, etc., to align with the "ECDIS" used in Figure 14.

# Action requested of the Committee

The Committee is requested to consider the proposals in this document and take actions as appropriate.

Input document number, to be assigned by the Committee Secretary

Leave open if uncertain