Guideline

GXXXX

THE USE OF THE AUTOMATIC IDENTIFICATION SYSTEM (AIS) IN OTHER

MARINE AIDS TO NAVIGATION

Edition 1.x

Date (of approval by Council)

urn:mrn:iala:pub:gnnnn:ed1.0

Revisions to this document are to be noted in the table prior to the issue of a revised document. The latest edition of the Guideline is the only version in force unless the Guideline is explicitly revoked by the Council.

| Date | Page / Section Revised | Requirement for Revision |
| --- | --- | --- |
| TBC 2022X |  |  |

1.	INTRODUCTION	5

2.	AIS AS A MARINE AID TO NAVIGATION	5

3.	PHYSICAL, SYNTHETIC, AND VIRTUAL AIS ATON	6

3.1.	Physical AIS AtoN	6

3.2.	Synthetic AIS AtoN	6

3.2.1.	Monitored synthetic AIS AtoN	6

3.2.2.	Predicted synthetic AIS AtoN	6

3.3.	Virtual AIS AtoN (VAtoN)	6

3.4.	EXAMPLE USE CASES FOR AIS AtoN	7

4.	THE AIDS TO NAVIGATION REPORT (MESSAGE 21)	7

5.	THE VHF DATA LINK	7

5.1.	How does it operate?	8

5.2.	Protocol for data transmission	8

5.2.1.	Timing	8

5.2.2.	Position	8

5.3.	VHF data link administration	8

6.	AUTHORISATION OF AIS ATON	9

7.	TECHNICAL STANDARDS FOR AIS ATON	9

7.1.	Type 1 AIS AtoN	10

7.2.	Type 3 AIS AtoN	10

8.	INSTALLATION AND MAINTENANCE OF AIS ATON	10

9.	SUPPLEMENTARY AIS ATON MESSAGES	10

9.1.	Reporting intervals for supplementary messages	11

10.	AIS DATA MANAGEMENT	11

11.	CYBERSECURITY	12

12.	IMPLEMENTATION	12

12.1.	AIS AtoN service availability	12

13.	DEFINITIONS	12

14.	ABBREVIATIONS	12

15.	REFERENCES	13

ANNEX A	CRITERIA FOR PROVIDING AIS ATON	14

ANNEX B	VIRTUAL ATON	17

ANNEX C	THE AIDS TO NAVIGATION REPORT (MESSAGE 21)	20

ANNEX D	PLANNING AND CONSIDERATIONS FOR AIS ATON	25

List of Tables

Table 1	Use cases for AIS AtoN	7

Table 2	Summary of most common AIS AtoN Station messages	10

Table 3	Potential risk mitigation measures	16

Table 4	Application of VAtoN	18

Table 5	AtoN and MMSI	20

Table 6	Codes for type of AtoN	21

Table 7	Dimension/reference for position, for a fixed AtoN	23

Table 8	Dimensions for a floating AtoN and both fixed and floating offshore structures	23

Table 9	Recommended use of status bits (light, racon and health)	24

List of Figures

Figure 1	Protocol for AIS data transmission	9

Figure 2	Flow chart for decision process, AIS AtoN	15

# INTRODUCTION

The Automatic Identification System (AIS) is an autonomous broadcast system, operating in the VHF maritime mobile band. It exchanges information such as vessel identification, position, course, speed, etc. between stations. It handles multiple reports, using Time Division Multiple Access (TDMA) technology ensuring reliable and robust operation. The carriage requirements for AIS equipment on vessels are set in Regulation 19.2.4 of Chapter V of the International Convention for the Safety of Life at Sea (SOLAS), 1974, as amended.

The main purpose of shipborne AIS is:

To enhance situational awareness and support collision avoidance in ship-to-ship mode;

to enable coastal States to obtain information relating to   ships and their cargoes; and

to supportVessel Traffic Services (VTS)  by facilitating ship-to-shore communications and traffic management,

AIS, as applied to other Marine Aids to Navigation (AtoN), fosters the safe and efficient movement of vessels for the benefit of the maritime community and the protection of the marine environment.

# AIS AS A MARINE AID TO NAVIGATION

Article 2.1 of the Convention on the International Organization for Marine Aids to Navigation  defines an AtoN  as a device, system, or service, external to a vessel, designed and operated to enhance safe and efficient navigation of individual vessels and vessel traffic. The application of AIS as an AtoN to another AtoN (AIS AtoN)is achieved through the broadcast of an Aids to Navigation report message (Message 21) from an AIS AtoN station or base station. AIS AtoN should only be used to enhance safety of navigation.

The primary purpose of AIS AtoN is to promote and enhance safety and efficiency of navigation by one or more of the following:

providing a positive, all-weather means of identification;

complementing other existing AtoN e.g. racons;

transmitting accurate positions of floating AtoN;

indicating if a floating AtoN is off position;

promulgation of Application Specific Messages (ASM) including:

delineating tracks, routes, areas, and limits, for example, areas to be avoided and Traffic Separation Schemes (TSS); and

providing weather, tidal, and sea state data;

providing additional AtoN capability through use of virtual AIS AtoN, where installation of physical AtoN is technically or operationally difficult;

enabling timely marking of new immobile hazards; and

enabling marking of mobile hazards by use of Mobile AtoN.

The criteria for the use of AIS AtoN  should be based on the navigational requirement derived from assessment of risk.

When planning AIS AtoN it is important to bear in mind that not all vessels are equipped with AIS. In addition, for those vessels that are AIS equipped, the display of AIS data can range from limited text display, to full ECDIS and Radar overlay. In the absence of ECDIS or Radar overlay users will not be able to fully use AIS AtoN functionality.

It is also very important when considering deploying AIS AtoN to note the mariner’s need for appropriate, relevant, accurate and unambiguous information.

Particular care must be exercised with the activation and promulgation of virtual AIS AtoN so as to avoid errors, particularly errors in position, and to avoid the unintentional creation of situations where mariners could be faced with too much information, irrelevant information, or information that results in confusion or distraction.

For further information regarding Mobile AtoN refer to Guideline G1154 Use of Mobile Aids to Navigation. More guidance on criteria for stablishing AIS AtoN can be found in Annex B.

# PHYSICAL, SYNTHETIC, AND VIRTUAL AIS ATON

AIS AtoN can be implemented in three ways – physical, synthetic and virtual.

## Physical AIS AtoN

A physical AIS AtoN is an AtoN that physically exists and is fitted with an AIS AtoN station broadcasting its own Message 21. It is displayed as a solid line diamond with crossed lines centred at the reported position of the AtoN.

## Synthetic AIS AtoN

A synthetic AIS AtoN is an AtoN that physically exists; however, its Message 21 is transmitted from a geographically separated AIS AtoN station. It is displayed as a solid line diamond with crossed lines centred at the reported position of the AtoN.

Synthetic AIS AtoNs are classified into two types: monitored synthetic AIS AtoN and predicted synthetic AIS AtoN.

### Monitored synthetic AIS AtoN

A monitored synthetic AIS AtoN has a communication link between the physical AtoN and the broadcasting AIS AtoN station. The location and status information of the physical AtoN is received by the AIS AtoN station and included in the Message 21.

### Predicted synthetic AIS AtoN

A predicted synthetic AIS AtoN does not have a communication link to update the location and status information of the physical AtoN to the broadcasting AIS AtoN station.

As a result, the predicted synthetic AIS AtoN Message 21 does not confirm the location and status of the AtoN, and therefore is not recommended for use on floating AtoN.

The use of predicted synthetic AIS AtoN broadcasts for fixed AtoN is acceptable as the location will not change, but the status of the AtoN is not verified.

## Virtual AIS AtoN

A virtual AIS AtoN  is transmitted as a Message 21 for an AtoN that does not physically exist. It is displayed as a thin dashed line diamond with crossed lines centred at the reported position of the AtoN.

When a virtual AIS AtoN is used, the AtoN symbol or information would be available for presentation to a mariner, even though there is no physical AtoN such as a buoy or beacon. A base station or geographically separate AtoN station would broadcast this message.

The virtual AIS AtoN Flag in Message 21 would be set to 1, to clearly identify this as a virtual AIS AtoN.

An example of where virtual AIS AtoN could be useful is the marking of hazards to navigation on a temporary basis, see Recommendation R1015 Marking of Hazardous Wrecks, and Guideline G1046 Response plan for the Marking of New Wrecks. More details on Virtual AIS AtoN are included in Annex B.

## EXAMPLE USE CASES FOR AIS AtoN

Example use cases for physical, synthetic and virtual AIS AtoN are shown below.

Use cases for AIS AtoN

| Element to be addressed | Physical | Synthetic | Virtual |
| --- | --- | --- | --- |
| Low power availability at site |  | X | X |
| Difficult to access site |  | X | X |
| Space limitations on site |  | X | X |
| Requirement for remote monitoring | X |  |  |
| Additional functionality – met./hydro. data, etc. | X |  |  |
| Location:difficult environmental conditions including tide/ current; cyclone/hurricane; extreme temperature |  | X | X |
| Need for spatial awareness | X | X | X |
| Waterway use for SOLAS vessels only |  |  | X |
| Waterway use mixed vessels | X | X |  |
| Temporary marking of new danger/ wreck |  |  | X |

# THE AIDS TO NAVIGATION REPORT (MESSAGE 21)

Establishing an AIS AtoN service enables AtoN providers to broadcast information about an AtoN. The Aids to Navigation Report (Message 21) can include the following information:

Type of AtoN

Name of the AtoN

Position of the AtoN

Accuracy of AtoN position

Type of position fixing device

Position status for floating AtoN

Whether physical or virtual AtoN

Dimension of the AtoN and reference positions

Status of AtoN systems  such as light/racon function

More details  on the information included in Message 21 are set out in Annex C.

# THE VHF DATA LINK

AIS uses TDMA protocols. This is the method used to communicate using pre-defined messages (sentences), which are exchanged between stations via two designated VHF frequencies, AIS1 (161.975MHz) and AIS2 (162.025MHz). The protocols, frequencies and the messages together are defined as the VHF data link (VDL). The VDL is similar to an internet email service where international compatibility is assured by a strict protocol, whatever the content of the message might be.

AIS AtoN typically transmit a message every three minutes.

## How does it operate?

The VDL is divided into a number of equal time slots which hold a set amount of data and are synchronized using GNSS time. The primary methods by which AIS devices access the link as detailed in ITU-R M.1371 are:

Self-Organized (SOTDMA) is the basic access method for mobile stations (vessels). Stations preannounce when they are going to transmit and plan their transmissions based on slot use information collected from other stations in order to prevent slot collisions, e.g.two stations picking the same slot to transmit their data packet.

Random Access (RATDMA) is used by AIS stations to access the link for unscheduled transmissions.

Fixed Access (FATDMA) is used by AIS stations that have a requirement to transmit data at predetermined intervals, and involves the reservation of particular slots for their exclusive use. Authorities in coastal States are responsible for the allocation and reservation of FATDMA slots.

Carrier Sense (CSTDMA) also known as polite behaviour is used by some mobile stations which can access the link only when they find an unused slot. These stations must listen to the beginning of a slot to detect occupancy and are therefore limited to a single slot transmission.

AIS base stations manage the network in a similar manner to telephone cell towers.

## Protocol for data transmission

### Timing

Correct synchronization of each user's equipment is imperative to ensure proper functioning of the TDMA.

For  the two AIS frequencies, each minute of time known as the frame is divided into 2250 slots, giving a total of 4500 slots. To ensure all units are synchronized, every AIS station contains a GNSS receiver, which provides Universal Time Co-ordinated (UTC) as a timing reference.

### Position

Although AIS relies on GNSS for timing it may also obtain its position reference from an external source,e.g. surveyed position for a fixed AtoN.

## VHF data link administration

AIS has been designed for short range VHF coverage, normally referred to as line of sight At. Although most AIS messages only use one slot, some can occupy up to five consecutive time slots. The greater the number of slots used by a message, and the larger the number of vessels in a coverage area, the greater the potential for data packet (slot) collisions. Since most AIS base stations typically have a high antenna position and large coverage area, this may result in messages not being decoded from more distant AIS units in an area where there are a very large number of AIS stations operating. However, data from these distant stations would continue to be transmitted and received correctly by closer AIS stations.

FATDMA reservations are required for Type 1 AIS AtoN stations,see section 7. Individual slots allocations for AIS AtoN stations require transmission of a message 20,Data Link Management, in the coverage area. This can be transmitted by an AIS base station that is capable of control of the VDL. Administrations should organize the usage of the VDL by appropriate FATDMA reservations and consider the total load on the VDL before introducing additional services which rely on AIS.

Efficient use of the FATDMA allocations can be improved by having several AtoN in the same area using the same slots but in different frames. For example, 3 buoys, each with a 3-minute reporting interval, in the same area could be configured such that Buoy A transmits in frames 0, 3, 6, … Buoy B transmits in frames 1, 4, 7,…. and Buoy C transmits in frames 2, 5, 8,…. all using the same slots.

The information exchanged via the VDL is safety related and the usage of the VDL should be monitored and controlled to safeguard its proper function and avoid overloading.

Even though there are potentially some scenarios in which data collisions may occur, AIS is quite robust and has been designed to work at high VDL loading.

Protocol for AIS data transmission

# AUTHORISATION OF AIS ATON

The use of AIS AtoN should be overseen by the competent authority. The competent authority should take care to control AIS AtoN provision to avoid overload on mariners’ displays.

AIS AtoN require a radio licence and the radio licensing authority may be different from the maritime administration. In such cases, there may be a need for a coordinated procedure to ensure that AIS AtoN are correctly licensed and authorised, without unnecessarily delaying or obstructing their deployment.

The authorities involved can be diverse and include maritime administrations, communications regulators, hydrographic services, lighthouse authorities and other governmental and non-governmental organizations and agencies.

The authorities should agree in advance who has responsibility for:

allocation of Maritime Mobile Service Identity (MMSI)

ITU reporting

AIS Slot map allocations where applicable.

# TECHNICAL STANDARDS FOR AIS ATON

There are two types of physical AIS AtoN with different functionality. They are summarised below and are fully described in IEC document IEC62320-2 AIS AtoN stations - Minimum operational and performance requirements - methods of test and required test results.

## Type 1 AIS AtoN

The Type 1 AIS AtoN is a transmit-only station, operating in FATDMA mode. The slots used by the Type 1 AIS AtoN need to be reserved by a national competent authority, using Message 20, transmitted from an AIS in the coverage area. The Type 1 unit must be configured to use the slots reserved for it before being placed into service.

This is the simplest type of AIS AtoN, with relatively low cost and power consumption.

## Type 3 AIS AtoN

The Type 3 AIS AtoN is more complex and incorporates two AIS receiving processes FATDMA and RATDMA that allow it to participate fully on the VDL.

The Type 3 AIS AtoN is therefore capable of:

Autonomous operation, not requiring slot reservations (RATDMA)

Autonomous operation using slots reserved by a national competent authority, using message 20, transmitted from another AIS Station in the coverage area (FATDMA)

Receiving and relaying AIS messages, including control and configuration messages for itself or for other AIS AtoN stations in a chain, see IEC 62320-2 for more details of chaining

Repeating AIS messages

# INSTALLATION AND MAINTENANCE OF AIS ATON

The installation of AIS AtoN requires careful placement to avoid system failures, damage or interference with other electronic devices. Particular attention is required to the correct programming of the AtoN.

More  advice on installation, maintenance and training is included in Annex D.

# SUPPLEMENTARY AIS ATON MESSAGES

In addition to Message 21 an AIS AtoN may also transmit other message types including 6, 7, 8, 12, 13, 14, and 25.

AIS also incorporates the facility for an authority to develop their own regional messages. These are referred to as Application Specific Messages (ASM). A summary of common AIS AtoN messages is shown in Table 2.

Summary of most common AIS AtoN Station messages

| Msg ID | Message Name | Message Description | Application examples |
| --- | --- | --- | --- |
| 6 | Binary Addressed Message | Binary data for addressed communication | Monitoring of AtoN lantern, power supply, etc. |
| 7 | Binary acknowledge message | Acknowledge of addressed binary message |  |
| 8 | Binary Broadcast Message | Binary data for broadcast communication | Meteorological and hydrological data |
| 12 | Addressed Safety Related Message | Safety related data for addressed communication | Warn AtoN malfunctioning |
| 13 | Safety related acknowledge message | Acknowledge of addressed safety related message |  |
| 14 | Broadcast Safety Related Message | Safety related data for broadcast communication | Warn AtoN malfunctioning |
| 25 | Single slot binary message | Binary data for addressed or broadcast communication | Status report |

The Organization maintains a register of regional ASM. The purpose of this register is harmonization. It can be found on the homepage of the Organization under  https://www.iala.int/asm/ for further information.

More than one AIS message format may be transmitted as noted above. The national competent AtoN authority should establish procedures to verify the broadcast information and the correct operation of the AIS AtoN station.

## Reporting intervals for supplementary messages

Reporting intervals for supplementary messages should be based on operational requirements as shown in the following examples:

Message 6 for AtoN monitoring

This message needs only to be sent as often as the operator data is required. However, in practice, power consumption by the AIS AtoN will be minimized if this message is sent just before or just after a Message 21. This is because most AIS AtoN devices will power down parts of their operating system between transmissions (sleep mode).

Message 8 for meteorological and hydrological data

Again, this should be coordinated with the wake-sleep cycle for Message 21. However, by its nature, this message is required less frequently, so that a multiple of the Message 21 reporting interval would be appropriate. In situations where the Message 8 for meteorological and hydrological data is repeated by an AIS Base Station, the reporting interval at the AIS AtoN station might as an example be reduced to 30 or 60 minutes.

Guideline G1095 Harmonized implementation of ASM includes more details.

# AIS DATA MANAGEMENT

AIS data can be gathered from local, regional, national or international network systems of AIS base stations and can be used to further improve and enhance services provided to mariners.

AIS AtoN could provide information including the following:

Monitoring the status of an AtoN

Tracking an AtoN that is off position

Identifying ships involved in collisions with AtoN

Gathering real-time information on the state of health of an AtoN

Remotely controlling changes in AtoN parameters

Providing statistics on reliability of AtoN

Extending the coverage of AIS monitoring

Guideline G1050 Management and monitoring of AIS  includes more information on the use of AIS data.

# CYBERSECURITY

AIS is an open communication protocol lacking in inherent security and this should be considering within any use in AtoN. Further guidance on cybersecurity can be found in Guideline G1182 Cyber security specifics from an IALA perspective.

# IMPLEMENTATION

## AIS AtoN service availability

The AIS AtoN service shall have a service availability corresponding to IALA Category 1, 2, or 3 depending on the importance of AtoN for the intended transmissions. The service availability for all AIS AtoN shall be calculated as a three-year rolling average. The Organization has published normative Recommendation R0130 Categorisation and Availability Objectives for Short Range Aids to Navigation.

The AIS AtoN transmissions containing the AtoN information shall have a signal level of greater than or equal to –107dBm when measured at the air-antenna interface of the user’s receiver following IEC 61993-2 standard within the following coverage areas:

5-10 M of the AIS AtoN for floating AtoN depending on height of AtoN

10-25 M of the AIS AtoN for fixed AtoN depending on height of AtoN

Synthetic and virtual AIS AtoN signal sent from an AIS base station might have different coverage areas depending on the location of the AtoN inside the AIS base station coverage area, e.g. the edge, centre, etc. National competent authorities shall develop their own procedures for dealing with coverage area and consider the use of multiple stations if necessary.

Expected signal strength can be computed for a suitable measuring point using standard propagation calculations.

# DEFINITIONS

The definitions of terms used in this Guideline can be found in the International Dictionary of Marine Aids to Navigation.

# ABBREVIATIONS

ASM	Application Specific Message

CSTDMA	Carrier Sensed TDMA

ECDIS	Electronic Chart Display and Information System

FATDMA	Fixed Access TDMA

GNSS	Global Navigation Satellite System

MMSI	Maritime Mobile Service identity

RATDMA	Random Access TDMA

SOTDMA	Self Organised TDMA

TDMA 	Time Division Multiple Access

VDL	VHF Data Link

VHF	Very High Frequency (30-300 MHz)

# REFERENCES

International Convention for the Safety of life at Sea (SOLAS) 1974, as amended.

IALA. (2020) G1154 Use of Mobile Aids to Navigation.

IALA. (2017) R1015 Marking of Hazardous Wrecks.

IALA. (2019) G1046 Response Plan for the Marking of New Wrecks.

ITU-R M.1371-5 (2014) Technical Characteristics for an automatic identification system using time division multiple access in the VHF maritime mobile frequency band.

IEC 62320-2:2016 Maritime navigation and radiocommunication equipment and systems - Automatic identification system (AIS) - Part 2: AIS AtoN Stations - Operational and performance requirements, methods of testing and required test results.

IALA. (2013) G1095 Harmonized implementation of Application-Specific messages (ASM).

IALA. (2005) G1050 Management and Monitoring of AIS information.

IALA. (2024) G1182 Cyber security specifics from an IALA perspective.

IALA. (2017) R0130 Categorization and Availability Objectives for Short Range Aids to Navigation.

IEC 61993-2:2018 Maritime navigation and radiocommunication equipment and systems - Automatic identification systems (AIS) - Part 2: Class A shipborne equipment of the automatic identification system (AIS) - Operational and performance requirements, methods of test and required test results.

CRITERIA FOR PROVIDING AIS ATON

# PHYSICAL AIS ATON

## Lighthouses and beacons

The primary purpose of providing AIS AtoN functionality to lighthouses and beacons is to provide the mariner with a fixed point of reference. The optional provision of AtoN status information advises mariners as to whether key AtoN are performing correctly and allows for revision of the passage plan if required.

AIS AtoN functionality may be provided on lighthouses and beacons where a navigational assessment identifies the requirement set out above. Typical locations for consideration would include offshore stations, headland stations, landfall stations, stations that are commonly used as waypoints, stations that mark points on featureless coastlines, or isolated dangers.

## Floating AtoN

In addition to the point of reference and AtoN information purposes described above for fixed AtoN the provision of AIS AtoN functionality on floating AtoN provides confirmation of the floating AtoN position. Confirmation of position provides the mariner with an assurance that the AtoN can be used and improves spatial awareness. Advance confirmation of the position of floating AtoN is a significant improvement in the service available to the mariner.

AIS can improve the conspicuity of AtoN on mariners’ displays. Typical locations for consideration would include major floating AtoN, gateway buoys at the approach to narrow channels, buoys that are commonly used as waypoints, buoys marking isolated dangers, buoys marking the extremities of shoal areas and buoys that are critical to the mariner’s spatial awareness.

# virtual ais aton

Permanent or temporary virtual AIS AtoN could be used where it is not possible to put physical AtoN on station, or where AtoN provision can be improved. Examples include ice or severe weather conditions, new wrecks and dangers, AtoN in very close proximity to shipping movements, marking of temporary fairways, marking of temporary hazards or debris fields, or where additional AtoN can be provided to enhance safety of navigation.

The need to rapidly mark wrecks and other new dangers with virtual AIS AtoN, often in advance of laying physical AtoN, will be based on an assessment of the danger posed to shipping in the area.

Any replacement of existing physical AtoN with virtual AtoN will require a detailed risk assessment.

Flow chart for decision process, AIS AtoN

# risks and limitations

AIS AtoN are increasingly portrayed on the displays of ships, though there are still some challenges to be addressed. These challenges include the lack of consistent symbology causing confusion, lack of interconnection between equipment, equipment not being properly configured to show data, information overload or absence, and some displaying a position offset.

Other risks and limitations may include GNSS or VHF vulnerability to interference, including jamming and spoofing. Overloading of the VHF datalink is also possible in congested areas.

## Limitations

### GNSS Vulnerability

In the event of GNSS service interruption due to interference, whether natural or intentional, ships may lose their positioning capability unless they have an alternative positioning system. Poor installation or failure of shipborne equipment can similarly interfere with or degrade GNSS reception.

Errors in the GNSS position, for examplecaused by time lag, will directly impact the appearance of the AIS AtoN symbol on the radar, potentially misleading mariners into thinking that the AIS AtoN broadcast position is faulty.

### Spoofing and Jamming of AIS signal

Both spoofing and jamming can compromise and/or shut down an AIS AtoN service. Jamming will typically block the service in a certain geographic area. Spoofing of virtual AIS AtoN is more sinister since the targeted receiver cannot detect the deception, which could mislead the navigator.

Some spoofing methods can be detected through careful monitoring of the transmission channel, for example by monitoring MMSI numbers within the service coverage area. Duplicated or non‐existent MMSI numbers within the coverage area may indicate spoofing.

Increased spoofing detection capability can be achieved through regional cooperation between neighbouring countries, exchanging valid MMSI numbers and cooperating on identifying invalid MMSI numbers.

Competent authorities who provide AIS AtoN services should maintain a database of all valid MMSI numbers assigned to AIS AtoN. This database should be shared with neighbouring countries.

# RISK mitigation

- This section provides some potential mitigation measures for the different risks.

Potential risk mitigation measures

| Risk | Potential mitigation measures |
| --- | --- |
| Reliance on AIS AtoN | Mariners should not rely solely on AIS AtoN but cross-check them with other data or information |
| Not all mariners will receive or be able to display AIS AtoN | - MSI should be maintained as the primary notification - Encourage the integration of AIS AtoN with navigational displays, where applicable |
| Information overload | - Use of lines and areas such as ASM instead of points - Only competent authorities are permitted to approve - Limit use of AIS AtoN in any area |
| VHF Data Link overload | Only competent authorities are permitted to approve AIS AtoN |
| Loss of AIS AtoN signal | - Publish standards for availability, continuity, and integrity - Verification of transmission by originator - Provide redundancy and integrity warnings - Include relevant information in MSI and chart - Shipborne navigational displays able to manage and display the lost target symbol |
| AIS AtoN vulnerability including Jamming andspoofing | - Verification of transmission by originator - Correlation with MSI / charts - Data link monitoring by competent authority |
| No confirmation of receipt of message | - Repeated or addressed/ acknowledged transmissions - Verification of transmission by originator |
| Erroneous message transmitted | - Procedures for message checking - Verification of transmission by originator |

VIRTUAL ATON

# general considerations

Virtual AtoN  can be used to inform mariners about dangers to navigation, safe waterways, areas where extra caution may be necessary and areas to be avoided. These are usually in the form of specific virtual AtoN i.e.as per Message 21 but can also be used to represent a line, area, position or other forms that may be displayed graphically in the form of ASM.

Virtual AtoN may be used as temporary deployments, where there is a time-critical consideration, and also in a permanent context, where permanent physical AtoN cannot be established or maintained. These two applications of virtual AtoN, temporary and permanent, should be reflected in Maritime Safety Information (MSI). Competent authorities should use all available means to ensure that mariners have the necessary information concerning the presence and purpose of virtual AtoN and notify their national hydrographic offices for inclusion and updates in nautical publications, including charts.

The potential benefits of virtual AtoN include enhancing safety and environmental protection, timely notification, quick deployment and modification, ease of presentation and low cost in comparison to physical AtoN.

# technical deployment

For a virtual AtoN using Message 21, the MMSI number represents the unique identity of the AtoN itself rather than the transmitting source.

The repeat indicator field is used to identify whether the signal is transmitted from another station. It allows an AIS base station to repeat the message type 21 of another entity which may extend the coverage of a less powerful mobile station. Competent authorities should consider broadcasting a Virtual AtoN from more than one AIS base station to ensure redundancy.

The nominal report rate of virtual AtoN broadcasts is specified by ITU. However, due to the limitations in data link capacity, a more flexible approach is recommended, taking data link capacity and power consumption at transmitting stations into account. Virtual AtoN transmitting sites should deliver a specified minimum signal strength at the user antenna within a specified service area.

It may be advisable to use virtual AtoN to identify temporarily restricted areas in order to prevent intrusion. If the area is marked with several virtual AtoN, the name for each object should be the name together with a serial number. It should be noted that the display of the user terminal may be limited, and the AtoN name may be too long to be fully displayed, so important information such as the serial number should be prioritised at the front.

The number of virtualAtoN and/or their reporting interval,i.e. refresh rate, that can be provided within a local area may be limited due to the capacity of the communication link.  There is a limit to the number of virtual AtoN that can be in the same area due to available timeslots in the AIS system, and multiple virtual AtoN could increase clutter on the user display. To mark areas, AIS ASM may be used, rather than multiple virtual AtoN.

# use cases for vaton

Potential use cases are listed in the table below.

Application of Virtual AtoN

| Application Mode | Function | Conditions | Suggested Type of Virtual AIS AtoN | Consideration |
| --- | --- | --- | --- | --- |
| Permanent Marking | Marking of shoals and reefs, fairway and its limits | Virtual AtoN can be effectively utilized where it is difficult to place or maintain a physical AtoN due to meteorological, topographical or hydrographical conditions | Isolated Danger Marks, Cardinal Marks,and Lateral Marks | Conditions could be sea state such as crashing waves and submerged reef, strong current, water depth, strong ice movement, poor holding ground, etc |
| Permanent Marking | Marking of Fairway | - Virtual AtoN can be effectively utilized where it enhances vessel traffic flow patterns. - Potential applications are to mark the centre of each traffic lane or the separation between two lanes of a TSS and other usual routes. | Safe Water Marks and Lateral Marks | The efficiency of vessel traffic flow patterns is the goal of this application |
| Permanent Marking | Marking of Fairway | Virtual AtoN can be effectively utilized in approaches to a harbour entrance where a ship changes its course and where it is difficult to install a physical AtoN | Safe Water Marks and Lateral Marks |  |
| Permanent Marking | Marking of Fairway | Virtual AtoN can be effectively utilized where it is difficult to deploy or maintain a physical AtoN due to vessel to vessel or vessel to AtoN interaction | Lateral and Cardinal Marks | Existing AtoN with history of impacts/contacts and with no repositioning options are good candidates |
| Permanent Marking | Marking Reference Points | - Virtual AtoN can be effectively utilized where there is a need to verify if a potential AIS position offset exists on shipborne navigational displays using schematic reference point patterns - In general to be considered in confined waters where high accuracy and performance of shipborne equipment is required | Reference Points | Predefined accurate coordinates as well as distance and bearing between virtual marks need to be used and made available through publications |
| Permanent Marking | Marking of Special Area, e.g., anchorage area, area to be avoided | Virtual AtoN can be effectively utilized where precaution or special caution required | Special Marks | A clear marking of special areas will improve safety of navigation |
| Temporary or Permanent Marking | Marking of Special Area | Virtual AtoN can be effectively utilized where it enhances the protection of the environment and/or the protection of species | Special Marks | - Risk of damaging the seafloor of a sensitive area with a physical AtoN might be a consideration - Timely speed reduction zones or restricted areas for protecting mammals or other species |
| Temporary Marking | Marking of a Navigational Restricted Areas,time critical only) | Virtual AtoN can be effectively utilized when navigation restriction is required due to military operations, marine accidents or when marking a wreck or offshore operations | Cardinal Marks, Emergency Wreck Marks, Isolated Danger Mark & Special Marks | - Need to be monitored/updated - Timely deployment is required where it is difficult to have the proper resource available for a physical AtoN deployment - The dynamic nature of the event, i.e. short duration or having to relocate an AtoN, requires the use of a Virtual AtoN |
| Temporary Marking | Designation of temporary fairway (time critical only) | Virtual AtoN can be effectively utilized for indication of fairways when a large scale disaster hits the area. | Lateral Marks & Safe Water Marks |  |
| Temporary Marking | Marking of AtoNs that are Malfunctioning or Off Position | Virtual AtoN can be effectively utilized when a physical AtoN has lost its ability to perform regular functions | Same as the physical AtoN, or Isolated Danger Marks | Timely deployment required |
| Temporary Marking | Pilot Boarding | Virtual AtoN will be useful in marking a pilot boarding station where the position is dependent on sea/ice conditions |  | Timely deployment required |

THE AIDS TO NAVIGATION REPORT (MESSAGE 21)

# Broadcast Information

All AIS AtoN must include a Message 21 as detailed in ITU-R M.1371.  The message configuration will determine the type of AIS AtoN,physical, synthetic or virtual, as well as the AtoN specifications.

The Message 21 broadcast includes the following information.

Maritime Mobile Service Identity (MMSI) number

Type of AtoN

AtoN name

Position of the AtoN (Latitude and Longitude)

Type of position fixing device

Position accuracy indicator

On/Off position status

Virtual AtoN flag,whether Physical or Virtual AtoN

Status of AtoN systems

Time stamp

Dimension of the AtoN and reference positions

# MMSI numbers for ais aton

All AIS AtoN Stations should have a radio licence.

They must include a MMSI number in its own messages. The MMSI is a unique identifier issued by the appropriate national MMSI issuing authority.

All AIS AtoN MMSI numbers have the format 99 followed by a three-digit Maritime Identification Digit (MID) followed by a four-digit unique identifier. The MID identifies the country that issues the VHF licence for the AIS AtoN station. The four-digit unique identifier starts with 1 (99MID1XXX) for physical and synthetic AtoN stations; 6 (99MID6XXX) for virtual AtoN stations, and 8 (99MID8XXX) for Mobile AtoN.

AtoN and MMSI

| Type of AIS AtoN or AtoN | MMSI format ITU-R M..585-8 | Virtual AtoN flag |
| --- | --- | --- |
| Physical | 99MID1XXX | 0 |
| Synthetic | 99MID1XXX | 0 |
| Virtual | 99MID6XXX | 1 |
| Mobile AtoN | 99MID8XXX | 1 or 0 |

The competent authority should be aware that the number of MMSIs available for use is a finite resource.

# type of marine aidS to navigation (AtoN)

The types of AtoN listed below are based on R1001 The IALA Maritime Buoyage System, where applicable.

The nature and type of AtoN can be indicated with 32 different codes:

Codes for type of AtoN

|  | Code | Definition |
| --- | --- | --- |
|  | 0 | Default, Type of AtoN not specified |
|  | 1 | Reference point |
|  | 2 | RACON or Mobile AtoN |
|  | 3 | Fixed structure, such as oil platforms, wind farms Note: This code should identify an obstruction that is fitted with an AIS AtoN. |
|  | 4 | Emergency Wreck Marking Buoy |
| Fixed AtoN | 5 | Light, without sectors |
|  | 6 | Light, with sectors |
|  | 7 | Leading Light Front |
|  | 8 | Leading Light Rear |
|  | 9 | Beacon, Cardinal N |
|  | 10 | Beacon, Cardinal E |
|  | 11 | Beacon, Cardinal S |
|  | 12 | Beacon, Cardinal W |
|  | 13 | Beacon, Port hand |
|  | 14 | Beacon, Starboard hand |
|  | 15 | Beacon, Preferred Channel port hand |
|  | 16 | Beacon, Preferred Channel starboard hand |
|  | 17 | Beacon, Isolated danger |
|  | 18 | Beacon, Safe water |
|  | 19 | Beacon, Special mark |
| Floating AtoN | 20 | Cardinal Mark N |
|  | 21 | Cardinal Mark E |
|  | 22 | Cardinal Mark S |
|  | 23 | Cardinal Mark W |
|  | 24 | Port hand Mark |
|  | 25 | Starboard hand Mark |
|  | 26 | Preferred Channel Port hand |
|  | 27 | Preferred Channel Starboard hand |
|  | 28 | Isolated danger |
|  | 29 | Safe Water |
|  | 30 | Special Mark |
|  | 31 | Light Vessel/ LANBY/ Rigs |

# name of aton

Message 21 has two name fields, the main field with 20 characters and the extended field  with14 characters, allowing for a total name length of 34 characters including spaces. It is important to consider that not all shipborne navigational equipment may display the extended field.

Given the lack of uniformity worldwide concerning naming, some guiding principles will enable more consistency. Some of the important elements to consider are:

The name should help to identify the location and function of the AtoN

Using a short name will prevent cluttering the shipborne display when users are displaying the name tag

Recognized international or national abbreviations or acronyms might help reduce the length

Use of numbering and lettering that respect R1001 The IALA Maritime Buoyage System

Avoid repeating of the information already available in other fields of the Message 21 and/or nautical publications, fixed, floating, MMSI, virtual, colour, etc.

# type of electronic position fixing device

For fixed AtoN and virtual AtoN the surveyed position should be used. The accurate position enhances its function as a radar reference target.

Floating AIS AtoN Station should transmit its current position as given by GNSS.

# position monitoring for floating aton

The position derived from GNSS can be used in conjunction with the reference, or chartedposition and a “guard zone” to monitor the position of floating AtoN.  When the GNSS position of the AtoN is outside guard zone parameters, indicating that the position threshold has been exceeded, an Off position -alarm sets the off-position indicator bit (flag) in message 21. If it is not properly set, it will either never alarm or alarm constantly.

Guard zone parameters for a moored floating AtoN  need to be determined and entered into the physical AIS AtoN unit configuration settings. Assuming no movement of its anchor, the area within  a floating AtoN will always be found is determined by the movement permitted by its mooring length, the minimum depth of water and errors inherent in the positioning method used including accuracy of the fixing method when the AtoN was deployed. When these factors are added together, they determine the area within which a floating AtoN should remain. A tolerance factor may be added to reduce the likelihood triggering an Off position- alarm.

More information on a floating AtoN swinging radius calculation can be found in the Guideline G1066 The Design of Floating Aid to Navigation Moorings.

Further information on position source and accuracy can be found in ITU-R M1371-5.

# physical or virtual aton

Where no physical AtoN exists, the virtual AtoN flag is set to one (1). Table 4 shows the relationship between the virtual AtoN flag and MMSI format each type of AIS AtoN.

aton dimension/reference for position

This field should indicate the size of the AtoN object itself and not the dimensions of the area in which a floating AtoN can move,guard zone or dimensions of a dangerous zone around the AtoN.

Fixed AtoN:

- A numeric value should be used as noted in the table below. The orientations established by the dimensions A, B, C and D should face true north, south, west and east respectively. For example setting A and C to zero, the reference point becomes the north-west corner.

Floating AtoN:

Larger than 2m x 2m. The dimensions of the AtoN should always be given as a circle, i.e. the dimensions should always be as follows: A=B=C=D>1. This reflects the fact that an orientation of the floating AtoN is not transmitted.

Smaller than or equal to 2m x 2m. The fields should be set to A=B=C=D=1.

Offshore structures that are not fixed, Table 4, Code 31  and fixed offshore structures,Table 4, Code 3:

- Dimension/reference for position parameter as determined below. Hence, all offshore AtoN and structures have the dimension determined in the same manner and the actual dimensions are contained in message 21.

Virtual AtoN:

- The dimension should be set to A=B=C=D=0 (=default). This should also be the case, when type of AtoN is set to reference point.

Dimension/reference for position, for a fixed AtoN

| Dimension | Numeric (m) |
| --- | --- |
| A | 0 - 511 |
| B | 0 - 511 |
| C | 0 - 63 |
| D | 0 - 63 |

Dimensions for a floating AtoN and both fixed and floating offshore structures

| A+B ≤ 2m | A=B=C=D=1 |
| --- | --- |
| A+B > 2m | A=B=C=D>1 |

# ATON STATUS BITS

AIS message type 21 provides for eight status bits that are intended to provide information on the operating status of the AtoN and/or its components. This includes light on/off/inoperative, Racon installed/operational/inoperative, and direction of movement (Course Over Ground) of a Mobile AtoN.  The format is shown below:

Recommended use of status bits (light, racon and health)

| AtoN light, RACON and health status 8 bits format: 111 XX XX X |  |  |  |
| --- | --- | --- | --- |
| Page id (8th, 7th and 6th bit) | RACON Status (5th and 4th bit) | Light Status (3rd and 2nd bit) | Health Status (1st bit) |
| 111 | 00 = No RACON installed | 00 = No light or no monitoring | 0 = Good Health |
|  | 01 = RACON installed but not monitored | 01 = Light ON | 1 = Alarm |
|  | 10 = RACON operational | 10 = Light OFF |  |
|  | 11 = RACON Error | 11 = Light fail or at reduced range |  |

NOTES

- Manufacturer’s default setting for the eight AtoN status bits of message type 21 should be all zeros.

Health status:The 1st bit is used for alerting that there is a problem at the AIS AtoN station. Health flag alarm should be set to 1 to indicate a fault in or failure of the AtoN system or AIS AtoN station, at this location.

Main Light status:For the main light, a fail is a situation where:

The light is off when it should be on.

The flash character is incorrect,e.g. an optic drive failure.

The main light fail may be set if the main light is operating at a reduced range, e.g. running on emergency lower range lanterns.

Racon status:For the Racon, a fail is a situation where the Racon unit signals a failure from an on-board built-in integrity test (BIIT). It may also signify a power failure for the Racon. If the Racon error flag is set, the health flag alarm should be set to 1.

# reporting intervals for AIS Aton messages

The reporting interval for message 21 should be chosen so that an approaching vessel receives the AtoN information message within an appropriate timeframe between entering into range of the AIS AtoN broadcast and reaching the AIS AtoN location.

Factors to take into account are:

- Vessel speed of approach

- Topology, for examplevessels approaching from around a headland

- Nominal transmission range

- VDL loading

PLANNING AND CONSIDERATIONS FOR AIS ATON

This Annex gives guidance on the selection, installation and maintenance of AIS AtoN equipment.

# selection of equipment

When selecting AIS AtoN equipment, the following points should be considered:

Life cycle cost analysis

Reliability

Size of unit compared to available space

Simplicity to configure and interrogate

Ease to upgrade if necessary

After-sales service and support

Ease of installation

Connectivity requirements, e.g. high-integrity external connections

Power consumption

# Installation

## Power Requirement

There needs to be sufficient power available to operate the system, noting the power consumption of co-located AtoN such as lights etc. to deliver the required autonomy. The power consumption of the overall configuration should be measured, rather than relied on the manufacturer’s generic data.

The power requirement of an AIS AtoN station is dependent on a number of factors which are usually available for setting via the unit configuration method. These are:

VDL access method: FATDMA will give substantially lower power drain than RATDMA

FATDMA slot selection:AIS1 and AIS2 slots should be close together in time to minimize the period for which processes in the AIS AtoN unit are active

Reporting interval: an extended reporting interval will reduce power drain, but the interval should satisfy the relevant guidance

Configuration of the AIS AtoN unit: the AIS AtoN unit could be designed or configured to enter into a sleep-mode when not active

Repetition of the AIS AtoN messages by a local AIS shore station, during the reporting interval of the AIS AtoN stationmay allow the reporting interval of the AIS AtoN unit to be extended reducing the power requirement. For example, the AIS AtoN may have a 10 minute reporting interval, but the local AIS shore station repeats the AIS AtoN message every frame, i.e. every minute. Consideration should be given to the coverage areas of the AIS AtoN unit and the shore station to ensure that operational requirements are met. An advantage of repeating from an AIS shore station may be to increase the coverage area of the AIS AtoN Station.

## Antennae

### VHF antenna

It is normal practice to use a dedicated VHF dipole antenna located as high as practically possible, whilst noting the physical vulnerability that results.

### GNSS antenna

When planning the installation of a GNSS antenna, it is a priority that it be clear of any vertical obstruction at all times and position as far as practical from any potential interference source.

### Choice of Location for Antennae

In the case of floating AtoN, installing VHF and GNSS antennae at the uppermost section of a superstructure may be a disadvantage from the structural point of view, as they are vulnerable to damage if the buoy is struck by a passing ship or when the buoy is being recovered for maintenance.

## Transmission range

The transmission range is a function of the transmitter power, antennae height and antenna gain. In areas of very heavy traffic, the volume of AIS transmissions may overload a receiver reducing the effective range.

In certain regions and specific environmental conditions, super-refraction or ducting of the VHF signal can occur, significantly enhancing the VHF coverage at times. This particular phenomenon should not be relied on to provide enhanced coverage.

## Selection of VHF antenna

The selection of VHF antenna should be specific for the AIS frequencies,161.975 MHz and 162.025 MHz, and be suitably designed for the maritime environment.

## Ingress Protection (IP) Rating

Electronic equipment installed on an AtoN will be subject to severe environmental exposure.

To safeguard the integrity of the equipment, increase its lifespan and ensure its reliability, the installation must prevent the condensation cycle from starting.  A sufficient IP rating of not less than IP56 should be specified.

## Lightning Protection

Surge protection to guard the equipment against an atmospheric discharge is essential in any installation of electronic equipment.

Lightning protection can take the form of diode-based surge protectors, varistors, gas discharge units and good grounding.

## Grounding

Good equipotential bonding between the AtoN superstructure, mounted equipment and sea water will provide some protection against raised electrical potential during a lightning storm, and will also prevent static build up.

## AIS Monitoring for AtoN units

AIS can be used for monitoring AtoN lantern and racon health information via message 6, and position via message 21.

AIS AtoN can be supplied as an integrated unit within lanterns or as a standalone unit, in which case a connection to the lantern is necessary for monitoring. It should be noted that when equipment has been integrated by the supplier there is a lower risk of failure to the internal connections, external antenna connection, etc. User implemented AIS equipment runs a higher risk of failing.

# commissioning and testing

## Configuration

AIS AtoN units require programming with the following parameters as a minimum:

MMSI

Name and type of Navigation Aid

Charted position

Guard ring (off-position alarm)

For type 1 AtoN: Transmission interval, Slot allocation

AIS function (Physical, Virtual or Synthetic)

Dimensions of the AtoN

Confirmation of all on site programmed information needs to be recorded with the competent Authority.

## Testing

Prior to deployment, the AIS AtoN should be set-up for transmission testing at an approved location and its performance monitored remotely. For floating AtoN testing of the off-position alarm should also be part of the testing sequence by moving it beyond its guard ring. When testing the AIS AtoN that is not in its assigned position, agreement must be gained with the licensing authority to avoid confusion to local shipping. After deployment, a functional test should be conducted including checks of the transmitted position and range of reception.

### After deployment, a functional test should be conducted including checks of the transmitted position and range of reception.

# maintenance and operation

## Maintenance Requirements

It is essential to maintain attention to detail when programming AIS AtoN as this is the most common point of failure.

Technical staff are responsible for verifying:

AIS AtoN operation

Watertight integrity of glands and connectors

VHF and GNSS antenna cables

Non-intrusive inspection of the condition of VHF antenna, of its flexibility and of the GNSS antenna if external

Condition of wiring and connectors, stiffness, cracks, moisture and oxidation

Power supply

Mounting arrangements

Transmission range and coverage measurement

The technician needs to be well acquainted with the configuration software. Good configuration management methodology should be in place to ensure accurate tracking of configuration of the AIS AtoN and the embedded software version.

## Training

The technical staff associated with the maintenance and operation of AIS AtoN should have specific training to enable them to work safely and competently on the AIS AtoN.

The skills required to maintain and programme AIS AtoN units at sea are at a higher level than thoset required to maintain basic AtoN. Since AIS AtoN maintenance and implementation on AtoN is not a routine task, their performance should be assessed and monitored periodically.

As with all training, manuals need to be plain, concise, reader-friendly and readily available to the maintainer.

## Spares

It is necessary to have spare transponders and antennae to replace those that fail, which are damaged by passing ships, vandalized or out of service. The best way to maintain the service is to replace a malfunctioning transponder onsite and to repair offsite if appropriate.