# INTERNATIONAL									30.06.2026

# ITU

# Report on ITU-R WP5B meeting 19 to 28 May 2026

Note by the IALA representative Stefan Bober

#### INTRODUCTION

ITU-R Working Party 5B (WP 5B) - Maritime mobile service including Global Maritime Distress and Safety System (GMDSS); aeronautical mobile service and radiodetermination service - held its meetings from 18th to 28th May 2026 as physical meeting in Geneva. This was the fifth meeting in the study cycle 2023 to 2027. Mr. Stefan Bober represented IALA.

IALA has a specific interest in the maritime mobile service including the Global Maritime Distress and Safety System (GMDSS) and the radiodetermination service, with particular emphasis on the development of VHF Data Exchange System (VDES), Automatic Identification System (AIS), Autonomous Maritime Radio Devices (AMRD), VHF digital voice, VDES R-Mode and e-Navigation.

The meeting of the WP5B-3 maritime focussed on the revision of ITU Recommendations and the two ITU study questions, namely study questions on “VHF digital voice” and on “VDES R-Mode”. WP5B-3 maritime has no direct agenda item to prepare for WRC-27. However, WP5B-3 maritime is involved in several other agenda item at WRC-27, e.g. WRC-27 agenda item 1.12, current and future use of the 1645.5 -1646.5 MHz band.

#### issues related to IALA work addressed during ITU-R WP 5B

##### Revision of Recommendation ITU-R M.1371-5 Automatic Identification System - AIS

The revision of Recommendation ITU-R M.1371-5 was finalized at last WP 5B meeting in November 2025 and was subsequently adopted by SG5. The Recommendation ITU-R M.1371-6 was published on the ITU website in February 2026.

However, an error was identified by IALA in the document concerning the specification of Message 28 - Aid-to-Navigation Report (Single-slot message).   The indication of which Type of aids-to-navigation applies to Mobile AtoN was missing in Table 85 - Type of aids-to-navigation. This error was corrected during the meeting by the ITU bureau.

The specification of Message 28 including Mobile AtoN is largely based on input from the delegation of USA and IALA. To ensure consistent implementation, it is recommended to pursue a harmonized approach in the application of this message.

##### New Report ITU-R M. [DIGITAL VOICE] – Assessment of the future use of VHF maritime mobile band for digital radio voice communication

##### WP 5B continued to draft a new report on the considerations of VHF maritime mobile bands for digital radio voice communications. The working document was renamed and restructured to allow a technical neutral approach of the report. The report evaluates candidate digital technologies for improving frequency efficiency and expanding channel capacity. Furthermore, it describes the technical and operational criteria that allows digital technologies to coexist alongside legacy analogue voice and other system (AIS, DSC, VDE, etc.,) channels without interfering with vital safety-of-life functions. The report is structured in four annexes, A1: Technical and operational characteristics of a VHF digital radio voice communication system, A2: A comparative analysis of candidate VHF digital radio voice communication, A3: Operational, migration and coexistence considerations in the VHF maritime frequency band; A4: Current trends and use in the band 156–174 MHz.  	 
The current version of the new report includes dPMR and DMR as candidate technologies for VHF digital voice.	
->	IALA is invited to contribute to the development of this report.

##### New Recommendation ITU-R M. [DIGITAL VOICE]

WP 5B continued working on a new recommendation on digital voice communication in the VHF maritime mobile band. The current version of the new report includes dPMR and DMR as candidate technologies for VHF digital voice.	
Noting the need for further considerations and additional studies on different technologies that could be used for the digitalization of VHF voice communication, the draft new Recommendation ITU-R M.[DIGITAL_VOICE] is retained as a working document.

##### New Report ITU-R M. [VDES R-MODE] - Impact of the possible introduction of a range mode on the VHF data exchange system

WP 5B continued work on a new report on the impact of the possible introduction of a range mode on the VHF data exchange system. The aim of this report is to describe the impact of R-Mode VDES.
Further information on VDES R-mode trials and experiments were provided by input documents. Although the draft report is nearly completed, additional information on VDES R-mode trials and experiments is expected to be submitted to next WP 5B meeting

->	IALA is invited to contribute to the development of this report.

##### Revisions of Recommendation ITU-R M.826 Transmission of information for updating electronic chart display and information systems (ECDIS)

WP 5B started work on the revision of Revision to recommendation ITU-R M.826 Transmission of information for updating electronic chart display and information systems (ECDIS). This Recommendation was published in 1992 to address the “need to provide radio transmissions of digital data for use by ships to automatically update and make corrections to their electronic charts”, and proposes several technical solutions to meet this requirement. However, the document has since been overtaken by new technical developments in maritime radiocommunication technology.

Noting the different views expressed for either suppressing or revising this Recommendation, it is agreed to seek comments from other concerned international organizations. A liaison statement to IMO, IHO, IALA, CIRM was drafted requesting whether Recommendation ITU-R M.826-0 should be revised or suppressed, to ensure regulatory clarity and remove unnecessary duplication across organizations.

->	IALA is invited to consider the issue and take action as appropriate.

##### Recommendation ITU-R M.489-2 - Technical characteristics of VHF radiotelephone equipment operating in the maritime mobile service in channels spaced by 25 kHz

WP 5B started the revision of revision of ITU-R M.489-2 - Technical characteristics of VHF radiotelephone equipment operating in the maritime mobile service in channels spaced by 25 kHz.  The main proposal is to introduce a blocking ratio from unwanted signals to protect GMDSS receivers from VDES transmissions. After consideration, the WG 5B-3 was not able to reach a consensus and agreed to further consider this proposal in the next WP5B meeting.

##### Revisions of Recommendation ITU-R M.2135-1 Autonomous maritime radio devices (AMRD)

WP 5B started the revision on the Recommendation ITU-R M.2135-1 Autonomous maritime radio devices (AMRD). The group noted the opinion of ETSI that this recommendation should be amended to raise the EIRP of group B AMRD from 100 mW to 1 W and to allow the use of Channel 2006 also for non AIS technology transmissions using polite channel access schemes, such as Listen Before Take (LBT). Further studies are encouraged on this topic.

WP 5B incorporated the proposed modifications to the Recommendation, including  the consequential changes resulting from the revision of Recommendation ITU-R M.585-8, which specifies the supplementary 12-character identity format for devices using freeform maritime identities, as well as the introduction of the MOB “off” and “cancel” indications in the text in message 14. The visual indication of open-loop or closed-loop operation remains unresolved.

##### Revisions of Report ITU-R M.2231-1 Use of Appendix 18 to the Radio Regulations for the maritime mobile service

WP 5B continued the revision of the Report ITU-R M.2231-1 to update the information on the use of Appendix 18, incorporating the changes adopted at recent WRCs, the revisions to the “Maritime frequency arrangements” and a new section entitled “A brief history of VHF maritime”.

##### Revisions of Recommendation ITU-R M.1080 (Digital selective calling system DSC)

WP5B finalised the revision of Recommendation ITU-R M.1080 “Digital selective calling system enhancement for multiple equipment installations”.

The draft revision of Recommendation ITU-R M.1080 was submitted to SG5 for approval.

##### Revision of Recommendation ITU-R M.541-11 (Operational procedures for DSC)

WP 5B continued the revision of Recommendation ITU-R M.541-11 - Operational procedures for the use of digital selective calling equipment in the maritime mobile service - to further clarify international callings through DSC and the specific frequency bands used by ACS in line with the latest Radio Regulations.

#### RELATED DOCUMENTS

- WD towards new report on the impact of the possible introduction of a R-Mode on the VDES

- WD towards new report on Assessment of the future use of VHF maritime mobile band for digital radio voice communication

- Revisions of Recommendation ITU-R M.826 Transmission of information for updating electronic chart display and information systems (ECDIS)

- Liaison statement to IALA on Request for inputs for the revision of Recommendation ITU-R M.826-0 - transmission of information for updating electronic chart display and information systems (ECDIS)

#### IALA IS REQUESTED TO

- IALA is requested to note the report on ITU-R WP5B meeting and to act accordingly.