IALA Guideline

ENG9-8.6.1

DRAFT Intersessional work ENG8/ENG9

Application of Retroreflecting Material on Marine Aids to Navigation

Edition 1.0

Document date

Revisions to this IALA Document are to be noted in the table prior to the issue of a revised document.

| Date | Page / Section Revised | Requirement for Revision |
| --- | --- | --- |
| month/year approved by Council | aaaaa | aaaaaa |

1	INTRODUCTION	6

2	AIMS AND OBJECTIVES	6

3	HISTORY	6

4	RETROREFLECTIVE MATERIAL FOR MARINE AIDS TO NAVIGATION PURPOSES	7

4.1	FLOATING MARINE AIDS TO NAVIGATION	8

4.2	FIXED MARINE AIDS TO NAVIGATION	8

5	TERMINOLOGY, PRINCIPLE, BASIC TECHNOLOGY AND DESIGN	9

5.1	REFLECTION AND RETROREFLECTION	9

5.2	OPTICAL PRINCIPLES	10

5.3	THE GLASS SPHERE OR BEAD	11

5.4	PRISMATIC OR CUBE CORNER REFLECTOR	11

5.5	COMPARISON	12

6	CHARACTERISING AND MEASUREMENT OF RETROREFLECTING MATERIAL	13

6.1	COLOUR	16

7	CLASSIFICATION AND STANDARDS OF RETROREFLECTIVE MATERIAL	17

8	CONSIDERATION WHEN APPLYING RETROREFLECTIVE MATERIAL	18

8.1	GENERAL CONSIDERATIONS FOR FLOATING AND FIXED MARINE AIDS TO NAVIGATION	18

8.1.1	COLOUR	18

8.1.2	PHOTOMETRIC PERFORMANCE WHEN WET	20

8.1.3	TEMPERATURE	20

8.1.4	FLEXIBILITY	20

8.1.5	CLEANABILITY	20

8.1.6	APPLICATION OF RETROREFLECTIVE MATERIAL	20

8.1.7	DEGRADATION OF RETROREFLECTIVE MATERIALS	20

8.1.8	PRECAUTION WHEN USING RADAR REFLECTOR IN COMBINATION WITH RETROREFLECTING MATERIALS	21

8.2	FLOATING MARINE AIDS TO NAVIGATION	21

8.2.1	SMALL BUOYS	21

8.2.2	LARGE BUOYS	22

8.3	FIXED MARINE AIDS TO NAVIGATION	26

8.4	CALCULATION OF SIZE OF RETROREFLECTORS FOR FIXED ATON	27

8.4.1	Example of calculation size of retroreflector in a leading line	29

9	BEST PRACTICE AND TYPICAL APPLICATIONS	30

10	DEFINITIONS	34

11	ACRONYMS	34

12	REFERENCES	35

List of Tables

Table 1	Matrix  RAα,β1	15

Table 2	Matrix RAα,β1 with colour details	15

Table 3	An example of data sheet for retroreflective film (Coefficient of Retroreflectivity)	16

Table 4	An example of data sheet for retroreflective film (Chromaticity and Luminance factor)	17

Table 5	Data Sheet of retroreflective film used in the case story	29

List of Figures

Figure 1	Cat's "glowing eyes" (https://en.wikipedia.org/wiki/Retroreflector)	7

Figure 2	Cat's eye for a bicycle (photo DMA)	7

Figure 3	Illuminated retroreflected ID number on a day board (DMA Greenland)	8

Figure 4	Reflective and Retroreflective (DMA)	9

Figure 5	Illustration of the cone of retroreflected light (CIE)	9

Figure 6	Retroreflection compared with other reflection patterns (DMA)	10

Figure 7	Glass Bead Retroreflectors (DMA)	10

Figure 8	Prismatic Retroreflectors (DMA)	10

Figure 9	Glass Sphere or Glass Bead (DMA)	11

Figure 10	Principle of Prismatic/ Cube Corner Reflector (https://en.wikipedia.org/wiki/Corner_reflector)	12

Figure 11	Prismatic Tape Overhead View (ifloortape.com)	12

Figure 12	Dispersion path from Glass Bead and Prismatic Retroreflectors (DMA)	12

Figure 13	Coefficient of Retroreflection (Ra)	13

Figure 14	CIE Goniometer system for specifying and measuring retroreflectors (CIE)	14

Figure 15	Spar buoy with green retroreflective band (DMA)	18

Figure 16	South Cardinal, Comprehensive Code, double blue band and single yellow band (DMA)	19

Figure 17	Movement pattern of a buoy (spar buoy)	21

Figure 18	Band on the full circumference on small buoys	21

Figure 19	Retroreflector applied at every 90 Degrees	22

Figure 20	Retroreflector applied at every 120 Degree	23

Figure 21	Retroreflector applied at every 180 Degrees	23

Figure 22	Buoys shape compared with the viewing angle	24

Figure 23	Spar buoy healing approx. 45 degree caused by wind and current	24

Figure 24	Eye height compared for a small and a large vessel	25

Figure 25	Searchlight Path and Observing Path	25

Figure 26	Retroreflectivity depending on the curvature shape of the buoy	26

Figure 27	Large day board of a leading line (DMA Greenland)	26

Figure 28	Quantities of Allard's Law	27

Figure 29	Quantities of Retroreflectivity	28

Figure 30	Leading line with retroreflectors (DMA)	29

Figure 31	Green lateral light buoy with horizontal retroreflective sheets	30

Figure 32	Red lateral spar buoy with horizontal retroreflective band on full circumference of the buoy	31

Figure 33	Retroreflective band on a North Cardinal and a West Cardinal buoys (Comprehensive Code)	31

Figure 34	South Cardinal with vertical retroreflective sheets (Comprehensive code)	32

Figure 35	Safe Water light buoy with horizontal retroreflective sheets (Comprehensive Code)	32

Figure 36	Example of a protective arrangement of retroreflective material (SeaHow and Sabik)	32

Figure 37	Offshore green lateral mark with horizontal retroreflective band on full circumference	33

Figure 38	Front day board with partly attached yellow retroreflective sheeting for night time use	33

Figure 39	Pier head attached with yellow retroreflective material	34

List of Equations

Equation 1	Coefficient of luminous intensity of a retroreflector	13

Equation 2	Coefficient of retroreflection of a plane retroreflective surface	14

Equation 3	Allard’s law (illuminance as a function of distance, luminous intensity and the meteorological visibility)	27

Equation 4	Calculation of incidence illuminance (E) at the retroreflector	28

Equation 5	Calculation of the luminous intensity of the retroreflector	28

Equation 6	Calculation of the size of a retroreflector	29

# INTRODUCTION

Retroreflective material has in general widespread use in many types of applications, including safe driving, safe navigation, safety of workers, industrial/technical automatization and for many other purposes.

The most commonly and well-known use is in enhancing the light returned from vehicle headlights to enable the drivers to detect warnings, impart information and aid the driver through the necessary manoeuvres. For this reason, road traffic signs and vehicles in general are equipped with retroreflective material.

Other applications are high-visibility clothing for workers, authority personnel, etc. to be safely and effectively visible and further for equipment for life rafts/boats and life vests according to SOLAS regulations.

Retroreflective material on marine aids to navigation (AtoN) has also significance. Retroreflective material has a widespread use in narrow and complicated waters and fairways for the marking at night time when a marine signal light is not suitable due to cost or size. However, retroreflective material has also importance on lighted fairways in case of the failure of a light.

The IALA Recommendation R0106 (E- 106) [1] describes the nautical codes for use of retroreflective material on floating AtoN according to the IALA Maritime Buoyage System (MBS) [5] .

A specific guideline on retroreflective material for marine AtoN is currently not available. Therefore, IALA provides information in this guideline

This guideline will give more detailed information on what retroreflectivity is and how retroreflectivity is achieved.

# AIMS AND OBJECTIVES

The guideline provides a general informative overview for AtoN authorities and suppliers of retroreflective material, including terminology, design, characterising, classification and guidance on its use for AtoN purposes.

# HISTORY

The earliest recorded information of spherical shapes made of glass dates back to early Chinese history. Observation of light reflected back towards the light source from these spheres was used to intensify the brightness or brilliance of art objects. Glass spheres of different coloured glasses can be found in ancient jewellery. Many of the cuts of the facets of diamonds are made to retroreflect the incident light, resulting in the brilliance of the diamond.

The first use of retroreflective material for purpose of enhancing the conspicuity of objects was probably created in Great Britain around the turn of the century (1800/1900). Observation of light reflected from animal’s eyes, especially those of cats, started the thought of using this technique to enhance the brightness of objects.

The explanation of the glowing nature of cat’s eyes stems from the highly reflective nature of their retinas. These animals were primarily nocturnal and relied on making the most use of the available low light level. The tissue (tapetum lucidum) immediately behind the retina is coated with material which reflects over 80 % of the incident light, and reflects visible light back through the retina, increasing the light available to the photoreceptors and then increase the detectability of objects.

Cat's "glowing eyes" (https://en.wikipedia.org/wiki/Retroreflector)

The first use of the “cat eye” principle was for the safety of drivers in road traffic application, therefore investigations were made resulting in the production of glass spheres with retroreflective features.

Cat's eye for a bicycle (photo DMA)

Retroreflective material is available today in arrays in different sizes and colours, sheeting of plastic and metal, adhesive tape, etc.  Numerous manufacturers for various applications provide many different colours and degrees of reflection intensity.

As with any retroreflector, sheeting glows brightly when there is a small angle between the observer's eye and the light source directed toward the sheeting but appears non-reflective when viewed from other directions.

# RETROREFLECTIVE MATERIAL FOR MARINE AIDS TO NAVIGATION PURPOSES

For floating AtoN retroreflective materials should be used with the code in according to the IALA R0106 (E-106) [1]

Further there are many applications for fixed AtoN to navigation, e.g.:

- Day boards for leading lines

- Beacons and lighthouse structures

- Signs for waterways

- Lettering and numbering

- Marking of obstacles and piers

## FLOATING MARINE AIDS TO NAVIGATION

Retroreflective material is usually used as an alternative to lights on buoys on less important fairways and fairways for leisure crafts; however, they also have importance on lighted fairways in case of the failure of a light.

The shape of the buoy provides mainly the daymark according to the IALA MBS [5]. The retroreflective material has a lower conspicuity at daytime compared to an ordinary or fluorescent surface colour. In addition, retroreflective material is much more expensive than a traditional surface colour and cannot be fixed to some curved surfaces. Therefore, it is not recommended to use retroreflective material for the whole shape of a buoy. Instead it should be applied as small sheets to the buoys and it should not interfere with the daymark. As a result, retroreflective material is for short range navigation (less than 1 NM).

The nautical codes of retroreflective material on floating AtoN is described and specified by IALA Recommendation R0106 (E-106), Use of Retro-reflective Materials on Aids to Navigation Marks within the IALA Maritime Buoyage System.

IALA Recommendation R0106 (E-106) [1] deals with two types of codes for floating AtoN – the Standard Code and the Comprehensive code.

The Standard Code provides only the identification of lateral and special marks. Safe water, cardinal and isolated danger marks have a white retroreflective sheet, which provides detection of the buoy only.

In the Comprehensive Code all types of marks of IALA MBS are individually translated into a code for the retroreflective material. As black cannot be translated to retroreflective material, the colour blue therefor replaces black.

## FIXED MARINE AIDS TO NAVIGATION

Retroreflective material is used for different types of fixed AtoN to navigation and for purposes related to safe navigation or warning of obstructions close to navigational water and fairways.  Applications where retroreflective materials are used can be for day boards of leading lines, lettering and numbering, signs for waterways, marking of edges, jetties, piers, etc.

In contrast to the application on buoys, the retroreflective material may be used for both the daymark and the marking at night. This means that the day board is completely covered with retroreflective material.

Illuminated retroreflected ID number on a day board (DMA Greenland)

# TERMINOLOGY, PRINCIPLE, BASIC TECHNOLOGY AND DESIGN

The Commission Internationale De L’éclairage/International Commission on Illumination (CIE), the international standardizing body responsible for light and colour measurement, defines retroreflection as follows:

“Reflection in which the reflected rays are preferentially returned in direction close to the opposite of the direction of the incident rays, this property being maintained over wide variation of direction of the incident rays”

Reflective and Retroreflective (DMA)

Illustration of the cone of retroreflected light (CIE)

## REFLECTION AND RETROREFLECTION

When looking at a reflecting surface’, the image may be complete blurred, as in surface with diffuse reflection, or it may be sharp, as in surface with direct/mirror reflection, see figure 6.

Most practical surfaces have mixed reflection, which is a mixture of diffuse reflection and mirror reflection in the surface, mostly with the surface reflection partly blurred by texture.

A particular kind of reflection is retroreflection, where most of the light is reflected back to the observer.

Retroreflection compared with other reflection patterns (DMA)

## OPTICAL PRINCIPLES

There are two basic optical systems for obtaining retroreflection

•	The Glass Sphere or Bead, see figure 7.

•	The Prismatic or Cube Corner Reflector, see figure 8.

Glass Bead Retroreflectors (DMA)

Prismatic Retroreflectors (DMA)

## THE GLASS SPHERE OR BEAD

The simplest retroreflector is a glass sphere or bead and this type was the first retroreflective element used for the so-called “Cat’s Eye”.

The incident light will be refracted at the front surface of the sphere and is focused towards the centre of the rear surface of the sphere. From there the light is reflected and refracted at the front surface again onto a course parallel to the direction from which it came.

To improve the reflectivity the rear surface it should have a reflector coating.

Only 28 % of the area of the sphere is at a sufficiently favourable angle to return the light beam. To enhance the reflectivity, the prismatic reflector can be used as an alternative.

Glass Sphere or Glass Bead (DMA)

Depending on the construction, the glass sphere/bead can be divided in three different types in respective order according to improvement

•	Exposed glass bead

•	Enclosed glass bead

•	Encapsulated glass bead.

## PRISMATIC OR CUBE CORNER REFLECTOR

The prismatic or cube-corner is the more efficient reflector.

If you take a solid glass cube and slice off one corner with a cut that passes along the diagonals of three adjoining faces you have the basic cube-corner unit which is a squat triangular pyramid shaped prism. When the pyramid is set up with the triangular base directed towards a beam of light, the light passes through the base with very little loss. Once inside the pyramid the light is internally reflected off each of the three side faces in turn, before being directed back through the base, exiting parallel to the incoming light beam. Reflected light is still parallel to the incoming beam as the light is equally refracted when entering and leaving the prism, see figure 9.

Principle of Prismatic/ Cube Corner Reflector (https://en.wikipedia.org/wiki/Corner_reflector)

Prismatic Tape Overhead View (ifloortape.com)

## COMPARISON

The prismatic retroreflector generally produces a very small cone of reflective light. In situations, where the observer is not exactly in the position of the light source, a broader returning angle may be preferred. Many glass bed sheets are more flexible and can be attached to curve surfaces more easily.

Figure 12 shows the difference in the reflected light between glass bead and prismatic tape.

Dispersion path from Glass Bead and Prismatic Retroreflectors (DMA)

# CHARACTERISING AND MEASUREMENT OF RETROREFLECTING MATERIAL

The retroreflective material is mainly characterised by two properties

- the coefficient of retroreflection RA (CIE 54.2-2001) [6]

- the colour chromaticity co-ordinates x, y and the luminance factor (CIE 1931).

Performance values for retroreflective sheeting are expressed with its efficiency in reflecting the incident light at a particular geometry.

The reflectivity is the ratio between the reflected light and the incident light.

retroreflective surface

incident light

reflected light

from light source 
(e.g. a search light)

towards observer



Coefficient of Retroreflection (Ra)

The incident light is described by the illuminance E⊥ the light source produces at the surface.

The reflected light is described by the luminous intensity I emitted towards the direction to the observer.

The ratio R between them is called the ‘coefficient of luminous intensity of a retroreflector’ [6].

Coefficient of luminous intensity of a retroreflector

R=reflected lightincident light=IE⊥

Unit:		 cdlx

This ratio still depends on the size A of the retroreflective sheet. To get a specification for the sheet independent of its size, the ‘coefficient of retroreflection of a plane retroreflective surface’ is defined as [6]:

Coefficient of retroreflection of a plane retroreflective surface

RA=RA=IE⊥*A

Unit:		 cdlx*m2

Remark: The IEC Vocabulary uses the symbol R'. The relevant specification use RA.

The important concept to understand is that the retroreflectivity value RA is a ratio. It’s similar to “percent” and does not tell how bright the sheeting will be on a given sign (even at the distance corresponding to the “geometry” for that particular value).

It is simply its “efficiency” at returning light to the source at that particular geometry (the set of observation and entrance angles).

The efficiency of a retroreflective material varies with different observation and entrance angles and the various materials vary in different ways.

These parameters influence the choice among different retroreflective materials for a particular application.

The following should be taken into account:

- geometry between light, AtoN and observer (angles and distance),

- type of searchlight

- practical viewing conditions and

- the viewing needs of the observer.

The coefficient of retroreflection RA depends on 4 different angles and is determined by a laboratory arrangement shown in figure 14.

CIE Goniometer system for specifying and measuring retroreflectors (CIE)

The ‘observation angle’ α is the angle between the light source and the observer, measured from the retroreflective sheet. In general the light source and the observer are nearly in the same position and the angle α is small. Typical values chosen for technical specification are in the interval [0.1° ... 2.0°].

The ‘entrance angle’ is the angle from the ‘light source’ axis to the ‘retroreflector axis’ and is characterised by two components, β1 and β2.

The retroreflection may depend on the rotation angle ε of the retroreflective surface around its perpendicular axis.

In consequence the coefficient of retroreflection is a set of numbers RA=RAα, β1,β2,ε. The total sets of coefficients are very difficult to handle in practice. To simplify the specifications the angles β1 and α are set to zero, which means that the influence of these angles is ignored.

With this simplification the coefficient of retroreflection RA becomes a matrix RAα,β1.

Matrix  RAα,β1

| RA | β11 | β12 | β13 |
| --- | --- | --- | --- |
| α1 | RA1,1 | RA1,2 | RA1,3 |
| α2 | RA2,1 | RA2,2 | RA2,3 |
| α3 | RA3,1 | RA3,2 | RA3,3 |

Where RA (i, j) is the coefficient for the angles α (i), β1 (j) with i, j = 1, 2, 3.Because each colour has its own coefficient RA the published matrices are rearranged.

Matrix RAα,β1 with colour details

| Geometry |  | Colour |  |  |
| --- | --- | --- | --- | --- |
| α | β1 | White | Yellow | ... |
| α1 | β11 | RA, White1,1 | RA,Yellow1,1 | RA,…1,1 |
|  | β12 | RA, White1,2 | RA, Yellow1,2 | RA, …1,2 |
|  | β13 | RA,White1,3 | RA,Yellow1,3 | RA,…1,3 |
| α2 | β11 | RA,White2,1 | RA,Yellow2,1 | RA,…2,1 |
|  | β12 | RA,White2,2 | RA,Yellow2,2 | RA,…2,2 |
|  | β13 | RA,White2,3 | RA,Yellow2,3 | RA,…2,3 |
| α3 | β11 | RA,White3,1 | RA,Yellow3,1 | RA,…3,1 |
|  | β12 | RA,White3,2 | RA,Yellow3,2 | RA,…3,2 |
|  | β13 | RA,White3,3 | RA,Yellow3,3 | RA,…3,3 |

Where RA,colour(i, j) is the coefficient for a specific colour and the angles α(i), β1(j) with i, j = 1, 2, 3.

Most of the available retroreflective material is designed for road traffic signals and commonly, the applied ‘observation angle’ is for this purpose. In practice, the angle is given by the vertical distance between the head light (light source) of the car and the driver’s position (observer) in the vehicle.

Traditionally the specifications of the coefficient of retroreflection are based on the illuminant A (2856 K), which is nearly the colour of the incandescent lamps of automobile head light. However, vehicles and marine searchlights xenon arc and LEDs with high colour temperature (> 5000 K) are in use, which might give different values in practice.

The table 3 below shows a typical datasheet values from a manufacture of retroreflective material.

The table shows at left the geometry of the measurements. Three sets of the ‘observer angle α’ and three sets of the ‘entrance angle β1’.

The actual retroreflective material is available in six different colours and the value of Ra [cd/ (lx*m2)] respectively of each colour is displayed under different geometry.

An example of data sheet for retroreflective film (Coefficient of Retroreflectivity)

## COLOUR

Retroreflective material is typically available in signal colours for white, yellow, red, green, blue and orange.

For buoys, orange may not be used because it is not in the codes of the MBS and may cause confusion with yellow and red.

For fixed AtoN or other marks orange may be used according to the regulations of the assessment of the competent authority.

The surface colour is described by a luminance factor β and two chromaticity co-ordinates x, y (see IALA Recommendation R0108 (E-108) [2]

Measurement of the colour of retroreflective material is carried out by same method as for other surfaces colours and in accordance with IALA Guideline G1134 [3]

The table 4 below shows an example of a specification for colours of a retroreflective material.

The specified chromaticity regions for retroreflective material is often much smaller than for surface colours.

The standard specifications are for daytime observations only. So, for the coefficient of retroreflection illuminant A (2856 K) is used and for the colour illuminant D65.

Retroreflective film in black colour is available. In daytime it appears black and during night time, when illuminated, it will appear white.

A black retroreflective film is recommended for the band of an isolated danger mark.

An example of data sheet for retroreflective film (Chromaticity and Luminance factor)

# CLASSIFICATION AND STANDARDS OF RETROREFLECTIVE MATERIAL

Standardisation of retroreflective material is related to the increase in use of automobiles and historically the first specification on the topic for safe driving was probably formulated in the United States in the 1930s.

The start of the international standardization on retroreflective materials began in Vienna in 1968 with the Convention of Road Signs and Signals by the CIE. Today CIE is the international organization for the standards in light and colour. Many technical reports and documents of the CIE are dealing with light signals, surface colours, illumination and light measurement. These are the basis for international agreements or national specifications.

For retroreflective material the most important international document is the CIE Technical Report 54.2 on Retroreflection, Definition and Measurement [6].

From this document all other international and national specifications are derived. The geometry of measurement differs somewhat among these specifications; however, today’s international trend is to specify the characteristics of retroreflectors in terms of road traffic.

Nearly all technical specifications on retroreflective material and the material themselves are for road traffic only.

Important standards on retroreflective materials are:

- European Standard EN 12899-1 (Fixed, vertical road traffic signs)

- US American Standard ASTM D 4956-04 Standard Specification for Reflective Sheeting for Traffic Control

- German Standard DIN 67520 (Retro-reflecting materials for traffic safety)

Frank: For ENG9 we should try to give information about the preferred classes of retroreflective material. Maybe we recommend two types and name the relevant classes of EN,  ASTM and DIN.

Two types: one for buoys / one for fixed AtoNs and some special buoys

European Standard EN 12899-1(Fixed, vertical road traffic signs):

•	Class RA1

•	Class RA2

DIN 67520 (Retro-reflecting materials for traffic safety):

•	Class RA1

•	Class RA2

•	Class RA3A

•	Class RA3B

Class RA1 refers to the lowest grade of retroreflective material, sometimes known as Engineer Grade.

Class RA3A and class RA3B includes florescent retroreflective material

# CONSIDERATION WHEN APPLYING RETROREFLECTIVE MATERIAL

Applying retroreflective materials for AtoN the type to use must be considered. In general, the application for floating AtoN and fixed AtoN differs. For example, floating AtoN are dynamic objects with limited size and fixed AtoN are static objects that may be very large.

## GENERAL CONSIDERATIONS FOR FLOATING AND FIXED MARINE AIDS TO NAVIGATION

### COLOUR

A retroreflective surface should not be used for daymarks throughout.

The retroreflective principle works only when the surface is near the observer and illuminated at night with a searchlight.

In daylight the retroreflective colours show low luminance and are not very conspicuous.

Manufacturers of retroreflective material have introduced retroreflective-fluorescent films. However, the colours available are made for road traffic signals and there is not a complete set of films for the IALA MBS colours.

Care should be taken that the amount of white retroreflecting material used on an AtoN does not detract from its daytime appearance. For cardinal marks in standard code, black retroreflective film showing a white colour when illuminated in night time.

Spar buoy with green retroreflective band (DMA)

Information regarding the coding of colours for floating AtoN is described in IALA Recommendation R0106 (E-106).

It may be difficult for the observer to discriminate between yellow and white retroreflecting materials, particularly where only one of these colours is being observed on its own. Thus, only one yellow band may be used on a special mark to avoid confusion with a West Cardinal mark in the comprehensive code (two yellow bands).

It may also be difficult for the observer to discriminate between green and blue retroreflecting materials, particularly where only one of these colours is being observed on its own. In principle, green buoys should carry only one green band, whereas blue is always used in combination with another colour, except in the case of East Cardinal marks which have two blue bands according to IALA R0106 (E – 106). However, this principle may be violated if one of the bands has become damaged.

The coefficient of retroreflection of blue and red is much lower than white or yellow, and to ensure proper recognition the following must be observed:

Safe water marks:

The red bands or stripes must be at least twice the width of the white bands or stripes. The separation distance between the colours must be at least twice the width of the white bands or stripes.

North and South Cardinal Marks:

The blue bands must be at least twice the width of the yellow bands. The separation distance must be at least twice the width of the yellow bands.

Isolated Danger Marks:

To ensure proper recognition of isolated danger marks the blue and red bands should be of equal width and separated by a distance at least equal to the width of a band.

When retroreflective material is simply used for detection purposes, use only white material in the form of letters, numbers or symbols.

South Cardinal, Comprehensive Code, double blue band and single yellow band (DMA)

### PHOTOMETRIC PERFORMANCE WHEN WET

The photometric performance of the retroreflective material can change under wet conditions. Particularly a glass bead retroreflector can have decreased performance under wet conditions. Tests have shown that micro prismatic retroreflectors, compared to glass beads, performs better under wet conditions.

### TEMPERATURE

High and low temperatures can cause deformation of the retroreflective material and the deformation can cause the loss of coefficient of retroreflection, cracking, distortion and reduced lifetime.

When specifying retroreflective materials, it is important to select materials with technical temperature specifications at least equal to or better than the local annual temperature variation.

### FLEXIBILITY

Buoys can have curved and sharp edges and it is necessary to select the retroreflective materials that can be bent as necessary.

If the flexibility is not adequate for the actual use, the retroreflective material will crack and there will be a loss in retroreflectivity.

### CLEANABILITY

To obtain high performance retroreflectivity and long lifetime of the material, it is important that the retroreflective material has good self-cleaning characteristics.

Gloss and uniform surfaces are recommended.

### APPLICATION OF RETROREFLECTIVE MATERIAL

Proper application techniques are very important to obtain optimal retroreflective performance and to ensure long lifetime of the retroreflective material.  The manufacture’s instruction for applying retroreflective material must be followed. A data sheet of the product will have detailed information.

Read all health hazard, precautionary and first aid statements before handling the material.

Frank: We should give some hints on how to attach the retroreflective sheets to the structure (problems with plastic, overlap when the sheet is bend to a cylinder, use of chamfer (DE: Fase) or rounding

### DEGRADATION OF RETROREFLECTIVE MATERIALS

The deterioration rate or service lifetime of retroreflective material depends of many factors and their influence is type dependent.

Plastic cube-corner prismatic materials are the most common type applied as retroreflective material at AtoN to navigation.

The colour may change due to pigment fading and loss of transparency caused by ultra violet (UV) exposure. The material for unbending cube corners may crack allowing moisture to penetrate and destroy the sheet. Due to the harsh environment the retroreflective materials are often exposed to mechanical injury caused by the scratching from the too close passing vessels, ice scratching, change in temperature and humidity, etc.

Microprismatic materials that are used in flexible materials will deform, changing the shape of faces and spreading the returned light, thus lowering the value of retroreflectivity at the same geometry.

### PRECAUTION WHEN USING RADAR REFLECTOR IN COMBINATION WITH RETROREFLECTING MATERIALS

Experiences have shown that retroreflective material consisting of metal folio can cause a lower Radar Cross Section (RCS) value. It is recommended not to apply the retroreflective material on the buoy surface where the surface covering an integrated radar reflector.

## FLOATING MARINE AIDS TO NAVIGATION

Floating AtoN is typically buoys and used for short range navigation. Buoys can have different designs and sizes. The hydrostatic properties of the buoy have great influence on how the buoy performs with the retroreflectors during different sea conditions, including wind, current, waves, etc.

When approaching and passing a buoy the observer sees the buoy from very different positions. In addition, the buoy may move vertically and may heel, see figure 17.

Movement pattern of a buoy (spar buoy)

### SMALL BUOYS

On small buoys and spar buoys, the retroreflective material are commonly applied as a band on the full circumference of the buoy – the retroreflector is therefore omnidirectional, see Figure 18.

Band on the full circumference on small buoys

### LARGE BUOYS

On larger buoys the shape, size and the structure of the buoy do not allow for the use of a band on the full circumference of the body of the buoy. In this case the retroreflective sheets are applied to the buoy body at specific suitable positions on the circumference.

There are three commonly used arrangements shown in figures below.

In each figure, the positions of the retroreflective sheets are shown and in addition the figures also show how they are seen from the observer’s position.

The retroreflective coverage is best at 90°, 120° and 180°, respectively. At the 180° application and with a viewing angle of 90° there is none or minor coverage depending of the lengths of the band and the diameter of the structure where the retroreflective band is fitted.

Retroreflector applied at every 90 Degrees

Retroreflector applied at every 120 Degree

Retroreflector applied at every 180 Degrees

Care should be taken with safe water marks and emergency wreck marking buoys when using a comprehensive code with vertical stripes. In this case an arrangement with an angle of 90° is preferred.

The shape of the buoy has influence on the retroreflectivity. As illustrated at figure 22, the entrance angle for the red buoy is better than for the green buoy and a heeling of the buoy due to wind and current can have excessive influence of the retroreflectivity. Figure 23 shows a spar buoy with an excessive heeling of approx. 45° and compared to data sheets of commonly used retroreflective material, the reduction in retroreflectivity is significant.

Buoys shape compared with the viewing angle

Spar buoy healing approx. 45 degree caused by wind and current

Movement of the buoy and movement of the observing vessel, including the eye height of the observer, should also be considered when selecting and specifying retroreflecting material for floating AtoN, see figure 24.

Eye height compared for a small and a large vessel

The ‘observation angle’ α is the angle between the light source and the observer, measured from the retroreflective sheet. In general the light source and the observer are nearly in the same position and the angle α is small and typical in the interval [0.1° ... 2.0°]. As show at figure 25 short range navigation combined with a searchlight installed rather below to the observation path can cause such an observation angle that it has crucial influence on the retroreflectivity, see Table 3

Searchlight Path and Observing Path

The curvature of the buoy has influence on the retroreflectivity as illustrated at figure 26. A searchlight illuminating two buoy body structures and lengths of the dotted lines present the retroreflected intensity. In the left illustration, the curvature of the buoy causes a lower retroreflectivity than the flat structure of the buoy illustrated to the right. The reason is that the entrance angle of the light is increased and a large entrance angle causes less retroreflectivity. In fact, a curved retroreflective band can have a very small ‘active’ area and consequently a very limited retroreflectivity. The reflectivity of a spar buoy, with a limited diameter, can be increased with a broader retroreflective band (vertical) causing a larger ‘active’ area in the observer’s direction.

For floating AtoN, it is preferable to choose retroreflective material with large entrance angle (β).

Retroreflectivity depending on the curvature shape of the buoy

## FIXED MARINE AIDS TO NAVIGATION

Fixed AtoNs are static and their use is closely comparable with the use of retroreflective materials for road signs and markings for traffic.

Therefore, the use of retroreflective material and estimation of range can be more detailed and effectively compared to floating AtoN with continual movements and continual observer geometry.

Fixed AtoN with retroreflector are commonly for longer distance navigation compared to retroreflector applied on floating AtoN.

Day boards in leading line can be of a size of several square meters.

Large day board of a leading line (DMA Greenland)

## CALCULATION OF SIZE OF RETROREFLECTORS FOR FIXED ATON

For the calculation of the approximately size of a retroreflector, the following quantities must be known:

- the intensity of the searchlight, Is [cd]

- the distance between the searchlight and the retroreflector, d [m]

- the retroreflectivity (datasheet from supplier), Ra [cd/lx/m2]

- the meteorological visibility, V [m]

Allard’s law allows the calculation of the illuminance as a function of distance, luminous intensity and the meteorological visibility [9]

Allard’s law (illuminance as a function of distance, luminous intensity and the meteorological visibility)

Ed=I0.05dVd2

Where:

E	is the illuminance at the eye of the observer [lx]

I	is the luminous intensity [cd]

d	is the distance between the light and the observer [m]

V	is the meteorological visibility (V=V0,05), see IALA dictionary

Quantities of Allard's Law

Quantities of Retroreflectivity

Calculation of incidence illuminance (E) at the retroreflector

E=Is0.05dVd2

Where:

E	is the illuminance at the eye of the observer [lx]

Is	is the luminous intensity of the searchlight [cd]

d	is the distance between the light and the observer [m]

V	is the meteorological visibility (V=V0,05), see IALA dictionary

Calculation of the luminous intensity of the retroreflector

Ir=Ra*E*A

Where:

Ir	is the luminous intensity of retroreflector [cd]

Ra 	is the retroreflectivity of the reflector, [cd/lx/m2]

E	is the incident illuminance at the retroreflector surface, [lx]

A	is the size of the retroreflector, [m2]

Calculation of the size of a retroreflector

A=IrRa*E

The luminous intensity, Ir must be equal to or greater than the user distance for the retroreflective material to be visible. The value derives from a table converting range in nautical miles to luminous intensity (cd).

### Example of calculation size of retroreflector in a leading line

Leading line with retroreflectors (DMA)

Data Sheet of retroreflective film used in the case story

Case Story Example:

A day board of a leading line is fitted with white retroreflective film. The retroreflective film conforms to EN 12899-1:2007 for class RA 2, and has a coefficient of retroreflection of 250 cd/ (lx*m2) covering a viewing angle β of +5°. Due to the distance vessel/retroreflector and the site of the observer on the bridge just below the searchlight, the observation angle α is selected to be 0.2°

The maximum user distance is one NM (1,852 metre) if a clear night with a visibility of 10 NM (18,520 meter), with no background lighting and taking into consideration that and inbound vessels have usually a searchlight with a luminous intensity of 100,000 cd.

Calculate the minimum size of the retroreflector in m2.

1. Calculate the incident illumination at retroreflector:

E=Is0.05dVd2

E=100,0000.0518521852018522

E=0,021608 lx

2. Calculate the minimum size of the retroreflector:

A=IrRa*E

A=1250*0,021608

(The observation distance in the example is 1 M and 1 M is equal to 1 candela, therefore I = 1 cd)

A=0,185 m2

Equivalent to a retroreflective board of 0.43 x 0.43 meter!

# BEST PRACTICE AND TYPICAL APPLICATIONS

In this chapter, diverse pictures show best practice methods on the application of retroreflective materials.

Green lateral light buoy with horizontal retroreflective sheets

Red lateral spar buoy with horizontal retroreflective band on full circumference of the buoy

Retroreflective band on a North Cardinal and a West Cardinal buoys (Comprehensive Code)

South Cardinal with vertical retroreflective sheets (Comprehensive code)

Safe Water light buoy with horizontal retroreflective sheets (Comprehensive Code)

Example of a protective arrangement of retroreflective material (SeaHow and Sabik)

Offshore green lateral mark with horizontal retroreflective band on full circumference

Front day board with partly attached yellow retroreflective sheeting for night time use

Pier head attached with yellow retroreflective material

# DEFINITIONS

The definitions of terms used in this IALA Guideline can be found in the International Dictionary of Marine Aids to Navigation (IALA Dictionary) at http://www.iala-aism.org/wiki/dictionary and were checked as correct at the time of going to print.  Where conflict arises, the IALA Dictionary should be considered as the authoritative source of definitions used in IALA documents.

# ACRONYMS

ASTM	American Society for Testing and Materials (today named ASTM International)

AtoN	Marine Aids to Navigation

CIE	COMMISION INTERNATIONALE DE L’ECLAIRAGE/ INTERNATIONAL COMMISSION ON ILLUMINATION

DIN	Deutsches Institut für Normung (German Institute of Standardization)

DMA	Danish Maritime Authority

EN	European Standards (translation from French/German as European Norms)

IMO	International Maritime Organization (Acronym style)

MBS	IALA Maritime Buoyage System

Ra	Coefficient of Retroreflection

SOLAS	The International Convention for Safety of Life at Sea

UV	Ultra Violet (light) (10 – 380 nm)

# REFERENCES

IALA Recommendation R0106(E – 106), Use of Retro-reflective Materials on Aids to Navigation Marks within the IALA Maritime Buoyage System

IALA Recommendation R0108(E-108) The Surface Colours used as Visual Signals on Aids to Navigation

IALA Guideline G1134 Surface Colours used as Visual Signals on AtoN

IALA Guideline 1094, Daymarks for Aids to Navigation

IALA Maritime Buoyage System (MBS)

CIE 54.2-2001 Retroreflection: Definition and Measurement, ISBN 978 3 900734 99 2, CIE (Commission Internationale De L’éclairage/ International Commission on Illumination, www.cie.co.at)

Handbook of Applied Photometry, Casmir DeCusitio

Test methods for the coefficient of retroreflection of microprismatic sheeting materials, Kai Sørensen, DELTA, April 2004

IALA E-200-2(or new guideline on calculation)

IEC 60050 International Electortechnical Vocabulary, IEV ref 845-04-96, Symbol R

IEC 60050 International Electortechnical Vocabulary, IEV ref 845-04-97, Symbol R'