ANM1 / 10 / 1.2

VTS17 / output / 01

AIS Binary Message Priorities

The Operations and Technical Working Groups of the VTS Committee reviewed VTS17/11/1, as requested by the AIS Committee.

A number of changes were identified to the table at Annex 1 to better reflect the requirements for transmission of the AIS Binary Messages between VTS/Shore stations and ships. In addition, the table at Annex 2 was reviewed and amendments made in respect of those matters related to VTS, as requested by the AIS Committee.

The AIS Committee’s attention is drawn to the fact that in some cases the “Name of Application” of a Function Identifier Group has been amended to better reflect the type of the information being transmitted

The VTS Committee is requested to approve this paper for submission to the AIS Committee and AtoN Management Committees.

Items added/changed by the VTS Committee are indicated in underlined italics.

Annex 1

Class of Message (security/ safety/ information = se,sa,in)

Occurrence of transmissions/receptions  Very frequent = ≤ 1 Per Minute, Frequent = ≤ 1 Per Hour, Infrequent = ≤ 1 Per Day

### Level of Importance of information = 1 High, 2 Medium, 3 Low or not at all

### Capable of being transmitted by other means e.g. NAVTEX, coast radio station

##### Overall Priority= 1 High, 2 Medium, 3 Low or not at all

| FI | FIG | Name of application | Broadcast/ Addressed | Class | Occurrence of transmissions/ receptions | Level of Importance of information |  |  |  |  | Length of message |  | Overall Priority |  | Capable of being transmitted by other means |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | To VTS Centre |  | Shore Station | Ship |  |  |  |  |  |  |
| 16 | VTS | VTS Targets (targets derived by means other than AIS) | ## A + B | Sa/ In | Very frequent | 2 |  | - | 2 |  | 1- 7 targets transmitted 2- 5 slots |  | 2 |  | Yes |
| 17 | VTS | Ship Waypoints and/ or Voyage plan report Sub row deleted, Column deleted | A + B | Sa/In | Frequent | 2 |  | 3 | 3 |  | 1-14 next waypoints 2-5 slots or text route description 20 characters (120 bits) |  | 2 |  | ## Yes |
| 18 | VTS | Advice of waypoints and/or sailing plan by the VTS | ## A | Sa/In | Frequent | 2 |  | - | 2 |  | 1-12 waypoints or text route description |  | 2 |  | Yes |
|  |  |  | B Sub row deleted | Sa/In | Infrequent | 2 |  | - | 2 |  |  |  | 3 |  | Yes |
| 19 | VTS | Extended ship static and voyage related data | ## A | In | Infrequent | 3 |  | 3 | - |  | 16 bits (1 slot) |  | 3 |  | Yes |
| 201 | Gen VTS deleted | Local navigation warnings Include this under General rather than VTS Specific | A + B | Sa | Very frequent - Frequent | 1 |  | 1 | 1 |  | Length of messages will be kept as short as possible, whilst still conveying all the necessary information required by the recipient. Required length of individual messages will be very variable |  | 1 |  | Yes |
|  |  | Local (temporary) corrections to ENCs | Considered covered under FI number 20 (note, if VTS remains deleted, than the FI number will have to change as this number is reserved fo VTS – Annex 5, ITU-R1371-1 refers)- Local Navigation Warnings |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Incidents requiring adjustment to navigation plans | Considered covered under FI number 20 - Local Navigation Warnings |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 | Gen | Pilot/tugs//berthing plan information including anchorage plans | A + B | Sa/In | Frequent | 2 |  | 2 | 1 |  |  |  | 1 |  | Yes |
| 32 | AtoN | Local AtoN status reports | B | Sa | Frequent - Infrequent | 1 | 1 |  |  | 1 |  | 1 |  | Yes |  |
|  |  |  | A | Sa/In | Frequent | 1 | 1 |  |  | - |  | 1 |  | Yes |  |

Notes:

1.   Local Navigation Warnings may be considered under both VTS and Gen as it may be outside a VTS area.  If outside a VTS area it may be a vessel generating the local navigation warning (eg. Divers operating)

#### Annex 2: Scoping Document –Binary Messages

| ## Variable | Number of Bits | FI |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 16 (VTS targets derived by means other than AIS) | 17 (Ship Waypoints and/or voyage plan) | 18 (Advise Waypoints and/or sailing plan by the VTS) | 19 (Extended Ship data) | 20 (Local navigation Warnings) | 8 (Pilot/tugs//berthing plan information including anchorage plans) | 32 (AtoN Status Report) |
| ### Message ID | 6 | Y | Y | Y | Y | Y | Y | Y |
| User ID (MMSI) | 30 | Y | Y | Y | Y | Y | Y | Y |
| IAI/DAC Code | 10 | Y | Y | Y | Y | Y | Y | Y |
| Repeat indicator | 2 | Y | Y | Y | Y | Y | Y | Y |
| Targets 1-7 | 120 (each) 840 Max | Y |  |  |  |  |  |  |
| Type of Target Identifier | 2 | Y |  |  |  |  |  |  |
| Target ID | 42 | Y |  |  |  |  |  |  |
| Latitude | 24 | Y |  |  |  | Y |  |  |
| Longitude | 25 | Y |  |  |  | Y |  |  |
| COG | 9 | Y |  |  |  |  |  |  |
| Time stamp | 6 | Y | Y | Y | Y | Y | Y | Y |
| ### SOG | 8 | Y |  |  |  |  |  |  |
| Position accuracy | 1 | Y |  |  |  |  |  |  |
| NWP (Number of waypoints) | 4 |  | Y |  |  |  |  |  |
| WP (lat) | 27 |  | Y | Y |  |  |  |  |
| WP (long) | 28 |  | Y | Y |  |  |  |  |
| Route specified (text) | 120 |  | Y (if lat/long not used) |  |  |  |  |  |
| AWP (advised Way Points) | 4 |  |  | Y |  |  |  |  |
| Advised turning radius | 12 |  |  | Y |  |  |  |  |
| Advised Route Text | 120 |  |  | Y (if lat/long not used) |  |  |  |  |
| Height Over Keel | 11 |  |  |  | Y |  |  |  |
| Ship Defects (if possible) |  |  |  |  | Y |  |  |  |
| PEC Number (if possible) |  |  |  |  | Y |  |  |  |
| ISM Code Compliance (if possible) |  |  |  |  | Y |  |  |  |
| Tidal data etc (if possible) |  |  |  |  | Y |  |  |  |
| Nav Warning Type | 12 |  |  |  |  | Y |  |  |
| Nav Warning Message | 120 |  |  |  |  | Y |  |  |
| Pilot Aboard |  |  |  |  |  |  | Y |  |
| Number of Tugs |  |  |  |  |  |  | Y |  |
| Assigned Berth/anchorage |  |  |  |  |  |  | Y |  |
| Time berth/ anchorage/ lock available |  |  |  |  |  |  | Y |  |
| Time of embarkation of pilot |  |  |  |  |  |  | Y |  |
| Time of disembarkation of pilot |  |  |  |  |  |  | Y |  |
| 1AtoN Status Report |  |  |  |  |  | Y |  | Y |
| 1Total No. Bits |  |  |  |  |  |  |  |  |

1 To be completed following consultation with AtoN Management Committee.