ANM1 / 10 / 1

RNAV17 / 11 / 1

AIS11/output/05

AIS Binary Message Priorities

The Operational Working Group of the AIS Committee reviewed the IALA NAVGUIDE Edition 4, Annex 5 of ITU-R M.1371-1, AIS11/2/2, AIS11/5/1, AIS11/6/1 and AIS11/6/2, to determine the most appropriate priority levels of significant importance for safety and information of the VTS and AtoN Binary Messages for VTS purposes and shipping.

It was decided the most appropriate method was to produce a draft paper for consideration using a table of the various Function Identifier Groups and their associated messages. This table, at Annex 1, takes account of the method of transmission of the Messages, the Classification of Message (security, safety, information), percieved frequency of use of the message, level of importance of the message and to whom. To assist in assessing the size of the individual Binary Messages a scoping document was prepared. This is attached at Annex 2 Account was also taken of whether the information can be conveyed by other means.

In addition to this some Function Messages may be either broadcast to all users or addressed to a single user, thus impacting upon the frequency of their use, level of significance and their length. This was therefore also taken into account when preparing the table at Annex 1.

The fact that a message may also be transmitted by other means is also indicated.

Class of Message (security/ safety/ information = se,sa,in)

Frequency of use of the application.  High = ≤ 1 Per Minute, Medium = ≤ 1 Per Hour, Low = ≤ 1 Per Day

Infrequent = ≤ 1 Per 2 days

### Level of Importance of information = 1 High, 2 Medium, 3 Low or not at all

### Capable of being transmitted by other means e.g. NAVTEX, coast radio station

##### Overall Priority= 1 High, 2 Medium, 3 Low or not at all

| FI | FIG | Name of application | Broadcast/ Addressed | Class | World/ Regional/ Local | Frequency of Use |  | Level of Importance of information |  |  | Length of message | Overall Priority | Capable of being transmitted by other means |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | To VTS Centre | Shore Station | Ship |  |  |  |
| 16 | VTS | VTS Targets (targets derived by other means than AIS) | ## B | Sa/ In | W | High |  | 2 | - | 2 | 1- 7 targets transmitted 2- 5 slots | 2 | Yes |
| 17 | VTS | Ship Waypoints and/ or route plan report | A | Sa/In | W | Low |  | 2 | 3 | 3 | 1-14 next waypoints 2-5 slots or text route description 20 characters (120 bits) | 2 | ## Yes |
|  |  |  | ## B | In | R/L | Low/ Infrequent |  | 2 | 3 | 3 |  | 2 | Yes |
| 18 | VTS | Advice of waypoints and/or route plan of theVTS | ## A | Sa/In | W | Medium |  | - | - | 2 | 1-12 waypoints or text route description | 2 | Yes |
|  |  |  | B | Sa/In | W | Infrequent |  | - | - | 3 |  | 3 | Yes |
|  |  |  |  |  | R/L | Infrequent - Medium |  | - | - | 2 |  | 2 |  |
| 19 | VTS | Extended ship static and voyage related data | ## A | In | R/L | Low |  | 3 | 3 | - | 16 bits (1 slot) | 3 | Yes |
| 201 | VTS/Gen | Local navigation warnings | B | Sa | L | Medium |  | 1 | 1 | 1 |  | 1 | Yes |
|  |  | Local (temporary) corrections to ENCs | Considered covered under 20 Local Navigational Warnings |  |  |  |  |  |  |  |  |  |  |
|  |  | Incidents requiring adjustment to navigation plans | Considered covered under 20 Local Navigational Warnings |  |  |  |  |  |  |  |  |  |  |
| 8 | Gen | Pilot/tugs/entry plan/berthing plan information | A | Sa/In | L | Medium |  | 2 | 2 | 2 |  | 2 | Yes |
| 32 | AtoN | Local AtoN status reports | B | Sa | R/L |  | Medium- Low | 1 | 1 | 1 |  | 1 | Yes |
|  |  |  | A | Sa/In | R/L |  | Medium | 1 | 1 | - |  | 1 | Yes |

1 Note: Local Navigation Warnings may be considered under both VTS and Gen as it may be outside a VTS area.  If outside a VTS area it may be a vessel generating the local navigation warning (eg. Divers operating)

#### Annex 2: Scoping Document –Binary Messages

| ## Variable | Number of Bits | FI |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 16 (VTS targets derived by other means than AIS) | 17 (Ship Waypoints and/or route plan) | 18 (Advise Waypoints and/or route plan of the VTS) | 19 (Extended Ship data) | 20 (Local navigation Warnings) | 8 (Pilot/tugs/entry plan/berthing plan information) | 32 (AtoN Status Report) |
| ### Message ID | 6 | Y | Y | Y | Y | Y | Y | Y |
| User ID (MMSI) | 30 | Y | Y | Y | Y | Y | Y | Y |
| IAI/DAC Code | 10 | Y | Y | Y | Y | Y | Y | Y |
| Repeat indicator | 2 | Y | Y | Y | Y | Y | Y | Y |
| Targets 1-7 | 120 (each) 840 Max | Y |  |  |  |  |  |  |
| Type of Target Identifier | 2 | Y |  |  |  |  |  |  |
| Target ID | 42 | Y |  |  |  |  |  |  |
| Latitude | 24 | Y |  |  |  | Y |  |  |
| Longitude | 25 | Y |  |  |  | Y |  |  |
| COG | 9 | Y |  |  |  |  |  |  |
| Time stamp | 6 | Y | Y | Y | Y | Y | Y | Y |
| ### SOG | 8 | Y |  |  |  |  |  |  |
| Position accuracy | 1 | Y |  |  |  |  |  |  |
| NWP (Number of waypoints) | 4 |  | Y |  |  |  |  |  |
| WP (lat) | 27 |  | Y | Y |  |  |  |  |
| WP (long | 28 |  | Y | Y |  |  |  |  |
| Route specified (text) | 120 |  | Y (if lat/long not used) |  |  |  |  |  |
| AWP (advised Way Points) | 4 |  |  | Y |  |  |  |  |
| Advised turning radius | 12 |  |  | Y |  |  |  |  |
| Advised Route Text | 120 |  |  | Y (if lat/long not used) |  |  |  |  |
| Height Over Keel | 11 |  |  |  | Y |  |  |  |
| Nav Warning Type | 12 |  |  |  |  | Y |  |  |
| Nav Warning Message | 120 |  |  |  |  | Y |  |  |
| 1Pilot Aboard |  |  |  |  |  |  | Y |  |
| 1Number of Tugs |  |  |  |  |  |  | Y |  |
| 1Assigned Birth |  |  |  |  |  |  | Y |  |
| 1Aton Status Report |  |  |  |  |  | Y |  |  |
| 1Total No. Bits |  |  |  |  |  |  |  |  |

1 To be completed following consultation with AtoN Management and VTS Committees.