Draft IALA Guidelines on AIS for use as an Aid to Navigation

1. Introduction

As the competent international body for Aids-to-Navigation, IALA defines an Aid-to-Navigation (AtoN) as: "a device or system external to vessels that is designed and operated to enhance safe and efficient navigation of vessels and/or vessel traffic." (IALA Navguide, Edition 4 2002, Chapter 3). It is noted that buoyage systems other than those recommended in IALA Navguide exist in inland waterways.

A special type of an Automatic Identification System (AIS) station fitted to an Aid to Navigation (AtoN AIS) will provide a positive identification of the aid without the need for a specialised shipborne display. In addition, this equipment can provide information and data that could:

- complement or replace an existing aid to navigation, providing identity, state of ‘health’ and other information such as real tidal height and local weather to surrounding ships or back to a shore authority;

- provide the position of floating aids (mainly buoys) by transmitting an accurate position (corrected by DGNSS) to monitor if they are on station;

- provide real-time information for performance monitoring, with the connecting data link serving to control changes in AtoN parameters remotely or switch on back-up equipment;

2. Aim

These Guidelines are intended to inform IALA Members of the ways in which Automatic Identification Systems (AIS) can be used as an Aid to Navigation (AtoN). Technical information is given in a number of annexes to assist with specification of equipment and testing for type approval.

AIS on AtoN may be considered as part of the AIS shore infrastructure, even though the AtoN may be positioned offshore.

3. IMO's Provisions For AIS Shore Infrastructure

IMO's SOLAS Convention, as revised, Regulation 19, §2.4.5., states with regard to the purpose of the AIS:

"AIS shall

- provide automatically to appropriate equipped shore stations, other ships and aircraft information, including ship's identity, type, position, course, speed, navigational status and other safety-related information;

- receive automatically such information from similarly fitted ships;

- monitor and track ships; and

- exchange data with shore-based facilities."

In addition, the IMO Performance Standards for the AIS state:

1.1.	The AIS should improve the safety of navigation by assisting in the efficient navigation of ships, protection of the environment, and operation of Vessel Traffic Services (VTS), by satisfying the following functional requirements:

- in a ship-to-ship mode for collision avoidance;

- as a means for littoral States to obtain information about a ship and its cargo; and

- as a VTS tool, i. e. ship-to-shore (traffic management).

1.2.		The AIS should be capable of providing to ships and to competent authorities, Information from the ship, automatically and with the required accuracy and frequency, to facilitate accurate tracking. Transmission of the data should be with the minimum involvement of ship's personnel and with a high level of availability.

1.3.	The installation, in addition to meeting the requirements of the Radio Regulations, applicable ITU-R Recommendations and the general requirements as set out in resolution A.694(17), should comply with the following performance standards."

From this the provision of IMO for AIS shore infrastructure can be inferred. It should be noted however, that there is neither a stipulation of IMO to any competent authority to implement a VTS nor to implement AIS into existing VTS. However, since IMO stated, that AIS improves the safety of navigation and operation of VTS, competent authorities should consider implementing AIS into VTS.

IMO's provision for AIS shore infrastructure was taken up by ITU-R when creating Recommendation ITU-R M.1371, which included a so-called AIS base station. When drafting a test standard for Class A and Class B shipborne AIS stations, IEC also took the existence and specific role of AIS base and repeater stations into consideration.

4. ITU's Additional Operational Requirements For AIS Shore Infrastructure

As a peer organisation to IMO, ITU recognized the potential of the AIS also for areas of shore-based application, other than ship reporting and VTS, namely maritime, safety-related information services, Aids-to-Navigation and Search and Rescue:

"The ITU Radiocommunication Assembly considering (...)

- that such a system should be used primarily for surveillance and safety of navigation purposes in ship to ship use, ship reporting and vessel traffic services (VTS) applications. It could also be used for other maritime safety related communications, provided that the primary functions were not impaired;

- that such a system would be capable of expansion to accommodate future expansion in the numbers of users and diversification of applications, including vessels which are not subject to IMO AIS carriage requirement, Aids-to-Navigation and Search and Rescue." (Recommendation ITU-R M.1371-1)

5. Aids to Navigation Report (Message 21)

This message should be used by an AtoN AIS station. The message should be transmitted autonomously at a Reporting Rate of once every three (3) minutes or it may be assigned by an Assigned Mode Command (Message 16) via the VHF data link, or by an external command. The message should not occupy more than two slots.

The main contents of the messages are:

- Type of aid to navigation;

- Name of the aid to navigation;

- Position;

- Position accuracy indicator;

- RAIM indicator;

- Type of position fixing device;

- Time stamp;

- Dimension of the aid to navigation and reference positions;

- 8 bits reserved for use by the regional/local authority (can include the technical status of the aid to navigation);

- Pseudo aid to navigation target flag.

The Details of the Composition of message 21 are given in Annex A.

6. Off Station Message

The IALA Navguide stipulates: "A floating aid to navigation, which is out of position, adrift or during the night is unlighted, may itself become a danger to navigation. When a floating aid is out of position or malfunctioning, navigational warnings must be given." Therefore, an AtoN AIS station, which transmits Message 21, could also transmit at the competent authority's discretion Safety Related Broadcast Message (Message 14) upon detecting that the floating Aid-to-Navigation has gone out of position or is malfunctioning,

7. AtoN Types

The nature and type of AtoN can be indicated with 32 different codes, as shown below:

|  | Code | Definition |
| --- | --- | --- |
|  | 0 | Default, Type of A to N not specified |
|  | 1 | Reference point |
|  | 2 | RACON |
|  | 3 | Off Shore Structure |
|  | 4 | Spare |
| Fixed A to N | 5 | Light, without sectors |
|  | 6 | Light, with sectors |
|  | 7 | Leading Light Front |
|  | 8 | Leading Light Rear |
|  | 9 | Beacon, Cardinal N |
|  | 10 | Beacon, Cardinal E |
|  | 11 | Beacon, Cardinal S |
|  | 12 | Beacon, Cardinal W |
|  | 13 | Beacon, Port hand |
|  | 14 | Beacon, Starboard hand |
|  | 15 | Beacon, Preferred Channel port hand |
|  | 16 | Beacon, Preferred Channel starboard hand |
|  | 17 | Beacon, Isolated danger |
|  | 18 | Beacon, Safe water |
|  | 19 | Beacon, Special mark |
| Floating A to N | 20 | Cardinal Mark N |
|  | 21 | Cardinal Mark E |
|  | 22 | Cardinal Mark S |
|  | 23 | Cardinal Mark W |
|  | 24 | Port hand Mark |
|  | 25 | Starboard hand Mark |
|  | 26 | Preferred Channel Port hand |
|  | 27 | Preferred Channel Starboard hand |
|  | 28 | Isolated danger |
|  | 29 | Safe Water |
|  | 30 | Special Mark |
|  | 31 | Light Vessel / LANBY |

Note:

- The types of Aids to Navigation listed above are based on the IALA Maritime Buoyage System, where applicable.

- There is potential for confusion when deciding whether an aid is lighted or unlighted. Competent authorities may wish to use the regional/local section of the message to indicate this.

8. Synthetic-AIS

In some cases, for practical or economic reasons it may not be appropriate to fit an AIS unit to an AtoN and the synthetic-AIS approach may be taken. In a synthetic-AIS, data from an AtoN is transmitted via a non-AIS datalink. The data is received ashore, for example at a VTS Centre, where it is converted into AIS format and re-broadcast as if it came from an AIS unit on the AtoN. The main reason for this approach is the high power requirement of an AIS unit. Many AtoNs are solar powered and cannot support significant extra loads.

9. Pseudo-AIS

An AIS broadcast can also be made from a shore-station for an AtoN that physically exists, but is not sending back data.

10. Virtual AtoNs

In other cases it may be appropriate to create a virtual AtoN on a display, where none exists. In this case the symbol would appear on the display for a certain location, even though there is no physical AtoN there. The associated information would clearly identify this as a virtual AtoN and safeguards would be needed against false representations. There are some cases where virtual AtoNs could be very useful, for example marking a new wreck until a buoy can be established, or marking Areas To Be Avoided.

11. Radar Reference Target

With the ever-increasing requirement for integrated systems, the problems of synchronising two or more systems on one display surface are increased. If two (or preferable three) AIS stations can be installed on prominent fixed radar targets within an area of special interest, such as a harbour or harbour approach, then the AIS GNSS positions, radar echoes and chart symbols of each target can be synchronised. This will result in reduced ambiguity and clutter on the display.

12. Other Functionality

An AtoN AIS station on a fixed aid to navigation such as a lighthouse could, in addition to transmitting its own identification, act as a relay for other aids to navigation. It can also provide local hydrographical and meteorological data, using application specific messages. These messages are currently undergoing development. When available, details will be provided in the IALA Guidelines.

13. Functional Layout of an AtoN AIS

The AtoN AIS needs to perform in accordance with the Recommendation ITU-R M.1371-1 and IALA's Technical Clarifications (latest edition) in order to maintain harmony with the AIS system requirements.

Figure 1 – Function Block Diagram of AtoN AIS

Presentation Interface

Controller

Internal Sync Source

BIIT

Power Supply

AIS TX

AIS RX

AIS RX

(Ext UTC Sync)

Met.data

Hydro data

I/O

AtoN Status

AIS on AtoN

Figure 2 – Block  diagram of a Synthetic AIS

Presentation Interface

Controller

Internal Sync Source

Power Supply

Datalink

Met.data

Hydro data

I/O

AtoN Status

Synthetic AIS AtoN Station

14. General requirements for receivers and transmitters

- The general requirements for receivers and transmitters can be found in Recommendation ITU-R M.1371-1, Annex 1.

- An AtoN AIS station should be capable of either 25 kHz or 12.5 kHz emission / reception depending on location.

- However, an AtoN AIS station need not be bandwidth agile.

15. Configuration means

The AtoN AIS station should provide the following configuration means:

- Configuration of AtoN report;

- Configuration of radio parameters;

- Configuration of alarm and warning thresholds;

- Configuration of reporting interval.

16. Proof of Performance/Certification

AIS equipment for AtoN application will need to be certified as complying with the technical requirements in ITU-R M.1371-1 to ensure compatibility with other AIS equipment and compliance with the Radio Regulations.

There seem to be two possible routes for the type approval of AtoN AIS equipment: an IEC Specification could be prepared, based on the Class A or Class B document, or IALA could provide a test specification derived from the same sources.

The IEC route may be difficult as there is no IMO requirement for AIS on AtoNs. There is nothing to prevent IALA from drawing up a specification and it would be appropriate as the competent body for AtoNs. However, It would have more credibility if it was recognised by the other international bodies.

It is therefore proposed that IALA draw up a test specification and provide it to IMO, ITU and IEC with a liaison statement explaining the reasons for taking this course.

The technical information on which to base these test specs, is provided in annexes B, C & D.

17. References

IMO References

- IMO Resolution MSC.74(69) Annex 3 Recommendation on performance standards for AIS

- Draft Revision of "IMO Resolution A.815(19):1995 Worldwide radionavigation system"; needed for requirements of any DGNSS service provided by AIS stations

ITU references

ITU-R Recommendation M.1371-1, Technical Characteristics for a Universal Shipborne Automatic Identification System using Time Division Multiple Access in the VHF maritime mobile band."

ITU-R Radio Regulations, Appendix S18

IALA References

- IALA Technical Clarifications on Recommendation ITU-R M.1371-1 [Clarifications] Edition 1.2 as of 13 February 2002.

- IALA Interim AIS Guidelines, December 2001.

- Draft IALA Recommendation on AIS shore stations and networking aspects related to the AIS service. Ed. 1.0 September 5, 2002

IEC references

- IEC 61993-2 "Maritime Navigation and Radiocommunication Equipment and Systems – Automatic Identification systems (AIS); Part 2: Class A shipborne equipment of the Universal Automatic Identification system (AIS) – Operational and performance requirements, methods of test and required test results" (2001)

- IEC 62......Future IEC Class B test standard

- 61162-series "Maritime Navigation and Radiocommunication Equipment and Systems"

- 61162-1 "Part 1: Single talker and single listeners", as revised 2001.

- 61162-2 "Part 2: Single talker and multiple listeners, high-speed transmissions", as revised 2001.

- 61162-3 "IEC Future Publication; Part 3: Multiple talker and multiple listeners, high-speed network bus".

Annex A

Aid to Navigation Report Message.

| Parameter | Number of bits | Description |
| --- | --- | --- |
| Message ID | 6 | Identifier for this message 21 |
| Repeat Indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. Refer to § 4.6.1; 0 - 3; default = 0; 3 = do not repeat any more. |
| ID | 30 | MMSI number |
| Type of Aids-to-Navigation | 5 | 0 = not available = default; refer to appropriate definition set up by IALA; refer to Table 34bis. |
| Name of Aids-to-Navigation | 120 | Maximum 20 characters 6 bit ASCII, "@@@@@@@@@@@@@@@@@@@@" = not available = default. The name of the Aid-to-Navigation may be extended by the parameter “Name of Aid-to-Navigation Extension” below. |
| Position accuracy | 1 | 1  high ( 10 m; Differential Mode of e.g. DGNSS receiver) 0  low ( 10 m; Autonomous Mode of e.g. GNSS receiver or of other Electronic Position Fixing Device) ; Default = 0 |
| Longitude | 28 | Longitude in 1/10 000 min of position of Aids-to-Navigation (180 degrees, East  positive, West  negative. 181 degrees (6791AC0 hex) = not available = default) |
| Latitude | 27 | Latitude in 1/10 000 min of Aids-to-Navigation (90 degrees, North  positive, South  negative, 91 degrees (3412140 hex) = not available = default) |
| Dimension/Reference for Position | 30 | Reference point for reported position; also indicates the dimension of Aid-to-Navigation in metres (see Fig. 18 and § 3.3.8.2.3.3), if relevant. (1) |
| Type of Electronic Position Fixing Device | 4 | 0 = Undefined (default); 1 = GPS, 2 = GLONASS, 3 = Combined GPS/GLONASS, 4 = Loran-C, 5 = Chayka, 6 = Integrated Navigation System, 7 = surveyed. For fixed AtoNs and virtual/pseudo AtoNs, the surveyed position should be used. The accurate position enhances its function as a radar reference target. 8 – 15 = not used. |
| Time Stamp | 6 | UTC second when the report was generated by the EPFS (0 –59, or 60 if time stamp is not available, which should also be the default value, or 61 if positioning system is in manual input mode, or 62 if Electronic Position Fixing System operates in estimated (dead reckoning) mode, or 63 if the positioning system is inoperative) |
| Off-Position Indicator | 1 | For floating Aids-to-Navigation, only: 0 = on position; 1 = off position; NOTE – This flag should only be considered valid by receiving station, if the Aid-to-Navigation is a floating aid, and if Time Stamp is equal to or below 59. For floating AtoN the guard zone parameters should be set on installation. |
| Reserved for regional or local application | 8 | Reserved for definition by a competent regional or local authority. Should be set to zero, if not used for any regional or local application. Regional applications should not use zero. |
| RAIM-Flag | 1 | RAIM (Receiver Autonomous Integrity Monitoring) flag of Electronic Position Fixing Device; 0 = RAIM not in use = default; 1 = RAIM in use) |
| Virtual/pseudo AtoN Flag | 1 | 0 = default = real A to N at indicated position; 1 = virtual/pseudo AtoN, does not physically exist, may only be transmitted from an AIS station nearby under the direction of a competent authority. (2) |
| Assigned Mode Flag | 1 | 0 = Station operating in autonomous and continuous mode =default 1 = Station operating in assigned mode |
| Spare | 1 | Spare. Not used. Should be set to zero. |
| Name of Aid-to-Navigation Extension | 0, 6, 12, 18, 24, 30, 36, ... 84 | This parameter of up to 14 additional 6-bit-ASCII characters for a 2-slot message may be combined with the parameter “Name of Aid-to-Navigation” at the end of that parameter, when more than 20 characters are needed for the Name of the Aid-to-Navigation. This parameter should be omitted when no more than 20 characters for the name of the A-to-N are needed in total. Only the required number of characters should be transmitted, i. e. no @-character should be used. |
| Spare | 0, 2, 4, or 6 | Spare. Used only when parameter “Name of Aid-to-Navigation Extension” is used. Should be set to zero. The number of spare bits should be adjusted in order to observe byte boundaries. |
| Number of bits | 272 – 360 | Occupies two slots. |

Footnotes:

The position derived from the internal GNSS receiver can be used in conjunction with a reference position and a ‘guard ring’ to monitor the position of the floating aids to navigation and to generate an ‘off station’ condition. This would be indicated I the AtoN AIS message and also be transmitted as a navigational warning message in the form of a Safety Related text message.

1. 	When using 3.3.8.2.3.3 for Aids-to-Navigation the following should be observed:

  - For fixed Aids-to-Navigation, virtual and pseudo A-to-Ns, and for off-shore structures, the orientation established by the dimension A should point to true north.

  - For floating aids larger than 2 m * 2 m the dimensions of the Aids to Navigation should always be given approximated to a square, i.e. the dimensions should always be as follows A=B=C=D≠0. (This is due to the fact, that the orientation of the floating Aid to Navigation is not transmitted. The reference point for reported position is in the centre of the square.)

  - A=B=C=D=1 should indicate objects (fixed or floating) smaller than or equal to 2m * 2m. (The reference point for reported position is in the centre of the square.)

2. 	When transmitting virtual/pseudo Aids to Navigation information, i.e. the virtual/pseudo Aids to Navigation Target Flag is set to one (1), the dimensions should be set to A=B=C=D=0 (default). This should also be the case, when transmitting „reference point“ information (see Table 34bis).

. message This should be transmitted immediately after any parameter value was changed

Annex B

Technical Specification for AtoN AIS Stations

[This specification is an edited version of the output paper from the IALA Engineering Committee in April 2001]

Two types of equipment are envisaged:

- a standard unit based on the Class A AIS shipborne station that could be used on lighthouses and other facilities where power consumption was not a major concern;

- a low powered version (LP AtoN AIS) for use on AtoN where mains power in not available.

An indicative set of design parameters include:

| Capability | AtoN AIS | LP AtoN AIS |
| --- | --- | --- |
| Desirable transmitting range | Up to 15 n.m. | Up to 15 n.m. |
| Nominal input voltage | 12 volts dc or 100-250 ac | 12 volts dc |
| Desirable power consumption | TBD | TBD |
| Desirable input voltage range | 8 to 32 dc | 8 to 32 dc |
| Enclosure | Rectangular box with mounting flange and meeting IP65 | Rectangular box with mounting flange and meeting IP67 |
| Cable terminations | Push fit connectors to IP 65 | Push fit connectors to IP 67 |
| Operating temperature range | -40 to +55 ºC | -40 to +55 ºC |
| Shock test |  | Equivalent to a racon |
| Sleep mode | Yes | Yes |
| Transmission cycle | Variable between 2s and 15 min | Variable between 2s and 15 min |
| Remote monitoring capability | RS232 input/output for: - equipment status - weather and oceanographic data collection | Digital inputs (voltage free) to cover basic monitoring requirements for: - normal - unreliable - failed - last lamp - low battery voltage |
| Remote control capability | Authority enabled remote switching. Digital output (voltage free) to cover basic control requirements. | Authority enabled remote switching Digital output (voltage free) to cover basic control requirements. |
| Messages | Authority enabled broadcast and addressed message | Authority enabled broadcast and addressed message |

Annex C

Required Parameter Settings for the Physical Layer of the AtoN AIS Station.

The following parameters are required for an AtoN AIS station. The Tables 1 to 3 are derived from Recommendations ITU-R M.1371-1, Annex 2. For the meaning of the symbols and additional information (footnotes) refer to the appropriate section of Recommendation ITU-R M.1371-1, Annex 2.

Table 1: Required parameter settings for an AtoN AIS station (part 1)

| Symbol | Parameter Name | Low Setting | High Setting |
| --- | --- | --- | --- |
| PH.RFR | Regional Frequencies | 156.025 MHz | 162.025 MHz |
| PH.CHS | Channel spacing | 12.5 kHz | 25 kHz |
| PH.AIS1 | AIS 1 (default channel 1) | 161.025 MHz | 162.025 MHz |
| PH.AIS2 | AIS 2 (default channel 2) | 162.025 MHz | 162.05 MHz |
| PH.CHB | Channel bandwidth | Narrow | Narrow |
| PH.BR | Bit rate | 9 600 bps | 9 600 bps |
| PH.TS | Training sequence | 24 bits | 24 bits |
| PH.TST | Transmitter settling time (Transmit power within 20% of final value, Frequency stable to within ± 1.0 kHz of final value) | ≤ 1.0 ms | ≤ 1.0 ms |
| PH.TXP | Transmit output power | 2 Watt | 2 Watt |

In addition, the constants of the physical layer of the AtoN AIS station should comply with the values given in the tables 2 and 3 below.

Table 2: Required Settings of Physical Layer Constants (part 2)

| Symbol | Parameter Name | Value |
| --- | --- | --- |
| PH.DE | Data Encoding | NRZI |
| PH.FEC | Forward error correction | Not Used |
| PH.IL | Interleaving | Not Used |
| PH.BS | Bit Scrambling | Not Used |
| PH.MOD | Modulation | Bandwidth adapted; GMSK/FM |

Table 3: Bandwidth Dependant Parameters of the Physical Layer of the AIS Base Station

| Symbol | Parameter Name | PH.CHB / Narrow | PH.CHB/Wide |
| --- | --- | --- | --- |
| PH.TXBT | Transmit BT-product | 0.3 | 0.4 |
| PH.RXBT | Receive BT-product | 0.3/0.5 | 0.5 |
| PH.MI | Modulation Index | 0.25 | 0.50 |

Requirements and Recommendations for the TDMA Receivers of the AtoN AIS Station

The technical characteristics as specified in table 4 below should apply to the TDMA receivers. The requirements indicated by (*) are compulsory for all AtoN AIS Stations, while all other parameters are recommended as a minimum. They may be subject to further regional requirements.

Table 4: Required and recommended receiver characteristics

| Receiver Parameters | 25kHz channels | 12.5kHz channels |
| --- | --- | --- |
| Sensitivity (*) | 20% PER for – 107 dBm | 20% PER for – 104 dBm |
| Co-channel rejection | -10 – 0 dB | -18 – 0 dB |
| Adjacent channel selectivity | 70 dB | 50 db |
| Spurious response rejection | 70 dB | N/A |
| Intermodulation response rejection and Blocking | 20% PER | N/A |

Annex D

Requirements for BIIT – Built in Integrity Test

The AIS Base station should monitor the following BIIT conditions and should generate the appropriate alarm and warning sentences on the PI (see below).  Some of the following BIIT conditions should be monitored only when the condition given is fulfilled.

#### BIIT Alarm conditions monitored by an AIS base station

| AIS: Tx malfunction |  |
| --- | --- |
| AIS: Antenna VSWR exceeds limit |  |
| AIS: Rx channel 1 malfunction |  |
| AIS: Rx channel 2 malfunction |  |
| AIS: General failure |  |
| AIS: No sensor position in use |  |
| AIS: Out of frame sync [Frame synchronisation failure?] |  |
| AIS: DGNSS input failed | If the base station transmits DGNSS data |
| AIS: DSC Tx Malfunction | If separate TX for DSC in same unit |
| AIS: DSC Rx Malfunction | If DSC in same unit |
| AIS: DSC Antenna VSWR exceeds limit | If separate antenna for DSC implemented |

#### BIIT Warning/Notification conditions monitored by an AIS base station

### AIS: UTC clock lost

AIS: external DGNSS in use

AIS: external GNSS in use

AIS: internal DGNSS in use (beacon)

A base station can use received messages 17 to correct its internal GNSS.

AIS: internal DGNSS in use (message 17)

AIS: internal GNSS in use

AIS: UTC clock ok

[AIS: Synchronisation unstable. How to measure?]