ANM 1 / 5 / 1

AIDS TO NAVIGATION REQUIREMENTS FOR HIGH SPEED CRAFT

Author:	Capt. Harald Andreassen

Head of department

Norwegian National Coastal Administration

Abstract

High-speed craft are used extensively on the Norwegian coasts as a reliable, rapid mean of transportation to link together communities and cities. It is proven that the high-speed craft to some extent should avoid rough weather and sea by using sheltered fairways in the archipelago. In the early beginning it was a tendency to state that the high-speed craft did not impose any new requirements as far as aids to navigation were concerned as better equipment. Training on navigation procedures was said to compensate for a more complicated and rapid decision process to navigate and manoeuvre.

Norway has, however, experienced some accidents with high-speed craft indicating that an increased effort is necessary to improve the overall safety in the operation of such craft. Specially designed and established aids to navigation in narrow fairways used for high-speed craft services are one important element in the process to improve the safety record.

The paper will present and discuss some issues related to improved safety in the operation of high-speed craft based on public hearings to investigate accidents. The Coast Directorate has independently of these investigations, in co-operation with the users and craft operators, conducted studies to improve the integrated aids for navigation systems. The paper will discuss the chain of occurrences that may influence on an accident in the accidental sequence of maloperation, lack of observation and use of aids to navigation in confirmation role to establish the correct position and heading. The paper will describe a new marking schedule for narrow fairways specially developed for high-speed craft.

R�sum�

Les navires � grande vitesse (NVG,) moyen de transport fiable et rapide permettant de relier entre elles communes et villes, sont tr�s utilis�s le long des c�tes de Norv�ge,. Il est prouv� que les NVG doivent �viter de naviguer par gros temps et en haute mer, et utiliser les voies prot�g�es des archipels. Au d�but, on tendait � penser que les NVG ne cr�aient aucun besoin nouveau en mati�re d�aides � la navigation. On disait qu�une formation aux proc�dures de navigation compensait un processus de d�cision compliqu� pour naviguer et man�uvrer.

La Norv�ge a, cependant, connu des accidents impliquant des NGV, ce qui indiquait que des efforts �taient n�cessaires pour am�liorer les conditions de s�curit� dans lesquelles �voluent ces navires. Des aides � la navigation sp�cialement con�ues et install�es dans les chenaux �troits emprunt�s par les services de  NVG constituent un �l�ment important du processus d�am�lioration des statistiques de s�curit�.

On pr�sentera et discutera de quelques probl�mes li�s � l�am�lioration des conditions de s�curit�, bas�s sur la perception qu�a le public des enqu�tes sur les accidents. La Direction des c�tes a, ind�pendamment de ces enqu�tes et en collaboration avec les usagers et les marins, men� des �tudes pour am�liorer les syst�mes int�gr�s d�aides � la

navigation. On pr�sentera aussi la cha�ne d��v�nements pouvant influer sur le processus d�accident r�sultant d�une erreur de navigation, d�un manque d�observation et d�utilisation d�aides � la navigation pour d�terminer correctement la position et le cap. Enfin, on d�crira un nouveau programme de balisage des chenaux �troits sp�cialement mis au point pour les NVG.

HISTORICAL

Different types of ships and their speed have been altered through decades. From sailing ships and slow going steamships, to modern ro/ro- and containerships and high-speed craft which are carrying hundreds of passengers, cars and other cargoes. The requirements to aids to navigation have also been into a development, but not worth mentioning to fast going vessels and high-speed craft. Passengers are transported with high-speed craft all over the world, in the Far East, Australia, and the American continent and in Europe. They are crossing between Sweden and Denmark and also between the south part of Norway and Denmark. On many of these countries and continents coasts are the waters and fairways easily accessible and conventional aids to navigation are therefor sufficient.

In the sheltered waters on the Norwegian coast, particular in the west- and north, the requirement to special aids to navigation for high-speed craft have been current.

In 1991 Norway had the first serious accident with a high-speed craft, the �Sea Cat accident�. The accident demanded two lives and injured 30 persons. After that accident, an initiative to increase the safety navigation in the fairways mainly used by the high-speed crafts was submitted.

The current fairways for such a marking were at the west- and north part of Norway:

> From Bergen to Sogn, from Trondheim to Sula, several places on the coast of Helgeland and from Namsos to R�rvik.

In November 1999 Norway had a fatal accident with a high-speed craft. The high-speed craft �Sleipner� was put in to service at the 25th of August 1999.  Tree months later, the 26th of November the craft run aground on a rock on the west part of Norway with 88 passengers onboard. 16 passengers and crew lost their lives.

That unfortunate accident touched the people, the whole nation and also the maritime authorities. The further work with specially marking for high-speed craft had to bee given high priority.

The 2nd Coastal District Office in Haugesund has been an initiator in that work. They have also taken part in development of new type of lights and marking especially for high-speed craft.

The accident aea

High-speed craft Sleipner

DEVELOPMENTS

Through several years different types of lights, the shaping and placing of these, have been tested and tried out in the many narrow fairways on the Norwegian coast. It was early found that ordinary street lamps could be an important aid to navigation. It was also important that the new type of marking could make it possible for the navigator onboard the high-speed craft to make a visible navigation at day- and night-time, safely and effective.

Ordinary light fittings were mounted on wooden poles or steel masts. These were placed on rocks, skerries and points at the sea edge, to illuminate the shore contour. The designation of this type of lights in Norway is indirect light or point light.

Later on has specialised installations with suitable lights, daymark with list of lights number and practical construction, have been designed. It is also decided to mount a light beacon with lateral markings on the top of light armature.

A standard for this type of light has been worked out.

To day we have more than 100 such aids to navigation for high-speed craft.

The 4th of September 2001 was the new and latest �light channel� opened between Haugesund - Fe�y � R�v�r.

This project which are closely integrated between the users, high-speed craft industry, the local harbour authority and the Norwegian National Coastal Administration, are a result of better navigation marks and increased safety for the passengers in that area. This new development within point light makes that �light channel� very exceptional.

The island society R�rv�r and Karm�y- and Haugesund municipality has shown great gratitude to the region office for all their engagement in the project. The crew on the maintenance vessel M/V Grinna established 31 �point lights�. The total costs of this project are 4,5 million NKr, of which 2,5 million NKr represents material costs and 2 million are work costs.

Channel for high-speed craft from Haugesund � R�v�r � F�yno (Utsira and Torvestad)

FINANCING

The Aids to Navigation Requirements for high-speed craft have been given high priority at the authority level and have been handled in official papers. And especially mentioned in the plan -  �Norwegian National Transport Plan -  NTP�. The plan gives high priority to the �light channel� and the approaches to ports.

The recommended strategy:		2002 � 2005				2005 � 2011

New establish:				36,25 million per year (4,5 mill. �.)  	35,8 million per. year

Renewal, service and maintenance

(RSM)					204 mill. Per year (25,5 mill. �.)	203 mil. Per year.

Requirement

Requirement toRSM to day:		260 million per year.

Requirement to RSM

after new establishy (10%):		+ 36 million per year.

The project is also detailed handled in the Norwegian National Coastal Administration Long Term-Plan (LTP).

ACTION PLAN / LONG TERM-PLAN

20022003200420052005 - 20111st coastal district 2,52,52,32,730 million 2nd coastal district 12,512,512,512,550 million3rd coastal district 6,256,256,256,2540 million4th coastal district8,758,758,758,7545 million5th coastal district3,753,753,953,5535 millionSpare2,52,52,52,515 million

SPECIFICATION

* Nautical requirements

In connection with the preparation of the specification following points was important:

* Light range

* Day marking

* Placing

* Colour of the light beacon (lateral)

* Numbering

* Reflection material

* Radar reflector

* Racon

* Fixed light on the light beacon

* Availability

* Placing of the point light

* Colour.

* Building and construction requirements

* Dimension on the pole

* Dimension on fittings

* Material

* Foundation wall

* Pole or mast stay

* Accessibility / ladder

* Placing a shore

* Placing at sea

* Placing in narrow and deep area on dykdalbe, bridge foundation

* Painting

* Electromechanical requirements

* Lamp / spotlight

* Light fittings

* Lantern

* Daylight switch

* Power supply

* Fuse

* Earth lead

* Power cabinet

* Cable

* Energy supply: Wind generator, battery, and solar panel

* Remote control.

It was also very important to define the requirements for �health � environment � and safety�, channel standard and evaluation of new channels.

A new type of marking

The construction  seen from below and the back

Installation of a new mark

The construction seen from below and the back.

VISION AND TASKS

The Norwegian National Coastal Administration has visions and aims to improve and further development of the system.  One of the aims is to use the system on different places on the cost also used by other vessels. The under mentioned points are important in the further work with aids to navigation for high-speed craft.

* One Standard

* IALA Recommendation

* Remote Control of the light (consumption of energy).

* Notice transmission to electronic chart system.

* References:

Recommendation for the rhythmic characters of lights on aids to navigation.

E. Short Range Aid System Configuration and Design,

Chapter 3 Establishment, Review, and Modification of Coast Guard Aids to Navigation Systems, US Coast Guard Aids to Navigation Manual.

Chapter 5 Dayboards, U.S. Coast Guard Aids to Navigation Technical.

ANM 1 / 5 / 1 page 7