## General Design Considerations.

### Until recently, mariners have used the radio aids to navigation systems and the short range systems in two distinct geographic areas.  The short range system was needed and used close to shore and in restricted waterways.  An offshore vessel was able to use the less accurate radio aids system.  However, with the low cost, high precision GPS/DGPS receivers and electronic charting programs available today, these two areas are beginning to overlap.  This is especially evident in those transitional areas where the mariner shifts from the low accuracy requirements of ocean navigation to the high accuracy needs of coastal and inshore piloting.  As electronic aids continue to improve, their use will increase in areas where previously only short range aids afforded the necessary accuracy.  This evolutionary change must be recognized and accounted for when conducting waterway analyses.

### User Interface.

#### Meeting and riding with the users of a waterway is one of the most important steps in waterway analysis and should be done prior to any decision to establish or modify an aid system.  Viewing the waterway from the user's perspective is critical.  Glaring deficiencies and redundancies may be apparent from a user's viewpoint that is not readily apparent from the viewpoint of the Coast Guard servicing unit.

#### The "wants" of the users must be carefully considered and evaluated by the waterway analyst, and then translated into user "needs".

### Shoreside Property.

#### Before establishing an aid, an easement or purchase must be obtained to ensure subsequent access.

#### Restrictions should be placed on the future use of property disposed of as excess to the needs of the Coast Guard if necessary to preserve the arc of visibility of an aid or the minimum distance at which it should be seen.  Restrictions may also be needed if a fog signal's sound pressure levels might be offensive to prospective occupants.

### Conditions.

#### The physical and environmental condition of a waterway, as well as the size of the vessels using the waterway, must be considered.  The design of the waterway's aid system should deal with, but not necessarily be driven by, the worst conditions the largest vessel might encounter.

#### Vessel traffic data can be obtained from several sources.  The local pilots and/or harbormaster may be able to provide it, or the data may be obtained from a local VTS.  The Army Corps of Engineers Waterborne Commerce of the United States Reports contain transit, tonnage, and commodity information.  The four parameters of vessel size that need to be considered are length, beam, draft, and dead weight tonnage.  Of these, research has shown dead weight tonnage to be the best predictor of vessel controllability.

#### The goal is to design aid systems for the largest vessel using the waterway.  There may be cases where the largest vessel is significantly larger than the majority of the vessel population using the waterway.  Such a large vessel may also make infrequent transits of the waterway.  In this situation, consideration should be given to other methods, besides aids to navigation, for helping ensure the safety of this vessel.  Speed and visibility restrictions, escort vessels, and one-way traffic are some options that the COTP could invoke to reduce risk to an acceptable level without significantly altering the aid system when exceptionally large vessels transit the waterway.

#### Physical conditions of the waterway must also be considered.  When channels are narrow compared to the beam of the user, ranges may be essential for safe navigation.  The bottom conditions and clearances may also mandate the use of particular aids.

#### Environmental conditions are more difficult to address.  The decision to mark for average or worst conditions, or something in between, cannot always be easily made.  The following guidelines may be helpful:

##### The design should accommodate the maximum or worst case, tidal current in the waterway.

##### Reduced visibility due to haze and fog must be considered.  Designing for worst case visibility is not practical.  The goal, or availability standard, for minor aids is to provide a system based on a meteorological visibility that is met or exceeded 80% of the time.  The system of major lights should be designed for a visibility that is met or exceeded 90% of the time.  Following the constraints in section E., these availability standards will help describe the necessary hardware combinations.

##### In some areas attaining these availability standards may not be possible because of very poor visibility or severe background lighting.  These occurrences should be infrequent.  In such cases, attaining the highest operational availability with current equipment is acceptable.

##### Selected aid types should be appropriate for the existing environmental category (exposed, semi-exposed, or protected).  The Aids to Navigation Manual - Technical, COMDTINST M16500.3(series), lists the design environment for all standard aid types.

##### The effects of ice during the winter and frequent heavy rain squalls during the summer also dictate aid mix.  Unlighted buoys, which have a tendency to surface through the ice, may be necessary in the former case.  Racons might prove useful in the latter case when all but the strongest of radar signals might be covered by return from a heavy squall.

#### Due to the significant potential for loss of life caused by bridge allisions, approaches to bridges over the water must receive careful attention.  This includes highway and railroad bridges over waterways marked with aids to navigation and over adjoining waters.  The following guidelines may be helpful:

##### Coordinate AtoN system design with district bridge administration officials.  Considerations include existing bridge marking (e.g. lighting, retro reflective panels, racons), physical characteristics of the bridge (e.g. height, span width), extent of fendering systems, and types of waterway traffic.  AtoN systems should facilitate safe transit of vessels on the centerline of main channels approaching bridges.

##### Where bridges cross over waters adjoining waterways marked with aids to navigation, consideration must be given to the accessibility of these waters from the marked waterways and the impact of their use by traffic as alternate routes or as staging areas.  The use of Information Marks may be appropriate in those areas where bridges may pose a hazard to vessel activity.

### Simulators.  Using simulators to analyze aid configurations will be more common in the future.  At the very least, simulators can provide a good overview of aid effectiveness in a waterway during daytime and for a clear night.

## Short Range Aid System Configuration and Design.

### General.  The short range aid system consists of four sub-systems.  They are daytime visual, nighttime visual, radar or reduced visibility, and sound systems.  Characteristics of each sub-system are:

#### The daytime visual system consists of the daymarks of beacons and buoys and daytime range lights.

#### The nighttime visual system consists of lights and retro reflective signals.

#### The radar system consists of radar reflectors, racons, and shore-based radar systems.

#### Sound systems consist of various non-directional sound producing devices.

### Since the above systems are collocated, they must be designed or evaluated concurrently.  Guidelines for the use of major lights, ranges, racons, and sound signals have been provided earlier in Section D of this chapter.  The following more specific guidelines should help determine the needs for the two largest categories of short range aids:  beacons and buoys.

### Procedure for Marking.  Whether designing a new system or evaluating an existing one, there are many factors to be considered.  The following procedure is intended to promote a systematic review of each of these factors, resulting in a thorough treatment of even the most complex situation.

#### Determine the Conventional Direction of Buoyage (CDB):  Direction of flood and ebb currents throughout U.S. waters have been determined by NOAA and are usually available in current charts and tide tables.  This information, as well as coastwise and Great Lake conventions provided in 33 CFR, defines our conventional direction of buoyage for assigning lateral markings.  The designer should indicate the direction of buoyage on the working chart, paying particular attention to intersections, islands, and headwaters where currents meet and part.

#### Mark the Approaches:  The approach to any restricted waterway is usually classified as a bay, sound, channel, inlet or strait.  The aid systems in these regions consist of mixtures of radio aids and short range aids.  Several procedures are used for marking in this area:

##### Short range aids in this region conform to the conventional direction of buoyage and usually mark good water.

##### Traffic separation schemes (TSS) may be found in this area.  Aids marking a TSS are usually special purpose, with the most seaward aid usually being a safewater mark.  For a TSS to be internationally recognized, the International Maritime Organization (IMO) must approve the layout and ATON provided to ensure that vessels can determine their position within the TSS.  Commandant (CG-3PW) will make the notification.  This approval will take a minimum of 24 months

##### Wrecks and obstructions should be marked if the traffic situation so dictates.

#### Prioritize Channels:  When more than one channel needs marking, they should be ranked by order of importance.  This ranking is usually based on width, depth, and predominant traffic flow.  Draw a continuous line, numbered "1", down the center of the primary channel in the direction of buoyage, as in Figure 4-1.  Continue with each subsequent channel, leaving short gaps where a subordinate waterway crosses or intersects one of a higher priority, until all are drawn and numbered.  With one exception, you should prioritize the ICW based on its importance relative to other proximate waterways, and not necessarily downgrade it to the lowest ranking.  For example, there may be places where the ICW is deeper or supports a higher volume of traffic than a nearby non-ICW waterway.  The exception arises when the conventional direction of buoyage for the ICW runs opposite to that of the normal waterway.  In that case, the normal waterway must have a higher priority than the ICW.

Prioritized Channels

Figure 4-1

#### Channel Subdivision.  When conducting an aid system design or evaluation, it is helpful to divide the channel or waterway into regions according to the unique requirements of the maneuvering tasks for each.  These descriptive maneuvering tasks are: turn, recovery (from a turn), and track keeping.

##### A turn region should generally extend a half mile either side of the apex of the turn.  Different turn types, such as cutoff, non-cutoff, or bends, require that the designer use discretion when establishing region boundaries.  The constraining consideration should be the desire to enclose the area in which the navigator is actually executing the turn.

##### The recovery region, as the name implies, should enclose the area, immediately following a turn, which is needed to regain a steady heading.  Research has shown this distance to be approximately three quarters to one and one-half miles.  A shallow turn, 15 degrees or less, would require the shorter distance for recovery, while a larger turn, up to 35 degrees or more, would require a greater distance to regain track.  Vessel size is also an important variable in delineating recovery region distances, with larger vessels, 50K DWT or greater, needing greater distances to regain track.

##### The remaining portion of the waterway necessarily becomes the track keeping region.

##### These regions should be plotted directly on the chart showing the channel under consideration.  Figure 4-2 illustrates the technique.  This is especially useful when evaluating the interaction between regions later.

Navigation Regions

Figure 4-2

#### Region Marking.  The next logical step is to determine the number and location of aids within each region.  By later analyzing the entire waterway as a system, the designer can identify excessive redundancy or apparent marking voids at common boundaries, and make necessary adjustments.  Some general guidelines for marking each region are listed below:

##### The turn is the most difficult task in the navigation process, so logically the turn region should contain the highest aid density.  The most important need in this region is to mark the inside of the turn.  This translates to one lateral mark at the inside apex of a non-cutoff turn and two lateral marks at the inside corners of a cutoff turn.  Bends should be treated as cutoffs.  Complicating factors such as two-way traffic, narrow channel width, large ship size, frequently reduced visibility, and excessive cross-currents will suggest the need for additional turning aids.  The exact number and configuration are subject to the designer's discretion.  In general, the goal when marking a turn region is to outline the available maneuvering space.  Figure 4-3 illustrates some possible arrangements.

Figure 4-3

##### When executing a turn, there is a tendency for a vessel to be displaced radially toward the outside of the turn.  Given this fact, the first consideration in the recovery region should be to provide a side mark to help the vessel pullout from the turn.  This translates to one aid along the outboard channel edge, usually within a half mile from the turn apex.  After the pullout, gated configurations promote the most precise navigation performance, followed by staggered and one-sided configurations, respectively.  Figure 4-4 illustrates each type.  Your choice should be based on the operational conditions discussed in section D.4. of this chapter.  AtoN spacing along the track usually falls between 1/2 and 1 1/2 miles.  Maximum spacing should be governed by the criteria in the following paragraph.

Figure 4-4

##### Track keeping, being the simplest of the three tasks mentioned, requires the sparsest aid density.  Ranges have been shown to promote the best performance in straight channel segments.  Even with ranges, however, side marks will nearly always be needed to promote safe navigation when environmental conditions preclude using the visual range or in passing or meeting situations.  The following paragraph lists possible arrangements in order of their effectiveness.  Maximum aid spacing is constrained by the following criteria.  The standards stated assume use by a mariner with a fifteen foot height of eye or greater.

###### During nighttime, a mariner aboard a vessel constrained to the channel by her draft should see at least two lighted aids forward, on at least one side of the channel, from any position in the waterway, 80% of the nights of the year.  Vessels not so constrained, such as small boats or vessels transiting open bays or sounds, need see only one lighted aid forward.  Consult the Visual Signal Design Manual, COMDTINST M16510.2 (series) to obtain the 80% visibility value for the appropriate geographic area.  The Allards Law program, available from G-SEC-2, can be used to determine optic/lamp combinations to meet minimum candela requirements.

###### During daytime, a mariner aboard a vessel constrained to the channel by her draft should see at least two aids forward, on at least one side of the channel, from any position in the waterway, at least 80% of the days of the year.  Mariners not constrained to the channel need see only one aid forward.  Enter Table 2-4, Change in Visual Range of Buoy Types Versus Change in Visibility (NM), in the Aids to Navigation Manual – Technical, COMDTINST M16500.3 (series) with the buoy type and the visibility to get visual range.  For dayboards, simply use the nominal range listed in the ATON Manual-Technical as the visual range.

###### Research has shown navigation performance deteriorates with increased aid spacing, especially for vessels larger than 50,000 DWT.  Consequently, when marking channels where vessels are constrained by their draft, no range lights are present, or natural or man-made landmarks are limited, maximum along-track spacing should normally not exceed 1.5 miles for vessels larger than 50K DWT and 2.0 miles for those less than 50K DWT.

###### By using the smallest aid spacing which results from applying these three criteria, each constraint will be satisfied.  Remember, this is only an upper limit on aid spacing.  Peculiarities of your waterway and its users may dictate shorter spacing and higher aid density.

##### Once each region is evaluated for proper marking, the region boundaries must be examined to ensure a smooth transition.  Adjust the initial design to eliminate excessive redundancy or fill in the gaps.

#### Assign Signals. With the aids in place, the appropriate signals may now be assigned.  Detailed specifications for all signals are contained in the following paragraphs and in the Aids to Navigation Manual – Technical, COMDTINST M16500.3 (series).  FIGURE 4-5 provides a quick reference for the most common signals assigned on aids in the U.S. and Western Rivers Marking Systems.

Figure 4-5

##### Daymarks:  The daymark is characterized by its color, shape, and number or letter.

###### Color:  Since most of our aids are lateral marks, assigning the proper shape and color is usually clear.  When looking in the direction of buoyage, aids to the right are red; those to the left are green. For those aids alongside two channels (junction, bifurcation, intersection), preferred channel marks are required.  The higher priority channel dictates the dominant or uppermost color, while the lower priority channel dictates the auxiliary or lowermost color.  Figure 4-6 shows an example of this application.  Safewater marks are painted with red and white vertical stripes.  Isolated Danger marks are black with a wide red, horizontal band.  Special marks are yellow.  Ranges can show any two of the colors red, green, black, white or yellow.  Certain non-lateral aids, such as lighthouses, leading lights, crossing, or sector lights may carry NB, NG, or NR dayboards.  Information daymarks are white with orange bands and orange geometric shapes.

Figure 4-6

###### Shape:  For fixed aids, the dayboard shape carries significance.  Lateral aids colored red are triangular; those colored green are square.  Preferred Channel marks are shaped according to the uppermost or dominant color.  Safewater marks are octagonal.  Special marks, crossing and other non-lateral marks are diamond shaped.  For large lighted buoys, shape is insignificant, except for the safewater mark and the isolated danger mark.  The safewater mark must carry a single red sphere as a topmark and the isolated danger mark must carry two black spheres as a topmark. Five and three foot lighted buoys have shape significance. For unlighted buoys, red aids are nuns, green aids are cans, and red and white striped aids are cans or pillars with a red spherical topmark.  Yellow aids may use various hulls.  As with dayboards, unlighted Preferred Channel buoys are shaped according to the uppermost color.

###### Numbers and Letters:  Channels should be numbered and lettered sequentially.  Lateral aids are numbered or lettered as consecutively as possible, beginning at the seaward end of the waterway, and increasing in the previously defined direction of buoyage.  Use even numbers on red aids and odd numbers on green aids.  In addition to a number, the first aid in a waterway may be suffixed with an identifying letter(s).  This provides positive identification where two or more buoys in an area have the same number (e.g. Buoy 2A).  Preferred Channel, Safewater, Isolated Danger, and Special marks shall be lettered.  On the Western Rivers, numbers displayed on distance markers, or suspended from lateral and crossing dayboards, indicate the mileage from a designated point.  Buoys on the Western Rivers are not numbered.  More guidance on numbering and lettering can be found later in the section on Aid Names.

##### Lights.

###### Terminology:  An aid's light characteristic consists of its color and rhythm. Authorized colors are red, green, white, and yellow.  Authorized rhythms are fixed, flashing (duration of light clearly less than duration of darkness), isophase (light and darkness equal in duration), and occulting (duration of light clearly longer than duration of darkness).  Flashing lights may be single flashing (one flash per period) or group flashing (flashes combined in groups with darkness between groups clearly longer than darkness between flashes).  Single flashing lights may be slow flashing (less than 30 flashes per minute; the adjective "slow" is usually omitted) or quick flashing (60 or more flashes per minute).  A Morse code light groups flashes (long and short) to form Morse characters.

###### Authorized characteristics:  A light's characteristic is determined by the aid's function.  Authorized characteristics for most aids are contained in Figure 4-5.  The quick rhythm is the most conspicuous, and should be used on important lateral aids.  Some examples are aids in turns, marking shoals, and marking wrecks.  Because a single fixed light can be mistaken for a vessel, use of fixed rhythm lights as lateral aids to navigation is not authorized.  Those non-lateral aids not subject to the IALA agreement (lighthouses, range lights, leading lights, sector and direction lights) are not constrained to particular characteristics.  The choice of characteristic should reflect a desire to provide the best conspicuity while avoiding conflicts with other aids which might mislead the mariner.  An example of such a conflict would be a major seacoast light showing a Morse Alpha rhythm, since, as shown in Figure 4-5, Morse Alpha indicates a safewater mark.  Common sense should eliminate any confusion.

##### ICW Markings.  Yellow symbols indicate that an aid marks the Intracoastal Waterway.  Yellow triangles indicate starboard hand aids, and yellow squares indicate port hand aids when following the ICW's conventional direction of buoyage:  southerly along the East coast, westerly along the Okeechobee waterway, and westerly along the Gulf coast.  Non-lateral aids such as safe water, isolated danger, and front range boards are marked with a horizontal yellow band.  Rear range boards do not display the yellow band because of the inconspicuity of the symbol.  At a junction with a federally maintained waterway, the preferred channel mark will display a yellow triangle or square as appropriate for the conventional direction of buoyage of the ICW.  Junctions with the ICW and privately maintained waterways are not marked with preferred channel buoys.

#### Other Marks and Considerations.  The designer must bear in mind the discussions presented thus far apply mainly to the relatively small portion of our total aid population marking dredged channels and restricted waterways.  Most aids simply delineate safe water or mark wrecks, obstructions, isolated dangers, etc.  These must be marked as the prevailing traffic situation dictates.  Common sense, experience, and user preference will provide the basis for their location and configuration.

##### Marking of Coaxial Waterways:  Occasionally, it may be necessary to mark the deep draft channel within a wider waterway while marking the wider boundaries for shallow draft traffic.  Begin by considering the limits of each traffic route independently.  Then refine the system to eliminate unnecessary aids where close spacing results.  Large potential for confusion to the mariner exists in naming and numbering the aids in this area.  The aids marking the deep draft route should be named and numbered first.  Included in the name should be a noun indicating a feature of that route, probably channel, traffic lane, cut, or canal.  An example would be Black River Channel Lighted Buoy 3 (see Figure 4-7).  The other aids that mark the broader expanse of water should then be named and numbered.  The name should not include the word "Channel" unless, by chance, it does lie on the channel edge.  For example, an aid between Black River Channel Buoys 3 and 5, but not lying on the channel, could reasonably be named Black River Buoy 3A.  It should not be numbered in pure numerical sequence with the channel.  In a narrow river, the use of alpha-numerics, as in the above example, may be the most obvious solution.  But, on a very wide river, the auxiliary lateral markings could be numbered totally independent of the channel numbering system.  The application of common sense toward the goal of creating simplicity and minimizing confusion should prevail.

Figure 4-7

##### Variations on Direction of Buoyage:  At times, strict adherence to using the direction of flood to determine the conventional direction of buoyage may cause confusion and result in an inconsistent marking scheme.  This situation is most likely to arise near islands or river mouths.  A little ingenuity and common sense should suggest the proper deviation from the rules.  The goal should be to design the simplest, most straightforward waterway while keeping exceptions to the rules to a minimum.  In Figure 4-8, an apparent confusing situation exists between points "B" and "C".  This can be eliminated by deviating from the rules and changing the direction of buoyage to go from point "B" to point "C".

Figure 4-8