From:	eNAV Committee

To:	ANM Committee							   March 2007

Liaison Note

IALA NAVGUIDE 2010 Edition

The e-NAV Committee requests that the following headings be added to the 2010 Edition of the IALA NAVGUIDE

Main heading:		e-Navigation (NEW)

Sub heading:		Background overview

Sub heading:		Definition

Sub heading:		Goals

Sub heading:		Elements

There is no longer a requirement for a section on radar transponders.

The e-NAV Committee further requests that the following text be added to the 2010 Edition of the IALA NAVGUIDE

AIS System Characteristics

Coverage Considerations

When implementing AIS within the VTS infrastructure the designer must recognize that every ship is using the two designated radio frequencies to organise transmission and reception of messages in an autonomous self-organising mode.  This forms a cellular system where every ship is his own centre, creating a secure zone around each ship which is half the distance of the radio range (which may vary based upon antenna placement and height).    However, there is a zone outside the secure zone called the Aloha zone.  The Aloha zone is four times the area of the secure zone, when ships are operating in the Aloha zone there is a high probability for message collision under heavy VDL load.

- Cautions when using AIS

The OOW should be aware that dynamic information may be inaccurate due to incorrect installations and sensor problems (e.g. incorrect GPS input).  The static and voyage data may also be inaccurate as this requires human input.   When these errors occur the VTS centre should take corrective action.

Note:  The AIS technology provides, for the first time, a means to detect and correct these navigational anomalies.