REPORT

on

the 11th session of the IMO Sub-Committee on Radiocommunications and Search and Rescue

(COMSAR)

Contents

# 1.	Introduction.

2.	Long-range identification and tracking of ships

3.	E-Navigation

4. 	Performance standards for AIS-SART

5.	XML format for ship reporting systems

6.	Establishment of new NAVAREAs in Arctic Waters

# 7.	Recommended actions by IALA

8.	Date of the next session

Annex 1

Proposed agenda of the Sub-Committee on Radiocommunication and Search and Rescue, 12th session (7 – 11 April 2008)

Annex 2

Draft Performance Standards for Survival Craft AIS Search and Rescue Transmitter (AIS-SART)

________

# 1.	Introduction.

Due to the refurbishment of the headquarters of the International Maritime Organization, the 11th session of the IMO Sub-Committee on Radiocommunications and Search and Rescue was held at the Royal Horticultural Halls and Conference Centre in London during the period 19 to 23 February, 2007. It was attended by representatives from 71 Member States and 21 Observer Organizations. The Secretary General and Jean-Charles Leclair were representing the Association.

The items of the agenda for the session of most interest to IALA members were related to the continuation of discussions on Long-Range Identification and Tracking of Ships and on the concept of e-navigation. On LRIT matters, it appears that a lot of works are still needed on the costing and billing policy recommendations and that the discussions remain very theoretical because the lack of information on the volume of traffic that the system would have to deal with. Contracting Governments are urged to provide data on their intent. Regarding e-navigation, the works of IALA seem on the same line as the IMO ones and complementary.

The Sub-Committee also approved draft performance standards for AIS-SART, for which IALA made a submission, and confirmed the use of XML format for ship reporting systems. It also approved the creation of 5 new NAVAREAs for the Arctic waters.

2.	Long-range identification and tracking of ships

At its 81st session, the Maritime Safety Committee adopted an amendment to SOLAS Chapter V introducing new obligations for ships regarding Long-Range Identification and Tracking of ships (LRIT). It is closed to be a worldwide system since there is no limit for flag States and port States to track ships and the limit for coastal States has been fixed at 1000 nautical miles from coasts. The new amendment is due to enter into force the 1st of January 2008 and new ships shall be fitted with a system to automatically transmit the information required – identity, position, date and time of the position provided – before the 31st December 2008, and for existing ships during the year 2009.  The ships involved are all passenger ships and cargo ships of 300 gross tonnage and upwards on international voyages, including high speed crafts and mobile offshore drilling units.

MSC 81 also adopted MSC Resolution 210(81) on Performance Standards and Functional Requirements for LRIT and MSC Resolution 211(81) on Arrangements for the Timely Establishment of the LRIT System. The latter established a programme of decisions that the MSC has to be taken during 2007 for the System to be able to work on time, and also established an Ad Hoc Working Group on engineering aspects of LRIT to develop:

- technical specifications for the International Data Exchange (IDE),

- technical specifications for the International LRIT Data Centre (IDC),

- technical specifications for communications within the LRIT System network (between the LRIT Data Centres and International LRIT Data Exchange and with the LRIT Data Distribution Plan),

- protocols for the development testing of the LRIT System and for testing the integration into the system of new LRIT Data Centres, and

- guidance which would assist the IMO in setting up and maintaining the LRIT Data Distribution Plan.

During its 82nd session, the MSC agreed to circulate, on an interim basis, draft technical specifications, as proposed by the Ad Hoc Working Group, under MSC.1/Circ.1219, “Interim LRIT Technical Specifications and other matters”, and decided to appoint IMSO as LRIT Co-ordinator, inviting IMSO to take whatever action it could in order to ensure the timely implementation of the LRIT System. The Committee also decided to re-establish the Ad Hoc Working Group to finalize its tasks in time for providing its conclusions and recommendations to MSC 83. In the meantime COMSAR 11 was invited to consider these LRIT issues with a high degree of priority.

The Ad Hoc Working Group met four times before the COMSAR meeting, including the previous week and the Sub-Committee mainly discussed two items prepared by the working group: a draft template of agreements necessary to implement the LRIT system, and the options, assessment and recommendations related to the LRIT costing and billing system.

Regarding the first point, the COMSAR Sub-Committee agreed in general with the matrix proposed and requested the IMO Secretariat to prepare first drafts agreements with the Contracting Parties. However, it was clear that many legal questions are still pendant and request further studies. The Member States and the Secretariat were invited to provide comments and proposals on this matter.

A lot of works are still needed on the costing and billing policy recommendations. The discussions appeared very theoretical because the lack of information on the volume of traffic that the system would have to deal with. In particular, despite several requests, IMO had not yet received any information from any Member States regarding the volume of LRIT information packages that they are likely to request and their intents in relation to the establishment of National, Regional or in Co-operation Data Centers. Therefore, the Sub-Committee strongly repeated the request from MSC to the SOLAS Contracting Governments to provide such information urgently.

At this stage, the Sub-Committee recommended to the Committee that:

- those SOLAS Contracting Governments that have decided to establish their own National, Regional or in Co-operation Data Centers will have to pay both for the initial setup costs for the centres, as well as for the ongoing cost for the 4 LRIT daily transmissions of their own flag ships;

- the cost of the LRIT Co-ordinator should be shared amongst all the Data Centers in the LRIT system and the costs should be equitably based on the level of effort that the LRIT Co-ordinator will expend to perform its duties;

- the cost of the International Data Exchange (IDE) should be shared on the basis of the volume of data sent through the IDE to a Data Center divided by the total amount of data sent through the IDE; and

- the cost of the International Data Center (IDC) should be shared on the basis of the volume of data requested by a Data Center or a SOLAS Contracting Government divided by the total amount of data requested from the IDC.

In a document submitted to COMSAR, the United-States were offering to build and operate an International LRIT Data Center and International LRIT Data Exchange until other suitable arrangements are in place to perform these functions. The Sub-Committee was of the opinion that this was not the time to discuss the offer but a number of delegations asked the United-States to resubmit their offer for consideration at MSC83. Indeed, the Sub-Committee expressed its concern that considerable time had elapsed since the adoption of SOLAS regulation V/19-1 without having made substantial progress in relation to the timely establishment of the LRIT system; and that the next MSC meeting in October will be the last chance to finalize the system before the fixed date of its entry into force, the 1st of January 2008.

3.	E-Navigation

The Maritime Safety Committee at its 81st session instructed NAV together with COMSAR Sub-Committee to develop an e-navigation strategy with a target completion date of 2008. At its 52nd session, the NAV Sub-Committee recognized that it would be essential, as a first step, to develop a clear definition and objectives for the concept of e-navigation. It agreed that, to progress the work for NAV 53, an intersessional Correspondence Group should be established with the following terms of reference:

The Correspondence Group on e-navigation should consider, provide comments and make recommendations on the following:

- The definition and scope of the concept of e-navigation in terms of its purpose, components and limitations to produce a system architecture;

- The identification of the key issues and priorities that will have to be addressed in a strategic vision and a policy framework on e-navigation;

- The identification of both benefits of and obstacles that may arise in the further development of such a strategic vision and policy framework;

- The identification of the roles of the Organization, its Member States, other bodies and industry in the further development of such a strategic vision and policy framework;

- The formulation of a work program for the further development of such a strategic vision and policy framework, including an outline migration plan and recommendations on the roles of the NAV and COMSAR Sub-Committees and the input of other parties concerned.

It was also agreed that the Correspondence Group should submit a document to COMSAR 11 raising specific questions that should be addressed by COMSAR.

Indeed, the Sub-Committee was able to consider such a document from the Correspondence Group. It also considered a document from the Republic of Korea proposing the utilization of technically improved GMDSS equipment as a data communication network for e-navigation, and a document from Australia containing a set of three diagrams explaining the concept of e-navigation.

The subject was discussed at large by a working group of whom the conclusions were agreed by the Sub-Committee. The IALA delegation indicated what the Association had already done on this matter, in particular the outcomes of the 1st session of the e-Navigation Committee, including the definition of e-navigation as well as the three prerequisite fundamental elements and the key communication aspects required for e-navigation.

Finally, it was agreed that existing GMDSS infrastructure could support SAR services and communications, but, with respect to e-navigation, broadband communication on a global basis using satellite technology would be necessary. It was further agreed that the lessons learnt from the development and operation of GMDSS and AIS, including human-machine interface, training and shore infrastructure, should be taken into account in the development of e-navigation.

The Sub-Committee concluded that the development of e-navigation must be user driven and not technology driven. There must be equipment standardization including a standard mode of operation. Lastly, software installed in operating systems must follow a formal change control process to ensure that all elements of the e-navigation system would operate efficiently.

4. 	Performance standards for AIS-SART

At its 78th session, the Maritime Safety Committee requested COMSAR to revise the Performance standards for SART (Resolution A.802(19)), in particular to include provisions for the AIS search and rescue transponder. COMSAR 10 prepared two liaison statements to ITU and IALA requesting their assistance in the matter. Within this framework, the AIS Technical Working Group of the IALA e-navigation Committee organised an intersessional meeting to consider the matter further in detail and developed a draft performance standards for survival craft AIS search and rescue transmitter (AIS-SAR Station).

The COMSAR Sub-Committee decided that the proposed Performance Standards for AIS-SART should be a separate resolution to avoid any confusion with that relating to radar-SART, and agreed with the IALA text proposal with few small changes, as set out in annex 2. The necessary amendments to SOLAS regulations III/6.2.2, III/26.2.5 and IV/7.1.3, to the 1994 HSC Code and the 2000 HSC Code, and to the MODU Code were also prepared.

COMSAR recognized that SART devices are not, and should not, be used for distress alerting; SART devices should be used for providing a means of locating after the transmission of a distress alert and are useful tools for SAR authorities. The Sub-Committee also invited the NAV Sub-Committee to consider the need for a presentation symbol for AIS-SART and requested ITU to consider pre-configured text formats for “test purposes” (SART UNDER TEST) and for “active state” (SART ACTIVE).

During the session, the United Kingdom provided a presentation concerning “sea trials of the AIS-SART”.

5.	XML format for ship reporting systems

The Sub-Committee received information from the European Commission on the use of XML formats for the submission and exchange of standard reports, notifications and requests for information as required by the European Directive 2002/59 on VTMIS and the supportive IT information and exchange system SafeSeaNet. NAV 52 already agreed that it would be appropriate to implement such standardized XML format with a view to improving safety and security. COMSAR expressed the same opinion and invited Member Governments and International Organizations to submit details of existing messages and proposals to COMSAR 12.

6.	Establishment of new NAVAREAs in Arctic Waters

The Sub-Committee considered the proposal from the Joint IMO/IHO/WMO Correspondence Group and agreed on the creation of five new Arctic NAVAREAs, extending up to 90 degrees North:  NAVAREA XVII to NAVAREA XXI and their respective co-ordinator, Canada for NAVAREAs XVII and XVIII, Norway for NAVAREA XIX and the Russian Federation for NAVAREAs XX and XXI.

# 7.	Recommended actions by IALA

# It is recommended that:

# 7.1	The Council

- Note the information on long range identification and tracking (LRIT) of ships (item 2).

- Note the information on e-navigation (item 3).

## 7.2	The VTS Committee

- Note the information on long range identification and tracking (LRIT) of ships (item 2).

- Note the information on e-navigation (item3)

- Note the information on XML format for ship reporting systems (item 5)

7.3	The e-Navigation Committee

- Note the information on long range identification and tracking (LRIT) of ships (item 2).

- Note the information on e-navigation (item3)

- Note the information on Performance standards for AIS-SART (item4)

- Note the information on XML format for ship reporting systems (item 5)

7.4	The ANM Committee

- Note the information on long range identification and tracking (LRIT) of ships (item 2).

- Note the information on e-navigation (item3)

- Note the information on Performance standards for AIS-SART (item4)

- Note the information on establishment of new NAVAREAs in Arctic waters (item 6)

# 8.	Date of the next session.

The 12th session had been scheduled to take place from 7 to 11 avril 2008 at the IMO headquarters in London.

*  *  *

J.Ch. Leclair.

Accredited IALA Representative to IMO

10 March 2007.

# Annex 1

Sub-Committee on Radiocommunications and Search and Rescue (COMSAR)

12th session – 7 - 11 April 2008

Agenda proposed to the Maritime Safety Committee

Opening of the session

1	Adoption of the agenda

2	Decisions of other IMO bodies

3 	Global Maritime Distress and Safety System (GMDSS)

- Matters relating to the GMDSS Master Plan

- Operational and technical co-ordination provisions of maritime safety information (MSI) services, including review of the related documents

4	ITU maritime radiocommunication matters

- Radiocommunication ITU-R Study Group 8 matters

- ITU World Radiocommunication Conference matters

5	Satellite services (Inmarsat and COSPAS-SARSAT)

6	Matters concerning search and rescue, including those related to the 1979 SAR Conference and the implementation of the GMDSS

- Harmonization of aeronautical and maritime search and rescue procedures, including SAR training matters

- Plan for the provision of maritime SAR services, including procedures for routeing distress information in the GMDSS

7	Developments in maritime radiocommunication systems and technology

8	Review of the IAMSAR Manual

9	Replacements for use of NBDP (radio telex) for maritime distress and safety communications in maritime MF/HF bands

10	Guidelines for uniform operating limitations of high-speed craft

11	Development of an e-navigation strategy

12	Work program and agenda for COMSAR 13

13	Election of Chairman and Vice-Chairman for 2009

14	Any other business

15	Report to the Maritime Safety Committee

__________

Annex 2

DRAFT PERFORMANCE STANDARDS FOR

SURVIVAL CRAFT AIS SEARCH AND RESCUE TRANSMITTER (AIS-SART)

1	INTRODUCTION

AIS Search and Rescue Transmitter (AIS-SART), in addition to meeting the requirements of the relevant ITU-R Recommendation and the general requirements set out in resolution A.694(17), should comply with the following performance standards.

2	GENERAL

The AIS-SART should be capable of transmitting messages that indicate the position, static and safety information of a unit in distress. The transmitted messages should be compatible with existing AIS installations. The transmitted messages should be recognized and displayed by assisting units in the reception range of AIS-SART.

2.1	The AIS-SART should:

.1	be capable of being easily activated by unskilled personnel;

.2	be fitted with means to prevent inadvertent activation;

.3	be equipped with a means which is either visual or audible, or both visual and audible, to indicate correct operation;

.4	be capable of manual activation and deactivation; provision for automatic activation may be included;

.5	be capable of withstanding without damage drops from a height of 20 m into water;

.6	be watertight at a depth of 10 m for at least 5 min;

.7	maintain water tightness when subjected to a thermal shock of 45C under specified conditions of immersion;

.8	be capable of floating (not necessarily in an operating position) if it is not an      integral part of the survival craft;

.9	be equipped with buoyant lanyard, suitable for use as a tether, if it is capable of floating;

.10	not be unduly affected by seawater or oil;

.11	be resistant to deterioration in prolonged exposure to sunlight;

.12	be of a highly visible yellow/orange colour on all surfaces where this will assist detection;

.13	have a smooth external construction to avoid damaging the survival craft;

.14	be provided with an arrangement to bring the AIS-SART antenna to a level of at least 1 metre above sea level, together with illustrated instructions;

.15	be capable of transmitting with a reporting interval of 1 minute or less;

.16	equipped with an internal position source and be capable of transmitting its current position in each message; and

.17	be capable of being tested for all functionalities using specific test information.

2.2	The AIS-SART should have sufficient battery capacity to operate for 96 h within a temperature range of -20˚C to +55˚C, and to provide for testing of the functions on the equipment. The AIS-SART should have an unique identifier to ensure the integrity of the VHF data link.

2.3	The AIS-SART should be so designed as to be able to operate under ambient temperatures of -20C to +55C.  It should not be damaged in stowage throughout the temperature range of -30C to +70C.

2.4	The AIS-SART should be detectable at a range of 5 nautical miles over water.

2.5	The AIS-SART should continue transmission even if the position and time synchronisation from the positioning system is lost or fails.

2.6	The AIS-SART should transmit within 1 minute of activation.

3	TECHNICAL CHARACTERISTICS

Technical characteristics of the AIS-SART should be in accordance with relevant ITU recommendations.

4	LABELLING

In addition to the items specified in resolution A.694(17) on general requirements, the following should be clearly indicated on the exterior of the equipment:

.1	brief operating and test instructions; and

.2	expiry date for the primary battery used.

___________