IALA Recommendation on the marking of fish farms

THE COUNCIL

RECOGNISING the increase in number of fish farms being established and the potential danger  posed to shipping,

RECOGNISING ALSO that it is a matter for a National Authority to decide on whether a fish farm needs to be marked, depending on the risk involved,

RECOGNISING FURTHER that marking is to improve the safety of navigation and protect the fish farm itself,

HAVING CONSIDERED the proposals by the IALA ANM Committee,

RECOMMENDS

1.	that when a fish farm is considered to represent a danger to navigation it should be marked in accordance with the IALA Maritime Buoyage System, using special marks, lateral or cardinal marks, or a combination thereof. The fish farm should be marked but depending on the size, extent and location of a particular farm, it may be sufficient to mark only part of the perimeter or the centre. Administrations should bear in mind that the marking recommendations herein may be adjusted considering traffic density, proximity to ports, proximity to dangers, tidal considerations and other factors.

Specific examples can be found in Paragraph 2.

1.1	Fish farms are normally marked by special marks.

1.2	If there is a requirement for vessel traffic between fish farms, then such a channel should normally be marked with lateral marks.

1.3	If the prevailing situation warrants it, cardinal marking alone may be used to direct mariners away from the fish farm.

To improve the effectiveness of the lighting, and taking into consideration background lighting, synchronisation of the various lights should be considered.

To improve the radar target and the visibility of the fish farm, Radar reflectors and  reflective material should be considered.

2.	Fish farms can be various regular and irregular shapes and sizes but are usually:

·	rectangular

	circular

2.1	Rectangular fish farms should be marked according to the length of the sides.

2.1.1	Example 1:

Size:		      		side A and side B    500 metres.

Recommended Marking: 	a light in the middle of the fish farm.

A

B

2.1.2	Example 2a:

Size:				side A (sea side and coast side)    2500 metres

side B (lateral side)    500 metres.

Recommended Marking:	a light on each sea corner and a daymark on each coast corner.

A

B

2.1.3	Example 2b:

Size:				side A (sea side and coast side)    500 metres

side B (lateral side)    2500 metres.

Recommended Marking:	A light on one of the sea corners, a light on the diagonally opposite coast corner and daymarks on the other two corners.

A

B

2.1.3	Example 3:

Size:	side A and side B  500 and    2500 metres
such as A  B ≤ 2.250.000 metres2

Recommended Marking:	lights on diagonally  opposite corners and daymarks on the other two corners.

A

B

2.1.3	Example 4:

Size:	side A and side B  900 and    2.500 metres
such as A  B  2.250.000 metres2 and  6.250.000 metres2

Recommended Marking:	a light at each corner of the fish farm.

A

B

2.2	Circular fish farms should be marked according to the length of the radius.

2.1.1	Type 1:

Size:				radius    250 metres.

Recommended:		A light in the centre of the fish farm.

2.1.2	Type 2:

Size:				radius   250 and    1000 metres.

Recommended:	two lights 180 degrees apart on the circumference of the fish farm and two daymarks on the circumference positioned 90 degrees to the lights.

2.1.3	Type 3:

Size:				radius   1000 and    2000 metres.

Recommended:	three lights 120 degrees apart on the circumference of the fish farm.

120°

2.1.4	Type 4:

Size:				radius   2000 metres.

Recommended:	three lights 120 degrees apart on the circumference of the fish farm and three daymarks on the circumference positioned 60 degrees to the lights.

 60°