Information Paper on AIS AtoN Display

Introduction

AIS as aids to navigation (AtoN) will be an important component of e-Navigation. It is essential to their effective introduction that they can be displayed in a standardised way on as many classes of vessel as possible.

The standards for the display of AIS AtoNs are covered in several different documents. This paper describes the current situation, identifies some inconsistencies and possible remedies.

# Summary of current situation

Class A AIS installations are required to display Message 21 (AtoN Report) on the MKD and on a graphic display, if provided. The processing and display of Message 21 by Class B units is optional. The symbols used for the display of AIS AtoNs are specified in the IMO Performance Standard and are implemented in the relevant IEC documents, explicitly or by reference. However, there are some inconsistencies.

The standard representation for AIS AtoNs is a diamond with a cross at the actual position. The same symbol is used for real and virtual AtoNs in the new standard for navigation displays (IEC 62288). The revised ECDIS standard (IEC61174 Ed. 3) does not mention AIS AtoNs, but refers to the navigation displays standard...In the new radar standard (IEC 62388) a ‘V’ is superimposed for virtual AtoNs. The current situation is set out in more detail in the Annex.

Proposal for harmonisation

The lack of distinction between Real and Virtual AtoN on ECDIS and other navigation displays could be a matter for concern. The best solution would be to add the superimposed ‘V’ to the standard for navigation displays (IEC 62288), which is also referred to in the ECDIS standard. If this is not possible (the standard is at the comment stage, but will be finalised early in 2008) then a notation could be added to the symbol in accordance with IMO SN/Circ. 243 & IEC 62288, which allow for alphanumeric labelling. For example, a V could be placed beside the symbol for a Virtual AtoN. However, this notation should be standardized to avoid confusion.

The superimposed ‘V’ has been proposed in a comment on IEC 62288 through the UK process. The IALA e-NAV Committee is being encouraged to support this addition via the inter-sessional meeting of the AIS TWG in January 2008. A Liaison Note could then be drafted to other international organizations drawing their attention to this harmonisation, or, if it is not approved, proposing the alternative of the ‘V’ notation beside the symbol.

ANNEX

AIS AtoN Display - Current situation

The standard for Class A AIS equipment (IEC 61993-2) specifies that  Message 21 should be displayed on the Minimum Keyboard and Display (MKD) (Name of AtoN, plus indication that it is an AtoN) and on a graphical display if provided (Position on chart, lat., long., range, bearing)

Class B equipment is covered by IEC 62287-1, ed.1, in which an actual display for received messages is optional. An external interface is also optional. However, if an interface is provided then all received messages, including Message 21, should be made available for presentation purposes.

IMO approved performance standards for the presentation of navigation-related information on shipborne navigational displays in MSC Resolution 191(79) and adopted Guidelines for the Presentation of Navigation Related Symbols, Terms and Abbreviations in Safety of Navigation Circular SN/Circ. 243 in December 2004. This specifies a blue diamond for AIS AtoNs, changing to red if it goes off station. No distinction is made for Virtual AtoNs, except for a limitation on size of diamond and the fact that the real AtoN symbol will be absent. The relevant text reads:

“AIS Aids to navigation (ATON) shall be presented as a diamond with crossed lines centred at the reported position of the ATON. The crossed lines shall not be more than 2 millimetres in length. For a virtual AIS ATON, the sides of the diamond shall be not more than 6 millimetres in length. For a real AIS ATON that is on position, the diamond shall

encompass both the charted ATON symbol and the reported position of the ATON. For a real AIS ATON that is off position, the diamond shall be drawn as for a virtual AIS ATON apart from the charted symbol. The diamond and crossed lines shall be drawn using a thin solid line style.

Recommended colour for an “on position” AIS ATON: blue.

Recommended colour for an “off position” AIS ATON: red.

An AIS ATON marking a fixed structure with dimensions larger than the diamond symbol shall be presented with an additional true scaled outline representative of the structure. In this case, the diamond symbol shall remain positioned at the reported position of the AIS ATON.

AIS ATON may be labelled. Alphanumeric text used to label an AIS ATON shall be the same colour as the AIS ATON symbol.”

This text and the associated graphics are reproduced in IEC Document 62288 – Presentation of navigation-related information on shipborne navigational displays – General requirements – Methods of testing and required test results. This document is at CDV (Committee Draft for Vote) stage, with a closing date of 11 Jan 2008.

The revision of IEC document 61174 Ed.3, ECDIS - Operational and performance requirements, also at CDV with the same closing date, includes the display of AIS targets, but not AtoNs. However, in Annex K: Presentation of navigation information, Annex A of IEC 62288 is referred to as applicable to 61174.

Display of AIS AtoNs is also addressed in IEC Document 62388, Shipborne radar - Performance requirements, methods of testing and required test results and this document is cross-referenced throughout to 62288, where radar and chart information appear on the same display. However, a late change was introduced in 62388 to superimpose a V on the symbol for a virtual AIS AtoN. IEC 62388 is at FDIS (Final Draft International Standard) and cannot be changed further, other than editorially.