R

| From: IALA | Formerly e-NAV 4-output-11 |
| --- | --- |
| To: IMO NAV Sub-Committee | February 29, 2008 |

Liaison Note to IMO NAV Sub-Committee

Proposal on Symbols for AtoN AIS

Introduction

IALA has been developing the Recommendation A-126, however it does not include the symbols for AtoN AIS.  In 2005, Japan established the Study Committee to study the operations and indication of the AtoN AIS for both real and virtual AtoNs.  The Committee was organized by the maritime experts such as professors, experienced engineers, pilots, captains and manufactures etc.  The Committee considered and developed the user friendly draft AtoN symbols for shipborne navigation display based on the paper chart.  By using the draft symbols, the Committee member conducted and evaluated the field trials in Tokyo area in 2006.  The trial report was submitted in September 2007and noted by IALA e-Navigation Committee.

In the same period, the real and virtual AtoN symbols were considered and decided by International Organizations which would be suitable for the ECDIS and other navigation displays.

This Liaison note proposes the draft AtoN symbols be considered for the future shipborne navigation display as user options.

Study and Field Trials for AtoN AIS Symbols

The Study Committee in Japan considered the draft AtoN AIS symbols in ANNEX 1 which is similar to the paper chart.  ANNEX 1 only shows the virtual AtoN symbols, however the real AtoN symbols are defined using solid line.

Using the draft symbols, the field trials were conducted and ANNEX 2 shows the display example of the shipborne navigation display.

As the result of field trials, the draft AtoN symbols were generally satisfied by the evaluators, however the following issues were identified as in ANNEX 3:

- readabilities depends on the display resolutions

- size and shape of the symbols need further studies at zooming

- symbols need simplification

Proposal

IALA has identified the several issues need to be further studied, however IALA kindly requests IMO NAV Sub-Committee to consider the proposed draft symbols as user options for the future shipborne navigation display in conjunction with the related organisations.

***

ANNEX 1

Consideration of AtoN AIS Symbols

The Study Committee in Japan developed draft symbols in Table 1 which were intended to give no confusion to the users and to enhance conveniences of the users in identifying descriptions presented by shapes and/or texts on a map, taking account of the following aspects as the basic criteria.

The draft symbols basically;

-  comply with the universally established symbols used for charted physical AtoN (real entity), and

-  not contain a symbol depicting a light in the illustration of virtual AtoN.

Noting these fundamental principles as the basis, symbols for each type of AtoN were designed and draft “Symbols of AtoN AIS” in Table 1 were produced.

Table 1  Draft “Symbols of AtoN AIS”

| No. | Type of AtoN |  | (draft) Symbols of AtoN (Real AtoN symbols are in solid line) |  |
| --- | --- | --- | --- | --- |
|  |  |  | Floating Aids | Fixed Aids |
| 1 | Cardinal Marks | North Cardinal Mark | Virtual AIS | Virtual AIS |
| 2 |  | East Cardinal Mark | Virtual AIS | Virtual AIS |
| 3 |  | South Cardinal Mark | Virtual AIS | Virtual AIS |
| 4 |  | West Cardinal Mark | Virtual AIS | Virtual AIS |

Table 1  Draft “Symbols of Virtual AtoN AIS” (continued)

| No. | Type of AtoN |  | (draft) Symbols of Virtual AtoN (Real AtoN symbols are in solid line) |  |
| --- | --- | --- | --- | --- |
|  |  |  | Floating Aids | Fixed Aids |
| 5 | Lateral Marks | Port Hand Mark | Virtual AIS | Virtual AIS |
| 6 |  | Starboard Hand Mark | Virtual AIS | Virtual AIS |
| 7 |  | Mark for Preferred Channel to Port | Virtual AIS | Virtual AIS |
| 8 |  | Mark for Preferred Channel to Starboard | Virtual AIS | Virtual AIS |

Table 1  Draft “Symbols of Virtual AtoN AIS” (continued)

| No. | Type of AtoN | (draft) Symbols of Virtual AtoN (Real AtoN symbols are in solid line) |  |
| --- | --- | --- | --- |
|  |  | Floating Aids | Fixed Aids |
| 9 | Isolated Danger Marks | Virtual AIS | Virtual AIS |
| 10 | Safe Water Marks | Virtual AIS |  |
| 11 | Special Marks | Virtual AIS | Virtual AIS |
| 12 | Lighthouse | Virtual AIS Virtual AIS |  |

ANNEX 2

Display Example of AtoN Symbols at Field Trials

ANNEX 3

Improvement of Symbols based on Results of Field Trials

In the field trial, a mobile type personal computer and the dedicated software were used as the display. The readable symbol size was approximately 2 cm in long sides due to the display size (14 inches) and resolution of XGA (1024 x 768).  During the trials, the symbols of the ship near the AtoN AIS were hidden due to the symbol overlapping.

As the result of the trials, the following issues were identified:

- readabilities depends on the display resolutions

- size and shape of the symbols need further studies at zooming

- symbols need simplification

As one of the solutions, the following simplified symbols are under considerations in the Study Committee in Japan.

Simplified symbol example for floating AtoN AIS

Simplified symbol example

for fixed AtoN AIS