IALA Guideline No. XXXX

On

Synchronisation of Lights

Draft

April 2008

AISM Association of Internationale de Signalisation Maritime  IALA

International Association of Marine Aids to Navigation and Lighthouse Authorities

20ter, rue Schnapper, 78100

Saint Germain en Laye, France

Telephone +33 1 34 51 70 0 Telefax +33 1 34 51 82 05

E-mail - iala-aism@wanadoo.fr      Internet -  http://iala-aism.org

Document Revisions

Revisions to the IALA Document are to be noted in the table prior to the issue of a revised document.

| Date | Page / Section Revised | Requirement for Revision |
| --- | --- | --- |

Table of Contents

1	GENERAL	5

2	DEFINITIONS	5

3	BENEFITS OF SYNCHRONISATION	5

4	Limitations/Considerations of Synchronisation	6

4.1	Environmental limitations	6

4.2	Maximum slippage time	6

4.3	Minimum angular separation	6

5	Synchronisation configuration for optimum conspicuity	6

5.1	Logical Grouping of lights	6

5.2	Use of different characters	6

6	Synchronization Methods	6

6.1	General methods of synchronisation	7

6.2	Timing Options	7

6.2.1	GPS	8

6.2.2	GSM-networks time	8

6.2.3	DCF77 – Frankfurt clock – frequency standard stations	8

6.2.4	Digital television (DVB)	8

6.2.5	Radio network (Yleisradio, Finnish National Radio Organisation)	8

6.2.6	TV stripe frequency	8

6.2.7	e-Loran	8

6.2.8	Synchronization using other radio signals	8

6.3	Master-slave synchronization	8

6.3.1	External master	8

6.3.2	Internal master	8

6.3.3	Protocol example	8

6.4	Peer-to-peer synchronization	8

6.4.1	Night time switch	8

6.4.2	Protocol example	8

7	Experiences with synchronization	9

7.1	Finnish Maritime Administration (FMA)	9

7.2	Farvandsvæsenet – Royal Danish Administration of Navigation and Hydrography	9

7.3	Sjöfartsverket – Swedish Maritime Administration	9

7.4	Kystverket – The Norwegian Coastal Administration	9

8	Synchronisation Examples	10

8.1	FMA - Fairway of Vuosaari	10

8.2	SMA – Synchronised fairways passage with buoys	11

8.3	AMSA  – Synchronised Channel lights	12

8.4	TTC – Synchronised Buoys	13

9	Summary	15

10	References	15

11	Annex 1 – GLA Synchronised Lights Viewing Trial	17

IALA GUIDELINE FOR SYNCHRONISATION OF LIGHTS

- Develop Guidelines on flash synchronization.

- Consider optical benefit of synchronisation and sequencing on conspicuity.

- Consider applications for synchronisation.

- Consider methods of providing synchronisation.

- Practical experiences

# GENERAL

Purpose of the document and benefits of synchronisation eg counteract the effects of background lighting etc.

# DEFINITIONS

# BENEFITS OF SYNCHRONISATION

Synchronising of two or more flashing lights is in use in signalling systems for various transport systems, including rail, aviation, and maritime. The purpose of synchronising is to increase the conspicuity of the signal, and or to indicate that the two or more lights are associated in some manner.  For example if two buoys form a “gate” in a channel, the lights on them might be synchronised to make that gate pair more conspicuous.

Sequencing of lights is when a series of lights is flashed in a time sequence to show the geographical relationship between them.  Such a set of lights is sometimes called a “flare path”.

It is also possible to combine the two effects, so that, for example, it there is a channel marked by pairs of buoys, the lights on each pair are synchronised, and in addition the pairs flash in a time sequence along the channel.

In each case the objective is to help the mariner distinguish which lights are pairs of buoys (or beacons) marking a channel, and in addition to indicate which pairs are closest and which are more distant.

This technique has been used in various countries over some decades, but no consensus has yet emerged on how synchronising and sequencing can be employed to best effect.  The reasons for this are not clear, but probably depend on the separation of the AtoNs along the channel, the length of the channel, the shape of the channel, and local visibility conditions.

# Limitations/Considerations of Synchronisation

## Environmental limitations

## Maximum slippage time

The General Lighthouse Authorities of the United Kingdom and Ireland have concluded that the timing error between synchronised lights should not be greater than 50ms. (Refer to GLA Report Version RPT-09-03-MN-IT-07.)

## Minimum angular separation

Trials carried out in the United Kingdom in August 2007 evaluated the effects of angular and temporal separation of synchronised lights. (Refer to same report, GLA Report Version RPT-09-03-MN-IT-07.)

The results showed that the minimum angular spatial separation of synchronised lights is 4.5 minutes’ of arc for reciprocal colours, and 3 minutes of arc for like colours.  The General Lighthouse Authorities of the United Kingdom and Ireland concluded that for synchronised lights of reciprocal colours, i.e. red and green, there should be a minimum angular separation of 5 minutes of arc, subtended at the observer, within the arc of utilisation of the synchronised lights.

# Synchronisation configuration for optimum conspicuity

## Logical Grouping of lights

## Use of different characters

# Synchronization Methods

Synchronisation and sequencing require having identical time information available at all the lights which are to be synchronised or sequenced.  This can be accomplished in at least two ways:

- A sufficiently accurate clock at each light location

- Reception at each light location of the same timing data from a single source

In practice the latter is usually employed, and implementation is done by a radio broadcast system set up for the purpose, or by reception of an existing timing signal, or by cable connection.  Early synchronisation systems often employed a special local radio transmitter, emitting a timing signal received at each light.

Nowadays various local or global time signals are available and dedicated radio transmitters are not required. The most commonly used timing signal is that contained within a GNSS system, such as GPS or GLONASS.

In some countries, radio timing signals are broadcast at VLF and these have been used for synchronisation of lights.  In Japan, the JJY signals at 40kHz and 60kHz have been used, with a small H-field antenna built into the lantern, but such systems are now largely phased out in favour of GPS.

Note that synchronisation of the timing of the lights in the group is all that is needed.  Synchronisation to GPS time or other standard is not needed.  The synchronisation accuracy need not be high.  An accuracy of 100 parts per million is acceptable in most cases.

## General methods of synchronisation

## Timing Options

General information regarding different timing options to be added here…..

| Timing Method | Benefits | Disadvantages |
| --- | --- | --- |
| GPS (1,5 GHz) | Low price, stable and accurate receiver, accurate atom-clock as a reference, small receiving antenna | needs direct visual contact |
| GSM-network (900 MHz, 1800 MHz) | Low price | Not all GSM-networks are UTC *- synchronized |
| response / impulse signal (selectable frequency) | many options, stable system | synchronising unit may have large power consumption |
| Radio signal (DCF77) (77,5 kHz) | stable and accurate | regional availability, atmospheric conditions may affect the signal |
| DVB (470–790 MHz) | works all over the world | no commercial receivers available |
| Broadcast radio network | works all over the world | no commercial receivers available |
| TV stripe frequency | works all over the world | no commercial receivers available |
| Loran-C | Stabile receiver, accurate atom-clock as a reference | more expensive than products using GPS, restricted receiving area, uncertain future |

* UTC (Coordinated Universal Time)

### GPS

### GSM-networks time

### DCF77 – Frankfurt clock – frequency standard stations

### Digital television (DVB)

### Radio network (Yleisradio, Finnish National Radio Organisation)

### TV stripe frequency

### e-Loran

### Synchronization using other radio signals

## Master-slave synchronization

### External master

### Internal master

### Protocol example

## Peer-to-peer synchronization

### Night time switch

### Protocol example

# Experiences with synchronization

## Finnish Maritime Administration (FMA)

## Farvandsvæsenet – Royal Danish Administration of Navigation and Hydrography

## Sjöfartsverket – Swedish Maritime Administration

## Kystverket – The Norwegian Coastal Administration

# Synchronisation Examples

## FMA - Fairway of Vuosaari

## SMA – Synchronised fairways passage with buoys

Galten

The fairway passage is situated in Mälaren just west to Stockholm.  The

synchronising was introduced as a permanent solution as the first place in

Sweden in year 2000.

The fairway passage changes directions three times and is marked with 14

buoys.

From the beginning before the synchronising was installed the character was

Fl R 3s and Fl (2) G 6s. The existing character was first used when

synchronising was tested and it was then perceived as inconspicuous.  Test was then conducting with Fl G 3s and Fl R 3s in the straight part and with Q where the fairway turns. This solution was considered as very good and is still in use today.

GALTEN 1:25000

## AMSA  – Synchronised Channel lights

In January 2005 trials of a sequenced (“flare path”) light system was carried out in the Point Richards Channel in the Port of Geelong, Australia.  Feedback from the pilots using the channel indicated that a system of synchronisation that allowed all of the lights to flash together would be preferred instead of the sequenced lights approach.

Closely following this trial was a project to upgrade the main channel beacons.  In consultation with the pilots a new improved lightning scheme for the channel that included synchronisation was agreed.  The channel was reviewed and it was decided to split the channel into four sections for the purpose of synchronising the lights, with sections 1 and 3 synchronised and sections 2 and 4 synchronised.

At the start of each section of the channel a fixed light plus a synchronised flashing light was implemented to clearly mark the start of that new section.

## TTC – Synchronised Buoys

In September 2006 the TTC discussed with the local office of Bremerhaven several ideas to increase the conspicuity of the lighted buoys in the approach of Bremerhaven. This was necessary because a new container terminal causes high background lighting and therefore the lighted buoys could hardly be seen against the relatively bright background.

Situation:

In the approach of Bremerhaven 6 pairs of lighted buoys are concerned. The distance of the lighted buoys is between 1.5 km and 2 km. All of them are in pairs and they have three different characters due to different meaning and their position.

In Germany lateral marks are allowed to use only selected characters according to a national regulation (Erlass vom 24.11.77):

General lateral marks: fl (flash) or oc (occulting)

Lateral mark to show “change of course”: q (quick) 0.5+ (0.5)

The time schedule of the characters for each pair of buoys is shown in fig.1.

Proposals:

Different proposals according the character of the buoys were made:

oc 4s: 3 + (1)), fl. 4s: (1 + (3) and iso 4s: (2 + (2). To install a “runway effect” each buoy should get the same character. These ideas could be simulated by a program on screen.

Result:

According to the demands of the marines (position identification by flash character) it was decided that all pairs of buoys shall be kept with their own characters.

- Due to the relatively great distances between the buoys and the weather condition (light haze) during the test period only the next pair was visible. The situation that you can observe all 6 buoys pairs at the same time can be simulated on the screen only.

- According to this experience that only one pair can be seen no further experiments were made. A “runway light” as used on airports was not examined and would bring no advantage in this case.

- All lights shall be synchronized: they start at the same time (“point of synchronisation” in fig.1) with their characters so that you see the next pair of buoys switching on and off at the same time.

Summary:

In this application the most important result is that you get the best improvement by synchronizing a pair of buoys. There was nearly no benefit of synchronizing a whole chain of buoys.

Remark:

Visual aids like buoys and the GPS system are considered to be redundant to each other. In case of a synchronisation by GPS a certain dependency arises so that in case of a failure of the GPS system the advantage of the synchronisation is lost.

This is especially remarkable for leading lights.

Q : 0,5s + (0,5)s

Oc(2) 9s : (1)s + 2s + (1)s + 5s

Fl(2) 9s : 1s + (2)s + 1s + (5)s

Pair: 41/42

Pair: 43/44

Pair: 45/46

Pair: 47/48

Pair: 49/50

Pair: 51/52

Oc(2) 9s : (1)s + 2s + (1)s + 5s

Fl(2) 9s : 1s + (2)s + 1s + (5)s

Q : 0,5s + (0,5)s

Point of Synchronization

# Summary

# References

[1] FMA, FMA´s Navguide, 2003.

[2] K-E Löfgren, Veneilijän merenkulkuoppi I saaristonavigointi, Suomen

Navigaatioliitto, 2003. (Book about Navigation theory)

[3] D. L. Mills, Internet time synchronization: the network time protocol, IEEE

Transactions on Communications, vol. 39, no. 10, pp. 1482-1493, Oct. 1991.

[4] S. Ganeriwal, R. Kumar, M. Srivastava, Timing-sync protocol for sensor

networks, Proc. First Int. Conf. on Embedded Network Sensor Systems, Los

Angeles, California, Nov. 2003.

[5] J. Elson , L. Girod , D. Estrin, Fine-grained network time synchronization using

reference broadcasts, Proc. of the 5th symposium on Operating systems design

and implementation, Boston, Massachusetts, December 09-11, 2002.

[6] S. PalChaudhuri, A. Saha, D. B. Johnson, Probabilistic clock synchronization

service in sensor networks, Technical Report TR 03-418, Department of

Computer Science, Rice University, 2003.

[7] G. Pottie, W. Kaiser, Wireless integrated network sensors, Communications of

the ACM, 43(5), pp 51-58, May 2000.

[8] http://www.sabik.fi/

[9] http://www.solarmarinelights.com/

[10] http://www.automaticpower.com/

[11] http://www.oi-electric.dk/

[12] http://www.idman.fi/page.asp?pageid=air,intro&languageid=EN

[13] http://www.fma.fi/media/merivayla/2004-2.pdf

Appendix A: List of people interviewed

Finnish Research Institute, ICT section - Risto Mutanen - Synchronising methods and equipment solutions

Finnish Research Institute, ICT section - Jan Holmberg - Synchronising methods and equipment solutions

Finnish Research Institute, ICT section - Heikki Seppä -  Synchronising methods and equipment solutions

Royal Danish Administration of Navigation and Hydrography - Jørgen Petersen

Swedish Maritime Administration - Hans Rosén

The Norwegian Coastal Administration - Oddbjørn N. Vik Norjan

Digita Ltd - Henri Viljasjärvi

TeliaSonera Ltd - Kirsi Lepistö

Appendix B: list of abbreviations

# Annex 1 – GLA Synchronised Lights Viewing Trial

***********************