# Draft Guidelines for Establishment of AIS AtoN

Introduction

The criteria for fitting AIS to any Aid to Navigation should be based on the navigational requirement derived from the assessment of risk. This document sets out general criteria to assist authorities in deciding whether AIS functionality should be provided, and if so, what message types and functions are appropriate.

The document looks at AIS use from an authority’s perspective. In deciding the level of service and type of system to be provided, consideration should also be given to whether sharing of data with other organisations or any commercial opportunity could dictate an alternative approach.

The fitting of AIS to AtoNs can provide the following services to AIS equipped vessels

- all weather identification of the AtoN

- an accurate position for floating AtoNs,

- an accurate position for fixed AtoNs which acts as reference target for verifying Radar

- status indication of the co-located traditional AtoNs

- Meteorological and Hydrological information by binary message

- AtoN status, Meteorological and Hydrological message by text message

In addition to the above the fitting of AIS to AtoNs provides the following benefits for the AtoN authority

- Near real-time monitoring of AtoN status and,

- Monitoring of the status of other support equipment at the site

There is a distinction between the use of AIS as an AtoN and its use for solely monitoring purposes or for collection of traffic data. Part 1 of this document considers factors related to use of AIS as an AtoN; Part 2 considers the available AIS AtoN Station and message types; Part 3 considers additional services such as provision of metrological and hydrological data; Part 4 deals with monitoring, tracking and other uses.

To mitigate against the possibility of transmitting false information, it is important to monitor the AIS transmission from a receiving station within the coverage area of the AIS AtoN Station. The primary source of this information will be the National Competent Authority (NCA) of the country.

If the AIS AtoN Station is not within the coverage area of the NCA network, then the authority may establish its own Base Station or AIS monitoring station at a site near the AIS AtoN Station.

IALA Recommendation A-126 On The Use of the Automatic Identification System (AIS) in Marine Aids to Navigation provides detailed information on the type of AIS AtoN services that may be provided.

1 – AIS AtoN Station Assessment Criteria

1.1 Lighthouses and Beacons

# 1.1.1 Requirement

The primary purpose of providing AIS AtoN functionality to lighthouses and beacons is to provide the mariner with a fixed point of reference on the shore, and to confirm the functionality of other AtoN provided on the station.  Such a point of reference is the e-Navigation equivalent of the physical station and is of assistance to the mariner in identifying a particular point of land, for spatial awareness, in taking bearings or distances for position confirmation, or in laying off parallel index lines. The provision of AtoN functionality information gives advance information to the mariner on whether key AtoN are performing correctly and allows for revision of the passage plan if required.

# 1.1.2 Criteria

AIS AtoN functionality should be provided on Lighthouses and Beacons where a navigational assessment identifies the requirement set out above. Typical locations for consideration would include offshore stations, headland stations, landfall stations, stations that are commonly used as waypoints, stations that mark points on featureless coastlines, or isolated dangers.

1.2 Buoys and Major Floating Aids

# 1.2.1 Requirement

In addition to the point of reference and AtoN information purposes described above for fixed AtoN the provision of AIS AtoN functionality on floating AtoN such as buoys and Major Floating Aids (MFA) provides confirmation of the floating AtoN position. Confirmation of position provides the mariner with an assurance that the AtoN can be used, spatial awareness, bearings and distances for position confirmation, or in laying off parallel index lines. Advance confirmation of the position of floating AtoN is a significant improvement in the service available to the mariner.

##### 1.2.2 Criteria

The importance of the floating AtoN in the navigational solution is the key to assessing need for provision of AIS AtoN functionality. Therefore, it is likely that all Major Floating Aids (MFA) and Buoys of Primary Navigation Significance (BPNS) will be AIS equipped.  Of the remaining floating AtoN, typical locations for consideration would include gateway buoys at the approach to narrow channels, buoys that are commonly used as waypoints, buoys marking isolated dangers, buoys marking the extremities of shoal areas and buoys that are critical to the mariner’s spatial awareness.

1.3 Virtual AtoN

# 1.3.1 Requirement

The requirement for virtual AtoN arises primarily from the need to mark wrecks and new dangers quickly. However, virtual AtoN may also have a future role in replacing some MFA and as AtoN for special craft such as Wing-In-Ground (WIG) vessels.

# 1.3.2 Criteria

The criteria for rapidly marking wrecks and other new dangers relate to an assessment of the danger posed to shipping in the area. The criteria for replacement of existing AtoN with virtual AtoN will involve an assessment of the navigational value of the physical characteristics. Typical locations would include deepwater AtoN intended primarily for Safety –Of –Life –At - Sea Convention (SOLAS) vessels. Possible future provision for new vessel types such as WIG craft arise from the need to mark a channel without having the physical obstruction of an AtoN.

2 – AIS AtoN Station and Message Types

# 2.1 Real and Synthetic AIS AtoN Stations

The criteria for deciding whether to use Real or Synthetic AIS AtoN Stations (or a hybrid of both) will depend on factors such as station type, power availability, transmit and receive requirements, monitoring requirements, reliability and continuity requirements and the availability of suitable locations.

For fixed AtoN, one method of providing an AIS AtoN service is by synthesised means from an AIS Base Station, providing that

- the fixed AtoN is within the coverage area of the transmitting base station, and

- there is an existing monitoring (non-AIS) system which can provide AtoN status data to the transmitting base station. However, in the absence of monitoring, Message 21 without the status information is still useful.

For floating stations, the preferred method of providing an AIS AtoN service is from a Real AIS AtoN Station. A Real AIS AtoN Station is a station that is fixed to the AtoN for which the AIS AtoN message is being broadcast. The power system on the AtoN must be able to support the extra equipment, and there must be sufficient space to mount the AIS AtoN Station itself and associated antennas.

If it is not possible to provide sufficient power on the station to support a Real AIS AtoN Station, then alternative means must be devised. This may consist of a monitored synthetic AIS AtoN Station, where monitoring of the AtoN condition is achieved by some other means, or combination of means. The AIS AtoN message is then transmitted from a nearby AIS station with available power.

It is also possible to use a hybrid of Real and Synthetic means where a Real AIS AtoN Station is fitted to the AtoN with an extended period between transmissions and the intervening transmissions are made from a nearby station.

Synthetic AIS AtoN Station broadcasts may also be used as a repeater where it is considered that extended range may provide additional value to the user. Whilst it is possible to broadcast a synthetic AIS AtoN message for an unmonitored floating AtoN, it is not recommended to do so due to the risk of transmitting misleading information.

Transmit and receive requirements will also dictate whether Real or Synthetic AIS AtoN Stations should be used. If there is a requirement for very frequent transmission, for example where a high speed craft would acquire an AIS AtoN Station broadcast at short range due to rounding a headland, then a Synthetic or hybrid solution may be required. If the AtoN is intended to repeat messages from adjacent stations, then clearly a Real AIS AtoN Station is required.

Monitoring requirements may also influence the choice between Real and Synthetic AIS AtoN solutions. Real AIS AtoN Stations will provide direct monitoring but will require some means of receiving and forwarding the messages to the control centre. Synthetic AIS will require separate monitoring but this may be provided using existing monitoring systems.

Reliability and Continuity requirements are also a factor. If the status of the AtoN can be confirmed independently, for example by radar or observation, then Synthetic or hybrid systems provide the ability to continue the AIS AtoN service even if the AIS AtoN Station on the AtoN fails. However, reasonably accurate, automated positioning information will be lost unless a separate position monitoring system is employed on the AtoN.

Availability of suitable locations is also a factor. Where there is no suitable adjacent station then Real AIS AtoN Station is the only option.

# 2.2 Virtual AIS AtoN Stations

Virtual AIS AtoN Stations will be used where a physical AtoN is not required or cannot be established, or where rapid deployment of emergency wreck marks is required, e.g. for new wrecks. The criteria for establishment of Virtual AIS stations are set out in Part 1 above.

2.3 Station Types

Recommendations ITU-R M.1371 and IALA A-126 are the primary sources for details of AIS AtoN Station and Message Types.

There are 3 types of AIS AtoN Station:

- Type 1 is a transmit only station,

- Type 2 is similar to Type 1 but has a receiver for remote configuration

- Type 3 is a full transmit and receive station

All three station Types are capable of switching off and sleeping between transmissions.

# 2.4 Criteria

The decision on station Type will depend on the level of service required. Station Types 1 and 2 can only be used if they are within the coverage range of a Base Station, for slot reservation purposes.

Type 3 Stations are required where the station is to be used as a repeater (e.g. for AIS AtoN Station configuration messages or future AIS Search-and-Rescue Transponder (SART) messages) or where there is a requirement to receive AIS data for monitoring or traffic analysis purposes, or where the AIS AtoN Station is outside the coverage of a base station and uses RATDMA.

Type 2 Stations provide very useful configuration functionality. In addition to the advantages of being able to remotely configure offshore AtoN, this would also provide the opportunity to modify, enable or disable message types and adjust transmission intervals where an AtoN has failed or moved off station.

Type 1 Stations are transmit only and are not configurable from ashore. They will typically offer lower power consumption than Type 2 or 3.

2.5 Message Types

The AIS AtoN message type options for transmission to the mariner are listed below.

Message 21 is the primary AIS AtoN information message. This message provides details of the identity, position and status of the AtoN and will warn the mariner if any of the AtoN functions are not performing correctly.

Message 12 is an Addressed Safety Related Message and may be sent from a Base Station or an AIS AtoN Station to an identified vessel to give them updated information in relation to the status of AtoN in the area.

Message 14 is a Broadcast Safety Related Message and may be sent from a Base Station or an AIS AtoN Station to all vessels within range to give them updated information in relation to the status of AtoN in the area.

Messages 12 and 14 supplement, but do not replace, other Maritime Safety Information (MSI) information such as Radio Navigation Warnings (RNW) and Navtex.

##### 2.6 Criteria

All AIS AtoN Stations transmit Message 21 as defined by ITU-R M.1371.

Where there is an associated Base Station, consideration should be given to providing Message 12 and 14 services either as a routine or when there is an AtoN casualty in the area.

3 – Additional Functionality.

In addition to the AIS AtoN Station itself, the platform can provide additional information to the mariner.

3.1 Message Types

## Message 8

Message 8 is a broadcast binary message. The International Maritime Organisation (IMO) has published a limited set of Message 8 formats for international use. In addition to these international messages, competent authorities are permitted to use other Message 8 formats on a regional basis.

The main Message 8 application of interest to authorities is the provision of Meteorological and Hydrological Data. Sensors on the AtoN, or in the immediate area, provide this data to the AIS AtoN Station, or a local Base Station, to be broadcast in the internationally agreed Message 8 format.

# Message 12

The use of Message 12 for promulgating AtoN information was considered in Part 2 above. Message 12 may also be used to transmit other safety related information. Likely uses include the repeating of AIS SART messages to the to the Search-and-Rescue (SAR) authority and provision of Meteorological and Hydrological information to identified vessels. This particular application arises from the fact that many receivers cannot decode the internationally agreed Message 8. Message 12 allows the transmission of this information as a text message addressed to individual vessels either on demand, on acquisition or on a schedule.

# Message 14

Message 14 is a Broadcast Safety Related Message. As with Message 12, the likely uses are repeating AIS SART messages or the transmission of Meteorological and Hydrological information in text format.

# 3.2 Criteria

The requirement for AIS SART repeating should be considered in consultation with the relevant SAR authority. It should also be noted that AIS SART is in still being developed and actual requirements may change in the future.

The requirement for Meteorological and Hydrological functionality should form part of the navigational review process. Typical areas where this functionality should be considered would include busy passenger and high-speed craft routes, areas adjacent to lifeboat and other SAR vessel stations, areas adjacent to major fishing ports (particularly where the port is removed from the true outside conditions), key offshore locations and busy leisure ports and harbours.

The Meteorological and Hydrological information provided can include Wind Speed and Direction, Wave Height, frequency and Direction, Tidal Height and tendency. The information provided will depend on the navigational review and the availability in the area of suitable platforms for the measurement devices. The accuracy of wave height data will depend on the measuring equipment used.

4 – Monitoring, Networks and Track Analysis.

4.1 Monitoring methods

There is a wide range of possible means of monitoring AtoNs. These range from physical observation, radar coverage, and electronic reporting through to full telemetry control systems. Whether an AIS AtoN Station is required for navigational purposes or not, AIS may still be used as an AtoN monitoring and networking tool.

Where AIS equipment at the AtoN is used for monitoring, consideration must be given to the possibility of the failure or the AIS itself while the AtoN continues to function. If a secondary monitoring system is available, any AIS messages being transmitted to the mariner may still be transmitted from an adjacent station as synthesised messages.

Networks of AIS Base Stations can provide a useful means of monitoring the integrity of transmitted AIS messages and for local storage of data.

The availability of detailed AIS vessel track information will contribute significantly to the navigational review process.

##### 4.2 Criteria

Where an AIS Base Station is being provided as part of the AtoN requirement, consideration should be given to utilisation of the station for monitoring of other AtoN in the area and for network, data storage and forwarding purposes. The criteria in this case are likely to centre on communications costs and reliability. While AIS data can be secured using time or event generated polling on dial-up lines, live streaming over broadband or similar high speed lines is preferred.

Where there is no AtoN requirement, the criteria for using AIS as an alternative means of monitoring will centre on cost, reliability and the value attached to the additional functionality available from the various systems.

In establishing an AIS station, consideration should be given to future as well as present requirements.