| Working Paper ANM 11 Task 1 | 2008 April 15th |
| --- | --- |

D R A F T

Guideline (Manual) on a minimum comprehensive mix of AtoN in fairways/waterways including dredged channels and canals

Background

A work programme item for the ANM Committee is to develop a Manual on channel design and a minimum comprehensive mix of AtoN.

Action Requested

ANM members are invited to discuss and revise this draft at ANM 11.

ANM 11 members are invited to discuss and distribute this draft in their countries, especially in their own National Authorities, for consideration and comments as input for ANM 12.

ANM 11 members are invited to make proposals for chapter 4.7 (examples from ANM 9 input and other examples).

D R A F T

Guidance on a minimum comprehensive mix of AtoN in fairways/waterways including dredged channels and canals

Table of contents

1	Introduction	4

2	Minimum requirements	5

2.1	General	5

2.2	Relevant IMO documentation	6

2.3	Relevant IALA Documentation	7

2.4	Relevant IHO documentation	8

2.5	Navigation Accuracy	8

3	Types and principles of Marking	10

3.1	Purpose of  Marking	10

3.2	General Principles of Marking	11

3.3	Principles for different Types of  Marking	11

3.3.1	Buoys and beacons	12

3.3.2	Leading lines	13

3.3.3	Sector lights	13

3.3.4	Electronic aids to navigation	13

This  sectionshall be written to define the possibilities of GNSS and other electronic aids with regards to the provision of accurate positions and their impact on conventional aids to navigation, especially proven provision of landfalls.…	13

3.4	Principles of Marking according to user groups	13

3.4.1	Commercial vessels	13

3.4.2	High speed craft	13

3.4.3	Uncertified Commercial Vessels	14

3.4.4	Leisure Craft	14

4	Description of typical types of fairways/waterways, dredged channels and canals and traffic patterns	15

4.1	Waters adjacent to the coast	15

4.2	Archipelagic waters	15

4.3	Straits	15

4.4	Estuarine rivers	15

4.5	Large river systems	15

4.6	Canals through narrow stretches of land.	15

4.7	Examples	15

5	Procedures for determining the requirements for short-range marine aids	15

5.1	Types of analysis and steps for each analysis	15

5.1.1	Site analysis	17

5.1.2	Needs analysis	17

5.1.3	Operational analysis	18

5.1.4	Simulation and GIS	19

- generating ‘what if’ scenarios – feasibility studies	20

5.1.5	Cost-benefit analysis	20

6	Conclusion	20

ANNEXES

...Examples for user groups, traffic patterns and so on

...Worked examples of marking

D r a f t

- Introduction

Until recently, mariners have used the radio aids to navigation systems (also referred as electronic aids to navigation) and the short range systems in two distinct geographic areas.  The short range system was mostly used close to shore and in restricted waterways.  On the opposite, offshore vessels were able to navigate safely using less accurate radio aids system.  However, considering the avaibility, reliability and relative low cost of high precision electronic aids systems (e.g. GPS/DGPS and electronic charting programs) available today, these two areas of navigation are beginning to overlap.  This is especially true in those transitional areas where mariners shift from the low accuracy requirements of ocean navigation to the high accuracy needs of coastal and inshore piloting.  As electronic aids continue to improve, their use will increase in areas where previously only short range aids provided the necessary accuracy.  This evolutionary change must be recognized and accounted for when conducting waterway analyses.

However, safe navigation by electronic means, does require a certain level of accuracy, availability and integrity in order to accurately portray the fairway/waterway, dredged channel or canal. There should also be an adequate level of  integration with the control of the ship either automatically or through the navigator. As there are uncertainties connected with this, visual navigation eventually combined with the use of radar is in principle either a primary or a secondary means of navigation.

So, despite the technical changes there remains a significant need to provide mariners with navigational information when piloting within restricted waters.  The best manner in which to provide information to this level of precision is through the use of a short range aids to navigation system consisting of buoys, beacons, sound signals, and electronic systems such as RADAR beacons (RACONS) , Automated Information System (AIS) devices and GNSS.

This guideline cater for this and provide guidance for Aids to Navigation Design regarding fairways/waterways including dredged channels and canals which should be both relevant to the mariner and cost effective to the National Authoritites. In order to cover different situations, the guideline is quite broad.

Aids to navigation systems may be provided for the safety of navigation in various kind of  waters such as:

- Waters adjacent to the coast

- Archipelagic waters

- Straits

- Estuarine rivers

- River systems

- Canals through narrow stretches of land.

- Fairways with natural navigation channel

- Dredged channels and canals

- Open Sea with Ship Routeing Systems

Different users are considered, as there are ships, vessels, boats etc. facilitating commerce, fishing, leisure and a number of other activities that can typically be found in marine environments

Aids to navigation systems may be established to:

a) assist landfall,

b)mark approaches to harbours, ports and waterways

c) mark channels or tracks

d) mark hazards

e) identify positions or courses

f) indicate preferred routes

g) separate traffic (e.g. to mark traffic separation schemes noted by specialized

symbols on Hydrographic charts and/or IMO Routeing Systems)

h) indicate special areas such as anchorages

- Minimum requirements

  - General

The primary goal of the design of waterway aids to navigation systems is to facilitate safe, economic and efficient movement of vessels.  Aid to navigation systems are designed to assist navigation and not to replace normal prudent navigational practices or the use of onboard navigational equipment.

The aids to navigation provided are normally intended to function in systems and therefore mariners should make use of the full information provided. These systems are intended to be used by the prudent mariner who knows and understands the

functioning of aids to navigation, uses them in conjunction with charts and other

published information, and conforms to the applicable Rules of the Road.

Whether designing a new waterway system or evaluating an existing one, there are many factors that must be considered.  The identification of these factors allows National Authorities to develop a greater understanding of the risks and threats that are present within a particular waterway.  Section 5 of this manual provides guidance for completing this analysis.  Recognizing that waterways are different, the site analysis, needs analysis, simulation, and operational analysis provide the necessary frameworks to generically evaluate the overall risks that may be present and provide mitigation measures..

These factors include the consideration of

- user “needs”,

- shoreside characteristics,

- physical and environmental condition of the waterway and

- significant traffic types and density using the waterway.

Once the evaluation has been completed, National Authorities should use this information to design the aids to navigation system that mitigates the risk to safe transit or reduces it  to an acceptable level.  In completing the design it is important to note that the entire waterway must be viewed using a systematic approach, recognizing that each individual element of the waterway design by itself will not reduce transit risk.  So while individual regions of the waterway must be considered, the overall aids to navigation system must support a smooth transit of the waterway as a whole. This way, the waterway must be examined “end-to-end” to ensure that an acceptable and uniform level of risk is present throughout the waterway. While the analysis process is a tool for the assessment of risk, it should be the National Authorities objective to design an aid to navigation system best able to manage those risks.

The specific aids to navigation system implemented should enable waterway users to transit an area safely and efficiently, while avoiding groundings, obstructions to navigation, and collisions with other vessels. Therefore, in order to satisfy the information requirement of the user, a system of aids to navigation must:

- Be available at the time it is needed

- Provide timely warning of danger from channel limits and fixed obstructions to navigation

- To quickly enable the mariner to determine their location within the channel, relative to fixed obstructions to navigation, and relative to other vessels

- Enable a safe course for the vessel to be determined

Once the system has been established, maintaining the availability of this system is critical to controlling overall risks.  Please refer to IALA Recommendation O-130 on short range AtoNs for additional information related to the categorization of individual aids to navigation, the calculation of availability targets, and recommended availability objectives.

  - Relevant IMO documentation

According to SOLAS, Chapter V, regulation 10, Ships' routeing systems contribute to safety of life at sea, safety and efficiency of navigation and/or protection of the marine environment. Ships' routeing may be established by the coastal state within its territorial sea. However, routeing systems are not always marked with physical AtoNs.

In the Exclusive Economic Zone and on the High Seas IMO is the competent body for Ships' routeing measures. Normally countries also submit Ships' routeing proposals that are within their territorial sea to IMO for adoption.

According to SOLAS, Chapter V, regulation 13, each of the Contracting Governments undertakes to provide, as it deems practical and necessary, Aids to Navigation  as density of traffic and the level of risk requires. IMO member states commit themselves to take into account the international recommendations and guidelines when establishing such aids.

According to the same regulation, Contracting Governments undertake to arrange for information relating to aids to navigation to be made available to all concerned. Changes in the transmissions of position-fixing systems which could adversely affect the transmissions of position fixing systems which could adversely affect the performance of receivers fitted in ships shall be avoided as far as possible and only be effected after timely and adequate notice has been promulgated.

Information on A to Ns has to be distributed to all concerned. Changes in the transmission of position fixing systems which could adversely affect the performance of receivers fitted in ships shall be avoided as far as possible.

According to IMO-resolution A. 915 (3), the position accuracy (95 %) for Global Navigation Satelite Systems for general navigation should be:

- Ocean			10 -100 m

- Coastal			10 m

- Restricted Waters		10 m

- Port			1 m

- Inland Waterways		10 m

  - Relevant IALA Documentation

The requirements of the IALA Maritime Buoyage System should be fulfilled in accordance with IMO requirements.

IALA Navguide, offers overall guidance.  In particular, chapters

3 on Aids to Navigation and 6.1.3 on Risk Management.

Following lists are to be checked and be confirmed:

IALA Recommendations

E-110 “For the rhythmic characters of lights on aids to navigation” (Edition 2.0 December 2005)

E-111 "For port traffic signals" May 1998.

O-130 "On categorization and availability objectives for short range aids to navigation", December 2004,

E-105 "On the need to follow national and international standards", May 1998

E-112 (with excel program)

O-138 on the use of simulation.

IALA Guidelines

1023

1058 Use of Simulation as a Tool for Waterway Design and Aids to Navigation Planning, December 2007,

1051 On the provision of Aids to Navigation in Built-up Areas; December 2005,

1033 On the provision of aids to navigation for different classes of vessels, including high speed craft, December 2003.

1018 on risk management

Other IALA recommendations and guidelines on VTS, AIS, Racons, DGPS must be considered.

  - Relevant IHO documentation

  - Navigation Accuracy

As indicated in IMO resolutions, the position accuracy (95 %) for Global Navigation Satelite Systems for general navigation should be:

- Ocean			10 -100 m

- Coastal			10 m

- Restricted Waters		10 m

- Port			1 m

- Inland Waterways		10 m

These accuracies correspond to an earth fixed earth centred reference system.

The visual and radar aids to navigation are relative to the user and the concept of accuracy may be different. For both instances, the result is referenced to the nautical chart. The position of the aids to navigation should be accurate and in accordance with IHO standards so that a ship sufficiently establishes its position and follows a track in the fairway by visual means or radar. The ability of a ship to stay on track within a fairway/waterway with the available aids could be used as a standard.

The concept of accuracy differs between methods and is in general more developed for the nautical chart and the GNSS. That is the case where nautical charts are surveyed to high accuracy and the ships position is available throughout the voyage and with high accuracy by the use of GNSS. The aids to navigation should be surveyed using the same accuracy as the nautical chart. The position obtained by the use of visual and radar aids cannot remain constant along the track. The concept of accuracy is in this case the ability to keep the intended track and to locate the movement of the ship along this track.

The position for the location of the fixed aid should be first determined on the nautical chart. The position of fixed aids to navigation should be made using existing standards of accuracy.

The fixed position and therefore the navigation accuracy of floating short range aids to navigation is sometimes difficult to define absolutely. The required level of accuracy is dependent upon the navigation information the aid is designed to provide, the availability of other aids and landmarks within the immediate vicinity and the overall level of risk that may be present within a particular section of the waterway.

Relative versus absolute accuracy.

Floating aids to navigation, on the other hand, are subject to a much greater degree of variation due to the capabilities of the servicing craft or vessel and of course depending on water level, tides and current.  Specifically, the servicing vessel or craft may not always be capable of returning a floating aid to navigation to its charted position.  The following section provides recommendations to National Authorities as to how close to the chart position the floating aid should be placed when returning it to its assigned location.

Desired Positioning Tolerance. (DPT) is the radius of a circle that describes the maximum allowed error for positioning the aid. It is based on the navigational needs of the mariner and the service that the aid provides.

The first consideration is the Waterway Risk Level as determined through factor analysis detailed in section 5. There are three waterway risk levels: great, moderate and low. The waterway risk level is based on the type and volume of traffic prevalent at the aid location, the environmental sensitivity of the water  way, and the bottom type (hard or soft), and the width of the channel compared to the beam of the average vessel that will transit the waterway.  The Width to Beam (W/B) ratio relates the size and maneuverability of the vessel to the physical size of the waterway.

Great Risk:

- Traffic Density. The amount of vessel traffic is substantial. Vessel congestion can be a problem.

- Type of Traffic. Vessel traffic consists of various types including large commercial vessels. Vessels containing hazardous cargo whose grounding and subsequent discharge would pose a substantial threat to public health or welfare, or result in critical public concern.

- Environment. The waterway has environmentally sensitive areas and endangered species areas.

- Waterway Conditions. Wind, wave, tide and current.

- Bottom Conditions. Hard bottom conditions (rock, coral, wreck, etc.) exist where the likelihood of holing a vessel is great.

- W/B Ratio.  Less than 3 in one-way channels or less than 5 in two-way channels.

Moderate Risk.

- Traffic Density. The amount of vessel traffic is moderate. Vessel congestion is not a problem under normal waterway conditions.

- Type of Traffic. Vessel traffic consists of commercial fishing fleets and smaller commercial vessels carrying non-hazardous cargo.

- Protection of Environment . The waterway has little effect on environmentally sensitive areas.

- Waterway Conditions. Wind, wave, tide and current.

- Bottom conditions are such that a possibility of holing a vessel exists if a grounding occurs.

- W/B Ratio.  Between 3-5 in one-way channels or between 5-8 in two-way channels.

Low Risk.

- Traffic Density. The amount of vessel traffic is light. Little or no vessel congestion exists.

- Type of Traffic. Vessel traffic consists of small pleasure craft, fishing boats and small commercial vessels carrying non-hazardous cargo.

- Environment. A marine casualty on the waterway will have little or no effect on environmentally.

- Bottom Conditions. A soft "forgiving" bottom makes the likelihood of holing a vessel remote if a grounding occurs.

- W/B Ratio: Greater than 5 in one-way channels or greater than 8 in two-way channels.

The second consideration is the Geographic Area Type. There are three area types:

- Area Type 1: Area Type 1 is a narrow or restricted waterway such as channels, rivers, canals and straits.

- Area Type 2: Area Type 2 has more vessel maneuvering room but may still be restrictive such as harbors and coves.

- Area Type 3: Area Type 3 has little restriction on the maneuverability of vessels which transit the area such as coastal regions and fairways.

Some waterways may fit into more than one category. In these instances National Authorities should determine the appropriate Area Type depending on the

size of the waterway and the importance of the aids.

The following table which is used by the USCG can be used to determine the Desired Position Tolerance for each floating aid to navigation. (The Table has to be checked against IALA Guideline 1018.)

- Locate the Area Type.

- Compare the waterway risk level

- Desired Positioning Tolerance is shown in meters

|  | Great Risk | Moderate Risk | Low Risk |
| --- | --- | --- | --- |
| Area Type 1 | 10 | 25 | 50 |
| Area Type 2 | 25 | 50 | 75 |
| Area Type 3 | 50 | 75 | 100 |

- Types and principles of Marking

  - Purpose of  Marking

The purpose of marking is to:

a) assist landfall, mark approaches to harbours, ports and waterways

b) mark channels or tracks

c) mark hazards

d) identify positions or courses

e) indicate preferred routes

f) separate traffic (e.g. to mark traffic separation schemes noted by specialized

symbols on Hydrographic charts and/or IMO Routeing Systems)

g) indicate special areas such as anchorage

The responsible provision of aids to navigation systems requires that systems be designed to meet the essential minimum requirements for safe and expeditious navigation through specific waters in accordance with the type and volume of traffic. In general, redundancy within, and between systems is to be avoided. Notwithstanding, duplication in function may be appropriate and necessary to

provide a degree of safety in the event of a discrepancy and to avoid excessive costs

of emergency repairs. Moreover temporary duplication may be provided when new or

alternative types of aids are being introduced in order to allow a safe transitional period.

  - General Principles of Marking

It is essential, that the Aids to Navigation be identified and located on charts (ENC, paper chart, synthetic radar screen).

In narrow or winding passages where the mariner has no time to correlate the vessel’s position with chart information, the navigational marks should be explicit and clear so that navigational decisions can be made directly using visual and/or radar information.

Short Range Aids to Navigation, especially buoys, should be designed regarding their visual information, radar information (active or passive) and other modes of information (Radionavigation, AIS).

In designing and redesigning systems, past incidents such as groundings, collisions or near-misses must be carefully considered. Such incidents should be well documented to ensure accuracy of the information used for a decision to change or not to change the configuration of aids in a system.

Aids to navigation systems should be designed to assist mariners under various weather, sea and ice conditions found during the local navigation season for the various types of vessels. Design elements must take into account the visibility and radar availability implications where buoys are removed for part of the navigation season and may be replaced with ice spars.

Designing for worst case visibility is not practical. Reduced visibility due to haze and fog must be considered.

During the design phase the required level of availability should be considered.  The level of availability is dependent upon the threats and risks that are mitigated through the use of a particular aid to navigation.   In those areas in which the level of risk has been determined to be high, the use of certain types of aids to navigation may prove to provide greater risk mitigations.  However, the planner must also consider the higher availability objectives that may also be required.

Reliance upon a single aid to navigation may result in a higher availability requirement which may prove difficult for the National Authority to meet.  If this is the case, then the implementation of multiple aids may be used to provide redundancy during periods of unavailability by certain aids.

  - Principles for different Types of  Marking

The fairway area can be marked in different ways, using the right mix of conventional short range aids to navigation and electronic aids to navigation:

- Consequent Lateral Fairway Marking

- Marking the critical points

- Marking the (middle) line(s)

- Start of the fairway

Each of these types of marking can use

- leading marks, which are not physically in that place they indicate,

- lateral or cardinal marks, which are physically in or very close to that place they indicate and

- safewater marks, which are physically in that place they indicate.

- Other aids to navigation including radio aids.

- AIS

One advantage of the edge marks is that they are well visible with shipbourne radar. One disadvantage of edge marks is that they can be damaged by ice, current, collision.

    - Buoys and beacons

The principles for buoyage can be also used for beacons if the beacons are situated directly on the fairway.

The types of buoys are described in the IALA Martime Buoyage System, which should be consulted for mandatory toolbox for buoyage. Thus the systematic, consistent and clear use of floating Aids to Navigation should be ensured.

If high navigation accuracy or a very clear and continous buoyage is required paired or “gated” buoys should be preferred.

The design of a continous buoyage of a fairway which is formed by straight lines and bends can be done in three steps:

1.	Select a conspicious buoy or pair of buoys for the point where the fairway starts.

2.	Place buoys on points

  - where ships have to divert their course or

  - where the fairway boundary line or the middle line has a bend or

  - where critical shallows and rocks or other hazards form the boundary of the fairway.

  - Fairway intersections

3.	Distribute buoys between these points according to their observation distance.

The observation distance at daytime depends on the dimensions of the buoy. It can be calculated from the assumption that the human eye is unable to distinguish an object under an angle of 2 or 3 minutes. The dimension of the object in this formula is the diameter of a circle with the same area as the visible shape.

| Buoy dimensions | Observation distance |
| --- | --- |
| To be filled in |  |

Edge Marking (drawings from Finland input ANM 9/6/6 and 9/6/7) including more of it here?

Safewater Marks indicate navigable water all around. They can be used to indicate the start of a fairway or to indicate the fairway line.

    - Leading lines

Leading lines (lighted or unlighted) can be established if there are stretches that form a straight line. Leading lines should be established

- if the buoyage could be affected by ice,

- if there is a channel inside the fairway, which has to be used by ships with big draft

- or .....

    - Sector lights

    - Electronic aids to navigation

- This  sectionshall be written to define the possibilities of GNSS and other electronic aids with regards to the provision of accurate positions and their impact on conventional aids to navigation, especially proven provision of landfalls.…

  - Principles of Marking according to user groups

    - Commercial vessels

These are vessels, used for commercial activities, which operate with charts, sailing

directions and which are equipped with certified on-board navigational aids such as a

standard compass, radar and radio positioning equipment (in accordance with

applicable regulations) to support long-range and/or low visibility navigation. These vessels are operated by professionally trained and certified personnel.

    - High speed craft

DEFINITION of HSC according to IMO has to be included here !

The following paragraph is a direct extract from Canadian Coast Guard design policy. It may need discussion in ANM 11 and revision from ANM:

Aid to navigation systems provided for certified commercial vessels operating in

confined waters shall be designed, where feasible, with a visibility range under long

term local weather conditions such that Design Availability will be 75%. Where adverse weather conditions occur sufficiently often that this target cannot be achieved, per se or cost effectively, by visual means alone, radar aids may be added to the system to improve availability. Visual systems for this category of vessels will not have a design visibility level of less than 0.5 mile. Where traffic continues to move when visibility is reduced below the design visibility level, radar aids may be added to the system as needs and usefulness of the radar aids are confirmed through user consultation. Design visibility levels for the straight sections of a channel may be lower than 75% wherever such radar aids are provided and DGPS coverage is available.

    - Uncertified Commercial Vessels

These are vessels being used for commercial activities, to which the applicable

regulations do not apply in terms of requirement for on-board navigational aids. Such vessels operate with charts and are normally equipped with a compass and timepiece suitable only for short range navigation however may be equipped with navigational aids prudent to safe navigation in the vessel’s operation area. These vessels are normally equipped with a searchlight where night-time use of reflective short range marine aids is intended or may be necessary and are operated by personnel who may or may not have a certificate and who rely to a considerable degree on local knowledge to navigate safely.

The following paragraph is a direct extract from Canadian Coast Guard design policy. It may need discussion in ANM 11 and revision from ANM.

Aid to navigation systems provided where uncertified commercial vessels operate shall be designed, where feasible, with a visibility range under long term local weather conditions such that Design Availability will be at least 75%. Visual systems for these vessels will not have a design visibility level of less than 0.5 nautical mile. Where adverse weather conditions occur sufficiently often that this target cannot be achieved, per se or cost effectively, by visual means alone, sound signals (i.e., by horn, bell or whistle) may be added to the system to attempt to meet this availability.

    - Leisure Craft

These are vessels being operated for pleasure activities by the owner or under

charter, rent or loan. They operate with charts and are normally equipped with a

compass and a searchlight where night-time use of short range marine aids is

intended or may be necessary.

The following  paragraph is a direct extract from Canadian Coast Guard design policy. It may need discussion in ANM 11 and revision from ANM.

Aid to navigation systems provided for use by pleasure craft shall be designed, where feasible, with a visibility range under long term local weather conditions such that Design Availability will be 75%. However such targets will not be met by designing systems for less than 1 mile visibility. Pleasure craft will be supported with visual aids only.

Further idea for discussion:

For a sailing boat the fairway should be as wide as possible to enable travelling in different wind conditions. As a minimum requiremet edge marks must principally be located at rocks and dangerous shallows.

For a motorboat – due to high speed - the preference may be that marking or buoyage is as clear as possible, continuous and visible from far away.

For fishery craft the marked fairway might be wished to be as narrow as possible, because they are allowed to fish outside the fairway.

- Description of typical types of fairways/waterways, dredged channels and canals and traffic patterns

  - Waters adjacent to the coast

  - Archipelagic waters

  - Straits

  - Estuarine rivers

  - Large river systems

  - Canals through narrow stretches of land.

  - Examples

The WG feels that it will be not possible to define a general minimum standard without considering the conditions of the waterway on a case by case basis.

It should nevertheless be tried to find some examples (typical) for marking and other A to Ns.

These examples can be used by IALA members as a first idea, how A-to-N should be in typical situations. These situations will be described for each example or “model”, so that it can be checked if they are adequate for and comparable with the existing situation.

... should these examples be in the ANNEX ?

- Procedures for determining the requirements for short-range marine aids

Procedures for determining the requirements for aids to navigation at any site are based upon four types of analysis.

  - Types of analysis and steps for each analysis

The design methodology can be described as a step by step procedures organized in a logical sequence in order to justify the conclusion and recommendations in determining the requirements for short-range aids to navigation.  These procedures consist of the following four analyses:

Site Analysis (5.1.1)

An analysis of users, traffic patterns, weather and possible navigational threats at the site.

The physical and environmental condition of a waterway, as well as the size of the vessels using the waterway, must be considered.

Needs Analysis (5.1.2)

An analysis of user needs for navigational markers and signals at the site (needs analysis) to help mariners cope with navigational threats.

Operational Analysis (5.1.3)

An analysis of outputs of various combinations of short-range aids that could be provided at the site (operational analysis) to identify feasible means of meeting user needs

Simulation and GIS may be used (5.1.5).

Cost-Benefit Analysis (5.1.4)

A comparative analysis of cost of effective combinations of aids (cost-effectiveness analysis) to select between viable alternatives.

In brief, the steps to be followed in each of these procedures are:

- Specify the navigation site to be analyzed by identifying the user’s traffic patterns and plotting the track or tracks on the chart. Gather all user and site information such as size, type and number of vessels, type, size, output and characteristics of all established aids to navigation, weather and sea conditions, previous reviews, marine incidents, requests, etc.

- Identify and classify the size and number of users according to the three basic categories.

- Rate the threats at each site for each type of user, using guidelines and professional judgement.

- Identify any local means, other than short-range aids to navigation, that may meet any of the needs identified.

- For each of the threats in the waterway, identify the related navigation needs and the aid or aids which address these needs.

- Identify any threats that are not adequately reduced and the recommended aids or other means that could meet the navigational need.

- Identify any aids to navigation that are provided where the threats are not significant or service is redundant and recommend action to reduce or discontinue aids.

- Calculate the coverage and hazard marking requirements at any open water landfall situation, for each user group.

- Calculate the equipment output required to achieve the required visual, aural and radar coverage at the specified level of availability.  Identify where the requirements are not being met with established equipment and propose viable options.

- Calculate the size and output requirements for each aid to navigation established or proposed, in the confined section of the waterway, to meet the proposed availability for each user group. Identify where the proposed levels are not being met and propose viable options.

- Establish the comparative direct costs of each proposed alternative, to assist in determining the most cost-effective system of aids to navigation.

    - Site analysis

The purpose of this first analysis is to gather site specific data for the review work.  In this phase, the following steps should be followed:

- Identify & classify the size, types & number of users

- Assess the volume of traffic and identify traffic patterns including navigation seasons and or trends for each users category.

- Obtain weather & sea conditions including tidal and current information, wind and reduced visibilitiy

- Obtain information on the physical geography of the river, waterway or shoreline such as silting, distinctive shore features, existing charted landmarks etc.

- Retrieve related historical information such as previous reviews, marine incidents, requests for aids to navigation, etc.

- Prepare an inventory of all aids provided by the authority at the site if applicable. This may include also private aids to navigation.

- Identify and rate specific threats to identify potential hazards for each type of users. This initial rating considers each of the following threats individually:

  - Minimum depth allowance (considering the vessel draft, wave height and under keel clearance)

  - Distance from hazards and from other vessels when passing

  - Minimum channel width and angle of turn in channel

  - Wind speed, wave height, current along and across track and restricted visibility.

    - Needs analysis

Once the site analysis has been completed, the authority should proceed with the analysis of the mariners needs.  This is where the authority establishes user requirements for safe and accurate navigation.  It is at this stage of the process that the authority:

- Consider the interactive effects and unique features of the site by rating the following composite threats according to a pre-determined scale:

  - Sea Conditions

  - Proximity of Hazards

  - Complexity of Channel

  - Diminished Room to Manoeuvre

  - Traffic Density / Mix / Crossing

  - Reduced Visibility

  - Darkness

  - Ice and Freezing Spray

  - Channel Siltation

  - Lack of Distinctive Feature for Positioning / Pilot Boarding Station

- Identify related needs, noting the most appropriate generic types of aids to reduce the threats (e.g. visual marker on relevant hazards, lighted visual marker for lateral motion, visual boundary marker, Aural signal on relevant hazard, Radar target, etc…) as well as rate their relative effectiveness in reducing each type of threat for each category of users.

- Note and assess to which extent local means and natural features (e.g. towers, spires, landmarks, etc.) can meet the needs,

- Note the specific aids provided (if there is an aids system already in place) and assess against the threats and/or the generic needs.

A five point scale is used for the purpose of rating composite threats. Under this scale, threats are rated as follows: 1 - Overriding Importance; 2 - Highly Significant; 3 - Significant; 4 - Little Significance; and 5 - Not Applicable.

The generic types of markers and signals that serve to reduce the various categories of composite threats have been rated in consultation with users for their effectiveness in reducing these threats for each user category.  These ratings are:

A. 	Greatly reduces the threat.

B. 	Significantly reduces the threat.

C. 	Somewhat reduces the threat or reduces the threat for some vessels in the category (e.g., radar targets will assist those uncertified commercial vessels who are equipped with radar but do not qualify as Category I vessels because of their lack of other electronic navigation aids, etc.).

    - Operational analysis

Through the operational analysis, the authority is able to examine the outputs of various combinations of aids to navigation, compare the end results in relation to the needs of mariners for the specific waterway and consider the best possible option.

The operational analysis is conducted by performing a series of steps to:

- Determine the system requirements (landfall or confined waters)

- Determine  the perception requirement (coverage)

- Compare the existing aids against the calculated requirements

- Determine the needs to meet design visibility (Where is design visibility defined ?)

- Examine and test options and or combination of aids to navigation

- Recommend final optimal system design.

- When reviewing an existing system, compare optimum design with existing system making appropriate observation and/or recommendations.

To complete the above steps, the following actions are also required:

- Calculation of the coverage and hazard marking requirements (perception requirement) for each type of users ensuring that said coverage is sufficient to allow the largest user time to complete an evasive manoeuvre.

- Calculation of the equipment output required to achieve the required visual, aural and radar coverage to meet the calculated perception requirement.

- Calculation of the size and output requirements for each aid to navigation established or proposed to meet the design availability for each user group.

    - Simulation and GIS

Prior to implementing a new AtoN system or changing an existing one, National Authorities can use simulation techniques to assess the overall safety and effectiveness of these changes. The use of Geographic Information System (GIS) technology can improve efficiency of AtoN deployment and waterway layout which, with volume of traffic overlaid (e.g taken from AIS data), can be used to assess risks and plan the disposition and type of AtoN to mitigate such risks and improve the provision of AtoN for all users. Having planned aids to navigation in this manner, the authority can use modern simulation tools where passage and combinations of various types of vessels can be simulated, in order to conduct a validation process, in consultation with appropriate stakeholders. To achieve a high level of realism in the simulations, GIS data can be integrated to the waterway models used in the simulator.

One of the purposes of simulation in relation to waterway design is to evaluate planning of placement and design of AtoN in a specific waterway, channel or port area. Other purposes could be  e.g. ensuring sufficient channel width, channel depth, and optimal orientation and design of breakwaters as well as ensuring that the lay-out of a channel and port is suitable seen from a manoeuvring point of view.

Simulation offers a method to help ensure that AtoN are appropriate and cost effective. Sophisticated computer simulation techniques are becoming increasingly available, and they provide an excellent mechanism for decision making as follows:

- - generating ‘what if’ scenarios – feasibility studies

- obtain input to placement and design in consultation with stakeholders

- validation of final design and lay-out

- training in a variety of areas.

Simulating the placement and operation of AtoN by day and night, and under various conditions of visibility assists in ensuring that AtoN are effective and provided in a cost effective manner that suits the purpose of providing a predetermined level of safety. This is particularly important as aids to navigation become more sophisticated (synchronised and sequential lights, LED with flicker, and other new light characteristics).

Please see IALA Recommendation (O-138) for further discussion on the use of simulation in waterway design and placement of AtoN. The recommendation includes a reference to a guideline that amongst other outlines how to select the right simulation tool in relation to the objectives and expected outcome of the simulations.

    - Cost-benefit analysis

The last phase of the design methodology consists of a cost-benefit analysis which is a comparative analysis of costs of the various scenarios evaluated during the previous steps. This cost-benefit analysis should be done using values and cost factors as accurate as possible.

At the end of the process, the design methodology produces various options for aids system, various combinations of aids and associated costs ready for consultations with the affected mariners.

- Conclusion

The art of navigation has evolved over many years, as well as ships, vessels and various other craft.

In principle navigation comprises:

- to plan a track / passage for a ship by the use of a nautical chart,

- to monitor or establish the ships position or advance along a predetermined track / passage, and

- the control of the ship, e.g. that to secure that it follows a predetermined track / passage.

To navigate there are a number of "tools" available:

- clock

- log

- compass

  - magnetic compass

  - gyro compass

- radar

- nautical charts

  - paper chart

  - electronic navigational charts

- visual and radar aids to navigation

  - single point aids (lights and daymarks)

  - sector lights

  - ranges

  - racons and radar reflectors

- flood lightning of conspicuous objects

- Global Navigation Satellite Systems (GNSS)

- local and regional electronic navigation systems

- auto pilot

Of these "tools" some can be alternate to another tool while some can be complementary to another tool. That is the tools can be used in various numbers and combinations. Subsequently principally a primary and secondary method of navigation is often used. Depending on the situation the primary method could be either a visual/radar based method or a GNSS based method.

The appropriate nautical chart is of fundamental importance to find a safe track/passage and later as a reference during the ships movement. It is also fundamental to have a compass, clock and log to be able to follow and monitor the advance along the track. To establish a position visual or radar aids can be used either independently or in combination, although for instance a Global Navigation Satellite System can also be used.

The navigation process is normally performed by the navigator integrating the chart, navigational data and the control of the ship. In this case visual and radar aids normally play an important role. On some ships this process is fully automated, relying completely on electronic/digital "tools." This put a demand on accuracy on the chart, the navigation system and the control system, that cannot be fulfilled under all circumstances today. In general it is considered that proper marking of waterways/fairways, dredged channels and canals by visual and radar aids is important to reduce the risk posed by the waterway and the traffic.

The concept of accuracy differs between methods and is in general more developed for the nautical chart and the GNSS. That is the nautical chart is surveyed to a high accuracy and the ships position will be available throughout the voyage and with high accuracy by the use of GNSS. The aids to navigation will be surveyed with the same accuracy as the nautical chart, but the position obtained by the use of visual and radar aids will not remain constant along the track. The concept of accuracy will in this case be the ability to keep the intended track and to know the advance along this track, leading to the provision of the right mix of conventional and electronic aids to navigation by the National Authorities to the mariners.

WELL DONE HENDRIK!!