Suggestions to the revision of IALA MBS

from China MSA

1. Suggest to redefine the special mark

1) To define the special mark as an aids to navigation mark.

The reason is that this kind of marks are used very often recent years in China. The special marks have worked  well in marking the anchorage boundary,oil pipe line , under water cable. They do have the function to provide navigational information for mariners.

2) The special marks should be distinguished their special purposes with marks on the upper structure to indicate the meaning . The followings are provided for consideration.

| Purposes | Mark type |  | Light character |  |  |
| --- | --- | --- | --- | --- | --- |
|  | color | Pattern of mark | Color | rhythm | period（s） |
| Anchorage | black |  | Yellow | Morse code “Q” --••- | 7 |
| Navigation prohibited area | black |  | Yellow | Morse code “P” •••--• | 7 |
| Marine operation | red/white |  | Yellow | Morse code “O” - - - | 7 |
| Traffic Separation zone | black |  | Yellow | Morse code “K” -•- | 7 |
| Underwater constructions | black |  | Yellow | Morse code “C” -•-•- | 7 |
| Recreation zone | red/white |  | Yellow | Morse code “Y” -•- - | 7 |
| Aquatic farming area | 黑 |  | Yellow | Morse code “F” ••••• ••••••- • | 7 |
| Small boat crossing area | 黑/白 |  | Yellow | Morse code “Z” - - • •••••• | 7 |

The option light period 12 is also available.

2. Suggest to add the Emergency Wreck Marking Buoy (EWMB)  into the MBS

China supports to add the EWMB into the MBS as a new kind navigational mark to mark the wreck only.  The reason is that this mark is very distinct and easy to identify for mariners.

The EWMB should remain on station for as long as required. For other new emergency dangers , they could be marked according existing rules in MBS.

3. For region A and B

The existing buoyage system may not possible to merge as one system for the whole world. But China encourages to unify the system as one in some adjacent regions. In such a way, mariners may avoid the possibility of confusion in a short voyage.