Draft Contribution of the Architecture TWG of the e-Navigation Committee to IALA NAVGUIDE

- The architecture of the e-Navigation – the shore perspective

  - IMO’s definition of e-Navigation and the information orientation

IMO adopted the following definition of e-Navigation :

“e-Navigation is the harmonised collection, integration, exchange, presentation and analysis of maritime information onboard and ashore by electronic means to enhance berth to berth navigation and related services, for safety and security at sea and protection of the marine environment”

Obviously, the key element of the e-Navigation concept is the information to the user and how it is presented to the user. What information actually constitutes is difficult to define: The probably best definition of information is as follows: “Information is the difference that makes a difference” to the user of that information.

Given this importance of the information to the e-Navigation to the user, the information flow between applications, which constitute sources and sinks for information, or both simultaneously, is a key to understanding the e-Navigation concept.

Therefore, the e-Navigation concept is quite abstract. Hence, also any formulation of the architecture of the e-Navigation must by necessity be abstract. This section tries to boil down the level of abstraction and thus apply the e-Navigation concept to what IALA is doing.

  - The e-Navigation architecture

A representation of the e-Navigation architecture employing the information flow concept is shown in Figure 1 (overleaf). The shipboard entities, the physical link(s) and the shore-based entities are included in this representation of the e-Navigation architecture.

On the left within Figure 1 is represented, for simplicity’s sake, a single ship technology environment. From the e-Navigation concept’s perspective the relevant devices within the ship technology environment are the transceiver station, the data sources and the data sinks connected to the transceiver station, the Integrated Navigation System (INS) and the Integrated Bridge System (IBS). The transceiver station is shown as a single station for simplicity’s sake, although there may be several transceiver stations.

The shipboard transceiver station interfaces with the physical link(s) to the appropriate technical e-Navigation services ashore.

The shore-based technical e-Navigation services, in their totality and by their interactions, provide the interfaces of the shore-based user applications to the physical link(s). They also encapsulate their technology to the whole of the common shore-based e-Navigation system architecture. The encapsulation principle hides the technology’s sophistication from the shore-based e-Navigation system as a whole and thus reduces complexity. In Figure 1, the entities which are involved with the specifics of the link technology are confined by the dotted line. Amongst other benefits, it allows for parallel work of the appropriate experts in the particular technology of a given physical link, provided the functional interfaces of the shore-based technical e-Navigation services are well defined. This is a very important point for the work of IALA.

For the precise technical structure of the shore-base technical e-Navigation services is currently under development as a common shore-base e-Navigation system architecture at IALA.

The shore-based technical e-Navigation services use data transfer network(s), which distribute(s) the data used by the shore-based technical e-Navigation services. The requirements for data transfer are dependent on the specifics of the technology of the technical shore-based e-Navigation service. For example, a shore-based Radar Service would require different data transfer network capabilities than a shore-based AIS Service. Hence, the individual requirements for data transfer network of a shore-based technical e-Navigation service are also encapsulated. This also reduces complexity.

It should be noted, that the shore-based technical e-Navigation services mutually exchange data between them at Nodes or Node sites by means of local area networking.

The operators ashore, e. g. VTS operators, need the user applications to perform their tasks in co-operation with shipboard applications. From their perspective, it is neither the physical links nor the shore-based technical e-Navigation services that matter – they have been encapsulated for that reason. It is the functional links between the shore-based user applications at hand and the peer shipboard applications which matter to the users on both sides.

Figure 1 also shows IMO’s World Wide Radio Navigation System (WWRNS) being presented as a system “external” to the e-Navigation architecture providing position and time information. The WWRNS includes GNSS but also augmentation radio navigation systems as well as terrestrial backup radio navigation systems. Obviously, e-Navigation is heavily dependent on the WWRNS.

IBS

World Wide Radionavigation System (WWRS) of IMO, including GNSS, GNSS augmentation and terrestrial backup

Appli-cation 1

Application 2

Application 3

Application 4

Link technology (encapsulated)

Transceiver station

Ship’s

sensors

Shipboard

Applications

INS

technical

e-Nav

ser-vices

Application-to-

application

(peer-to-peer)

functional

connection

Physical

Link (e.g. radio link)

other

ships

other

ships

mariner

Shore-based

users

Figure 1: 
e-Nav architecture

Ship technology 
environment

  - Features of the e-Navigation architecture

In addition, storage of information and /or data over longer periods of time together with the retrieval of that information / data are part of the e-Navigation concept.

To achieve that end, operational requirements (user requirements) can be represented in terms of appropriate state-of-the-art data models with abstract descriptions of the processes and of the functionality which are required to properly process the data to arrive at meaningful information for the stakeholders involved.

Furthermore, the co-operation in data exchange ship to ship, ship to shore, shore to ship, shore to shore and other users is at the core of the e-Navigation architecture.  Hence the co-operative nature is a core value of e-Navigation concept.

To provide seamless service, the existing non-co-operative services and aspects must be integrated in this predominantly co-operative e-Navigation architecture by proper engineering models.

It is appropriate, when considering abstract flow of information between applications, to use the term of functional links between participating applications.

The above abstract statements constitute the core values of the e-Navigation concept in analysis and design. Eventually the data exchange and the processes and functionalities using that data need to be manifested in physical links, e.g. a physical communication link, and physical entities and devices, e. g. computer systems. Hence, requirements and limitations stemming from that physical world will have their impact on the system architecture.

Finally, the different dimensions of application-to-application information flow need to be considered to understand the e-Navigation concept.  These different dimensions include the following:

- Location of the participating applications: on board ships, on board floating AtoNs, onboard aircrafts interacting with the maritime community and on shore, etc. This dimension highlights the requirement of a high connectivity within the e-Navigation concept.

- Distributed responsibilities of stake holders in the e-Navigation concept, e.g. different authorities ashore operating in the same area, but with different tasks.

- The large variety of the nature and amount of data to be exchanged between applications will result in a large variety of appropriate technologies to be considered.

- Quality requirements: The attributes of accuracy, integrity, reliability, continuity, time behaviour etc. need to be defined when analysing the operational requirements (user requirements).