| From: Roger Barker and Hendrik Eusterbarkey as a contribution to task 1 |  |
| --- | --- |
| To: ANM 12 | 2008 Sept. 5th |

Guidance on A to N design for channels

Meeting with PIANC Marcom WG 49 with the task to review “Approach Channel – A Guide for Design”

Input paper for point 6 of the provisional agenda

Task 1

Develop guidance on channel design and a minimum comprehensive mix of AtoN

- Background

According to ANM 11 action items “Hendrik Eusterbarkey is requested to examine output from his meeting with Dr. Mark McBride and PIANC on channel design and incorporate into the draft Guideline on Channel Design (ANM11 / WG1/ WP1) as necessary”.

- Report on outcome from meeting with PIANC WG

  - General

Roger Barker (IALA Risk Management Tool Group) and Hendrik Eusterbarkey (IALA ANM) were joining the meeting on the 18th April 2008 in Wageningen, The Netherlands, at the location of MARIN.

After a plenary the WG was divided in Subgroups:

- The Vertical Subgroup to develop guidance on vertical dimensions of fairways

- The Horizontal Subgroup to develop guidance on horizontal dimensions of fairways

- The General Subgroup dealing with all aspects of the report, except the two main chapters dealing with vertical and horizontal dimensions of fairways

Roger Barker was joining the Horizontal Subgroup. Hendrik Eusterbarkey was joining the General Subgroup.

  - Horizontal Subgroup

The horizontal dimensions, which will be required for a channel depend significantly on the Navigation Accuracy and, thus, on the provision of Aids to Navigation. The Horizontal Subgroup is very much focussing on risk models to find the “safe” channel width for a given situation though some members of the PIANC WG prefer a “deterministic” approach.

The Risk models used by this PIANC WG und used by IALA (IWRAP) were discussed and compared. The importance of analysing both the quantative value of a Risk Model such as the IALA IWRAP model and also a qualitative model such as PAWSA, were discussed. When considering the qualitative models the difference in case by case assessments, bringing all relavant experts to the table were emphasised.

Under 3.1.1.2 in the draft for the report of PIANC Marcom WG 49 some details on Fairway marking and positioning systems are mentioned. These were discussed in the Horizontal Subgroup regarding the need to consider the individual nature of each case and the benefits of both types of Risk Assessment.

  - General Subgroup

There is another chapter in the draft of the report of PIANC Marcom WG 49, which is relevant to consider for IALA. This is Chapter 4. It was discussed in the General Subgroup. The chaper has been written by Esa Sirkiä from Finland, who is well known in IALA, e. g. because of his participation in several Risk Management Workshops.

The chapter itself does not need any major changes. However, it should be distributed to E-Nav and VTS committee to check, change and amend the paragraphs on On-board navigation systems and VTS/VTMS.

However, a former issue from ANM picked up and more or less solved. In an ANM 8 input paper it was asked – assuming  the wording in table 5.2 of the existing PIANC “Approach Channel – A Guide for Design” will not change –what excellent, good and moderate AtoNs are, and if it is right to keep to an additional Width of 0.0 for “excellent A to N”.

So, if excellent in the sense of  table 5.2 of the existing PIANC “Approach Channel – A Guide for Design” is when no additional width is required for the inaccuracy of AtoN, is this a required positioning accuracy of  0 (zero !) m?

No! Because there is a difference between “positionning  accuracy” and “navigation accuracy”.

If the positioning accuracy is better (that means less) than the navigation accuracy it should be supposed as already included in the navigation accuracy. In case of “excellent AtoN” the positionning  accuracy should be already included in the navigation accuracy.

So, it is agreed, that there could be something in the new PIANC guide like (f) in Table 5.2 in the existing PIANC Guide and that the additional Width factor can be 0.0 (zero).

What is left to deal with, is to get an idea of the difference between excellent, good and moderate A to N in the table 5.2 of the existing PIANC “Approach Channel – A Guide for Design” and to check the additional Width factors

This could be estimated as follows (example for “good AtoN)”:

Considering table 5 of IALA-paper “Navigation Accuracy by the use of buoys and leading lights in restricted waterways” an example is calculated in the following table. The “standard number” of buoys is calculated by simple calculations under 2.1 of the IALA paper “Navigation Accuracy by the use of buoys and leading lights in restricted waterways”.

The difference between “excellent” and “good AtoN” would be caused by a bigger buoy distance of only 10% or less. This does not seem realistic.

| Navigation Channel width | Excellent A to N |  |  | Good A to N |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | PIANC additional width | Navigation accuracy (IALA paper) 1) | a) Mix, b) buoys: 1: percent of stand. number 2: separation distance | PIANC additional width on each side 2) | Navigation accuracy (IALA paper) 1) | a) Mix, b) buoys: 1: percent of stand. number 2: separation distance |
| 100 | 0 | 10 | a) only buoys 1: 100 % 2: 300 m | 0,6 | 10,6 | a) only buoys 1: 94 % 2: 320 m |
| 200 | 0 | 20 | a) leading lights and buoys 1: 90 % 2: 660 m | 1,25 | 21,3 | a) leading lights and buoys 1: 86 % 2: 700 m |
| 400 | 0 | 40 | a) all included b) buoys 1: 90% 2: 1300 m | 2,5 | 42,5 | a) all included b) buoys 1: 84 % 2: 1430 m |

- based on the width of fairway – caution if this is much bigger than the width of the navigation channel !

- based on ships beam B, considered to be 1/8 of channel width

So, the additional width for “good” and “poor” A to N in table 5.2 of the PIANC “Approach Channel – A Guide for Design” might be bigger in the new Guide.

  - Two more remarks

After consulting the IALA experts, e. g. Omar Eriksson, there is no sufficient scientifc verification and/or validation of  IALA paper “Navigation Accuracy by the use of buoys and leading lights in restricted waterways”. So, this can not be used in the report of the PIANC working group.

The the only true way to "verify"  the proposed additional width for navigation inaccuracy or for any proposal for additional AtoN will be the inclusion of both forms of Risk assessment – qualitative and quantutativw – and the appropriate use of simulation, as part of the qualitative Risk assessment (Ref : IALA recomendation O 138 “Use of GIS and Simulation by Aids to Navigation Authorities” and IALA-Guideline 1058 on simulation).

- Action Requested

- 3.1 Hendrik Eusterbarkey should examine, if some of the consideartions arising from the cooperation with the PIANC Marcom WG 49 have to be included into the IALA Guidance on Q to N design for channels.

- 3.2 All ANM members are invited to examine, if they feel that some of the considerations arising from the cooperation with the PIANC Marcom WG 49 should be included into the IALA Guidance on Q to N design for channels.

3.3 As we found that there is no sufficient scientifc verification and/or validation of the IALA paper “Navigation Accuracy by the use of buoys and leading lights in restricted waterways”, which is the basis for section 3.1.15 of the IALA-Navguide, we should consider if we feel that section 3.1.15 in the IALA-Navguide should be removed.

- Annexes:

- Terms of Reference for PIANC Marcom WG 49

- Draft of Chapter 4 of PIANC Marcom WG 49 – Report

- part of Draft section 3.1.1.2 of PIANC Marcom WG 49 – Report

- Table 5.2 of the existing PIANC “Approach Channel – A Guide for Design”

ANNEX 1 to Input paper ANM 12, Task 1, “Develop guidance on channel design and a minimum comprehensive mix of AtoN”

# PIANC MARCOM  49 Working Group :

# Horizontal and vertical dimensions of fairways

# TERMS OF REFERENCE

# HISTORICAL BACKGROUND – DEFINITION OF THE PROBLEM

Ensuring the continued safety and efficiency of ships transiting dredged channels in the years ahead will require channel designers and naval architects alike to better understand the handling and manoeuvrability of both existing and new generation ships in shallow and restricted waters.  In particular, PIANC (INA) has a desire to address the issues of horizontal and vertical dimensions (clearances) relating to ship channel and manoeuvring area design.

# OBJECTIVES OF THE STUDY

The objectives of the Working Group examining the horizontal and vertical dimensions of fairways will be to review, update and, where appropriate, expand on the design recommendations on horizontal and vertical dimensioning as presented in the Working Group 30 report of 1997 on approach channels. In doing so, the Working Group should consider recent developments in simulation and other design tools and the sizes and handling characteristics of new generation vessels.

# earlier reports to be reviewed

The design of approach channels and fairways was first considered by PIANC in a report published by Working Group 2 of the PIANC International Oil Tankers Commission (IOTC) in 1972. Some years later, this work was reviewed by Working Group 4 of the PIANC International Commission for the Reception of Large Ships (ICORELS) in a report published in 1980. The subject was most recently considered by the joint PIANC-IAHP Working Group PTC II-30 in co-operation with IMPA and IALA. Their findings were published, first as a preliminary set of concept design guidelines in 1995, followed by the 1997 final report "Approach Channels – A guide for design".

# Matters to be investigated

The Working Group should consider the following issues and items:

| Design Vessels(current and new generation) | Channel Bends |
| --- | --- |
| Water Datum (CD/MLLW/MSL) | Channel Design/Alignment Studies |
| Tidal Ranges | Obstructions (submerged, overhead) |
| Wind/Wave/Currents | Pitch/Roll/Heave |
| One/Two Way Traffic | Draught and Trim |
| Tug Assistance and Tug Efficiency | Squat |
| Restrictions on pilot boarding and tug connection | Underkeel Clearance |
| Protected Channels | Shallow water effects |
| Entrance Channel/affects | Sinkage in Fresh Water |
| Speed Restrictions | Seawater Intake Clearance |
| Bank Clearance | Air draught |
| Manoeuvring Lanes | Use of High Tides |
| Manoeuvring areas and turning circles | Safety criteria and risk assessment |
| Manoeuvring in adverse conditions | Study methodologies and appropriateness |
| Clearance Between Ships(underway and moored) | Visibility |
| Buoy Clearance | Ice |

# METHOD OF APPROACH

- Collation and review of recent developments in information and design tools (such as desk study methods, mathematical and physical modelling and simulation techniques) available to inform decisions on channel dimensioning, including research work in hand.

- Review and, where appropriate, update the PIANC-Working Group 30 on guidelines for horizontal and vertical design/dimensioning of approach channels published in 1997

- Preparation of an inventory of channel dimensions and channel restrictions at existing terminals and harbours.

- Particularly, the Working Group should give priority to :

- Vertical motions of ships in approach channels (due to squat, wave-induced motions, dynamic effects etc.)

- Vertical clearances under bridges, overhead cables etc.

- Safety criteria, assessment of levels of risk and appropriate clearance margins

- Simulation of ships in channels

- Methods for assessing environmental (Metocean) operating limits

- New and future generation ship dimensions/manoeuvring characteristics

- Manoeuvring limits in adverse conditions (e.g. tug effectiveness at speed and in adverse wave, current and wind conditions)

- Restrictions on pilot boarding, tug attachment and detachment and the time required.

It is anticipated this Working Group will be complete with its research of available literature, an inventory of channels, terminals and harbours with existing restrictions, projected vessel sizes and handling characteristics, associated clearance/dimension requirements within a 2 year time frame.

# suggested final product of the working group

The final report of the Working Group will provide guidelines and recommendations on the horizontal and vertical dimensions of approach fairways/channels to harbours and terminals, and the manoeuvring areas within harbours, for the purpose of assisting in the design process, along with defining restrictions to operations within a channel.  This will incorporate establishing vertical bridge clearances and depth requirements.

The final report should be presented as an update the PIANC-Working Group 30 on guidelines for horizontal and vertical design/dimensioning of approach channels published in 1997.

The final report should also identify any topic on dimensioning of channels and fairways which may require further work/research.

# desirable disciplines of the members of the Working Group

It is proposed this working group be composed of:

- Port engineers with contacts to their peers

- Marine engineering consultants

- Naval architects

- Port pilots

- Tug operators.

# ANNEX 2 to Input paper ANM 12, Task 1, “Develop guidance on channel design and a minimum comprehensive mix of AtoN”

- Draft of Chapter 4 of PIANC Marcom WG 49 – Report

# 4. Aids to navigation

This section refers to positioning systems and facilities used to assist in navigating. Navigation itself may be understood as monitoring one's position and directing oneself as accurately as possible both geographically and relatively, in this case within a constrained waterway.

There is several navigation methods widely used in the marine world:

- Visual navigation that uses optical observations

- radar navigation that uses radar observations

- Electronic navigation that uses positioning signal from satellites and other systems.

- Celestial  navigation

Today, all   are generally used in combination as appropriate. Also, VTS/VTMS systems are monitoring and assisting safe navigation from shore via radio in many port and coastal areas.

Aids to navigation are needed where there is lack of adequate natural visual leads. Aids to navigation are of four basic types:

- Directional guidance i.e. lateral positioning reference to fairway

- Longitudinal positioning reference to fairway

- Acting as fixed point while navigating i.e. position reference

- Warning of possible hazardous areas and objects

References /1/, /2/ and /3/ give useful information about types and arrangement of AtoN.

## 4.1 Fairway markings

Fairway markings are prescribed for both visual and radar navigation. Fairway markings may be located either along the sideline of fairway e.g. buoys or fairway centre line e.g. leading lights. The properties of markings may be summarised thus:

- Leading lines along centre line act as very powerful tool for lateral positioning but poor for longitudinal positioning

- Leading lines crossing the fairway are accurate tool for longitudinal positioning.

- Single markers are mainly tool for longitudinal positioning.

- Paired markers act for both, longitudinal and lateral positioning

- Single marker on curve turning point may act as position reference around turn

- Single marker may be used for warning purposes, for example warning about a hazardous wreck or a rock

By combining types of markings as appropriate, one may design a fairway that is safe to navigate. The aids required   are related to fairway properties, width and curvature. Aids required are also dependent on weather and traffic conditions.

There is a very strong connection between fairway dimensions and markings. Basically better the lateral positioning is, less there is need for additional fairway width. A Curvy fairway needs good lateral and longitudinal positioning and all the curves should have turning point reference or some other provision.

Theoretically more there are fairway markings easier it is to navigate. However, there is a saturation point where adding aids to navigation does not help positioning further. One has to appreciate that too many markers may be confusing and weaken the result. The task is to find the optimal solution between fairway markings and fairway dimensions. Usually this point has to be found by simulations in detailed design phase. Designer should pay attention to possible fairway crossings to avoid confusing navigational situations with too many markers.

Minimum requirement of fairway marking is that at least one marker should always be visible on either side of the fairway /2/. With this rule and knowledge of visibility conditions in area of interest one can calculate maximum distances between markers. Maximum marker spacing is then less than chosen minimum visibility. The ship size, speed and bridge visibility may dictate that the minimum distance that is practically required is greater than the minimum meteo distance otherwise considered.

One possible way to mark fairway is to place a marker in all vertexes of sideline. If straight parts are longer than what maximum spacing defined, there is always an option to place markers also on straight sideline. This marking technique is common in curved fairways. An alternative to both straight and curved channels is the use of centre line buoyage, particularly in 2 way channels. Fewer buoys are needed and the factor of paired buoys creating additional channel obstructions and small craft restrictions is avoided.

In simulation studies has been showed that gated markers are superior in straight parts of fairway compared to single markers on sideline vertexes/2/.  As mentioned earlier, paired makers are effective for both, lateral and longitudinal positioning.

Conclusively can be said that every fairway is individual case and should be studied as such. There is no optimal solution that may be used everywhere but variety of marking solutions and techniques. Chosen marking should always be studied in simulator at least on its critical points.

## 4.2 On-board navigation systems

Basic goals.

Both the vessel's absolute geographical position within the area and the vessel's relative position to known fixed and moving objects both natural and man-made need to be considered.

Visual navigation.

The primary means of visual navigation is manually plotting on a paper chart using two or more compass bearings of geographical features. The use of horizontal and vertical sextant angles etc., can be considered impractical and obsolete. Relative position fixing  can be facilitated through the observation of various forms of leading marks including geographical features and  channel buoyage.

Night vision equipment and binoculars are probably the only optical aids that may be found in use today.

Electronic aids.

Radar can be used for both geographical position fixing and  for relative position monitoring through a variety of  techniques. Radar technology continues to develop.  For channel marking, the addition of Radar beacons or racons can significantly improve the prominence of important buoys etc.

ECS is a non IMO approved navigation information system that electronically displays vessel position and relevant nautical chart data and information from an ECS Database on a display screen.

ECDIS equipment is specified in the IMO ECDIS Performance Standards and with adequate back up arrangements, can be accepted as complying with the up-to-date chart required by regulation V/19 & V/27 of the 1974 SOLAS Convention.

Global Navigation Satellite System (GNSS) is the standard generic term for satellite navigation systems.

Differential GPS (DGPS) is an enhancement to GPS, the result is a significantly more accurate and reliable position fix.

eLORAN is an independent, dissimilar complement to Global Navigation Satellite Systems (GNSS). The international maritime community now understands that its future digital eNavigation environment needs an internationally agreed alternative system to GNSS. eLORAN is the only viable candidate.

e-Navigation Whilst not strictly a position fixing system, is a navigational concept which will far reaching influences on the future of shipboard navigation systems and techniques. It is an International Maritime Organization (IMO) led concept based on the harmonisation of marine navigation systems and supporting shore services driven by user needs.

AIS The Automatic Identification System is a system used by ships and Vessel Traffic Services (VTS) principally for identification and locating vessels. Work is now ongoing to facilitate the transmission of data such as identification, meteo and tides from navigation marks via AIS.

Portable Pilot Unit (PPU) is a notebook computer based system which depending on its level sophistication and manufacturer is able to provide the Pilot with his own wholly or partially independent navigation and manoeuvring monitoring console.

Other systems. In addition to those mentioned above, more fundamental equipment is also in use and includes but is not limited to Gyro, Magnetic, Fluxgate and GPS based compasses. Echo sounders and sonar devices etc..

## 4.3 VTS/VTMS systems and impact

Vessel traffic services (VTS) often also referred to as VTMS,  are shore based port or coastal region traffic management systems.   The systems range from the provision of simple information messages to ships, to extensive management of traffic within a port or waterway.

There are three basic types of VTS:

- Information Service Defined by IMO as ‘a service to ensure that essential information becomes available in time for on-board navigational decision-making’.

- Navigational Assistance Service. Defined by IMO as ‘a service to assist on-board navigational decision-making and to monitor its effects, especially in difficult navigational or meteorological circumstance or in case of defect or deficiencies.’

- Traffic Organisation Service. Defined by IMO as ‘a service to prevent the development of dangerous maritime traffic situations and to provide for the safe and efficient movement of vessel traffic within the VTS Area.’
A Traffic Organisation Service includes a comprehensive and dedicated service, throughout the declared service period, without which the long term planning of traffic movement and developing situation would not be possible.

Potential for confusion arises as it is incumbent upon the navigator to be familiar with the type of VTS system established in a particular area.  The nature of a specific VTS must therefore be widely and clearly promulgated.

Traffic management implications from a channel design and operation perspective are primarily that it must be established what level if any of VTS is available to assist the navigator. For example a narrow channel in a busy port approach may require a full Traffic Organisation Service whereas a  relatively quiet port may only need an Information service.

Shore based pilotage is a concept much mooted in various forums. It is a different concept to Navigational Assistance.  It is practised in a limited fashion in some ports but is generally considered within many learned and professional maritime circles to be unsafe, impractical and very limited.

References:

/1/ Aids to Navigation Guide, IALA (Navguide) Edition 4, Dec. 2001

/2/ Hydraulic design of deep-draft navigation projects, USACE Engineering manual EM 1110-2-1613, 31 May 2006, chapter 10

/3/ ROM 3.1-99 Design the maritime configuration of ports approach channels and flotation areas, 2nd edition, Dec. 2003.

ANNEX 3 to Input paper ANM 12, Task 1, “Develop guidance on channel design and a minimum comprehensive mix of AtoN”

- part of Draft section 3.1.1.2 of PIANC Marcom WG 49 – Report

Fairway marking and positioning systems(MH) Refer to sections 4.1 and 4.2

1) Fairway marking

As the basis of fairway marking, a pair of buoys which indicate the maneuverable zone are to be arranged on the both sides of fairway. In general, the light buoys or the light beacons are placed in order to enable the night navigation. The clearance between two buoys in the direction perpendicular to the course line should be determined in consideration of the design ships, their sailing speed and visibility such as the frequency of fog occurrence. There should be placed fairway markings on the bend region of fairway, and the leading light and guide marking are recommended to be placed in the fairway with successive bends.

2) Positioning Systems

As the ship positioning system, GPS or D-GPS with electronic charts are widely utilized nowadays in the ship handling operation. However, although electronic charts are increasingly found on board ships, the vast majority are not official ECDIS but ENC’s and therefore come with a warning that they must not be used for navigation. Ship handlers judge and recognize the ship position by an image of GPS information on the display of electronic chart. There exist two kinds of errors with respect to GPS utilization. One is the perception error on the electronic chart display. Image information on the electronic chart is definitely presupposed sufficiently accurate, but the positioning of ship is made solely by perceiving ship movement on the display by naked eyes. For this reason, some perception error (a half of the ship breadth or so) should be taken into account in the design of maneuvering lane. The other is the error of GPS information itself, for which an error of about 30 meter may be supposed. No error may be considered for the ship positioning by D-GPS. Actually there are errors to be considered, not least the accuracy of the antenna installation and calibration. GPS is not the only method for ship positioning.

ANNEX 4 to Input paper ANM 12, Task 1, “Develop guidance on channel design and a minimum comprehensive mix of AtoN”:

Table 5.2 of the existing PIANC “Approach Channel – A Guide for Design