To:	Aid to Navigation Management Committee

From:	e-NAV Committee (5th Session)

19 September 2008

Liaison Note to ANM re Development of Virtual Aids to Navigation

The IALA e-NAV Committee at its 5th session (e-NAV 5) considered the Report on a Virtual AtoN Demonstration (document e-NAV 5/11/01) and agrees with the conclusions of the report, namely that:

- virtual AtoNs may have a useful role as a quick and cost efficient method of providing the mariner with navigational information,

- the possible applications include near instantaneous emergency wreck marking, selective channel marking (where only certain vessels can 'see' a Virtual AtoN marked channel), and dynamic channel marking that can quickly respond to changing conditions (such as a shifting sandbank).

- where a national network of AIS base stations is deployed, it will be possible to create and deploy Virtual AtoN targets anywhere within range of the network, from a single control centre.

It concurs that for ships to benefit from this technology their onboard systems must be capable of displaying AIS AtoN messages and they should be provided with electronic chart software capable of displaying virtual AtoN targets.

It would however like to highlight several considerations that should be taken into account when continuing the development and procedures for use of virtual AIS AtoN:

- the mariner’s requirements to receive Maritime Safety Information (MSI) in line with current standard practices should not be overlooked (i.e. WWRNS and GMDSS),

- it is recommended that for the foreseeable future there should be no changes to current navigation warning procedures without full consideration of the possible implications,

- any potential impacts for the AIS VHF Data Link (VDL),

- as yet not all ships are equipped to be able to benefit from the graphical presentation of AIS virtual AtoN (e.g. vessels fitted with AIS MKD),

- from a Human Element perspective, mariners not familiar with the application of virtual AtoN may be confused by the graphical presentation of AtoN which they are not able to physically see, and

- it may be less confusing to move to the next logical step for the transmission of information concerning new dangers, such as wrecks, and instead of using virtual AtoN to develop a means of transmission of the graphical depiction of the actual danger or the provision of an area to be avoided.

In summary, the Committee has some concerns about the development of virtual AIS AtoN and will aim to provide further advice in the future.