IALA Guideline No. ####

On

Buoy Lights

Edition 1

[Date issued]

AISM Association Internationale de Signalisation Maritime      IALA

International Association of Marine Aids to Navigation and Lighthouse Authorities

20ter, rue Schnapper, 78100

Saint Germain en Laye, France

Telephone: +33 1 34 51 70 01  Fax:  +33 1 34 51 82 05

e-mail:  iala-aism@wanadoo.fr       Internet:  www.iala-aism.org

This document is not being proceeded with

Document Revisions

Revisions to the IALA Document are to be noted in the table prior to the issue of a revised document.

| Date | Page / Section Revised | Requirement for Revision |
| --- | --- | --- |

Table of Contents (Title style)

To update, right click anywhere on the table, then update field / update entire table.

Document Revisions	2

Table of Contents (Title style)	3

Index of Tables	3

Index of Figures	4

BUOY LIGHTS	5

1	Introduction	5

1.1	Purpose and Scope	5

2	The Application of buoy Lights	5

2.1	Technical Considerations for Buoy Lights	5

2.1.1	Design	5

2.1.2	Display	5

2.1.3	Vertical divergence	6

2.1.4	Range	6

2.1.5	Focal Plane	6

2.1.6	Colour	6

2.1.7	Light character	6

2.1.8	Synchronization scheme	6

2.1.9	Environmental considerations	6

2.1.10	Ecological	6

2.1.11	Considerations for cleaning	6

3	Basic Principles	6

3.1	Types of Buoy Lights	6

3.1.1	Integrated power system lanterns	6

3.1.2	Modular Lantern System	7

3.1.3	Ice Hardened Lantern	7

4	Types of Buoy lights	7

4.1	Integrated Power System Lanterns	7

4.2	Modular Lantern Systems	7

4.2.1	Power Source	7

4.2.2	Lantern Components	7

5	Further considerations	8

5.1	Design and specification of buoy lights	8

5.2	Installation considerations	9

ANNEX 1	Annex Title	12

1	Heading 1	12

1.1	Heading 2	12

1.1.1	Heading 3	12

APPENDIX 1	Appendix title	13

1	Heading 1	13

Index of Tables

Table 1	Comparison of the basic types of buoy lights	7

Index of Figures

No table of figures entries found.

BUOY LIGHTS

# Introduction

Followed by body text

Information that is used to introduce the document, including reference to lead-up to the creation of the document.  This should also include references to any IALA Conference or Symposium recommendations that led to the document creation.

All text should be English UK

## Purpose and Scope

Followed by body text

# The Application of buoy Lights

A buoy light is part of an aid to navigation that displays different colours and/or rhythms over designated arcs.( usually 360 deg in the case of a buoy light)  The colour of the light provides directional information to the mariner.

There are many different applications for buoy lights.

Some examples are:

- Markers in all IALA recommended colours

- Fairway

- Channel

- Special Area markers

- Danger

- Direction

- Wreck Emergency

## Technical Considerations for Buoy Lights

### Design

It is important to consider the buoy and the lantern as a system. Both must be compatible in order to get the desired level of service.

Reference IALA Guideline on Buoys ….

Items to consider are outlined in the following sections.

### Display

The location of the light on the buoy should guarantee the visibility and conspicuity in all horizontal directions.

Buoy lights in a fairway may need to be visible from above so that the buoy can be identified from the bridge of a large ship that is close to the buoy. This may be for collision avoidance rather than navigation.

### Vertical divergence

Depends on the location of the buoy and the sea conditions.  In general the largest vertical divergence is recommended.  Reference IALA document on vertical divergence.

### Range

The range should be selected with vertical divergence consideration.  A practical recommendation for the maximum range would be no more than 10 nautical miles. Note that the effective range is also influenced by the flash character which defines the light on time. If ranges approaching 10 miles are considered then the required focal plane height needed to achieve this range must be carefully evaluated along with the effects of anticipated maximum wave heights.

### Focal Plane

Depending on the location of the buoy and the sea conditions. A higher focal plane is preferred.  Reference IALA guideline on vertical divergence.

### Colour

Reference IALA standard ….

### Light character

Reference IALA standard ….

Reference presentation of Trinity house.

Note that the flash character defines the light on time which should take into consideration the buoy movement.

### Synchronization scheme

Reference IALA guideline on synchronization.

### Environmental considerations

The design of the light should consider the operating conditions that it will be exposed to. The lantern must survive extreme environmental conditions.  This includes wind, temperature, humidity, ice, salt spray, etc…

Ensure that the design of the lantern considers corrosion, is mechanically strong to sustain the shock and vibration of a floating buoy

### Ecological

Both operating and end of life must be considered. During operation, all ecological considerations are important.  At end of life, recycling and disposal methods must be considered.

### Considerations for cleaning

Grime, dust and ice build-up can interfere with the lanterns operational characteristics as well as the power supply’s effectiveness. Consideration for cleaning is important. Consideration should be given to vertical solar panels to prevent build-up.

# Basic Principles

## Types of Buoy Lights

There are two major types of lanterns, integrated power system lanterns and modular lantern systems.

### Integrated power system lanterns

This is a solar powered LED lantern complete with battery and electronic charging system. The lantern characteristics are programmable via some type of programmer. The unit is a complete system that is bolted onto the buoy.

Additional integrated systems can provide other functions such as AIS, GPS, GSM monitoring, synchronization capabilities.

This is low a maintenance system, only need to clean and easy to install.

In order to determine whether a lantern will function correctly at a particular location, the same power supply factors must be considered as you would for a modular system.

### Modular Lantern System

This system is comprised of a separate power source and lantern component. The power source is commonly a solar panel, a battery, charge controller. The lantern component is commonly a flasher module and LED light source or a flasher module, lamp changer and standard lamp.

The use of LED light sources are recommended for buoys due to the vibration and shocks experienced which will shorten the life of conventional lamps.

### Ice Hardened Lantern

This system is a special application with an ice hardened LED lantern that is capable of submerging under the ice as the ice flows.

- Comparison of the basic types of buoy lights

| Light Type | Practical Range | Light Source |
| --- | --- | --- |
| Integrated power system lantern | Up to 6 nm | LED |
| Modular - LED | Up to 10 nm | LED |
| Modular – Incandescent | Up to 10 nm | Incandescent |

# Types of Buoy lights

Over the years several types of lantern systems have been used. Today’s LED technology provides low power, reliable and long life lights that are very robust in this application.  It is recommended that either a integrated power system lantern LED unit or an LED insert be used as the light source.

## Integrated Power System Lanterns

This type of lantern is a complete system with LED light source, programmable flasher, and integrated power source consisting of solar panels, charge controller and battery.

Include sketch of an integrated power system lantern diagrams with arrows, Sketch of installed on a buoy.

For a discussion on LED technology, reference IALA Guideline on LED Technology …..

## Modular Lantern Systems

### Power Source

There are different power systems available for use on buoys. Solar, wind, wave, primary battery. These are discussed in IALA guideline ……

The majority of installations are solar which provides a self sufficient buoy with the minimum of moving parts. Maintenance is minimized. A solar powered system is recommended if the geographical location permits.

Sketchs….

### Lantern Components

#### Housing and Lens

According to IALA guidelines, the housing and lens must provide an omni-directional horizontal beam of light. Both must be able to withstand the harsh environmental conditions experienced by aids to navigation.

The lamp components are housed inside the housing and lens. This can either be an LED insert or an incandescent lamp.

Sketchs…..

#### LED Inserts

Modern technology has incorporated an LED light source, electronic flasher and photocell into a single unit.  This replaces the incandescent lamp within the housing.  When using an LED insert, care must be taken to ensure that the LED is focussed within the lantern.  Manufacturers provide information to ensure that the LED is focussed.

Sketchs…..

#### Incandescent lamp

IALA guideline 1043 discusses the various lamp light sources.

Due to the small filament size, special consideration should be made in the design of the lens system in order to provide enough vertical divergence.

Dual filament lamps can increase the service life by providing an operating and a standby filament.  However the use of a single high performance lamp combined with planned maintenance periods may provide suitable reliability.

#### Lamp Changer

Due to the limited lifespan of a incandescent lamp, this device holds 4 or 6 lamps and rotates a new lamp into the focal point once the existing lamp has failed. In doing so, a longer maintenance period can be accommodated.

Sketch.

#### Flasher

A programmable electronic device that controls the incandescent lamp to produce the desired flash characteristic. It also uses a photocell to turn the light on at night and off in the morning.

Sketch ……

#### Synchronization

At times it is desired to synchronize the flash characteristic among buoys.  This can be accomplished by the use of a GPS, radio or GSM module.  Refer to IALA guidelines on the synchronization of lights.

# Further considerations

## Design and specification of buoy lights

The following steps are recommended when preparing a specification for a buoy light.

Seasonal or year round operation?

The first decision is whether a seasonal or year round signal is required.  If a seasonal light is required then the use of a seasonal timer is required.  Also the turn on and turn off dates need to be determined. This assumes that the buoy is not removed from station at the end of the operational period.

Determine Required Intensity

Intensity is determined from the range required and the conditions under which viewing occurs (transmissivity of atmosphere, level of background lighting).  IALA recommendations should be consulted.

Choose Light Source

An LED light source is recommended where ever possible due to reliability, lifetime and low power requirements.

If an LED light source will not perform in this location, then based on the range requirements, choose the appropriate lamp type and size.  Ensure that the design of the power source is capable of supporting light source needs. Reference IALA guideline ….

Check Vertical Divergence

This assumes a detailed knowledge of the riding performance of the buoy and the wave regime at the planned station.  This information may be very difficult to obtain.

Ensure that the vertical divergence of the light allows the mariners to recognize the signal in all of the operational conditions.

Specify Flash Character

Determine the character for the buoy light, bearing in mind the characters of other nearby aid to navigation lights. Ensure that the power system is capable of providing the power required.

Connections to Other Devices

Determine if connection or synchronization is required with other lights, special controllers, monitoring.

Choose equipment that matches the locations environment

## Installation considerations

Ensure that the lantern system is mounted at the appropriate focal plane height.

Choose a location on the buoy for the lantern system that provides as nearly as possible 360 degree unobstructed path for the light beam as outlined in the IALA guideline ……

Follow the manufacturer’s mounting instructions for the lantern system.

Solar panels should be placed such that sufficient charge is always available, regardless of the position of the buoy.

Solar panels should be oriented to minimize the buildup of dirt and grime.

To ease the maintenance of a discrete lantern system, consider installing the flasher, charge controller and any other electronic modules together in a waterproof box close to the base of the buoy.  This will alleviate the need to climb the buoy when maintaining it.

Commercial LED lanterns will invariably have the flasher and probably a charge controller built into the lantern housing.

Modern electronic equipment should not require any maintenance during the ‘on station’ period of the buoy.

Equipment mounted high up on the tower will be less liable to wave damage than items mounted nearer the body.

Where ever possible, install heavy items of the system as low on the buoy as possible so as to minimize the affect on the stability of the buoy.  This will make maintenance easier and safer as well.

Check from boat

Carry out a practical viewing trial from a suitable vessel to

Verify that the lantern is visible from the required range.

References

- IALA Engineering Environment and Preservation of Historic Lighthouses Committee, EEP4 Input EEP4/6l/2, LED Sector light, F. Hermann, C.Mauel, Traffic Technologies Centre, Koblenz, Germany.

- IALA Recommendation for a definition of the nominal daytime range of maritime signal lights intended for the guidance of shipping by day, April 1974. IALA.

- IALA Navguide 2001 Edition 4, Section 3.4.1.  IALA, 2001.

- IALA Recommendation on the Determination of the Luminous intensity of a marine aid to navigation light, IALA December 1977.

- Annex Title

Guidelines should have Annexes.  Appendices are attached to Annexes.

# Heading 1

Body text

To restart the numbering at 1:

- Office 2003, go to Format / Bullets and Numbering / Restart numbering (lower left in the box)

- Office 2007, go to down arrow next to Numbering icon and select Set Numbering Value

## Heading 2

Body text

### Heading 3

Body Text Indent

#### Heading 4

Body Text Indent 2

- Appendix title

# Heading 1

Followed by Body Text