COMMENTS ON THE DRAFT IALA MBS-MODIFIED

(Juan F. Rebollo, Puertos del Estado, Spain)

INTRODUCTION 

Firstly I wish to congratulate the working group responsible for this draft for the work done.  In this paper I have tried to collect my comments and suggestions on this matter, as an input to the Committee.

While I cannot attend this meeting, I’m available electronically for any clarification.

The first part will present general aspects, conceptual or which apply in different parts of the draft. The second one will be specific comments on specific points of the draft and new items.

Finally in this introduction, I suggest the possibility of holding an ad-hoc meeting in order to exchange points of view with the greatest number of members and make a final review and know the level of acceptance of the new system, before final approval.

GENERAL 

According to the beginning of the second paragraph of "General Principles of the System" (page 2), I think the title should be:

IALA Aids to Navigation System

Part A: Maritime Bouyage System (MBS)

Part B: Other Visual Marine Aids to Navigation

It should be clear what it’s the MBS and what it’s the Other. While the first is mandatory (in fact), the second one is descriptive. It can be added from time to time, other parts (C, D,..) for other AtoN, but the other AtoN must be defined in the document in any case.

Should also be included at the end of the first paragraph of "General Principles of the System" (page 2), some mention about this system is applicable for all types of ships and all type of mariner.

In Part A: MBS, 6:  I suggest the structure as:

6 SPECIAL MARKS

6.1 Definition of Special Marks

6.2 Description of Special Marks

My point of view is that the Special Marks are widespread and are known as "yellow", so I suggest keeping this color only for this name. Accordingly, by allowing other colors, etc, the current 6.2 can be moved to the end, after Marking New Dangers, to a new point 8 named "Undefined Marks".

The next part will explain in detail this proposal.

Where appropriate, to comment on the use of the lowest period in the rhythm of the lights in order to improve their recognition.

Now, other issues of use in various parts of the draft:

▪ In the tables describing the marks where it says "Shape (Buoys) should just say     "Shape ".

▪ Use in a consistent way throughout the document, where applicable, the term "competent Authority".

▪ To refer to the area covered by a light signal using the term "arc", consider using "area".

Part B: Other Visual Marine Aids to Navigation; can include two sections:

▪ B-1: Main Aids

▪ B-2: Complementary devices or secondary Aids

The type B-2 include the "information Marks (p. 4) and the pointer devices (new).

DETAILED REVIEW

The indication is only the modification suggested.

▪ Page 2, right column, "General Principles of the System", end of first paragraph: "This booklet provide guidance on the IALA Aids to Navigation System: Marine Bouyage System and Other visual marine aids to navigation"

Taking into account this comment on other parts of the draft.

▪ p. 2, right column, "General Principles of the System", end of second paragraph: "This is primarily a physical system, however all the marks described may be provided complementarily by electronic (digital) means.”

I don’t think that it should use electronic means as the only way, as an independent mark. Not all users will have capacity for the reception. It should consider including virtual marks with AIS as an option in the MBS. My opinion is no jet, but it can be used in some emergency cases and for a limited time.

▪ p 2, right column, "General Principles of the System," beginning and end of the third paragraph: "Within the MBS there are six types of ...... , the other five types of. "

▪ p 3, left column, Lateral Marks:

Comment: do any indication about the sense of obligation for these marks.

▪ p 3, left column, Cardinal Marks

Comments: to indicate that this type of mark is informative, but does not prohibit navigation in any sector (I think this is truth)

▪ p 3, left column, Cardinal Marks

Comment: show that the periods are predetermined and that is recommended, if possible, the smaller.

▪ p 3, right column, Safety Water Marks

Include text to indicate its use to indicate the place of passage under a bridge, using the assigned light and by daymark a circular board with red and white vertical bars.

▪ p 4, left column, Special Mark, the end of the third paragraph: "the use of a pictogram to indicate their purpose, according with the IHO symbols, if available."

▪ p 4, left column, Marking New Dangers, first bullet: "* marking a danger using Lateral, Cardinal or Isolated danger marks, or"

I suggest removing the word "normal" since it’s not defined. Furthermore Safety Water and Special marks are not included in "normal".

▪ p 4, right column, beginning: "In both cases, marking of a ..."

Other Visual Marine Aids to Navigation (new name) must start at new page. We suggest the following:

OTHER VISUAL MARINE AIDS TO NAVIGATION

These marks are intended to aid navigation as general information to mariners for navigation around the coastal, and not necessarily regarding channel and obstruction marking.

▪ B-1: Main Aids

Fixed whose function is to serve as general navigation aids, including singular points on the coast to facilitate safe navigation in the area:

- Lighthouses and beacons

- Sector Lights

- Leading Lines / Ranges

- PEL Devices

- Major floating aids

(Later more details)

▪ B-2: Complementary devices or secondary Aids

An element that while not has direct function as aids to navigation, allowing the mariner to improve the identification of the main aids, or provide useful information on some aspects of the area.

- Information marks: marks not included in the MBS that provide information related to the sea near area, even with written text or pictogram. For example: beach marking or marks for permitted or prohibited activities in marine reserves. .

- Pointer devices: The objective is to improve the conspicuity and identification of the main visual aids and can be installed attached to the main structure of the aid or in a place near. It can show a line or arc shape. For example: littepipe-line, arch-beacon.

• p. 5 (draft): Title: Maritime Bouyage System - RULES (deleting And Other Aids to Navigation)

• p. 5, 1.1: "The Maritime Bouyage System provides rules that apply to all fixed, floating, and the complementary mark by electronic mean, serving to indicate:”

• p. 5, 1.2.2: "... to indicate where the mariner may find the deepest waters" (as on page 2)

• p. 5, 1.2.5: "They are not generally intended to obstruction or marks channels, but to indicate an area or feature referred to in nautical documents" (consistent with p. 4)

• p. 5, 1.2.6:  Delete.

• p. 5, 1.3 (text): "The significance of the mark depends of the following features:

1.3.1 By night: color and rhythm of light

1.3.2 By day: one or more of the color, shape or top-mark. If light should be the same features than by night.

1.3.3 By electronic (digital) symbol, like complementary information to the physical marks. "

• p.7, p.8 and 2.4.3, 2.5.3: Rhythm - "Composite group flashing (2 +1), with lest than 16 sec period."

• p.9, 3.1.2:  Comment –  to use the term “should be” or “must be”..

• p.10, end of 3 - Note: "... with a clear separation between the cones, according the IALA technical documents.

• p.10, 4.2 Rhythm "flashing Group (2) with a period of 5 sec.”

• p.11, 5.1 - Comment:  some reference indicating its use to indicate the best place to pass under a bridge. Indicate that the daymark is a circular board with red and white vertical bars.

• p.11, 6.1: replace the last sentence ... they are not ..., by "They are not generally intended to obstruction or marks channels, but to indicate an area or feature referred to in nautical documents" (consistent with p. 4)

• p.11, 6.2: Removing and replacing at the end  8: Undefined marks.

• p.12, 6.3: Move it after 6.1

• p.12, 7.2.1: Delete the word "normal”. Modified according to the first bullet on page 4.

New Item

8- Undefined marks:

The same text in the draft for Section 6.2, modifying the beginning: "With a similar application to the special marks, these marks can use the other color as listed in ...."

Comment: as an example include the use of blue lights to marking the end of the floating pontoons in marinas.

• Page 16 and 17 move back to point 8 (new) at the end of the MBS.

Up to this point the MBS Rules

Now, new page:

OTHER VISUAL MARINE AIDS TO NAVIGATION

TECHNICAL SPECIFICATIONS / RECOMMENDATIONS

(I’m using the numbers of the draft)

• p.13, 8.2.1, final sentence: "The color of the light provides directional information to the mariner, as well as danger areas that should be avoided"

• p.13, 8.2.2, new bullet: "To provide information on hazard areas that should be avoided."

• p.14, 8.3.1: include in the definition is "lonely" and that the nominal range of the light must be equal to or greater than 10 nautical miles and not be a mark of the MBS. (this is the Spanish definition)

• p.14, 8.3.2: Delete the text before the table.

• p.14, 8.3.2: Replace "Shape (buoys), for " Shape " (remove buoys).

• p.14, 8.4.1: Copying the same definition as for Lighthouse, but in this case the nominal range must be less than 10 nautical miles or be a mark of the MBS. If they are floating then calling buoys.  (This is the definition in Spain)

• p.14, 8.4.2: Delete the text.

After the sector lights add a new sections devoted to PEL devices.

PEL definition: These devices are a sector light that has a similar use of leading Lines (ranges), show directional information to the mariners and, if any, showing sectors with red-green lateral mind. It must remain in operation by day and by night. It has a inimum angle of indetermination, which allows you to use very small sectors.

• p.15, 8.5: Comment - Include a reference to Lanby.

Suggestion: Delete this item.

• p.15, 9: "IALA documentation provides information on design, planning, operating, and implementing the marks described in the IALA Aids to Navigation System."

And, perhaps can finish with a large photo of a lighthouse