## Vessel Traffic Services (VTS)

### Definition

A VTS, as defined by IMO Resolution A857(20), Guidelines for Vessel Traffic Services, is :

“A service implemented by a competent authority, designed to improve the safety and efficiency of vessel traffic and to protect the environment.  The service should have the capability to interact with the traffic and respond to traffic situations developing in the VTS area”.

The purpose of vessel traffic services is to improve the safety and efficiency of navigation, safety of life at sea and the protection of the marine environment and/or the adjacent shore area, worksites and offshore installations from possible adverse effects of maritime traffic.

### Services

Participation in a VTS may be mandatory or voluntary, depending on local regulations.  In either case, vessels operating in a VTS area may avail themselves of the following services.

- Information Service: is a service to ensure that essential information becomes available in a timely manner to support on-board navigational decision making.  This may include information on the position, identity, intentions and destination of other vessels; meteorological and hydrological conditions, notices to mariners, status of aids to navigation, or any other condition or situation on the waterway that could affect a vessel’s transit.

- Traffic Organisation Service: is a service to prevent the development of dangerous maritime traffic situations and to provide for the safe and efficient movement of vessel traffic within the VTS area.  Traffic Organisation concerns the forward planning of movements to maintain vessel safety and to achieve efficiency.  This service may include the allocation of water space, establishing and operating a system of traffic clearances, managing traffic around certain vessels with limited manoeuvrability, establishing routes to be followed and speed limits to be observed, or any other measures that may be considered necessary and appropriate by the VTS.  All or certain classes of vessels may be required to participate in this service.

- Navigational Assistance Service: is a service to assist in the on-board navigation decision making and to monitor its effect.  This may include bearing and range to a nearby danger or landmark, a course to make good, or assistance in determining a vessel’s position.  Navigational assistance may be provided at the request of a vessel or when deemed necessary by the VTS.

- Support of Allied Services:  VTS are often closely aligned with SAR services, government agencies or commercial port services and may be of assistance in aiding vessels to contact or communicate with these allied services.  These services may benefit from correct and timely information about vessel positions, movements, destinations and times of arrival.  This enables those allied services to enhance their own efficiency, whilst at the same time to better plan and utilise their resources.

### VTS System

To respond to developing traffic situations and to decide upon appropriate actions, a VTS should be capable of generating a comprehensive overview of the traffic in its service area combined with all traffic influencing factors.  Examples of data that should be collected to compile the traffic image include:

- data on the fairway situation, such as meteorological and hydrological conditions and the operational status of aids to navigation;

- data on the traffic situation, such as vessel positions, movements, identities and intentions with respect to manoeuvres, destination and routing;

- data on vessels in accordance with the requirements of ship reporting and any additional data required for the effective operation of the VTS.

The traffic image provides situational awareness and facilitates the VTS operator to evaluate conditions and make decisions accordingly.  The information presented in the traffic image is communicated to participating vessels through the VTS capability to interact with traffic.

Publications that provide more information on VTS:

IALA Recommendations V-102 Application of “User pays” principle to Vessel Traffic Services V-103 Standards for Training and Certification of VTS personnel V-119 Implementation of Vessel Traffic Services V-120 Vessel Traffic Services in Inland Waters V-125 Integration and Display of AIS and other information at a VTS Centre V-127 Operational Procedures for Vessel traffic services V-128 Operational and Technical Performance Requirements for VTS equipment A-123 The Provision of Shore Based AIS A-124 AIS Shore Station and networking aspects relating to AIS service A-126 Use of AIS in Marine Aids to Navigation

IALA Guidelines 1014 Accreditation of VTS Training 1017 Assessment of training requirements for existing VTS Personnel, candidate Operators and the revalidation of VTS Operator Certificates 1018 Risk management 1026 AIS as a VTS Tool 1027 Designing and implementing simulation in VTS training 1032 Aspects of Training of VTS Personnel relevant to the introduction of AIS 1045 Staffing Levels at VTS Centres 1046 Response plan for marking new wrecks 1055 Preparing for a voluntary IMO Audit on VTS Delivery 1056 Establishment of VTS Radar Services

IALA Manuals VTS Manual (2008) Aids to Navigation Guide (Navguide)

IALA Model Courses of Training V-103/1 VTS Operator – Basic training V-103/2 VTS Supervisor – Advancement Training V-103/3 VTS Operator & VTS Supervisor – On-the-Job Training V-103/4 VTS OJT Instructor