| From: Hendrik Eusterbarkey, as Task Leader “Channel Design” and as PIANC WG observer, not as a national representative |  |
| --- | --- |
| To: ANM 13 | 2009 February 25th |

Input paper for task

“Develop guidance on channel A to N design and a minimum comprehensive mix of AtoN”

- Background

A work programme item for the ANM Committee will be to draft a Manual on channel design and a minimum comprehensive mix of AtoN. This includes cooperation with the WG 49 of  the International Navigation Association (PIANC) dealing with the review of the report “Approach Channel – A Guide for Design” from June 1997.

After some discussions the PIANC WG is also feeling that the role of Aids to Navigation and other Traffic Technology Services for the design of navigation channels might have been underestimated in the Guide from June 1997. This was on the screen for ANM since ANM 8 (one of the relevant input papers is added as annex to this input paper.

Hendrik Eusterbarkey, one of the IALA observers in the PIANC WG, was asked intersessionally by a member of the PIANC WG, sub-Worlkung-group “Horizontal dimensions”, how table 5.2 of the PIANC “Approach Channel – A Guide for Design” from June 1997 could be improved.

- Proposal of PIANC WG

The discussion within the PIANC WG is to change the figures in table 5.2 of the PIANC “Approach Channel – A Guide for Design” as follows:

Additional width of fairway; B = ships beam

| Quality of A to N |  | Existing Guide | New Proposal |
| --- | --- | --- | --- |
| Excellent with shore traffic control |  | 0.0 | 0.0 |
| Good |  | 0.1 B | 0.5 B |
| Moderate with infrequent poor visibility |  | 0.2 B | 1.0 B |
| Moderate with frequent poor visibility |  | > 0.5 B | 1.0 B |

- Comments on the Proposal of PIANC WG

The 0.0 for excellent A to N should be acceptable, because the postioning accuracy with excellent AtoN will be so good, i. e. such as low value, that it will disappear in the navigation accuracy, which is - in that case – significantly determined by other factors caused by ship and waterway.

- Proposal of Definitions of A to N Quality for the PIANC Guide

A member of the PIANC WG, subgroup “Horizontal Dimenssions” asked HE as a IALA observer to help to define the three AtoN qualitities. A first draft is as follows:

Excellent A to N s for a restricted channel (proposed definition by PIANC WG):

- Paired lighted buoys with radar reflectors, optimal buoy separation distance (determined by A to N experts)

- Lighted leading lines

- VTS service with navigational assistance

- Differential Global Navigation Satellite System (DGNSS)

- Electronical Chart Display and Information System (ECDIS) on-board

Good A to N s for a restricted channel (definition to be discussed and decided at ANM 13):

- Paired lighted buoys with radar reflectors, buoy separation distance not more than 1.2 of optimal distance (determined by A to N experts)

- Lighted leading lines

- Differential Global Navigation Satellite System (DGNSS)

- Electronical Chart Display and Information System (ECDIS) on-board

or

- Paired lighted buoys with radar reflectors, optimal buoy separation distance (determined by A to N experts)

- Lighted leading lines

- VTS service with navigational assistance

- Global Navigation Satellite System (GNSS)

Moderate A to N s for a restricted channel (definition to be discussed and decided at ANM 13):

- Lighted buoys with radar reflectors, buoy separation distance not more than 1.5 of optimal distance (determined by A to N experts)

- Differential Global Navigation Satellite System (DGNSS)

- Electronical Chart Display and Information System (ECDIS) on-board

or

- Paired lighted buoys with radar reflectors, buoy separation distance not more than 1.2 of optimal distance (determined by A to N experts)

- Lighted leading lines

- VTS service with navigational assistance

- Global Navigation Satellite System (GNSS)

- It has to be noted, that these figures are only valid for a very first estimation when designing a new fairway. For the detailed design of a new fairway or when changing the A to N system for an existing waterway A to N experts have to be asked and more investigation and analysis has to be done carefully, using also tools like risk assessment and simulation.

- Action Requested

- IALA is participating in the reviev of the PIANC Guide for Approach Channels by sending IALA experts as observer to the PIANC WG and by liason notes. There should be a confirmed opinion or “IALA expertise” to help the PIANC WG to find the right consideration of AtoN in their document.

- The relevant committee for this issue in IALA is the ANM committee. So, ANM13 members are invited to consider, comment and/or confirm this proposal for the way how to include AtoN aspects into the PIANC Guide for Approach Channels.