By the Secretary

The following are comments by IALA ANM Committee members, on the Revised draft Guideline on a mix of AtoN in fairways, to PIANC WG49 (Approach channels), which meets during the week commencing 19 October.  It is provided solely for information.

Based on the latest communication regarding a revision of the PIANC guideline and your request for comments we have the pleasure of offering our point of view in this.

Basically we still believe that the present method of calculating the width of fairways in general are adequate and that there is no need to introduce measures that will increase the result of a calculation. But a more detailed review of the elements contributing to the calculation reveals that there may be room for clarification/adjustment of some of the definitions of the factors as follows.

Length of vessel

The length of the vessel is not taken into consideration. A drift angle for a given vessel will incur a given swept path which depends on the length of the vessel. Hence, there is a relation between the length of the vessel and the fairway width which is not addressed.

Tug assistance

The increasing use of tugs actively (escorting, as breaker etc) when assisting e.g. tankers in fairways is influencing the overall manoeuvrability of the assisted vessel. Studies within recent years have shown that the width of fairways can be reduced when introducing these operations. The question is how to take this into consideration within the present calculations. Should it be taken into consideration when evaluating if the manoeuvrability (Basic Manoeuvring Lane) is good, moderate or poor? Or should there be a separate reduction factor for this? In addition, it may be considered if this issue should be addressed separately in the text.

Aids to Navigation

We are of the opinion that elements with no connection to marking will confuse the selection of factors. We are concerned that this part in section 5.2 is mixing the quality of marking with VTS, visibility etc. Also ship related input as ECDIS and DGNSS should not make any impact on the calculations. Ships beyond the SOLAS requirement are irrelevant since you can not design a fairway for certain ships with specific equipment. The consequence will then be that ships without this equipment will not be allowed to enter the fairway.

Positioning accuracy for AtoN

A factor that should be further discussed is the effect of the fairway width if the area is hydrographical surveyed and the AtoNs are positioned according to IHO/S-44 or not. As the calculations are made up now we understand it as these matters are taken for granted. This might be obvious when constructing new fairways but not always when dealing with existing ones.

In addition to above we have discussed other parts of the present guideline.

Section 5.3.6.5 Berthing and Swinging Areas

This section would benefit from some kind of elaboration. The figures could consider different type of ships and their manoeuvring capabilities for instance. In addition, we are concerned about the figures 1.8 – 2.0. Many ports in Sweden have less but are still considered having sufficient safety margins.

6.4.2 Ship handling and simulation

We suggest that this section is replaced by a short introduction with a reference to IALA’s Guideline 1058.

The following text could be used as an introduction:

“The following is the introduction from IALA’s Guideline 1058 on the use of simulation as a tool for waterway design and AtoN planning where various simulation tools, their use and limitations are presented.

Simulation is the process of designing a model of a real system and conducting experiments with this model for the purpose either of understanding the behaviour of the system or of evaluating various strategies (within the limits imposed by a criterion or set of criteria) for the operation of the system.(R.E. Shannon, 1975)

Simulation tools are capable of providing realistic and accurate results and input to the investigation and evaluation of channel and port design. The purpose of simulation for AtoN design, planning and evaluation is to identify and mitigate the risks for the mariner operating in a specific waterway, channel and port area. It also includes evaluation of channel layout, placement and technical specification of AtoN and manoeuvring aspects.

Simulation offers a relatively low cost method to help ensure that the AtoN solution provided meets the users’ requirements in an effective and efficient manner. Simulation can incorporate both physical and digital methods. By providing a simulation tool to the user, an overall improvement in safe and efficient operation can be realised by assisting in demonstrating the operation of the waterway, channel design and associated AtoN, before the reality of navigating a vessel in the area.

User consultation should be an integral part of all AtoN planning and simulation processes. Accurate simulation tools can potentially improve the usefulness of the feedback obtained from users.”

We hope that this might contribute in your further discussions in the PIANC working group in October.

Peter Sörensen

Pernilla Bergstedt

Johan Pettersson

Raul Escalante

Michael Skov