Report from Rapporteur M1

The use of the IALA Guidelines on Risk / development on the IALA Risk Assessment Tool

Once again I am ready to fill you in on some of the developments known to me since my last report in April 2009 at ANM13. An extract from this paper will be presented at ANM14 as a Power Point presentation.

Proposed working package from ANM13 was:

- To follow and report from the work in the IALA Risk Management Steering Committee including the progress of the work with IWRAP MkII (IALA Waterway Risk Assessment Program)

- To report from the training seminar for the new IWRAP MkII tool in Kuala Lumpur

- To follow and report from the final work in the BaSSy project.

- To collect and report members use and experience of the IALA Risk Management Tool. Members are encouraged to report to Rapporteur M1 accordingly.

Risk Management

Risk management is a term applied to a structured (logical and systematic) process for

- Identifying, analysing, assessing, treating, monitoring and communicating risks for any activity, and

- Achieving an acceptable balance between the costs of an incident, and the costs of implementing measures to reduce the risk of the incident happening.

The IALA Risk Management Tool

In late 2004 a Council approved steering group to oversee the application of the Tool and to assess and approve any proposed changes to the models was established.

The IALA Risk Management Steering Committee has now held their 17th meeting (held as a teleconference on 14 September 2009).

The purpose of the IWRAP MkII is to provide authorities a standardized quantitative method to evaluate the probability of collisions and groundings in a given waterway. On the basis of a specific route layout, waterway attributes, specified traffic intensity and composition, the tool allows the user to efficiently evaluate and estimate the annual number (frequency) of collisions and groundings along the route.

IWRAP MkII is capable of taking into account the risk reduction effect of Aids to Navigation, however this has not yet been automated, since there still is a lot of work to be done on quantifying the risk reduction effect of different types of Aids to Navigation.

The validation of the software was done in October and November 2008 and reported satisfactory. For the areas tested, the results obtained were found to be close to the historical evidence data at hand.

The program is presented to users together with both a practical User Manual and a Theoretical Manual describing the underlying theory and mathematical models. Each individual user would obtain from the IALA Secretariat, a license, enabling the use of IWRAP MkII on a single computer. Each license key would have an expiration date determined by IALA.

It was proposed that one Technical Committee (ANM) would be in charge of using the results send to IALA by the users of IWRAP MkII.

Since last ANM meeting both a grisksetup_v2_1_1.exe and an IWRAPMK2SETUP_BETA.exe has been added to the list of files and programs at ftp.gatehouse.dk.

The GRISK manual: grisk_manual_v1_0_19.pdf. (Equals IWRAP MkII manual) and the theoretical document: IALA_Frequencies-v8.pdf are both unchanged.

At the IALA Risk Management Steering Committee 15th meeting GateHouse explained that version 1.0.18 and version 1.0.19 of the GRISK/IWRAP MkII model were issued. A lot of modifications and improvements were made.

Input in the IWRAP MkII model is normally derived from AIS data and it has to be inputted manually. Some are capable of inputting this AIS data automatically, but it requires some technical skills.

The IALA Training Seminar on IWRAP MkII was held 20-24 April 2009 in Kuala Lumpur, Malaysia. It had 36 delegates from 15 countries.

The 16th meeting of the IALA Risk Management Steering Committee was held in Kuala-Lumpur just after the training seminar.

At the training seminar all participants got the IWRAP MkII, version 2 (GRISK v.2.0) at the end of day one. As we progressed at the training seminar the version was almost changed online by the GateHouse representative. It showed that the training seminar concept was a very good approach/idea. All laptops were able to run the programme (if they were allowed by there administrations to upload programmes to there laptops!).

The next two days were dedicated to further training and exercises for the participants including some presentation on the theoretical background.

On the final day of the training seminar some additional thoughts were presented for the participants. One presentation focused on the calibration of charts/maps, spatial accuracy of charts, polygon editing, depth interpolation and traffic distribution. The steering group will continue to develop IWRAP, continue to work on causation factors, update recommendations/guidelines, gather examples of project and monitor the use of IWRAP.

GateHouse has developed the free version further, but as this is a manual way of working a commercial version will also be available. Here it will be possible to import AIS-data automatically, extract ground polygons from ENC, e.g. S-57 format, do consequence analysis and make a pollution/emission report. At this time I have no idea of the costs.

The Council concurred with the conclusions of the Steering Group that IWRAP Mk2 should not be used by people not having been received the ad hoc training, and agreed to organise training sessions as necessary. It was agreed to hold the IWRAP Mk2 training session from 16 to 18 February 2010 in St Germain.

GateHouse has made a commercial in confidence document regarding IWRAP Extensions.

The BaSSy Project (Baltic Sea Safety)

The BaSSY Project which involved Sweden, Finland and Denmark consisted of 4 work packages:

- WP0: Project management

- WP1: Development of the GRACAT (GRounding And Collision Analysis Toolbox) program incorporating human factors into risk assessment

- WP2: Bridge and VTS-centre analysis

- WP3: Harmonized FSA-process for different areas of the Baltic Sea

The scope of the BaSSy Project was to establish a framework – toolbox that would allow estimation of the risk to human, assets and environment caused by collision and grounding in the Baltic. It should then be possible to analyse risk reducing effect of safety improving measures such as modified route layout, introduction of traffic separation zones, VTS etc.

The BaSSy Toolbox, also known as GRISK or IWRAP MkII depending on who you are talking to, is capable of doing calculations for multiple ships sailing multiple routes

The risk analysis method has been tested in two areas in the Baltic Sea which are open water and winter conditions in the Sea of Åland and dense traffic in the Bornholm Gat.

The BaSSy Toolbox evaluation made by the consultant company COWI was attached the report from the steering groups 16 meeting. In general, it can be said that GRISK uses state-of-practice methods. The example results largely resemble those from COWI’s analysis, which confirms the reliability of the approach. A standardised approach such as GRISK does not provide the same level of detail as a customised analysis like COWI’s. This is reflected by some of the results, which differ to certain, but definitely acceptable extent (parallel collisions, crossing collisions).

The BaSSy Project was planned to finish in late 2008. However, it was agreed to prolong the project until the end of June 2009 at which state all reporting should be finalized.

I received a draft report in early July 2009 for comments and I have just received the latest draft report on 10 September 2009. It was a risk analysis using GRISK performed by SSPA (Statens SkeppsProvnings Anstalt, Sweden) and the derived results were then compared with local and global accident frequencies in order to get more knowledge about the magnitude of the causation factors. The toolbox estimates the collision and grounding frequencies non-conservative compared to local and global statistics.

The toolbox has been found to be a user friendly programme which needs a more unified input and more research on parameters steering the causation factors for the collision and grounding model.

Some further conclusions:

- The input data collection demands a high level of expert knowledge

- Modelling restricted fairways can be time consuming

- The programme underestimates collision frequencies in the area, as the model is sensitive to the causation factor

- The grounding model is sensitive to the deep curves which are chosen for the calculations

- The programme can be used as a common platform for maritime collision and grounding frequency analysis.

So, all in all, they are positive about the programme, but it still needs to be developed further.

Collect and report members use and experience of the IALA Risk Management Tool

Only received information from one member who stated that the IALA Risk Management Tool was not used in the period since ANM13.

Unless otherwise decided at the IALA Conference in March 2010 the proposed working package until next meeting in October 2010 is as follows:

- To follow and report from the work in the IALA Risk Management Steering Committee including the progress of the work with IWRAP MkII

- To collect and report members use and experience of the IALA Risk Management Tool. Members are encouraged to report to Rapporteur M1 accordingly.

- To follow and report from the final work in the BaSSy project.

Michael Skov

Head of Division

Aids to Navigation Management and Policy

Danish Maritime Safety Administration (DaMSA)