IALA MARITIME BUOYAGE SYSTEM BOOKLET

DOCUMENT HISTORY:

The 2006-2010 Work Program for the ANM Committee, agreed at the IALA Conference in Shanghai, contained the Task of reviewing the IALA Maritime Buoyage System (MBS). This was undertaken by ANM Working Group 3 and following extensive stakeholder consultation the Council agreed that the ANM Committee should produce a revised IALA MBS Booklet to be launched at the 2010 IALA Conference.

- THIS DOCUMENT WAS AGREED BY AMN13 (APRIL 2009) AS THE FINAL DRAFT AND LEFT WITH THE SECRETARIAT TO ACCEPT THE TRACKED CHANGES, CARRY OUT AN EDITORIAL CHECK AND SUBMIT TO COUNCIL FOR APPROVAL

- THE DOCUMENT WAS SUBMITTED TO COUNCIL WITH TRACKED CHANGES STILL SHOWN AND WITHOUT EDITORIAL CHECK

- THE DRAFT REVISED IALA MARITIME BUOYAGE SYSTEM BOOKLET WAS REJECTED BY COUNCIL AT THEIR MEETING MAY 2009

- COUNCIL UNDERTOOK TO PROVIDE COMMENTS ON THE REJECTED DOCUMENTMTO SECRETARIAT BY 20TH JUNE

- DRAFT REVISED DOCUMENT AGREED AT ANM13 HAS TRACKED CHANGES ACCEPTED AND COUNCIL COMMENTS RECEIVED ARE ADDED AS NEW TRACKED CHANGES TO THIS VERSION.

02.07.09 	Tracked changes accepted and existing front cover replaced with proposed new cover from Japan Coastguard.

03.07.09	Proposed changes from IALA Secretariat added as tracked changes in Green

Changes proposed by Germany added as comments

Proposed changes from Ireland added and highlighted in turquoise

04.07.09	Proposed changes from South Africa added as tracked changes in Pink

    - Proposed changes from Spain added and highlighted in yellow

    - ANM WG3 Members met ‘virtually’ to consider Council comments.

23.07.09	MN Accepted all track changes and distribute to WG.

Maritime Buoyage System

And

Other Aids to Navigation

IALA

International Association of Marine Aids to Navigation and

Lighthouse Authorities

International Association of Marine Aids to Navigation and Lighthouse Authorities

MARITIME BUOYAGE SYSTEM

HISTORICAL BACKGROUND

PRIOR TO 1976

There was once more than thirty different buoyage systems in use world-wide, many of these systems having rules in complete conflict with one another.

There has long been disagreement over the way in which buoy lights should be used since they first appeared towards the end of the 19th century. In particular, some countries favoured using red lights to mark the port hand side of channels and others favoured them for marking the starboard hand.

Another major difference of opinion revolved around the principles to be applied when laying out marks to assist the mariner. Most countries adopted the principle of the Lateral system whereby marks indicate the port and starboard sides of the route to be followed according to some agreed direction. However, several countries also favoured using the principle of Cardinal marks whereby dangers are marked by one or more buoys or beacons laid out in the quadrants of the compass to indicate where the danger lies in relation to the mark, this system being particularly useful in the open sea where the Lateral buoyage direction may not be apparent.

The nearest approach to international agreement on a unified system of buoyage was reached at Geneva in 1936. This Agreement, drawn up under the auspices of the League of Nations, was never ratified due to the outbreak of World War II. The Agreement proposed the use of either Cardinal marks or Lateral marks but separated them into two different systems. It provided for the use of the colour red on port hand marks and largely reserved the colour green for wreck marking.

At the end of World War II many countries found their aids to navigation destroyed and the process of restoration had to be undertaken urgently. In the absence of anything better, the Geneva rules were adopted with or without variation to suit local conditions and the equipment available. This led to wide and sometimes conflicting differences particularly in the crowded waters of North Western Europe.

In 1957 the, then, International Association of Lighthouse Authorities (IALA) was formed in order to support the goals of the technical lighthouse conferences which had been convening since 1929.

Attempts to bring complete unity had little success. Fresh impetus was given to the task of the IALA Technical Committee, by a series of disastrous wrecks in the Dover Strait area in 1971. These wrecks, situated in one lane of a traffic separation scheme, defied all attempts to mark them in a way that could be readily understood by mariners.

There were three basic issues to address:

- the need to retain existing equipment as far as possible to avoid undue expense

- the need to define how the colours green and red were to be used when marking channels

- the need to combine Lateral and Cardinal rules.

To meet the conflicting requirements, it was thought necessary as a first step to formulate two systems, one using the colour red to mark the port hand side of the channels and the other using the colour red to mark the starboard hand side of channels. These were called System A and System B, respectively.

The rules for System A, which included both cardinal and lateral marks, were completed in 1976 and agreed by the International Maritime Organization (IMO) The System was introduced in 1977 and its use has gradually spread throughout Europe, Australia, New Zealand, Africa, the Gulf and some Asian Countries.

From 1980

The rules for System B were completed in early 1980. These were considered to be suitable for application in North, Central and South America; Japan; Korea and The Philippines.

The rules for the two Systems were so similar that the IALA Executive Committee was able to combine the two sets of rules into one, known as “The IALA Maritime Buoyage System”. This single set of rules allows Lighthouse Authorities the choice of using red to port or red to starboard, on a regional basis; the two regions being known as Region A and Region B.

At a Conference convened by IALA in November 1980 with the assistance of IMO and the International Hydrographic Organization (IHO), Lighthouse Authorities from 50 countries and the representatives of nine International Organisations concerned with aids to navigation met and agreed to adopt the rules of the new combined System. The boundaries of the buoyage regions were also decided and illustrated on a map annexed to the rules. The Conference underlined the need for cooperation between neighbouring countries and with Hydrographic Services in the introduction of the new System.

From 2010

Although the maritime buoyage system (MBS) has served the maritime community well since its inception in the 1970s, after the 2006 IALA Conference in Shanghai, China, it was decided to review the system in light of changes in the navigation environment and the further development of electronic aids to navigation.

Worldwide consultation revealed that the fundamental principles of the MBS should be retained. However, due to changes in navigation practices and patterns, as well as innovations and technological developments, some enhancements to the MBS were needed.

Ideally, a unified marking arrangement would, in principle, be desirable for Regions A and B.  All IALA Members view this change as impractical, detrimental to safety, and probably unachievable. However, with the aim of improving navigational safety, advances towards a global unified system can be achieved through adoption of common characteristics, such as consistent lighting rhythms, on port and starboard hand Marks regardless of region.

The most significant changes in the 2010 revision are the inclusion of aids to navigation used for marking recommended by IALA that are additional to the floating buoyage system previously included. This is aimed at providing a more complete description of aids to navigation that may be used. It includes the Emergency Wreck Marking Buoy, descriptions of other aids to navigation specifically excluded from the original MBS, and the integration of electronic marks via radio transmission. With regards to aids to navigation, the changes provided by this revision will allow the emerging e-Navigation concept to be based upon the marks provided by this booklet.

Thus, the IALA Maritime Buoyage System will continue to help all Mariners, navigating anywhere in the world, to fix their position and avoid dangers without fear of ambiguity, now and for the years to come.

GENERAL PRINCIPLES OF THE SYSTEM

The responsibility for safe navigation resides with the mariner, through the appropriate use of aids to navigation in conjunction with official nautical documents and prudent seamanship, including voyage planning as defined in IMO Resolutions.  This booklet provides guidance on the Maritime Buoyage System and other aids to navigation for all users.

The IALA Aids to Navigation system has two components: The Maritime Buoyage System and other aids to navigation comprised of fixed and floating devices.  This is primarily a physical system, however all of the marks may be complimented by electronic means.

Within the Maritime Buoyage System there are six types of marks, which may be used alone or in combination. The mariner can distinguish between these marks by identifiable characteristics.  Lateral marks differ between Buoyage Regions A and B, as described below, whereas the other five types of marks are common to both regions.

These marks are described below:

LATERAL MARKS

Following the sense of a “conventional direction of buoyage”, lateral marks in Region A utilize red and green colours (refer to section 2.4) by day and night to denote the port and starboard sides of channels respectively. However, in Region B (refer to section 2.5) these colours are reversed with red to starboard and green to port.

A modified lateral mark may be used at the point where a channel divides to distinguish the preferred channel, that is to say the primary route or channel that is so designated by the competent Authority.

CARDINAL MARKS

Cardinal marks indicate that the deepest water in the area lies to the named side of the mark. This convention is necessary even though for example, a North mark may have navigable water not only to the North but also East and West of it. The mariner will know it is safe to the North, but shall consult the chart for further guidance.

Cardinal marks do not have a distinctive shape but are normally pillar or spar. They are always painted in yellow and black horizontal bands and their distinctive double cone top-marks are always black.

An aide-memoire to their colouring is provided by regarding the top-marks as pointers to the positions of the black band(s):

- North:	Top-marks pointing upward: black band above yellow band

- South: 	Top-marks pointing downward: black band below yellow band

- East: 	Top-marks pointing away from each other: black bands above and below a yellow band

- West:		Top-marks pointing towards each other: black band with yellow bands above and below.

Cardinal marks also have a special system of flashing white lights. The rhythms are basically all “very quick” (VQ) or “quick” (Q) flashing but broken into varying lengths of the flashing phase.  “Very quick flashing” is defined as a light flashing at a rate of either 120 or 100 flashes per minute, “quick flashing” is a light flashing at either 60 or 50 flashes per minute.

The characters used for Cardinal marks will be seen to be as follows:

- North: Continuous very quick flashing or quick flashing;

- East: Three “very quick” or “quick” flashes followed by darkness;

- South: Six “very quick” or “quick” flashes followed immediately by a long flash, then darkness;

- West: Nine “very quick” or “quick” flashes followed by darkness.

The concept of three, six, nine is easily remembered when one associates it with a clock face. The long flash, defined as a light appearance of not less than 2 seconds, is merely a device to ensure that three or nine “very quick” or “quick” flashes cannot be mistaken for six.

It will be observed that two other marks use white lights; Isolated Danger Marks and Safe Water Marks. Each has a distinctive light rhythm that cannot be confused with the very quick or quick flashing light of the Cardinal marks.

ISOLATED DANGER MARK

The Isolated Danger mark is placed on, or near to a danger that has navigable water all around it. Because the extent of the danger and the safe passing distance cannot be specified for all circumstances in which this mark may be used, the mariner shall consult the chart and nautical publications for guidance. Distinctive double black spherical top-marks and Group flashing (2) white lights, serve to associate Isolated Danger marks with Cardinal marks.

SAFE WATER MARKS

The Safe Water mark has navigable water all around it, but does not mark a danger. Safe Water marks can be used, for example, as fairway, mid-channel or landfall marks.

Safe Water marks have an appearance different from danger marking buoys. They are spherical, or alternatively pillar or spar with red and white vertical stripes and a single red spherical top-mark. Their lights, if any, are white using isophase, occulting, one long flash or Morse “A” ( ─ ) rhythms.

# SPECIAL MARKS

Special marks are used to indicate a special area or feature whose nature may be apparent from reference to a chart or other nautical publication. They are not generally intended to mark channels or obstructions where the MBS provides suitable alternatives.

Special marks are yellow. They may carry a yellow “X” top-mark, and any light used is also yellow. To avoid the possibility of confusion between yellow and white in poor visibility, the yellow lights of Special marks do not have any of the rhythms used for white lights.

Their shape will not conflict with that of navigational marks. This means, for example, that a special buoy located on the port hand side of a channel may be cylindrical but will not be conical. Special marks may be lettered or numbered, and may also include the use of a pictogram to indicate their purpose using the IHO symbology where appropriate.

MARKING NEW DANGERS

“New Dangers” are newly discovered hazards, natural or man-made, that may not yet be shown in nautical documents and publications, and until the information is sufficiently promulgated, should be indicated by:

- marking a new danger using appropriate marks such as; Lateral, Cardinal, Isolated Danger marks, or equally

- using the Emergency Wreck Marking Buoy (EWMB)

If the competent Authority considers the risk to navigation to be especially high at least one of the marks should be duplicated.

The Emergency Wreck Marking Buoy has blue and yellow vertical stripes in equal number, with a vertical/perpendicular yellow cross top-mark, and displays a blue and yellow alternating light.

Marking of a new danger may include use of a Racon coded Morse “D” (─ ) or other radio transmitting device such as automatic identification systems as an Aid to Navigation (AIS as an AtoN).

Marking of a new danger may be discontinued when the appropriate competent Authority is satisfied that information concerning the “New Danger” has been sufficiently promulgated or the danger has been resolved.

OTHER MARKS

Other Marks include lighthouses, beacons, sector lights, leading lines, major floating aids, and auxiliary marks. These visual marks are intended to aid navigation as information to mariners, not necessarily regarding channel limits or obstructions.

- Lighthouses, Beacons and other aids of lesser ranges are fixed aids to navigation that may display different colours and/or rhythms over designated arcs. Beacons may also be unlighted.

- Sector lights display different colours and/or rhythms over designated arcs. The colour of the light provides directional information to the mariner.

- Leading lines / Ranges allow ships to be guided with precision along a portion of a straight route using the alignment of fixed lights (leading lights) or marks (leading marks), in some cases a single directional light may used.

- Major floating aids include lightvessels, light floats and large navigational buoys intended to mark approaches from off shore.

- Auxiliary Marks are those other marks used to assist navigation or provide information. These include aids of non-lateral significance that are usually of defined channels and otherwise do not indicate the port and starboard sides of the route to be followed as well as those used to convey information for navigational safety.

- Port or Harbour Marks such as breakwater, quay/jetty lights, traffic signals, bridge marking and inland waterways aids to navigation (further described in section 8.7)

International Association of Marine Aids to Navigation and Lighthouse Authorities

MARITIME BUOYAGE SYSTEM

And Other Aids to Navigation

RULES

- GENERAL

  - Scope

The Maritime Buoyage System and Other Aids to Navigation provides rules that apply to all fixed, floating and electronic marks serving to indicate:

    - The lateral limits of navigable channels.

    - Natural dangers and other obstructions such as wrecks.

    - Landfall, course to steer, and other areas or features of importance to the mariner.

    - New dangers.

  - Types of marks

A Mark is defined as a signal available to the Mariner to convey guidance in safe navigation.  The Maritime Buoyage System and Other Aids to Navigation provide the following types of marks that may be used in combination:

    - Lateral marks, used in conjunction with a “conventional direction of buoyage”, generally employed for well defined channels.  These marks indicate the port and starboard sides of the route to be followed.  Where a channel divides, a modified lateral mark may be used to indicate the preferred route.  Lateral marks differ between Buoyage Regions A and B as described in MBS Sections 2 and 8.

    - Cardinal marks, used in conjunction with the mariner's compass, to indicate where the mariner may find navigable water.

    - Isolated Danger marks to indicate isolated dangers of limited size that have navigable water all around them.

    - Safe Water marks to indicate that there is navigable water all around their position, e.g. mid-channel marks.

    - Special marks to indicate an area or feature referred to in nautical documents, not generally intended to mark channels or obstructions.

    - Other marks used to provide information to assist navigation.

  - Method of characterising marks

The significance of the mark depends upon one or more of the following features:

    - By night, colour and rhythm of light and/or illumination enhancement

    - By day, colour, shape, top-mark, and/or light (including colour and rhythm).

    - By electronic (digital) symbol e.g. as a complement to physical marks.

    - By electronic (digital) symbol solely.

- LATERAL MARKS

  - Definition of “conventional direction of buoyage”

The “conventional direction of buoyage”, which must be indicated in appropriate nautical charts and documents, may be either:

    - The general direction taken by the mariner when approaching a harbour, river, estuary or other waterway from seaward, or

    - The direction determined by the proper authority in consultation, where appropriate, with neighbouring countries.  In principle, it should follow a clockwise direction around land masses.

  - Buoyage Regions

    - There are two international Buoyage Regions A and B, where lateral marks differ. The current geographical divisions of these two Regions are shown on the world map on the inside front cover of this booklet.

  - General Rules for Lateral Marks

    - Colour

The colour of lateral marks must comply with the IALA MBS Regions as specified in Sections 2.4 and 2.5

    - Shapes

Lateral marks should be of cylindrical and conical shape. However, where they do not rely on a distinctive shape for identification, they should, where practicable, carry the appropriate topmark.

    - Numbering or lettering

If marks at the sides of a channel are numbered or lettered, the numbering or lettering shall follow the “conventional direction of buoyage” i.e. numbered from seaward. The protocol for numbering lateral marks, especially in confined waterways, should be “even numbers on red ~ odd numbers on green”.

    - Synchronisation

If appropriate, synchronised lights (all flash at the same time) or sequential lights (flash one after another) or a combination of both may be utilized

  - Description of Lateral Marks used in Region A

|  | 2.4.1 Port hand Marks | 2.4.2. Starboard hand Marks |
| --- | --- | --- |
| Colour | Red | Green |
| Shape of buoy | Cylindrical (can), pillar or spar | Conical, pillar or spar |
| Topmark (if any) | Single red cylinder (can) | Single green cone, point upward |
| Light (when fitted) |  |  |
| Colour | Red | Green |
| Rhythm | Any, other than that described in section 2.4.3. | Any, other than that described in section 2.4.3. |

2.4.3 At the point where a channel divides, when proceeding in the “conventional direction of buoyage”, a preferred channel may be indicated by a modified Port or Starboard lateral mark as follows:

|  | 2.4.3.1. Preferred channel to Starboard | 2.4.3.2. Preferred channel to Port |
| --- | --- | --- |
| Colour | Red with one broad green horizontal | Green with one broad red horizontal |
| Shape of buoy | Cylindrical (can), pillar or spar | Conical, pillar or spar |
| Topmark (if any) | Single red cylinder (can) | Single green cone, point upward |
| Light (when fitted) |  |  |
| Colour | Red | Green |
| Rhythm | Composite group flashing (2 + 1) | Composite group flashing (2 + 1) |

  - Description of Lateral Marks used in Region B

|  | 2.5.1. Port hand Marks | 2.5.2. Starboard hand Marks |
| --- | --- | --- |
| Colour | Green | Red |
| Shape of buoy | Cylindrical (can), pillar or spar | Conical, pillar or spar |
| Topmark (if any) | Single green cylinder (can) | Single red cone, point upward |
| Light (when fitted) |  |  |
| Colour | Green | Red |
| Rhythm | Any, other than that described in section 2.5.3. | Any, other than that described in section 2.5.3. |

2.5.3 At the point where a channel divides, when proceeding in the “conventional direction of buoyage”, a preferred channel may be indicated by a modified Port or Starboard lateral mark as follows:

|  | 2.5.3.1. Preferred channel to Starboard | 2.5.3.2. Preferred channel to Port |
| --- | --- | --- |
| Colour | Green with one broad red horizontal | Red with one broad green horizontal |
| Shape of buoy | Cylindrical (can), pillar or spar | Conical, pillar or spar |
| Topmark (if any) | Single green cylinder (can) | Single red cone, point upward |
| Light (when fitted) |  |  |
| Colour | Green | Red |
| Rhythm | Composite group flashing (2 + 1) | Composite group flashing (2 + 1) |

- CARDINAL MARKS

  - Definition of Cardinal quadrants and marks

The four quadrants (North, East, South and West) are bounded by the true bearings NW-NE, NE-SE, SE-SW, and SW-NW, taken from the point of interest.

    - A Cardinal mark is named after the quadrant in which it is placed.

    - The name of a Cardinal mark indicates that it should be passed to the named side of the mark.

    - The Cardinal Marks in Region A and Region B, and their use, are the same

  - Use of Cardinal Marks

A Cardinal mark may be used, for example:

    - To indicate that the deepest water in that area is on the named side of the mark.

    - To indicate the safe side on which to pass a danger.

    - To draw attention to a feature in a channel such as a bend, a junction, a bifurcation or the end of a shoal.

    - Competent Authorities should consider carefully before establishing too many cardinal marks in a waterway or area as this can lead to confusion, given their white lights of similar characteristics.

3.3 Description of Cardinal Marks

|  | 3.3.1. North Cardinal Mark | 3.3.2. East Cardinal Mark |
| --- | --- | --- |
| Topmark (a) | 2 black cones, one above the other, points upward | 2 black cones, one above the other, base to base |
| Colour | Black above yellow | Black with a single broad horizontal yellow band |
| Shape of buoys | Pillar or spar | Pillar or spar |
| Light (when fitted) |  |  |
| Colour | White | White |
| Rhythm | VQ or Q | VQ(3) every 5s or Q(3) every 10s |

|  | 3.3.3. South Cardinal Mark | 3.3.4. West Cardinal Mark |
| --- | --- | --- |
| Topmark (a) | 2 black cones, one above the other, points downward | 2 black cones, one above the other, point to point |
| Colour | Yellow above black | Yellow with a single broad horizontal black band |
| Shape of buoys | Pillar or spar | Pillar or spar |
| Light (when fitted) |  |  |
| Colour | White | White |
| Rhythm | VQ(6) + Long flash every 10s or Q(6) + Long flash every 15s | VQ(9) every 10s or Q(9) every 15s |

Note: (a) The double cone top-mark is a very important feature of every Cardinal mark by day, and should be used wherever practicable and be as large as possible with a clear separation between the cones.

- ISOLATED DANGER MARKS

  - Definition of Isolated Danger Marks

An Isolated Danger mark is a mark that is established on or near to an isolated danger that has navigable water all around it. Because the extent of the danger and the safe passing distance cannot be specified solely by the mark for all circumstances, thus the mariner shall consult their chart and nautical publications for guidance. Electronic means may be used to complement this with additional information.

  - Description of Isolated Danger Marks

| Top-mark (b) | 2 black spheres, one above the other |
| --- | --- |
| Colour | Black with one or more broad horizontal red bands |
| Shape of buoys | Optional, but not conflicting with lateral marks |
| Light (when fitted) |  |
| Colour | White |
| Rhythm | Group flashing (2) |

Note: (b) The double sphere topmark is a very important feature of every Isolated Danger mark by day, and should be used wherever practicable and be as large as possible with a clear separation between the spheres.

- SAFE WATER MARKS

  - Definition of Safe Water Marks

Safe Water marks serve to indicate that there is navigable water all round the mark. These include centre line marks and mid-channel marks. Such a mark may also be used to indicate channel entrance, port or estuary approach, or landfall. The light rhythm may also be used to indicate best point of passage under bridges

  - Description of Safe Water Marks

| Colour | Red and white vertical stripes |
| --- | --- |
| Shape of buoy | Spherical; pillar or spar with spherical topmark |
| Topmark (if any) | Single red sphere |
| Light (when fitted) |  |
| Colour | White |
| Rhythm | Isophase, occulting, one long flash every 10s or Morse “A” |

- SPECIAL MARKS

  - Definition of Special Marks

Marks used to indicate a special area or feature whose nature may be apparent from reference to a chart or other nautical publication. They are not generally intended to mark channels or obstructions where other marks are more suitable.

Some examples of uses of Special Marks are:

    - Ocean Data Acquisition Systems (ODAS) marks.

    - Traffic separation marks where use of conventional channel marking may cause confusion.

    - Spoil Ground marks.

    - Military exercise zone marks.

    - Cable or pipeline marks.

    - Recreation zone marks.

    - Boundaries of anchorage areas

    - Structures such as offshore renewable energy installations

    - Aquaculture

  - Description of Special Marks

| Colour | Yellow |
| --- | --- |
| Shape of buoy | Optional, but not conflicting with lateral marks |
| Topmark (if any) | Single yellow “X” shape |
| Light (when fitted) |  |
| Colour | Yellow |
| Rhythm | Any, other than those reserved for cardinal, isolated danger and safe water marks. |
| Pictogram | The use of pictograms is authorized, as defined by a competent authority. |

- MARKING NEW DANGERS

  - Definition of New Dangers

The term “New Danger” is used to describe newly discovered hazards not yet shown in nautical documents. “New Dangers” include naturally occurring obstructions such as sandbanks or rocks or man-made dangers such as wrecks.

  - Marking of New Dangers

    - “New Dangers” should be appropriately marked using Lateral, Cardinal, Isolated Danger marks or by using the Emergency Wreck Marking Buoy. If the Authority considers the risk to navigation to be especially high, at least one of the marks should be duplicated.

If using a Cardinal or Lateral lighted mark for this purpose a VQ or Q light character shall be used.

    - Any duplicate mark shall be identical to its partner in all respects.

    - In addition it may be marked by a Racon, coded Morse “D” (─ ).

    - In addition it may be marked by other electronic means, such as automatic identification system (AIS as an       AtoN).

    - Virtual Aids to Navigation may be deployed solely or in addition to physical Aids to Navigation.

    - The marking of the new danger may be removed when the competent Authority is satisfied that information concerning the “New Danger” has been sufficiently promulgated or the danger otherwise resolved.

    - Emergency Wreck Marking Buoy is described below:

| Colour | Blue/Yellow vertical stripes in equal number dimensions (minimum 4 stripes and maximum 8) |
| --- | --- |
| Shape of buoy | Pillar or spar |
| Topmark (if any) | Vertical/perpendicular Yellow cross |
| Light |  |
| Colour | Yellow/blue alternating |
| Rhythm | One second of blue light and one second of yellow light with 0.5 sec. of darkness between |

- OTHER MARKS

  - LEADING LINES / RANGES

    - Definition of Leading Lines/Ranges

A group of two or more marks or lights, in the same vertical plane such that the navigator can follow the leading line on the same bearing.

    - Description of Leading Lines

Leading Line structures can be any colour or shape that provides a distinctive mark that cannot be confused with adjacent structures.

| Colour | No colour significance. Competent authority determines the optimum colours to contrast with the dominant background colour at the location. |
| --- | --- |
| Shape | No shape significance. Rectangular or triangular figures are recommended. |
| Light (when fitted) |  |
| Colour | Any colour. Competent authority determines the optimum colour to contrast with the dominant background colour at the location. |
| Rhythm | Any, however fixed characteristics should be used sparingly and the use of synchronisation can assist in the overcoming background light. |

  - SECTOR LIGHTS

    - Definition of Sector Lights

A sector light is a fixed aid to navigation that displays a light of different colours and/or rhythms over designated arcs. The colour of the light provides directional information to the mariner.

    - Description of Sector Lights

A sector light may be used:

- To provide directional information in a fairway

- To indicate a turning point, a junction with other channels, a hazard or other items of navigational importance

- To provide information on hazard areas that should be avoided

- In some cases a single directional light may be used.

| Colour |  | Not applicable |
| --- | --- | --- |
| Shape |  | None, light only |
| Light |  |  |
| Colour | If using to mark channel limits follow convention for IALA region indicated in Section 2. Lights may have ‘oscillating boundaries’ |  |
| Rhythm | As appropriate |  |

  - LIGHTHOUSES

    - Definition of a Lighthouse

A lighthouse is a tower, or substantial building or structure, erected at a designated geographical location to carry a signal light and provides a significant daymark. It provides a long or medium range light for identification by night.

    - Description of a Lighthouse

It may provide a platform for other AtoNs such as DGNSS, racon or AIS as an AtoN to assist marine navigation. A lighthouse is a structure that and may provide a daymark for identification by day. A sector light may also be incorporated into the structure.

| Colour / Shape | Lighthouse structures can be of any colour, shape, or material generally designed to provide a distinctive daymark. |
| --- | --- |
| Light |  |
| Colour | White, Red, or Green |
| Rhythm | Any number of flashes, isophase or occulting or as appropriate, to allow light to be readily identifiable. |

  - Beacons

    - Definition of a Beacon

A fixed man-made navigation mark that can be recognised by its shape, colour, pattern, topmark, or light character, or a combination of these.

    - Description of a Beacon

- Can carry a signal light and in this case is termed a light beacon or lighted beacon

- If not fitted with a light it is termed an unlighted or unlit beacon and provides only a day mark

- As a leading line/range or conspicuous radar mark

- It may also carry a topmark

| Colour | Any |
| --- | --- |
| Shape | As appropriate, including cardinal marks |
| Topmark (if any) | As appropriate |
| Light (when fitted) |  |
| Colour | White, Red, or Green |
| Rhythm | As appropriate |

  - MAJOR FLOATING AIDS

    - Definition of Major Floating Aids

Major floating aids include lightvessels, light floats and large navigational buoys.

    - Description of Major Floating Aids

Major floating aids are generally deployed at critical locations, intended to mark approaches from offshore areas, where shipping traffic concentrations are high. It may provide a platform for other AtoNs such as, racon or AIS as an AtoN to assist marine navigation.

| Colour | As appropriate - predominantly red |
| --- | --- |
| Shape of buoy | Vessel or buoy shape with light tower |
| Light (when fitted) including off station lights |  |
| Colour | As appropriate |
| Rhythm | As appropriate |

  - AUXILIARY MARKS

    - Definition

Minor aids that have not been previously described.

    - Description

These marks are usually outside of defined channels and generally do not indicate the port and starboard sides of the route to be followed or obstructions to be avoided. They also include those marks used to convey information related to navigation safety. These marks shall not conflict with other navigational marks and shall be promulgated in appropriate nautical charts and documents. Should not generally be used if a more appropriate mark is available within the MBS.

- IALA RECOMMENDATIONS AND GUIDELINES

IALA Recommendations and Guidelines provide information on planning, operating, managing, and implementing the marks authorized by the MBS and can be found via the IALA website at:  www.iala-aism.org

N

Comment on the charts from Ireland:  These charts do not seem to be referenced in the document.  Perhaps this is ok!  The symbol on the top right of the chart is presumably a satellite, not mentioned in the document.  Lower centre is a “Virtual EWMB” not referenced in the document.  A little further west is an “Electronic (Digital) Guard Ring” not mentioned in the document.  Nor is mention made of the “AIS Shore Station”.     Action by WG to be agreed.

Additional comment from South Africa:  As the opportunity was used to include other AtoN in the MBS, this document should, in my view also address the following, some of which are based on the MBS principles, seeing that these are not addressed in any other IALA documentation that I know of.  Principles of marking harbour/ports, e.g. those listed below (which may not be all inclusive).

Breakwater / Entrance lights.  Also address the situation: When the one breakwater / entrance light may not be opposite the other, but extend further out to sea (as a protruding point).  When should a breakwater light be red / green and when can it be a white light.   A long breakwater needs to be marked halfway.  Quay/Jetty lights.  Also when lights are required to be on both sides of the jetty.   Other points of interest: Bridge markings. Port Traffic Signals - perhaps refer to the IALA Recommendation.  Principles of marking inland waterways with a combination of  fixed and floating AtoN.

INTERNATIONAL ASSOCIATION OF MARINE AIDS TO NAVIGATION AND LIGHTHOUSE AUTHORITIES

ASSOCIATION OF INTERNATIONALE de SIGNALISATION MARITIME

20 ter Rue Schnapper

78100 St Germain-en-Laye

France

Tel : 33 1 34 51 70 01

Fax : 33 1 34 51 82 05

e-mail: iala-aism@wanadoo.fr

www.iala-aism.org