Dear Sea Going Correspondence Group Member

We have been asked to provide the IALA Aids to Navigation Management Committee with a brief update on user experience of AIS. Would you please provide feedback/comment on the following:

- Do you use AIS for collision avoidance, spatial awareness, Aids to Navigation identification?

- Is AIS improving in terms of data reality

- Can you see the AIS targets on ECDIS/radar

- Overall assessment of AIS

- Any other comments on the use of AIS.

- 1

- Do you use AIS for collision avoidance Yes, spatial awareness Yes , Aids to Navigation identification Not in the port I work in at this stage

- Is AIS improving in terms of data reality. Not really sure – I access ships’ data on the AIS system in the pilot station (we have it on a big TV screen in the office overlaid on to an electronic chart) and some of it is out of date, incorrect or just plain wrong. Crap in = Crap Out as always! A ship the other day had the wrong name displayed.

- Can you see the AIS targets on ECDIS/radar Yes and Yes

- Overall assessment of AIS A very useful aid to navigation to be treated with the caution that any aid to navigation is normally treated

- Any other comments on the use of AIS. I have to turn the vectors from the AIS targets on the RADAR screen off as when the tugs approach the ship on an inbound job the alarm continuously sounds, causing annoyance and distraction. We can see which way the tide is setting by looking at the AIS headings of the ships on the ECDIS in the pilot station which is handy to see if the tide has changed. We do not have a RADAR facility at the pilot station so the AIS is essential for our traffic management of the port. We can see vessels arriving and call them up on VHF to ensure they are going to the correct anchorage or inform them of outbound vessels. We can also see if they have anchored in position and warn other ships if the anchored vessel is out of position. Once ships commence proceeding from their anchorage to the pilot boarding ground we can check to see if they are moving which ensures that we know they are on the move before we head out to pilot them in. Sometimes the ship is not moving and a call on VHF can ensure that they are aware of their berthing instructions and are proceeding inbound. For us here it is almost essential that we have AIS as without it we would be quite lost.

2

Do you use AIS for collision avoidance, spatial awareness, Aids to Navigation identification? 
 
Yes all of the above. But the real benefit to us on the Ship is target identification. At last we can identify vessels at a distance. It means communication can be established directly if necessary.
Nav is less important but I can see that when on a sailing boat with less reliable equipment then AIS checks would be most useful.
 
Is AIS improving in terms of data reality? 
 
Not sure what data reality is but if you mean - are people putting in the correct information for others to see? Then often this is the case. If you mean, are the automatic interfaces to log and compass working satisfactorily? - that is a case of technology.
The point is that if you put your mind to it you can always adjust or remove data.
 
Can you see the AIS targets on ECDIS/radar ?
 
Yes, our interfaces put targets through to both. The trouble is that many officers tend to use the ECDIS as a radar with worrying consequences! Actually I've caught myself doing that......must remember to not do it!!
 
Overall assessment of AIS 
 
For us here on the Ship it's a useful addition to the navigation/collision avoidance portfolio. But above all it is useful for ship identification.
 
Any other comments on the use of AIS. 
 
You read a lot of paranoiac nonsense about AIS. People worried that terrorists will target them if positions are broadcast far and wide. But this has proved so far to be just that. The truth is that we have all benefitted from the information input. Internet access to AIS sites mean that we can follow ships. At last we are all visible and accountable!

3

Subject: Re: AIS

I don’t use AIS alone for collision avoidance, however I do check a ship’s destination when planning ahead, but I am always conscious of the fact that a vessel’s voyage related data is only as reliable as the person that put that info in there.  I often use the MKD unit and ECDIS to check for AIS targets that are further away than the range scale I have set on my radar.

Where AIS is fitted to AtoNs, I don’t usually look at the AIS targets on the Radar or ECDIS for spatial awareness, I prefer to look out the window and get instant visual confirmation.  However in poor weather conditions the clarity of an AIS target on the Radar is of utmost benefit in creating spatial awareness for the mariner.

Yes I use AIS for AtoN identification, it is particularly useful for highlighting an AtoN at a much greater distance than the mariner would normally take notice of it, and for letting the mariner know that the AtoN is on station.

Yes there has been a significant improvement in terms of data reality since AIS first came out, but ship’s are still displaying incorrect voyage related data, with ETAs, destinations and incorrect status being the most common errors.  All pre-sailing checklists and watch handover checklists should be updated to include, updating of AIS and checking correct heading input.

Yes we have AIS overlay on ECDIS and Radars onboard.  On one particular make of radar, I find on occasion I have to switch off the AIS input.  The reason for this is if I acquire and select an Arpa target and have already selected the same target as an AIS target, and if the CPA of the AIS target is less than that of the Arpa target, the radar understands it’s the one target but will chose only to display the target with the least CPA that being the AIS target.  I’m just concerned with the Arpa target info, and as a result I end up switching the AIS feed to the radar off.

There are many problems associated with using AIS, such as, not all vessels are fitted with AIS, it is totally dependent on the correct and continued operation of GNSS, and care is required where AIS is overlaid on Radar or ECDIS as some manufacturers have chosen only to overlay ship messages,  Important messages on the MKD may not be visible on the overlay.

At the moment the biggest advantage of AIS to the mariner is the clear identification of AIS targets in the area and the much earlier identification of AIS targets over ARPA targets.  Regardless of weather conditions the AIS targets are always available for the mariner to see on their AIS unit and or Radar/ECDIS if overlaid.  An AIS target is also much better detected in a radar shadow area.

I fully agree with the use of VatoNs, to supplement the normal procedures for marking a hazard.  I would also like to see the broadcast of Radio Navigational Warnings being supplemented by being promulgated as an AIS target.  However an entirely different ECDIS/ Radar symbol than that of a ship AIS target or an AtoN AIS target would need to be introduced so as not to confuse the mariner.

AIS when used correctly and fully understood is a remarkable tool that has changed the way the mariner will navigate forever, however traditional navigation and communication skills must not be allowed to deteriorate due to these advances in maritime technology.  AIS training needs to be standardised and carried out in all the maritime and fisheries colleges.

4 Reference your below email , Please find my observations regards usage and limitations of the AIS: - Do you use AIS for collision avoidance, spatial awareness, Aids to Navigation identification? - Use for Identification of Vessels name and other details only - Is AIS improving in terms of data reality - Yes - Can you see the AIS targets on ECDIS/radar - Yes - Overall assessment of AIS - Good, Definitely an aid to navigation - Any other comments on the use of AIS - The AIS equipment that I have worked on were that of JRC and Furono. Have found that there was no alarm system provided on the AIS to remind us that the NAVIGATION STATUS has to be updated whenever we sailed out, Anchored, moored or were at NUC,etc. Would suggest that there be some sort of Sensing points which remind the navigator that there is a chance that he has forgotten to update the AIS Navigation Status. This information is very useful to the other vessels, especially when in Narrow Channels.

5

Referring your items:

1. We use AIS together with radar (and of course visual...) for collision avoidance and general awareness, but almost all the time integrated with the radar.

For AtN identification not so much, basically because the number of AtN that are available is still low. But the answer to your question is "yes".

2. I do not have any complaints about data reality in technical terms. I do have complaints about the data input by operators, with a few ships not updating their equipment, or switching it off. For example, if I want to know the destination of a certain vessel leaving the port, in order to better assess the next manoeuvre, I find many "moored" still on.

3. Yes, we can see and the feature is normally used.

4. I find the equipment useful and contributing to the general safety of the ship.

5. I think that the next steps, with more AtN included (also the virtual ones) and close interaction with the ECDIS, should be good.

6

Do you use AIS for collision avoidance, spatial awareness, Aids to Navigation identification?

I use AIS for collision avoidance but only to identify name of vessel and next port of call. This is to identify which vessel is calling and to determine what the vessels intended track is. My OOWs are instructed not to use the CPA on the AIS as a safe means of collision avoidance. Not used for spatial awareness and the AIS unit on my vessel is basic and cannot identify AtoN.

·         Is AIS improving in terms of data reality

Yes. However, as recently as yesterday encountered two vessels underway but listed as still moored on the AIS.

·         Can you see the AIS targets on ECDIS/radar

Yes. It is a very effective tool for identifying other vessels and their intended track (separation schemes etc). There is one very significant drawback in that it has the potential to cause a close quarters situation to the unwary. As you know AIS is GPS stabilised. When switching the AIS feed on the water track ARPA (used for collision avoidance) it is difficult to identify which is the AIS vector and ARPA calculated vector at first glance. The ARPA operator MUST manually plot the radar target or no vector will be produced leaving the OOW to think that the vector displayed is an accurate reflection of the situation. This knowledge applies to young and old OOWs, as on a number of occasions, the ‘lazy’ option has been taken by switching on AIS so that the radar target does not need to be plotted, fine until one is in an area of significant tide/current.

Another drawback is significant numbers of vessels are not listed i.e. fishing/pleasure/military and OOWs are instructed to be aware of this.

·         Overall assessment of AIS

As with all technology, a very effective tool when used correctly. Requires better instruction on its use and its limitations to maximise its potential and limit the dangers of poor use.

·         Any other comments on the use of AIS.

From my experience the equipment supplied is poorly thought out and had little or no thought given to the practicalities of its use. To give an example, on the JRC unit fitted to my vessel, to go from moored to underway and back takes five keystrokes passing along the way ‘not under command’, ‘aground’, ‘anchored’ and ‘under tow’. How ridiculous is that!

Should you require any further clarification please do not hesitate to make contact.

7

AIS has become an excellent tool in supplementing radar information and observation of other vessels when assessing risk of collision. During a recent, rather busy transit, of the Singapore Straits, using AIS to closely monitor the course and speed changes of vessels in close proximity, as soon as they occur was extremely helpful. AIS is even more useful during restricted visibility with high traffic density. Predicting what a vessel might do next i.e. when an course or speed alteration might be anticipated is possible knowing her next port and ETA. This again is very useful.

When AIS first became widely available, the data available from other vessels needed to be used with extreme caution . Often we would see a vessels’ static and voyage data  wrongly described;  for example, the ETA for next port maybe sometime in the past or the vessel steaming away from the “next port” . The data from one VLCC we past, displayed an overall length of only 60m.

In recent years with the development of VTIS around the world,  vessels are often contacted if the AIS data appears incorrect or inconsistent. AIS may also checked during SIRE inspections (for tankers) and PSC will often verify data during bridge inspections. As a result, in recent years, it appears that  bridge officers have become more alert in ensuring the data entered is correct and we are definitely seeing clear improvements in data quality which is becoming increasingly more credible and consistent.

On the vessel I command, AIS targets and data can been seen on both ECDIS and radar, although I do not permit displaying AIS data on radars as it is really unnecessary. Acquisition and tracking targets on ARPA is sufficient for collision avoidance.

The availability of static and voyage data of vessels in close proximity, allows the maneuvers to avoid collision or close-quarters situation to be discussed using VHF radio and agreed by bridge officers, before being executed. Worryingly and all too often, we witness agreements and manoevers which are totally contrary to the International Collision Regulations. Whatever the regulations I am convinced that use of AIS and VHF as basic tools for collision avoidance will continue and it is really time that the use of AIS and bridge to bridge VHF communications are considered whenever the time comes to revise the International Collision regulations.

The overall assessment is that AIS is proving to significantly improve situational awareness and is contributing to assisting collision avoidance. Obviously, like all bridge instruments, it is a tool which is required to be used in conjunction with other systems, particularly radar and visual observations.

The original requirement for carriage of AIS under SOLAS Chapter 13/ISPS Code for security reasons has been somewhat less effective.

I hope this is helpful

8

Please see my responses in red:

Dear Sea Going Correspondence Group Member

We have been asked to provide the IALA Aids to Navigation Management Committee with a brief update on user experience of AIS. Would you please provide feedback/comment on the following:

- Do you use AIS for collision avoidance, spatial awareness, Aids to Navigation identification?

YES

- Is AIS improving in terms of data reality

YES

- Can you see the AIS targets on ECDIS/radar

YES

- Overall assessment of AIS

AN EXCELLENT ADDITIONAL ELECTRONIC AID TO NAVIGATION.

- Any other comments on the use of AIS.

NONE

9

Reference your mail for AIS use:

- Do you use AIS for collision avoidance, spatial awareness, Aids to Navigation identification?  - YES

- Is AIS improving in terms of data reality - YES

- Can you see the AIS targets on ECDIS/radar - YES

- Overall assessment of AIS - VERY GOOD

- Any other comments on the use of AIS. - NEVER USE MESSAGING. I DO NOT ALOW IT BECAUSE YOUNGER OFFICERS START TO PLAY, ETC.

10

- Do you use AIS for collision avoidance, spatial awareness, Aids to Navigation identification?

- Short answer is no, excepting for target identification by name to match up with VTS radio traffic

- Is AIS improving in terms of data reality

- Not noticed any improvement, still a number showing wrong info etc

- Can you see the AIS targets on ECDIS/radar

- Yes, this is the most useful function, radar targets are identified by name

- Overall assessment of AIS

- Some useful infomation is presented to add to the mix, but I don't place any reliance on it

- Any other comments on the use of AIS.

- Some models seem to have a limited working life, my vessel is on the second replacement unit in five years

11

1. Collision avoidance (to identify target)

2. Yes, though many vessels dont have certain data displayed or even wrong data

3. On some ships - not all

4. A very useful aid

5. Data displayed to be verified regularly and a method of checking the transmitted data

and strength of transmission

12

- Do you use AIS for collision avoidance, spatial awareness, Aids to Navigation identification?

I use AIS for spatial awareness and primarily as an initial indication of other vessel’s CPA / TCPA.

- Is AIS improving in terms of data reality

No comment.

- Can you see the AIS targets on ECDIS/radar

Yes on both counts. ECDIS style display is a ‘must have’ for instant awareness. To me radar display just adds confusion.

- Overall assessment of AIS

An extremely useful aid to navigation!

- Any other comments on the use of AIS.

No comment.

I hope that the above comments are of use to you.

13

sorry for the delay in reply! have been coasting and hence busy.

following feedback:

- Do you use AIS for collision avoidance, spatial awareness, Aids to Navigation identification

Yes! AIS is being extensively used for collision avoidance.

- Is AIS improving in terms of data reality

yes. Other than the possibility that small vessels/fishing vessels may not have AIS or may have their unit switched off,  a real picture of the traffic around our vicinity is possible.

can you see the AIS targets on ECDIS/radar

- AIS info is available on ECDIS / Radar

- Overall assessment of AIS

Very Good

- Any other comments on the use of AIS.

Any target sighted on radar with which a collision risk exists is easily identifiable by its name and Callsign which is displayed on the Radar (as well as on Ecdis).

A positive identification is possible by comparison of info available on the Radar screen with that of info available on AIS screen. .. -  Compass bearing, Radar Bearing, Distance, GPS position of target, Name and callsign display on radar screen.

Helps to eliminate doubt in case of using VHF for collision avaoidance action confirmtion after positive identification Also during course changes, newly acquired target etc the radar info is delayed and AIS can be used during this period for continued update of activity of other target.

At very close range when clutter has smudged the target still AIS info can be used in the absence of Radar info of known targets.

Identification of type of vessel - FV, SV, RAM or NUC helps in deciding whose responsibility it is at a much longer range than that of lights. Helps in taking early action Identification of Destination: In many situations the destination helps us to predict what could be the other vessels next possible move ( which way he is likely to alter and when  )

14

I consider AIS to be an incredibly valuable tool for situational awareness.  Particularly when integrated with radar and/or ECDIS, AIS provides an unmatched ability to identify and effectively deal with other ships.

I utilize AIS for collision avoidance to some degree, recognizing that AIS data may reflect course changes before an ARPA plot of the same ship.

The ability to now positively identify a ship by name, rather than “vessel on my port bow”, has improved radio calls in traffic situations by an order of magnitude.  As more Deck Officers become proficient with obtaining a target vessel’s MMSI from the AIS, then using the MMSI to switch the target vessel’s radio to a working channel to confirm intentions, radio traffic will continue to improve and the volume of calls decrease.  AIS has also significantly reduced the radio calls and work load in VTS areas, as the VTS already has the data on screen.

When properly updated, a vessel’s voyage data can be a very useful tool.  Just recently, while approaching a safety fairway with a close CPA on another ship, a query of the other ship’s AIS data revealed a pilot station ETA half an hour earlier than ours, which was then confirmed by radio.  Rather than spend an entire night with the other vessel in a slow overtaking situation, we maneuvered to allow them to proceed ahead, to our mutual benefit.

I believe that AIS data should be required to be integrated with radar and/or ECDIS.  The fusion of AIS and ECDIS is a huge multiplier in situational awareness, particularly in or approaching pilotage waters.

With more major Aids to Navigation being fitted with AIS, there is a significant improvement in the ease of confirming these A to N.

My primary complaints with AIS tend to be operator-related:

-          Failure to maintain voyage data up to date.

-          Inaccurate gyro input, where heading and COG differ significantly, with the heading data not matching visual.

Finally, I strongly oppose the use of minimum keyboard display AIS units on deep sea vessels.  While they may be useful for small coastal traffic, their difficulty of use makes operators on deep sea vessels fail to update data as often as needed.

15

Writing as an Estuarial and River Pilot with some coastal pilotage (outer estuary) on an almost daily basis.....

- Do you use AIS for collision avoidance, spatial awareness, Aids to Navigation identification?

For collision avoidance, no. But it does provide useful initial information on vessel's basic dynamic status (ID, co & Speed etc).

Spatial awareness, definitely not. AIS is still far to unreliable with frequent incidences of AIS Icons not coinciding with a ships actual position (when overlayed on radar) and often incorrect status being displayed (e.g. Moored whilst underway). Furthermore the frequency of inoperative AIS sets means that no reliability can be placed on all ships being displayed.

AtoN - No but that is as there are no AIS equipped AtoNs in the Thames.

- Is AIS improving in terms of data reality

Short answer - NO not really. In reality it probably is technically but there are frequent experiences daily of incorrect or missing data being transmitted (or not).

- Can you see the AIS targets on ECDIS/radar

Yes if the ship is suitably equipped. However, it can lead to a very cluttered display if the labelling is not switched off leaving only the icons to be individually interrogated.

- Overall assessment of AIS

Useful but not reliable enough to base critical decisions upon.

- Any other comments on the use of AIS.

The MKD should be outlawed and replaced by a proper display with a proper QWERTY keyboard. Its inadequacy in both setting up systems and reading other ships data has lead to misuse and abuse of AIS in some cases.

16

Thank you very much for sending another query to SGCG. Pleas note comments:

1. Do you use AIS for collision avoidance, spatial awareness, Aids to 
Navigation identification?

Yes, we use AIS for all above mentioned purposes. Recent developments of 
virtual AtoNs are very useful for navigation.

Some officers prefer to keep AIS targets on ECDIS and one radar, and on 
another to keep only ARPA plotting. Another use AIS targets on both radars. 
Both ways proved to be very useful for collision avoidance, especially 
constant comparing AIS and ARPA CPA / TCPA data. 
Good practice in some countries is to equip fishing fleet with AIS. This 
gives both long range warning and close range avoidance help, especially 
when under reduced visibility or high sea & rain clutter on radars.

As UKHO includes Automatic Identification System (AIS) chapter in in NP282, 
this is very helpful for AtoN identification.

2. Is AIS improving in terms of data reality?

As the technology goes forward and new models are being produced, AIS is 
constantly improving in all aspects.

3. Can you see the AIS targets on ECDIS/radar?

Yes, on both radars and ECDIS

4. Overall assessment of AIS?

Very helpful navigation instrument

5. Any other comments on the use of AIS?

Quality of data provided on the radars and ECDIS depends off course from 
maker to the equipment and in types of connections between AIS and radars 
and ECDIS. It is worth to mentioned that some AIS units or radar / ECDIS AIS 
menus are user friendly while some are not, thus potential of AIS is not 
fully utilized on such "unfriendly" equipment.

17

From A Pilot’s Perspective

## Collision Avoidance, Spatial Awareness & VAtN

From an anti-collision perspective, AIS data is time-lagged and does not represent real-time information; In pilotage districts and close-quarters situations, vessels can be altering course/speed and these changes are not readily apparent until some time afterwards. In other words, a ship may be making an alteration of course and have a considerable rate-of-turn, but the AIS target data may still be displaying a steady course, for example.

Provided this delay is acknowledged, the information displayed gives a useful insight into another vessel’s movement. The AIS information is no “older” than information conveyed by VTS of the particular target’s changing condition and, thus, during blind pilotage is extremely useful.

(Collision avoidance data offered by VTS is thus no better nor worse than can be obtained from own ship’s equipment save that it is available without a VHF-conversation between VTS.)

The use of AIS data can be useful for confirmation of an individual’s spatial position, but from a Pilot’s point-of-view, it is perhaps of secondary importance as one would expect a Pilot to be familiar with his/her surroundings and familiarity of district; nonetheless, it is a useful back-up tool if the information is displayed on a chart or radar screen.

In my pilotage district, there are currently no Virtual Aids to Navigation and no impending plans to use the facility; navigation marks are of the fixed or floating type, and the preferred method to aid spatial awareness when looking through wheelhouse windows.

Again, from a Pilot’s perspective, there are some extremely good radar units that provide easily-accessed context menus for accessing AIS data; however, there are one or two makes and models that do not appear to have been well-designed to offer straightforward display of the necessary data. Ease of use is important to a boarding Pilot who arrives on a ship’s bridge and has then to familiarise himself with differing ship’s equipment.

## Data Reality

As regards data reality, there are still a small number of vessels who are requested by VTS to update their voyage-specific data, but these are isolated instances and usually boil-down to omissions by crew. As a rough guide, generally, all other data comes through quite satisfactory in my area of operation off East coast of England, however, as with all other aids to navigation, it is prudent not to rely solely on one form of collision-avoidance and data should be cross-checked by other means.

## Viewing AIS Targets on ECDIS & Radar

On most ECDIS, I find the display of an AIS target is generally lost amid all other colours on the screen. A target does not stand out readily against the background of depth soundings, depth-contour colouring/shading and other navigation symbols, let alone any super-imposed charted tracks/WO points/etc.. One has to inspect the display very closely to find an AIS target. It is an area where I believe some improvement could be made.

Radars are somewhat different. A radar displaying only echo returns, with AIS overlay, are usually very good and quite clear; the AIS symbols are usually brighter and stand-out against the ‘painted’ radar picture. Much depends on the radar software version in this case and, again, there is great variance between manufacturers.

Those radar models that allow chart-overlay, radar echo overlay and AIS display can be confusing; there is simply too much data displaying and requires a concerted effort distinguish information.

I speak from a Pilot’s perspective here, of course, and certainly accept the usefulness of chart data being readily-available to a navigator who may be a stranger to [my] locality and may prefer to have all this data on one display, rather than cross-referencing collision avoidance data with positional information, etc..

## Overall Assessment of AIS

Generally, the overall concept is extremely good and is a welcome additional source of information to the navigator on the bridge of a vessel. Like any other tool, development has helped improved AIS and further upgrades will no doubt improve its reliability the system suffered since its inception.

Best regards

18

Currently when navigating vessels within the NorthWest Europe area as a Deep-Sea Pilot I am using AIS in the following ways:

- Connection via Portable Pilot Plug for input to my laptop, using Seamaster programme and ARC’c charts.

- Use of ships stand alone AIS equipment as required by SOLAS

- When fitted to the vessel I use AIS overlay onto vessels radar and ENC equipment.

I consider that it is necessary to monitor AIS to be in full compliance with the Collision Regulations.  (Please see attached article from the EUROPILOTS Website.)

In my opinion it is wrong to use the phrase “spatial awareness” in this context I prefer the term “situational awareness”.  This I believe allows for the displacement and distortion of the targets as seen for example on vessels radar.  I feel that this situational awareness provided on radar and ENC is very important, provided it is used with caution.

Aids to navigation are one area where the use of AIS will improve navigational safety but so far we only very early in their development stage.

Many vessels are now operating with AIS overlays onto ENC/radar but due to inadequacies of training in both AIS and ECDIS there are many operators who are not totally happy with the concept and do not appear to fully understand the possible problems or benefits.

There are still many vessels that are not routinely updating their dynamic information with regards to navigational status, destination, draft etc.

In the absence of any guidelines that I am aware of the Port of destination information can be confusing and may be give:

- By Port name

- By Pilot station destination

- By ISPS port identification code.

Confusion can be observed with vessel types.  For example fishing vessels when not fishing are still identified as such and may expect preferential treatment as indicated by Rule 18.  It would be better if privileged vessels were only identified as such when they are restricted or hampered.

There is obviously a weakness within the AIS guidelines because AIS information from Aids to Navigation can be confusing.  On a recent transit through the Dover Straits, I observed the following information:

- ZC2 buoy, type/cargo fishing

- Ruytingen SW, buoy type/cargo WIG reserved for future use

- Sandette N buoy, WIG all ships of this type

- Northforeland Lighthouse, vessel type, Passenger.

AIS as the potential to greatly enhance navigational safety and awareness.  Unfortunately due to the lack of training and the large number of vessels still operating with MKD the full benefits will not be achieved in the near future.

It will be interesting to see how AIS develops over the next few years and one exciting area for this will be E-nav Port State Bridge Local Area Network.

Europilots Article

Use of AIS for collision avoidance.

Although not specifically mentioned in the current International Regulations for Preventing Collisions at Sea it can be assumed that in the event of an incident, AIS will be amongst the information considered by an inquiry.

It is likely that use of AIS would be considered prudent under the need to provide a proper lookout by all available means, (Rule 5)

AIS information can be very useful to assist the OOW when assessing if risk of collision exists, (Rule 7).

As an aid to avoiding collision and close quarter situations at sea AIS offers the following potential advantages over radar:

- Information provided in near real time

- Capability of showing near instantaneous target alterations of course and/or speed

- No target swap

- Not lost in sea or rain clutter

- Detection possible within VHF range, including around bends and behind small islands

However the use of AIS should NOT be considered to replace the need for a visual lookout and it must be used to supplement information obtained from systematic radar plotting. It is possible that if an over reliance is placed on AIS information the OOW could be in breach of Rule 7 (c), assumptions made on the basis of scanty information.