# IALA ANM 16 – Rapporteur Report

# Agenda item 5.4. Development of e-Navigation within AtoN Authorities

### Background

It is the responsibility of the ANM Committee to monitor the development and implementation of e-Navigation from the perspective of the mariner and the shore based AtoN authorities.  This report tracks progress by shore based AtoN authorities in planned and actual implementation of services encompassed by the e-Navigation concept and operational framework.

### Current and planned e-Navigation developments by IALA members

Australian Maritime Safety Authority

Australia has established an e-Navigation consultation group who receive IMO and IALA information from AMSA and have the opportunity to input their views ahead of IMO and IALA meetings on e-Navigation.  e-Navigation was heavily featured on the program for AMSA’s Aids to Navigation symposia held in 2007 and 2010 and another specific e-Navigation forum is planned to be held in 2011/12.

AMSA is about to commence the trialling of AIS AtoN in all its forms.  Some ports have already been successfully trialling AIS AtoN for up to two years and have now made their operation ongoing.

Australia participates extensively in the various IMO and IALA forums devoted to the development of e-Navigation.  Australia is implementing a under keel clearance management system in the Torres Strait, which is seen as a key e-Navigation test bed and was presented on at the recent “e-Navigation Underway” conference.

Canada

In 2010-11, the Canadian Coast Guard prepared an e-Navigation vision/strategy in collaboration with other federal partners such as Environment Canada, Transport Canada and the Canadian Hydrographic Service.  The document includes a high level implementation plan for e-Navigation along with guiding principles and a detailed user-needs matrix (refer Annex 1) depicting by navigational areas the various e-Navigation service interests from users.  The matrix was developed with input from industry stakeholders.  This work will serve to guide implementation of e-Navigation in Canada within the various federal Departments and to share the Canadian plan with international organization.

The vision/strategy document is not approved yet and cannot be shared at this time with IALA partners. However, the matrix, although still in draft mode, has been provided by Canada (refer Attachment A).

Also, Canada installed a national AIS service in 2010-11 and is now under initial operational status. The full operational status (FOS) is scheduled for March 2012.

Ireland

In Ireland responsibility for e-Navigation matters is across a number of agencies. CIL are responsible for all AtoN issues, the Department of Transport would be responsible for most of the remaining issues but individual ports would also do their own thing.

On the AtoN front Ireland are well advanced on AIS with 19 buoys and 10 lighthouses fitted. They will roll out a total of 64 AIS AtoN in the next 2 years and are moving towards using AIS for AtoN monitoring where we do not require control.

CIL have arrangements with the Coast Guard to provide a data feed and to reserve slots and provide virtual AtoN where required. They have trialled Virtual AtoN and would intend using them if a wreck / new danger situation arises where they could contribute.

Have also trialled AIS Met Hydro messaging providing wind, wave and tidal data using the IMO message and text messaging. However, it is worth noting that the GLA (Trinity House, Northern Lighthouse Board and Irish Lights) have decided not to provide Met Hydro information and therefore no further work is being done in this area.

Ireland are active in various committees developing e-Navigation and in a good position to advance the project.

The Irish Coast Guard have a full coastal AIS network and will be ready for other e-Navigation initiatives as they develop. At Department of Transport level there is support for e-Navigation through participation in IMO. There is an active working group on Marine Surveillance with the aim of bringing together data from all agencies at both domestic and European level.

Individual ports are using AIS and we have our first application for a Synthetic AIS AtoN in a port.

Norway

The Norwegian Branch of the Nautical Institute held a seminar at the Regional Head Quarters of the Norwegian Coastal Administration in Haugesund on Monday 18 October 2010.

The key presentation was by Lee Alexander a leading figures within IMO and IALA’s work on developing the e-Navigation framework.  One of the key points made by

Lee Alexander was that mariners must influence” what e-Navigation becomes and identify user requirements; what are (or will be) e-Navigation equipment, systems and services; what are (or will be) the major presentation/ and display issues.    Users already know the limitations of existing ship borne equipment/ systems standards and are well placed to identify harmonisation challenges and opportunities and to recommend guiding principles and best practices.

Lee Alexander also said that a wider view on potential information sources for use in e-Navigation was needed.  Data associated with ice hazards, tidal movement, surface traffic and transit timings of deep-draught vessels in critical areas like the Torres Strait and River Scheldt were obvious examples of information for e-Navigation.  However, environmental data associated with “saving the Planet” and, in particular, can also be utilised.  He gave the example of the North Atlantic Right Whales where members of this endangered species were straying into sea lanes off the North East Coast of the USA but, thanks to tracking technology, their movements could be displayed on the bridges of ships that might otherwise be unwitting threats to the species.

Other presentations included ECDIS and the use of Portable Pilot Units.

### e-NAV Conference

The 2011 “e-Navigation Underway” conference gathered 136 delegates from 21 countries.

Experiences gained from existing e-Navigation test beds such as the use of Portable Pilot Units and underkeel clearance management systems and planned test bed activities were presented.  Live demonstrations were given on the existing e-Navigation services provided within the EfficienSea project such as:

- Meteorological and Oceanographic data on route

- Maritime Safety Information (MSI)

- Route Exchange.

The conference arrived at 19 conclusions (refer Annex 1) concerning the e-Navigation process, the need for standards and regulations, the value of test beds, bridge solutions, user needs, and the need for similar forums to share knowledge and coordinate test bed activities.

Feedback was very positive with participants feeling that the conference successfully clarified the various aspects of e-navigation and in particular made the topic more realistic through practical examples.

### AtoN information standards

### Outcomes of workshop on harmonisation between the IALA and IHO data model registers.

To ensure the harmonization and interoperability of a framework for data access and information services under the scope of SOLAS, NAV 56 agreed that the IMO NAV Correspondence Group on e-Navigation could be tasked with its development.  The Correspondence Group and the IALA e-NAV Committee have been debating the merits and ultimate choice of a suitable data framework for e-Navigation between:

- IALA Universal Maritime Data Model (UMDM) being developed in the e-NAV Committee’s Architecture WG; or

- IHO’s S-100 model – this model supports a variety of hydrographic-related digital data sources, products and customers. S-100 is not an incremental revision of S-57 but it is a new standard that includes both additional content and support of new data exchange formats.

S-100 is designed to accommodate any geospatial requirement related to maritime navigation and as such it is very much user-need driven.  S-100 could accommodate the requirements of IALA for e-Navigation, and also other stakeholders, if the stakeholders are prepared to use supplementary registers, or domains within a supplementary register, within S-100.

The IALA’s UMDM concept appears to have some structural superiority in terms of the idea of using layers.  IALA’s work on UMDM has significant potential to form a good foundation in support of e-Navigation as it aligns well with several of the identified future directions of e-navigation. The current design and scope of S-100 appears sufficient and adequate to cater for e-Navigation.

There would be a problem if IALA stakeholders encode aids to navigation according to an IALA registry and IHO encode aids to navigation according to an IHO registry describing many common real world elements.

Norway hosted a workshop (held at the International Hydrographic Organization headquarters in Monaco 4-5 November, 2010) that looked at how to achieve the IMO’s envisaged ‘common data structure’ by harmonization between the IALA and IHO data model registers.

The workshop agreed thirteen (13) conclusions and recommendations (refer Annex 2)

### Status of development of Australian AtoN Information Metadata standard

Australia is currently in discussions with the IHO for the adoption of its AtoN Information Metadata standard (shown at Annex 1) as an S-100 product specification.  These unified product specifications are used by IHO member authorities when providing services to the maritime community.  The Australian Hydrographic Service will be meeting with the IHO in April 2011 to progress adoption.

A number of other IALA national members have developed similar schemas.

### Annex 1 – Conclusions of the “e-Navigation Underway” Conference

### The e-Navigation Process

1. The e-Navigation implementation should be a process of evolution rather than revolution. e-Navigation will progress incrementally and iteratively.

2. There is a need to consider the alignment between e-Navigation and other related initiatives such as e-Maritime. This could be achieved through harmonization and cooperation across these initiatives.

3. The IMO Strategy Implementation Plan should identify a method for testing the usability and effectiveness of new additional elements of e-Navigation in terms of operations, technical, regulatory aspects and training.

4. It is important to evolve core elements of e-Navigation, meeting common global needs before expanding to non-core applications.

### Need for standards and regulations

5. Some e-Navigation elements, user driven, should be regulated while others should only be guided to the extent of design principles.

6. Standards are needed to ensure global compatibility. Regulations may be needed for core components of e-Navigation.

7. There is a compelling need to address the IMO, ITU, IEC processes for dealing with new developments and maintaining existing systems that are based on rapidly changing technology and evolving user needs.

### Value of e-Navigation Test Beds

8. Test beds can contribute to the GAP analysis and subsequently provide useful input to the development of operational systems i.e. “closing the gap”.

9. As users‟ needs and levels of training will vary, it needs to be recognized that test bed results from one region are not necessarily globally applicable. The challenge is to take this into account whilst seeking global solutions.

10. Existing standards and data structures should be used in test beds, in particular S-100, in order to explore their suitability on a global scale.

11. A test bed driven approach to the development of e-Navigation is useful given similar test methodologies and evaluation criteria as well as proper dissemination of results.

### Bridge Solutions

12. An e-Navigation bridge environment should provide for value added applications guided by design principles rather than regulations to encourage innovation.

13. INS is a possible basis for the future development of on board e-Navigation. Use could be made of the IMO modular approach used in the INS performance standards.

14. INS could be a core platform on the bridge enabling the addition of specialized applications that may be required by specific users and in particular regions.

15. There is a need to start the process for INS carriage requirement. This should be a part of the e-Navigation strategy implementation plan.

### User Centric

16. User centred design is important to ensure the success of e-Navigation both onboard and ashore. Attention should be paid to the operational context and procedures as well as technology.

17. It is important to inform, educate and work with users prior to and during test bed projects. In working with users, the ability to observe them and properly assess their needs is paramount.

### Value of Discussion Forums

18. There are many pioneering activities apart from test beds that can contribute to e-Navigation.  Events such as the e-Navigation Underway Conference are useful as forums for information exchange on such developments.

19. The wide range of participants and views expressed during the e-Navigation Underway Conference has shown the value of sharing test bed information and results.

Annex 2 – Conclusions and recommendations of the workshop on harmonisation between the IALA and IHO data model registers

1.	S-100 should be considered as a baseline, as an important element in the IMO e-navigation concept.

2.	IMO, in consultation with other organizations, should consider establishment of a harmonization group based on the example of the IMO/IHO Harmonization Group on ECDIS.

3.	Any organization which proposes to develop and/or maintain its own data structure relevant to e-navigation, shall be encouraged to join the Harmonization Group. Such a data structure shall be interoperable with existing data structures and relevant international standards that are already supporting the e-navigation concept. Harmonization should be achieved through interoperability, which should be considered a shared responsibility.

4.	Noting that IALA and IHO are the stewards of two important e-navigation domains, technical representatives of IALA and IHO should continue their close liaison.

5.	The e-navigation community is aware of other data structures already developed within other domains relevant to e-navigation. It is essential that owners of these data structures cooperate with the Harmonization Group to achieve interoperability, where appropriate.

6.	Consideration should be given to different knowledge domain communities both in terms of their requirements and contributions in connection to e-navigation development.

7.	As both IHO and IALA have been modelling e-navigation data structures within their own domains, the working groups within both organizations need to explore how IALA and IHO could share, correlate, cross-reference and harmonise, as appropriate, to create and maintain a data model that draws on the expertise of both organizations.

8.	Domain registries shall be on-line in order to be easily searched.

9.	Consideration should be given to providing technical advice/support to organizations that do not have technical capability or competency.

10.	Implementation of any domain data structures shall be harmonised and interoperable both technically and across different geographical regions.

11.	It can be expected that the availability and use of internet on ships at sea will increase and may provide extra opportunities for maritime data exchange and information services. Note needs to be taken of the multitude of Spatial Data Infrastructures (SDI) being developed in related domains (eg. meteorology, oceanography, etc) which are largely internet based. IMO should give careful consideration to the implications for e-navigation.

12.	The concept of re-use rather than re-invent should be an e-navigation data framework ground rule.

13.	Consideration needs to be given to the standards being used for the passing of data between individual components of systems both onboard and ashore.

### Annex 3 – Australia’s AtoN Information Metadata Schema

| AID TO NAVIGATION FORM |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| AtoN Owner |  |  |  |  |  |  |
| Date established |  | <Input Date (dd/mm/yyyy)> |  |  |  |  |
| Country |  | <Input Country> |  |  |  |  |
| State |  | <Input State> |  |  |  |  |
| AtoN owner/authority |  | <Input Aton Owner> |  |  |  |  |
| Aton ID |  | <Input Aton ID(State/Port Authority Asset No)> |  |  |  |  |
| Location ID |  | <Input Location ID (eg No 1, No 2, etc)> |  |  |  |  |
| AHS Ausnum |  | <Input AHS Ausnum by AHO Office Only> |  |  |  |  |
| UKHO Vol K Number |  | <Input VolK number> |  |  |  |  |
| Location |  | <Input Location Name> |  |  |  |  |
| Common name |  | <Input Common Name> |  |  |  |  |
| AtoN type |  | <Input AtoN Type (eg Racon, Light, Ramark etc)> |  |  |  |  |
| Mark structure |  | <Input Mark Structure (eg Tower - Lattice, Tower - Masonry etc)> |  |  |  |  |
| Mark type |  | <Input IALA Description (eg North Cardinal etc)> |  |  |  |  |
| Geographic or Grid Co-ordinates |  |  |  |  |  |  |
| Datum |  | <Input Datum> |  |  |  |  |
| Latitude degrees |  | <Input Latitude Degrees (eg 24) |  |  |  |  |
| Latitude minutes |  | <Input Latitude Minutes (eg mm.mmmm - 4 decimal places) |  |  |  |  |
| North/South |  | <Input North/South |  |  |  |  |
| Longitude degrees |  | <Input Longitude Degrees (eg 24)> |  |  |  |  |
| Longitude minutes |  | <Input Longitude Minutes (eg mm.mmmm - 4 decimal places)> |  |  |  |  |
| East/West |  | <Input East/West |  |  |  |  |
| Horizontal accuracy |  | <Input Suitable Estimated Accuracy (minutes) |  |  |  |  |
| Projection |  |  | <Input Name of Projection> |  |  |  |
| Zone |  |  | <Input Zone> |  |  |  |
| Easting |  |  | <Input Easting - 4 decimal places> |  |  |  |
| Northing |  |  | <Input Northing - 4 decimal places> |  |  |  |
| Positional Survey Information |  |  |  |  |  |  |
| Method of survey |  | <Input Method of Survey (or position fixing)> |  |  |  |  |
| Surveyed by |  | <Input Surveyed by> |  |  |  |  |
| Survey date |  | <Input Survey Date (eg dd/mm/yy)> |  |  |  |  |
| Light Details |  |  |  |  |  |  |
| Charted light characteristics |  | <Input Chart Light Characteristics (as per Symbols and Abbreviations used on Admirality Charts)> |  |  |  |  |
| Light colour |  | <Input Light Colour> |  |  |  |  |
| Eclipse |  | <Input Exlipse> |  |  |  |  |
| Period of operation |  | <Input EXCLIT (Night, Day, 24 hour, occasional)> |  |  |  |  |
| Nominal light range |  | <Input Nominal Light Range (in Nautical Miles to 1 decimal place) - requires IALA Definitions> |  |  |  |  |
| Elevation |  | <Input Height of Light Focal plane (in Meters to 1 decimal place)> |  |  |  |  |
| Elevation datum |  | <Input Height Datum of Light Focal plane> |  |  |  |  |
| Vertical accuracy |  | <Input Suitable Estimated Accuracy (meters)> |  |  |  |  |
| Visible sectors |  | <Input Arc of Visibility/Sectors (From:)> |  |  |  |  |
| Visible sectors |  | <Input Arc of Visibility/Sectors (To:)> |  |  |  |  |
| Obscure sectors |  | <Input Arc of Obscurity/Sectors (From:)> |  |  |  |  |
| Obscure sectors |  | <Input Arc of Obscurity/Sectors (To:)> |  |  |  |  |
| Beacon/Buoy/Structure Details |  |  |  |  |  |  |
| Situation |  | <Input Situation> |  |  |  |  |
| Attached to Fixed Infastructure |  | <Input Yes/No> |  |  |  |  |
| Infastructure Details |  | <Input Infrastructure Details> |  |  |  |  |
| Shape |  | <Input shape> |  |  |  |  |
| Colour(s) |  | <Input Colour(s) (Top to Bottom)> |  |  |  |  |
| Special mark |  | <Input Special Mark (Yes/No)> |  |  |  |  |
| Purpose of special mark |  | <Input Purpose of Special Mark> |  |  |  |  |
| Daymark/Topmark Fitted |  | <Input Daymark/Topmark (Yes/No)> |  |  |  |  |
| Daymark/Topmark Colour |  | <if 'Yes' Daymark/Topmark fitted, Input Daymark/Topmark Colour> |  |  |  |  |
| Daymark/Topmark Shape |  | <if 'Yes' Daymark/Topmark fitted, Input Daymark/Topmark Shape> |  |  |  |  |
| Sign Fitted |  | <Input Yes/No> |  |  |  |  |
| Sign Information |  | <Input Wording on Sign> |  |  |  |  |
| Photograph |  | <Input Photo Supplied (Yes/No)> |  |  |  |  |
| Leading Lights/Beacon Details |  |  |  |  |  |  |
| Orientation of leading marks/transits |  | <Input Orientation (eg 035)> |  |  |  |  |
| Type of Navline |  | <Input Type of Navline: Recommended Track/Clearing Line/Transit Line - requires IALA Definitions> |  |  |  |  |
| Direction of traffic |  | <Input Direction of Traffic: One Way/Two way/Inbound/Outbound> |  |  |  |  |
| Racon/AIS Details |  |  |  |  |  |  |
| Racon character |  | <Input Racon Character (Morse Code)> |  |  |  |  |
| Racon type |  | <Input Racon Type (X/S/Frequency Agile)> |  |  |  |  |
| Racon range |  | <Input Racon Range (in Nautical Miles to 1 decimal place)> |  |  |  |  |
| AIS station type |  | <Input AIS Station Type> |  |  |  |  |
| AIS - Tx/Rx |  | <Input AIS (Transmit/Receive)> |  |  |  |  |
| MMSI Number |  | <Input MMSI Number> |  |  |  |  |
| Elevation |  | <Input height of antenna (AIS or racon, metres to 2 decimal places)> |  |  |  |  |
| Range (calculated) |  | <Input Calculated Range (in Nautical Miles to 1 decimal place) > |  |  |  |  |
| General Comments |  |  |  |  |  |  |
| Comments |  | <Input General Comments> |  |  |  |  |
| Record completed by |  | <Input Your Name> |  |  |  |  |
| Title |  | <Input Your Title> |  |  |  |  |
| Date completed |  | <Input Date Completed (eg dd/mm/yy)> |  |  |  |  |
| Email |  | <Input Your Email Address> |  |  |  |  |