| Application Mode | Function | Description | Type of Virtual AtoN AIS | Consideration |
| --- | --- | --- | --- | --- |
| Permanent Marking of Obstacles | Marking of Shoals and Reefs | Virtual AtoN can be effectively utilized where it is difficult to place a physical AtoN due to sea state, winds or other environmental conditions. A clear marking of the shoals and/or reefs will improve safety of navigation | Isolated Danger Marks & Cardinal Marks | - Integrity monitoring - Chart datum used - ECDIS and radar reliability |
| Permanent Marking (navigation support) | Marking of Fairway Limits | Virtual AtoN can be effectively utilized where lighted buoys cannot be installed for some reason (i.e. sharing of waterway). A clear marking of the fairway limits serve for ordinary flow of marine traffic and improved safety of navigation. | Lateral Marks | - Too much info for mariners |
|  | Marking of Fairways | Virtual AtoN can be effectively utilized where a physical AtoN placement is difficult due to the water depth, seabed, etc. A clear marking of fairway will improve safety of navigation. | Lateral Marks, & Safe Water Marks | - Too much info for mariners |
|  |  | Virtual AtoN can be effectively utilized in approaches to a harbor entrance where a ship changes its course and where it is difficult to install a physical AtoN. A clear marking of the point on approach will serve for an orderly flow of ships at a entrance and improve safety and efficiency of shipping. | Safe Water Marks | - Too much info for mariners |
|  | Marking of Fairways & Marking of the Limits of Safe Waters | Virtual AtoN can b effectively utilized where navigation becomes difficult due to a thick fog, heavy rain, etc. (This application can also be adapted as a temporary marking during a limited visibility.) Marking of a recommendable fairway during times of limited visibility will serve to improve safety of navigation and efficiency of shipping. | Lateral Marks & Safe Water Marks | - Should be temporary |
| Permanent Marking | Special area (i.g. anchorage area, area to be avoided) | Virtual AtoN can be effectively utilized where pre-caution or special caution required. A clear marking of special area will improve safety of navigation. |  | - Too much info for mariners |
| Temporary Marking | Marking of a Navigational Restricted Areas | Virtual AtoN can be effectively utilized when navigation restriction is required due to e.g., marine accidents or when marking of wreck or offshore operations. A temporary navigation restriction can prevent subsequent incidents from developing. | Cardinal Marks, & Special Marks | - Need to be monitored/updated - Timely deployment required |
|  | Designation of Temporarily Recommendable Fairways | Virtual AtoN can be effectively utilized for indication of fairways when a scale disaster hits the area. A clear marking of temporarily recommendable fairways will be expected to serve for the relief ships dispatched to the site and to support safe and effective relief activities. | Lateral Marks & Safe Water Marks | - Need to be monitored/up dated - Timely deployment required |
|  | Marking of Aids to Navigation that are malfunctioning or off position | Virtual AtoN AIS can be effectively utilized when a physical AtoN has lost ability to perform regular functions due to a natural disaster. When a physical AtoN lost ability to perform regular functions due to natural disasters, recovery actions are required at the earliest opportunity. Virtual AtoN can respond to the circumstance even if the actions by personnel cannot be achieved due to meteorological and/or hydrographical conditions, and keep the influence of the trouble reach to ships in navigation at a minimum level. | Cardinal Marks, Lateral Marks, Isolated Danger Marks, Safe Water Marks & Other Position Marks | - Timely deployment required |
| Temporary Marking | Pilot boarding station | Virtual AtoN will be useful to mark a pilot station where position depends on sea condition. | Special Marks | - Timely deployment required |