Maximizing the Potential of AIS

Capt. Roger Barker, Trinity House, London.

This paper examines:

- Assessing the requirements for Aids to Navigation

- Use of AIS as an Aid to Navigation

- Possible limitations to be considered with AIS data

- Conclusions.

Assessing the requirements for Aids to Navigation

The use of AIS traffic analysis, together with use of an appropriate GIS overlay system, is proving to be instrumental in 3 key areas with regards to Aid to Navigation Requirements and the overall Risk Assessment process.

- Emergency Wreck or Obstruction Marking.

- Confirmation of requirements for existing and new Aids to Navigation for General Navigation.

- Assessment of new or existing shoals and shallow areas subsequent to Hydrographic Survey.

A significant improvement in the AtoN assessment can be made which can also result in a more cost effective operation where the appropriate Aid is deployed, thus ensuring maximum efficiency with no detriment to the safety of the Mariner and general requirements of the User.

From initial and instantaneous snapshot of AIS traffic at one moment in time, Fig 1., data can be accumulated to provide long-term traffic patterns  for the real assessment of where vessels are actually transiting:

Figure 1

AIS data for traffic around the UK at one moment in time

Many AtoN providers will already be using long-term overlays of vessel traffic giving an indication of overall traffic volumes, see fig.2, but this can easily be misinterpreted.

The area in question has large traffic volumes but the significance for each individual Aid to Navigation, or group of Aids is not identifiable from this data.

Figure 2. Four weeks of vessel traffic data on the South coast of England.

Raw AIS data, as received from a network of base stations is of little use for traffic analysis purposes.

It is necessary for the data to be processed such that it can be plotted in user selected periods using a suitable GIS overlay. Once plotted the data must then be further re processed to give the user specific information he requires such as; only vessels over a certain draft;  vessels bound for specific destinations or vessels of a certain type.

The traffic overlays must be used in conjunction with suitable charts, and then combined with the users own specific data sets, for example the range and sectors of existing Aids to Navigation, new or proposed routing measures that will affect traffic patterns, the assessment of any new or planned developments which may include offshore renewable energy sites and changes to bathymetry which require a re-assessment of AtoN requirements.

Specific Applications

1 - Emergency Wreck or Obstruction Marking

When a new wreck or obstruction has been announced and the decision on whether a vessel and personal must be deployed, firstly for accurate survey and location and then to deploy, if required, the appropriate Aid to Navigation, an informed assessment must be made.

Before traffic data was available this decision relied almost entirely on the expertise of the assessor with no supporting evidence which could underpin the decision.

Information is and has been available for many years regarding vessel trading routes which helped to inform the process but this information was not able to provide data on specific traffic lines or indeed an assessment of the size of vessels actually passing over a particular location.

Considering each situation the assessor of requirements must make a rapid but informed decision on the worst possible situation. With a wreck this means that , particularly with a fishing or leisure vessel, the overall length of the vessel could in exceptional circumstances be the actual height above the seabed. An assessment of the clearance between the wreck and the sea surface at chart datum or lowest astronomical tide (LAT) must be made and a determination of the Risk to the Mariner.

Using an appropriate analysis tool with a range of AIS traffic data over a significant period a search of only vessels over a specific draft can be made.

As an example, if we consider an obstruction that has been reported with a maximum height above the sea bed of 10 metres which is located in an area with surrounding depths of 20 metres, then the available depth over the obstruction is 10 metres and this is the delineating draft to be assessed.

Figure 3

Example of an obstruction

With AIS data

In detailed analysis of the 8 weeks of AIS traffic data filtered for vessels with a draft in excess of 7 metres, to give an allowance for error over the 10 metres assessed as the requirement and also an allowance for under keel clearance.

Figure 4

In this case it was found that only 5 tracks with vessels over 7 metres encroached into the designated area.

Further analysis can then be carried out to ascertain the name, type, and destination and in this case two of the tracks were the same vessel, a 7.2m dredger and the other three were vessels under 7.5 metres draft engaged on coastal work.

It can therefore be seen that an informed decision can be made as to whether a lighthouse tender must be deployed immediately to carry out a survey and secondly whether floating marks are also required immediately.

2 - Confirmation of requirements for existing and new Aids to Navigation for General Navigation.

SOLAS Chapter 5, Regulation 13, states that:

Each Contracting Government undertakes to provide, as it deems practical

and necessary either individually or in co-operation with other Contracting Governments, such aids to navigation as the volume of traffic justifies and

the degree of risk requires.

By considering the actual coverage of current AtoNs provided, including accurate assessment nominal ranges and both charted and uncharted sectors of fixed lights, and combining this data with up to date traffic patterns, an informed decision on the ongoing requirements for existing Aids and the requirement for additional or different aids can be made.

Figure 5 shows a group of lighthouse on the SW coast of England

Figure 5

Figure 6

The ranges and sectors of lights overlaid on the chart, as in figure 5, can help to assess the requirements.

By careful use of a GIS overlay system, however, the nominal range, sectors and discontinuance of lights in sequence can be really assist in making an assessment of the requirements.

When this information is combined with AIS traffic data further evaluation of requirements can be made. From figure 6, which depicts traffic over a four week period, it can be seen that further in depth analysis is required.

Survey vessel

Tater Du Lighthouse

Figure 7

After filtering down to one week and studying the vessel traffic patterns, Figure 7, in this case it was possible that a significant reduction in nominal range of the light at Tater Du could be achieved without detriment to the Mariner. This reduction means that best use can be made of modern LED technology with a consequential reduction in power requirements, maintenance and therefore overall costs, providing significant value for money.

(The area of heavy tracks to the NW of the peninsula are from a survey vessel working during this period)

3 -  Assessment of new or existing shoals and shallow areas subsequent to Hydrographic survey.

When new survey information is received which has relevance to the Aids to Navigation deployed by a Lighthouse Authority, a decision is required on whether new or amended marking is required and importantly if changes are required, the time frame within which they must be carried out.

Figure 8

Figure 9

Figures 8 and 9 show an area in the approaches to the Thames on the English East Coast. Preliminary Hydrographic notes were issued by the survey contractor during the survey and before full promulgation of the survey. These notes indicated significant reduction in depths to the North East of the Black Deep buoy.

In this case the overall AIS data was assessed across a gate (red line) and the vessel data recorded accordingly.

| Draught (m) | No. of Vessels | % |
| --- | --- | --- |
| Unspecified | 0 | 0% |
| < 4 | 2 | 11% |
| 4 to 6 | 2 | 11% |
| 6 to 9 | 1 | 6% |
| 9 to 12 | 8 | 44% |
| > 12 | 5 | 28% |
| Total | 18 | 100% |

Figure 10

The table in Figure 10 shows data recorded from the gate analysis and this showed that over a 4 week period some 72% of the vessels passing this area were over 9 metres draft.

This information guided the assessor in his decision to arrange for a buoy move as soon as possible.

The converse may of course happen where a change in the AtoN requirements, although considered prudent in due course but with no urgency. As such in these cases best use of resources can be made such that the change can be carried out during routine operations.

Use of AIS as an Aid to Navigation

The concept for using AIS as an Aid to Navigation is covered elsewhere in the conference.

It is for note here, in the discussion of maximizing the use of AIS, that when a thorough traffic analysis has been carried out, the data received from that analysis may indicate that use of AIS AtoN, whether real, synthetic or virtual may be considered either in conjunction with or as an alternative to physical Aids.

One of the main problems surrounding the use of AIS AtoN is that there are currently limited vessels able to observe the AtoN on a suitable display. Traffic analysis can be used to ascertain those vessels that need to see the AtoN and as such inform the decision on requirements.

Possible limitations to be considered with AIS data

The data and information that is gained through AIS traffic analysis must be treated with a certain amount of caution.

- The data received from the vessel, other than position information, will always be dependant on the input from the vessel. There are many instances where draft, Port of destination and type of vessel inputs are incorrect or suspect

- Many Mariners/users of AtoNs are not required to carry AIS transceivers. Although many leisure craft now carry AIS class B, this is not mandatory and as such this sector of the user community must be further considered using other data inputs. In the UK we are able to use the Royal Yachting Association routing atlas, once again with GIS overlays, to consider the requirements.

- AIS data is generally received from a network of base stations using VHF frequencies. As such the data will become less reliable as the area being considered is further offshore. One problem here is that there is not a firm line between good and bad data with differing atmospheric conditions affecting reception at different times. As such data received from further offshore must be treated with caution.

Conclusions

The risk assessment process for the requirements of Aids to Navigation must rely mainly on the qualitative assessment of experts. The diversity of the various aspects that have to be considered will include the vast assortment and variety of vessels, the experience of the Mariners involved and the very different parameters that exist at the different locations.

This qualitative assessment can, however, now be informed by the quantitative elements provided by AIS traffic analysis data and GIS overlays.

It is considered essential that every opportunity is taken to provide the correct control measures to mitigate the risk which include the most appropriate mix of both Physical and Electronic Aids to Navigation and in consideration of Navigation Aids available aboard the vessels in question.

It is also essential that resources are used appropriately. As recorded earlier, without any detrimental effect on the Safety of the Mariner, in depth AIS traffic analysis can inform decisions to ensure that maximum efficiencies are achieved in all areas.

1971 words

Acknowledgements:

The chart extracts are courtesy of the UK Hydrographic Office and are of course not to be used for Navigation

The AIS data is by courtesy of the UK Maritime and Coastguard Agency