| From: ANM | ANM2/output/01 |
| --- | --- |
| To: AIS Committee (AIS13) | Date: April 17, 2003 |

Liaison Note

Preliminary Draft Recommendation on AIS for AtoN

- Introduction

The ANM Committee reviewed the Preliminary Draft Recommendation on AIS for AtoN as per the liaison note from AIS12, presented as ANM2/4/4.  The ANM Committee recognises that this is a preliminary draft document, which includes many technical aspects, and is not mature.  As such, the ANM Committee found parts of it difficult to understand and interpret.  In preparing the comments below, the ANM Committee is attempting to provide clarification on some items, and welcomes comments from the AIS Committee to assist in understanding the document.

- Comments

The ANM Committee provides the following comments on the Preliminary Draft Recommendation on AIS for AtoN.

  - General

- Recommendation - that the title of this draft paper be changed from “AIS for Aids to Navigation” to “AIS Technology for Aids to Navigation”.

  - Introduction

Since this document would be of benefit as a stand-alone paper, a preamble or introduction to this paper would be useful. This introduction could be similar to the introduction to other AIS Recommendations, and provide a response to the questions ‘What is AIS?’ ‘What does AIS do?’

- Recommendation - that an introduction be added to the paper.  An example of such an introduction is attached to this liaison note.

  - Section 1.0

- Question - Section 1 – Application of AIS to aids to navigation – bullet point 8 regarding marking tracks.  Is this a message 21 or a message 8?

- Comment - Section 1. – Application of AIS to aids to navigation – bullet point 3 in ‘…further benefits’.  With regards to identifying if a ship hits an AtoN – this would be a use of AIS as a monitoring tool rather than AIS as an AtoN.

  - Section 3

- Recommendation - that a new paragraph be added under Section 3 stating:   ‘When connected to an AIS unit, external GPS/DGPS receivers must be configured to transmit co-ordinates in WGS84 datum. Failure to do so will result in misleading information being transmitted to the mariner’.

- Comment – the recommended wording would affect the mobile station as well.

- Question – Section 3.1.1 – Message 21, Configuration – item 3.  It is stated that ‘all co-ordinates for AIS positions are to be given in WGS84’.  The committee notes that not all administrations have converted their charts to WGS84.  For example the China representative pointed out that Beijing 54 is their standard at present. Has this been taken into consideration?

- Comment / Request for Clarification – Section 3.1.1 – Message 21, Configuration – item 4, Dimension of AtoN field for a fixed AtoN.  There is confusion as to the significance of this paragraph. East/West dimension is specified at (0-63) metres and North/South dimension as (0-511).  This is an asymmetric shape. The reason for this is not clear and the committee seeks clarification.

- Comment - If message 21 were to be used for Wind Farms, these dimensions would not be sufficient.

- Comment - there appears to be a conflict between what is written in 3.1.1 and table 34 – footnote No.1 with regard to placement of the reference point.

- Question - Wind farms (and their coverage area) - are they message 21 or message 8?

- Action Requested

The ANM committee requests that the Preliminary Draft Recommendation is forwarded to ANM3, prior to finalising the document.  This may conflict with the aspiration to finalise the document at AIS13 in September 2003, but could still enable approval of the document at Council Session 33 in December, 2003, if an electronic review process can be agreed.

IALA Recommendation on AIS Technology for Aids to Navigation

Suggested Text for Preamble

# Background

Automatic Identification System (AIS) is an autonomous and continuous broadcast system, operating in the VHF maritime mobile band.  It is capable of exchanging information such as vessel identification, position, course, speed, etc. between ships and shore.  It can handle multiple reports at rapid update rates and uses Self-Organising Time Division Multiple Access (SOTDMA) technology to meet these high broadcast rates, ensuring reliable and robust operation.

Chapter V of the 1974 SOLAS Convention (as amended) requires mandatory carriage of Automatic Identification System (AIS) equipment on all vessels constructed on or after 01 July 2002.  Implementation for other types and sizes of SOLAS Convention vessels is continuing and will be completed not later than 31 December 2004.

SOLAS Chapter V, Regulation 19, section 2.4.5 states that:

"AIS shall

- provide automatically to appropriate equipped shore stations, other ships and aircraft information, including ship's identity, type, position, course, speed, navigational status and other safety-related information;

- receive automatically such information from similarly fitted ships;

- monitor and track ships; and

- exchange data with shore-based facilities."

In addition, the IMO Performance Standards for AIS state:

"1.2	The AIS should improve the safety of navigation by assisting in the efficient navigation of ships, protection of the environment, and operation of Vessel Traffic Services (VTS), by satisfying the following functional requirements:

.1	in a ship-to-ship mode for collision avoidance;

.2	as a means for littoral States to obtain information about a ship and its cargo; and

.3	as a VTS tool, i. e. ship-to-shore (traffic management).

  - The AIS should be capable of providing to ships and to competent authorities, information from the ship, automatically and with the required accuracy and frequency, to facilitate accurate tracking. Transmission of the data should be with the minimum involvement of ship's personnel and with a high level of availability.”

AIS Technology can also be applied to Aids to Navigation (AtoN) to further improve and enhance services provided to mariners.  The purpose of this document is to provide guidelines and recommendations for the use of AIS technology in this field.

# Application of AIS to Aids to Navigation

The application…………………………………