## Radio-navigation Committee

18th Session								19 March 2003

# Integration of Radio-navigation Systems Infrastructure

## Introduction

The report of the RNAV Committee to the 2002 IALA Conference identified the requirements for integration and inter-operability of radio-navigation systems as one of the trends driving future developments. This paper notes the current status, and assesses the potential for integration of the infrastructure, of the various systems currently deployed, planned or under development, to determine the effects this might have on IALA Radio-navigation Policy. Another trend noted in the report was towards multi-modal systems, therefore this analysis will not be limited to maritime uses or systems provided for maritime use.

## Systems Considered

Principal consideration is given to the maritime radiobeacon Differential GNSS service (DGNSS), operated by IALA Members. However, Space Based Augmentation Systems (SBAS) are being promoted for maritime use, therefore it is appropriate to consider whether there is scope for sharing infrastructure. Differential GNSS signals can also be broadcast via the Automatic Identification System (AIS) at VHF frequencies, and via  Loran transmissions as Eurofix signals. The scope for integration of Loran systems with DGNSS and future GNSS is also considered. Co-location of reference stations and integrity monitors for different systems, or shared use of such facilities are considered in terms of how to make best use of resources.

## DGNSS as a component of GNSS

The IALA DGNSS service is currently the only recognized augmentation system for maritime users. The presently available GNSS – GPS, has insufficient accuracy for many specialised purposes and inadequate integrity for safety of life applications. Therefore in these situations the use of some form of augmentation is essential and that augmentation can be considered a component part of the GNSS. Future GNSS, particularly Galileo, could make use of existing facilities such as DGNSS stations as part of their ground sector.  However, the ground infrastructure of a GNSS is limited to a small number of control, monitoring and uplink sites, selected for their security and satellite coverage, whereas DGNSS are much more numerous and are generally selected for their coverage of locations important to the user.

### Integration of GNSS and SBAS

Three SBAS, WAAS, MSAS and EGNOS, are currently at an advanced stage of development. These systems broadcast wide area differential GNSS corrections, integrity messages and additional ranging signals from geo-stationary satellites and are primarily provided for the aviation user. None of these systems have currently been declared operational, however combined GNSS/SBAS receivers are readily available. Two wide area commercial differential GNSS systems, targeted at the survey market, are also currently available to subscribers. Unlike the ground wave transmission mode of DGNSS, SBAS uses line-of-sight signals and is therefore not optimized for maritime use. This is particularly relevant in built-up port areas, fjord scenarios and inland waterways, especially in higher latitudes. SBAS produces correction signals by monitoring the GNSS signals at a number of sites within the coverage area and calculating an appropriate wide area correction model and there is obvious scope for sharing reference station infrastructure with DGNSS.  However, this raises questions about the validation and recognition of SBAS for maritime use.  IALA policy should be based on the results of studies being carried out on these matters.

## DGNSS via AIS

A new method of broadcasting DGNSS corrections has emerged recently with the adoption of AIS as a carriage requirement under the SOLAS Convention by IMO. AIS Shore-Stations will have the capability to broadcast a message (Type 17) containing DGNSS corrections, thereby providing an integrity service for GNSS. The IALA Guidelines for the Operation of AIS contain guidance on the architecture of the system for providing DGNSS from AIS.  Integrity monitoring of the differential data is essential.  To optimise the use of existing infrastructure, it is recommended that already established radiobeacon DGNSS stations are used as the source of the correction data, ideally via a landline but, if necessary, off-air.

This concept could be extended to provide DGNSS coverage in areas where radiobeacons do not exist, or where they are difficult and expensive to maintain. AIS repeater stations are likely to be less costly to establish and service than radiobeacons.

## Integration of GNSS and Loran

Loran and GNSS are mutually complementary systems.  An integrated receiver benefits from the accuracy and coverage of GNSS and the ground wave propagation mode and resistance to jamming of the Loran signal. Loran also provides an alternative method of broadcasting a time reference. The use of Loran stations as sites for the ground infrastructure of a GNSS would result in economies in terms of land purchase, supporting services and security. Mutual interference problems are unlikely, given the large difference in frequencies, modulation and antenna design.

## Integration of DGNSS and Loran/Eurofix

Co-location of DGNSS and Loran may be possible and would offer infrastructure economies, particularly with regard to broadcast antenna.

Where operational, Eurofix offers an alternative method of broadcasting differential GNSS corrections utilizing the Loran frequency, which benefits from increased coverage compared to DGNSS. Co-location of DGNSS and Eurofix may be possible and would seem an obvious economy since the equipment required is similar.  Furthermore, Eurofix was designed to use the RTCM data format broadcast by DGNSS.

Loran/Eurofix sites would not necessarily be in the right locations to provide DGNSS for maritime users.

## Integration of DGNSS and SBAS

The use of DGNSS to re-broadcast SBAS corrections is being explored to overcome the problem of limited geo-stationary satellite visibility in high latitudes and where the topography masks the signals. Dual frequency GNSS reference stations may also offer augmentation to the SBAS system by providing water vapour measurements.

## Integration of Eurofix and SBAS

Eurofix has been proposed as an en-route aid for the increasing number of polar flights. The financial advantages to the airlines of routing flights over the North Pole are very significant, but the GNSS integrity function cannot be performed by SBAS, because of the limited visibility of geo-stationary satellites. The potential use of Eurofix (& radiobeacons) for this purpose is also very important for the opening up of northern shipping routes. There are few conventional aids to navigation in these regions, but existing Loran/Chayka stations and radiobeacons can provide the integrity function to enable GNSS to be used as the “sole means” of navigation.

## Conclusions

It can be seen that there is some scope for integration, or at least sharing of infrastructure between GNSS and its augmentations and between those augmentation systems. This may open up navigation services in areas where they were previously unavailable. However, this raises questions about the recognition and validation of other systems such as SBAS for maritime use and IALA policy should be based on the results of studies being carried out on these matters.  The potential for integration of systems should be taken into account in future radio-navigation policy and planning.