AISM Association Internationale de Signalisation Maritime      IALA

International Association of Marine Aids to Navigation and Lighthouse Authorities

IALA Guideline No. 1004

On

Level of Service

Edition 1.2

[Date issued]

Edition 1; October 1997

Edition 1.1; December 2005

10, rue des Gaudines

78100 Saint Germain en Laye, France

Telephone: +33 1 34 51 70 01  Fax:  +33 1 34 51 82 05

e-mail:  contact@iala-aism.org       Internet:  www.iala-aism.org

Document Revisions

Revisions to the IALA Document are to be noted in the table prior to the issue of a revised document.

| Date | Page / Section Revised | Requirement for Revision |
| --- | --- | --- |
| December 2005 | Entire document | Reformatted to reflect IALA documentation Hierarchy |
| April 2012 | Entire document | Restructured and refocused to provide clear information on Levels of Service. |

Table of Contents

2.1	Description	4

2.2	Components	4

2.2.2	Means	4

2.2.3	Quality	5

3.1	Determining the Extent	5

3.2	Determining the Means	5

3.3	Determining the Quality	5

3.4	Consultation and Review	5

3.6	Benefits	6

3.7	References	6

4.1	Contracting Out	6

4.2	Disclaimer	7

Index of Tables

- Table 1	Title required	3

Index of Figures

- Figure 1	Title required	3

Level of Aids to Navigation Services

# Introduction

This guideline provides information on levels of service for aids to navigation provided by Competent Authorities including related description, components and benefits. It also provides guidance on how to development appropriate levels of service.

# Level of service

## Description

A level of service is a statement which articulates a commitment of service by the Competent Authority to mariners who are navigating or operating in the area, as well as, clients and/or governments responsible for funding the provision of the aids to navigation services.

Level of service statements should be clear, easy to understand and available to all concerned.

## Components

A level of service statement should include, at minimum, the following components:

- Extent

- Type

- Quality

It may be beneficial to also include an overview and description of the Competent Authorities’ objectives and mandate in any statement.

### Extent

The extent should address whether a service will be provided by the Competent Authority. Ideally, it describes where and why the service is provided.

Most Competent Authorities are bound by SOLAS Chapter 5, regulation 13, which states that “Each Contracting Government undertakes to provide, as it deems practical and necessary either individually or in co-operation with other Contracting Governments, such aids to navigation as the volume of traffic justifies [where] and the degree of risk requires [why].”

The extent of service provided may also vary by Competent Authorities for specific areas, category of users, or because of national obligations.

### Type

### The type of service should address what the Competent Authority will provide. It is a description of the service provided, such as, visual aids to navigation, Radionavigation System, Vessel Traffic Service, maritime safety information, etc.

### Quality

The quality should address to what level the Competent Authority will provide a service. It is a minimum standard at which clients can expect a service to be performed, also known as a service standard.

Service standards can be expressed as one, or a combination of the following:

- an overall percentage availability that any aid to navigation can be expected to be operational,

- percentage availability targets based on the relative importance of an aid to navigation or system of aids to navigation,

- maximum response times for a given service or within a specific area.

# Development of Level of Service

## Determining the Extent

The extent of the service provided can be determined by identifying national obligations of the Competent Authority. Obligations may be in the form of legislation, international conventions (such as SOLAS) as well as other priorities defined by the Competent Authority.

Competent Authorities bound by the SOLAS Convention should employ risk assessments and other tools to determine the extent of the service provided based on the volume of traffic and the degree of risk.

## Determining the Type

The type of service provided is primarily determined by identifying a variety of users, meterological and oceanographic conditions, traffic volume, hydrography, etc. in which navigation occurs. Competent Authorities may also have other national obligations to provide a specific type of service.

Other factors may require evaluation in order to select the appropriate service type, such as offshore energy infrastructure, aquaculture, environmental considerations, etc.

## Determining the Quality

Competent Authorities must achieve a careful balance between user requirements and what is reasonably possible given resource and funding constraints when establishing a service standard to ensure quality. Consideration must also be given to the ways by which the service is delivered and any related guidelines or recommendations.

## Consultation and Review

Consultations with stakeholders (users, partners, personnel, etc.) are encouraged and can occur at any stage during the development of a level of service. A periodic review of established levels of service may be required to address changes to user requirements, national obligations, and environmental conditions.

It is recommended that Competent Authorities put in place procedures to monitor the performance of individual components of the service provided in order to assist in the review of the level of service.

  - Promulgation

A formal mechanism for promulgating levels of service and changes thereto, accessible by all potential users, should be established. Publishing levels of service on the Competent Authorities’ website may be an ideal mechanism to promulgate this information to the appropriate stakeholders.

## Benefits

Established levels of service are integral to efficient planning and delivery. They provide users with a clear understanding of the expected services and they also contribute to ensuring that services are delivered in a nationally consistent, integrated, predictable, measurable and fair manner.

# Risk Assessment and Waterway Design

Once the level of service has been developed, Competent Authorities should employ risk assessments and other waterway design tools to determine the appropriate type, mix, and quantity of aids to navigation services.  IALA Guideline 1018 and IALA Guideline 1078 provide specific guidance for assessing waterway risks and determining the appropriate mix of aids to navigation therein.

Competent Authorities should conduct periodic review of waterway risk to determine the efficacy of the aids to navigation service.

# Other Considerations

## Contracting out Aids to Navigation Services

Where the responsibility for the provision of an aid to navigation service is delegated, for example to state, territory or local government organizations, or to port, harbour or waterway authorities, or local private groups, responsibility to ensure and/or enforce levels of service shall remain with the Competent Authority concerned.

## Disclaimer

Levels of service are generally for planning purposes only. They are intended to communicate the services and response levels clients can expect from a Competent Authority under normal conditions. In some circumstances, due to factors outside of their control (weather, maritime traffic, unanticipated events, etc.), a Competent Authority may be unable to meet the levels of service. A disclaimer to this effect should be included in any level of service document.

# References

When developing the aids to navigation level of service, the following IALA Guidelines, and Recommendations should be consulted:

- IALA Guideline 1018 on Risk Management

- IALA Guideline 1035 on Availability and Reliability of Aids to Navigation

- IALA Guideline No. 1052 on Quality Management Systems for Aids to Navigation Service Delivery

- IALA Guideline No. 1078 on The Use of Aids to Navigation in the Design of Fairways

- IALA Recommendation O-130 on Categorisation and Availability Objectives for Short Range Aids to Navigation

- IALA Recommendation O-132 on Quality Management for Aids to Navigation Authorities

- IALA Recommendation O-134 on the IALA Risk Management Tool for Ports and Restricted Waterways

Note: An example of a Level of Service (LOS) is provided at Annex A.

- CANADIAN COAST GUARD AIDS TO NAVIGATION LEVELS OF SERVICE STATEMENT

AIDS TO NAVIGATION

- Description

- The Aids to Navigation program involves the provision of short-range marine aids, including visual aids (fixed aids, lighthouses and buoys), aural aids (fog horns), radar aids (reflectors and beacons) and long-range marine aids, including electronic aids, such as the Differential Global Positioning System (DGPS).

- The benefit to mariners is safe, accessible and effective vessel transit in Canadian waters.

  - Objective

- To facilitate safe and expeditious movement of maritime traffic.

  - Services

- Provision of visual and aural aids to navigation such as fixed aids, lighthouses, buoys and fog horns.

- Provision of electronic positioning systems such as the Differential Global Positioning System.

- Provision of navigation safety information.

| Program: Aids to navigation |  |
| --- | --- |
| Service: Provision of visual and aural aids to navigation such as fixed aids, lighthouses, buoys and fog horns |  |
| Service | Service Standard |
| An aids to navigation system is provided where the volume of traffic justifies and the degree of risk requires aids (per program directives and procedures manual) under the following conditions: - To guide mariners to and from harbours operated under Port Authorities or the CCG - To facilities supported by federal funds - In areas of adequate charts (CHS Charts) in conjunction with other marine services as part of an agreement made by the CCG - To allow re-supply of isolated communities that are dependent upon marine transportation, even where there is a lack of adequate charting - In harbours that predominantly serve commercial fishers Short-Range Marine Aids may be established to: - assist landfall, mark approaches to harbours, ports and waterways - mark channels or tracks - mark hazards - identify positions or courses - indicate preferred routes - separate traffic (e.g. to mark traffic separation schemes noted by specialized symbols on Canadian Hydrographic charts) - indicate special areas such as anchorage Short-Range Marine Aids will not be provided: - In waters for which this responsibility has been delegated to other authorities through legislation or signed agreements - In waters where there is a lack of adequate charting that restricts the safe use to those with local knowledge - In waters where adequate depth of water is not available for common use - In waters where the aid(s) cannot be maintained to targeted reliability levels - Exclusively for purposes other than navigation - To mark obstructions outside marked channels and away from charted routes and tracks. However, isolated dangers in waters which are known by adequate charting to be otherwise safe, and which are regularly frequented by an appreciable number of users, may be marked. - For other than public use and, thus, exclusively for the benefit of single or a small number of users, or to mark access to private or municipal facilities. | - Visual aids are designed, where feasible, to be visible at least 75% of the time during the worst month of the navigation season. This is calculated based on long-term weather observations from the Meteorological Service of Canada – Environment Canada. - Aural aids may be provided when the design availability target of 75% cannot be achieved by visual means alone, for uncertified commercial vessels only. - Radar aids may be provided when the design availability target of 75% cannot be achieved by visual means alone, for certified commercial vessels only. - The overall target level for operational reliability for the short-range aids to navigation system is 99%, calculated over a three-year period. |

* * *