.

20ter, rue Schnapper, 78100

Saint Germain en Laye, France

Telephone +33 1 34 51 70 01 Telefax +33 1 34 51 82 05

E-mail - iala-aism@wanadoo.fr     Internet - http://iala-aism.org

IALA Recommendation

O-139

On

The Marking of Man-Made Offshore Structures

Edition 1

December 2008

AISM Association Internationale de Signalisation Maritime  IALA

International Association of Marine Aids to Navigation and Lighthouse Authorities

Document Revisions

Revisions to the IALA Document are to be noted in the table prior to the issue of a revised document.

| Date | Page / Section Revised | Requirement for Revision |
| --- | --- | --- |

# Recommendation on The Marking of Man-Made Offshore Structures

(Recommendation O-139)

THE COUNCIL:

RECALLING the function of IALA with respect to Safety of Navigation, the efficiency of maritime transport and the protection of the environment;

RECOGNISING that there is an increase in new and emerging uses of ocean and coastal waters, subsoil and seabed, an increase of seaborne trade, increasing demands of energy resources, increasing recreational use and increasing pollution threats from both ocean uses and an expansion of coastal populations.

RECOGNISING ALSO the number and types of man-made structures being built in the maritime environment are increasing.

RECOGNISING ALSO the need to provide consistency in marking different types of offshore structures which may be a danger to navigation,

RECOGNISING ALSO that it is a matter for a National Authority to decide on whether a man-made structure needs to be marked, depending on the risk involved and the level of traffic.

RECOGNISING ALSO that IMO Resolution A.672(16), dated 6th December 1989, established Guidelines and Standards for the Removal of Offshore Installations and Structures on the Continental Shelf and in the Exclusive Economic Zone, which incorporate requirements for such installations and structures, whilst being removed, to be marked in accordance with IALA Recommendations.

RECOGNISING FURTHER that marking is to improve the safety to navigation and protect the structures themselves,

ADOPTS the recommendation on the Marking of Man-Made Offshore Structures as set out in the following Sections.

NOTING that the content of recommendations for marking as outlined in IALA Recommendations:

O-114 “The Marking of Offshore Structures” Edition 1 May 1998

O-116 “The Marking of Aquaculture Farms” Edition 2 June 2007

O-117 “The Marking of Offshore Wind Farms” Edition 2 December 2004

O-131 “The Marking of Offshore Wave and Tidal Energy Devices” Edition 1 June 2005

Have been fully incorporated into O-139 which now supercedes the above withdrawn documents

RECOMMENDS that Members ensure that the marking of man-made structures conforms to the standards and practices specified in the following Sections of this recommendation; and

CONSOLIDATES IALA Recommendations, O-114, O-116, O-117 and O-131 within one document.

*    *    *

# Table of Contents

Document Revisions	2

Recommendation on The Marking of Man-Made Offshore Structures	3

Table of Contents	5

Index of Figures	6

Annex	8

1	INTRODUCTION	8

1.1	Scope	8

1.2	Field of Application	8

1.3	Information	8

2	marking of offshore STRUCTURES	9

2.1	MARKING OF OFFSHORE STRUCTURES IN GENERAL	10

2.1.1	General	10

2.1.2	Marking	11

2.1.3	Considerations During Construction / Decommissioning	12

2.1.4	Additional marking	12

2.2	MARKING OF OFFSHORE AQUACULTURE FARMS	13

2.2.1	General	13

2.2.2	Marking Examples	17

2.3	MARKING OF OFFSHORE WINDFARMS	23

2.3.1	General	23

2.3.2	Marking of Individual Structures (Wind Turbines)	23

2.3.3	Marking of Groups of Structures (Wind Farms)	25

2.3.4	Considerations During Construction	26

2.3.5	Additional Considerations	26

2.4	MARKING OF OFFSHORE WAVE AND TIDAL ENERGY DEVICES	27

2.4.1	General	27

2.4.2	Marking	27

2.4.3	Considerations During Construction / Decommissioning	29

2.4.4	Contingency Plans	30

2.4.5	Additional Considerations	31

3	FUTURE REQUIREMENTS	32

4	glossary	32

APPENDIX 1	INVENTORY OF OFFSHORE STRUCTURES	33

1	oFFSHORE STRUCTURES: definitions	33

1.1	Aquaculture Farms	33

1.2	Articulated Loading Platform (ALP) (Oil & Gas Permanent Installations)	33

1.3	Catenary Anchor Leg Mooring (CALM) (Oil & Gas Permanent Installations - Loading / Discharge Buoys)	34

1.4	Floating Production Storage Offloader (FPSOs) (Loading / Discharge Buoys)	35

1.5	Flotel (Oil & Gas Temporary Structures)	35

1.6	LNG Offloading Points	35

1.7	Meteorological Masts	36

1.8	Minimum Facility Platform (MFP)	36

1.9	Offshore docks / Loading Islands	37

1.10	Pipes	37

1.11	Production Platform / Drilling Rigs	39

1.12	Seaplane berth	39

1.13	Seawater Intakes / Sewage Outfalls	40

1.14	Single Point Mooring (SPM) (Loading / Discharge Buoys)	40

1.15	Tension Leg Platform (TLP) (Oil & Gas Permanent Installations)	41

1.16	Tidal Generator	42

1.17	Tidal Generator Field	42

1.18	Underwater manifolds / obstructions	43

1.19	Water Injection Platforms or booster station (WIPs)	43

1.20	Wave Generator	44

1.21	Wave Generator Field	44

1.22	Wind Generator	46

1.23	Wind Farm	46

# Index of Figures

Figure 1	A light in the middle of a rectangular aquaculture farm	17

Figure 2	A light on each sea corner and a daymark on each coast corner of a rectangular aquaculture farm.	18

Figure 3	A light on one of the sea corners, a light on the diagonally opposite coast corner and daymarks on the other two corners of a rectangular aquaculture farm.	18

Figure 4	Lights on diagonally opposite corners and daymarks on the other two corners of a rectangular aquaculture farm.	19

Figure 5	A light at each corner of a rectangular aquaculture farm.	20

Figure 6	A light in the centre of a circular aquaculture farm	21

Figure 7	Two lights, 180 degrees apart on the circumference of a circular aquaculture farm and two daymarks on the circumference positioned 90 degrees to the lights.	21

Figure 8	Three lights, 120 degrees apart on the circumference of a circular aquaculture farm.	22

Figure 9	Three lights, 120 degrees apart on the circumference of a circular aquaculture farm and three daymarks on the circumference positioned 60 degrees to the lights.	22

Figure 10	Sample marking of an individual wind turbine	24

Figure 11	Sample marking of a wind farm	26

Figure 12	A diagram of an Articulated Platform	33

Figure 13	A Catenary Anchor Leg Mooring buoy	34

Figure 14	Floating Production Storage Offloader	35

Figure 15	An example of a Flotel	35

Figure 16	An LNG Offloading Point	36

Figure 17	Meteorological Masts	36

Figure 18	An example of a Minimum Facility Platform	37

Figure 19	Offshore docks / Loading Islands	37

Figure 20	Typical underwater piping	38

Figure 21	Underwater pipe maintenance	38

Figure 22	An example of a Production Platform	39

Figure 23	An example of a Seaplane berth	39

Figure 24	A Seawater Intake / Sewage Outfall	40

Figure 25	A Single Point Mooring buoy	40

Figure 26	A diagram of a Tension Leg Platform	41

Figure 27	Tidal Generators	42

Figure 28	A Tidal Generator Field	42

Figure 29	An example of an underwater obstruction	43

Figure 30	A Water Injection Platform	43

Figure 31	A Wave Generator	44

Figure 32	A Wave Generator Field	44

Figure 33	A Wind Generator	46

Figure 34	A Wind Farm	47

# Annex

Recommendation O-139 – The Marking of Man-made Offshore Structures

# INTRODUCTION

There has been an increasing development in man-made structures at sea, in coastal or near coastal waters, which may affect shipping.  These structures can be: isolated or in groups, near or far from the shore, small or large, and close to or far from commercial and non-commercial navigation zones. Define Coastal & near Coastal waters

IALA is monitoring the developments of these structures and will continue to create and update documentation as required to ensure clear and unambiguous marking of waterways for safe navigation, protection of the environment and protection of the structures themselves.  Authorities facing problems in this field are invited to bring them to the attention of IALA to obtain advice on current practice.

An inventory and examples of Man-Made Offshore Structures can be found in Appendix 1.

The following sections of this Document detail the updated IALA Recommendations for the marking of each offshore structure type identified considered in Appendix 1.

The marking of offshore structures should be implemented using these recommendations as a minimum requirement. Note, National Authorities may require more stringent marking requirements

In section 3 of this Annex, attention is drawn to the need to re-examine as part of a periodic review basis to and if necessary update these Recommendations, with respect to from time to time with the advances in this technology.

## Scope

These recommendations are for the guidance and information of stakeholders such as National Administrations, Lighthouse Authorities, Aviation Authorities and other competent Authorities, Aids to Navigation providers, and the Contractors and Developers involved in each type of  the structures mentioned in the following sections. Add – Shipping Companies, Ships Agents, Port Authorities and Port State Control Regions (PARIS MOU - TOKYO MOU etc).

## Field of Application

These Recommendations apply to all structures fixed in position temporarily or permanently which extend above or below the surface of the sea and which are possible obstructions to navigation, e.g. structures used for drilling or exploring for oil and/or minerals, oil production platforms, oil well protective jackets, renewable offshore energy installations or ocean data platforms.  Structures specifically specially established as aids to navigation are excluded from these recommendations.

## Information

Responsible authorities must ensure that shipping and other all stakeholders will be informed of all installed AtoN and marking’s in accordance with these Recommendations, which are published in  nautical documents and by promulgation of navigational  warnings (in accordance with the SOLAS Convention)

## Definitions and Acronyms

- WTG - Wind Turbine Generator

- SPS – Significant Peripheral Structure

- I – Intermediate Structure

- MBS – IALA Maritime Buoyage System

- MHWS - Mean High Water Springs

- AIS – Automatic Identification System

- FPSO – Floating Production Storage Offloader

- MFP – Minimum Facility Platform

- SPM – Single Point Mooring

- SBM – Single Buoy Mooring

- Coastal Waters -……………to discuss…….

- Near Coastal Waters - …… to discuss…….

- Etc

# marking of offshore STRUCTURES

Man Made Offshore structures present very different characteristics.  Therefore, these structures have been grouped as follow’s; in the following manner.

Section 2.1: Offshore Structures in General not included in Sections 2.2 to 2.4

Section 2.2: Aquaculture Farms

Section 2.3: Wind farms

Section 2.4: Wave and Tidal Energy Devices

It is important to consider that the marking of offshore structures is essential important during the different phases of their existence, i.e. construction, operation and decommissioning, when the structure may remain as a hazard to navigation.

Table X (draft) below lists the marking recommendations and considerations for offshore structures

| * = RECOMMENDED + = TO BE CONSIDERED | Light (white)1 | Light (yellow)2 | Sub Light (Red)3 |  | Sound Signal | Radar Beacon | AIS | Buoys |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Offshore Oil or Gas Platform – Temporary or Fixed (2.1) Tension Leg Platform | * |  | * |  | * | + | + | + |
| Floating Production Storage Offloader (FPSO) (2.1) | * |  |  |  | + | + | + |  |
| Floating Petrochem Offloading Points/ Single Point Mooring (SPM) | * |  |  |  | + | + | + |  |
| Meteorological Mast(2.3) | * |  |  |  | + | + | + | + |
| Minimum Facility Platform (MFP) (2.3) | * |  | + |  | * | + | + | + |
| Offshore Docks/Loading Islands | * |  | * |  | * | + | + | + |
| Underwater Pipes, Underwater Manifolds | + |  |  |  |  |  |  | + |
| Tidal/Wave Generator | * |  | * |  | + | + | + | + |
| Tidal/Wave Generator Field | * |  | * |  | + | + | + | * |
| Offshore Wind Farm (2.3) |  | * |  |  | + | + | + | + |
| Isolated WTG (2.3) | * |  |  |  | + | + | + | + |
| Windfarm Transformer Station (2.3) | * |  | + |  | + | + | + |  |
| Intermediate structures on the periphery of OWF (2.3) |  |  |  | * |  |  |  |  |

1 – Minimum intensity 1400cd Morse <U> ev 15s

2 – Minimum intensity 75cd

3 – Minimum intensity 15cd

## MARKING OF OFFSHORE STRUCTURES IN GENERAL

### General

Consultation between the stakeholders such as Developers, National Administrations, Aids to Navigation (AtoN) Authorities, Competent Authorities and Contractors should take place at an early stage.  In general, development of all structures mentioned in this Section should not prejudice the safe use of Traffic Separation Schemes, Inshore Traffic Zones, recognised sea lanes and safe access to anchorages, harbours and places of refuge.  On a case-by-case basis, National Authorities may consider establishing Recommended to Avoid, Exclusion or Safety Zones, which would prohibit or restrict vessels from entering groups of Offshore Structures in General.  Such information should be shown must be identified on the navigation chart, as appropriate.

The IALA Recommendations of 1965, with later amendments in 1968, did not fully meet the requirements of the United Kingdom, Ireland and Norway. This was due mainly to the exposed position of structures and to the environmental conditions in the North Sea. These countries therefore agreed to enforce more stringent regulations.

These more stringent regulations have been in general use since 1977, and have met the operational requirements of certain authorities responsible for marking of structures, and those of shipping. These, and the Norwegian regulations, can be requested from those authorities.

In this connection respect the Authorities mentioned as indicated have further agreed to inform neighbouring countries and IALA about the establishment of any additional marking equipment that might otherwise might lead to confusion.

### Marking

The Offshore Structures mentioned in this section should be marked as a single unit, or a block or field as follows:

Any structure shall be marked at night by one or more white lights so constructed and fixed as to ensure that at least one light is visible upon approaching the structure from any direction.

The lights shall be placed not less than 6 metres and not more than 30 metres above Mean High Water Springs (MHWS) with a minimum effective intensity of 1400 candelas (approximately 10 Nautical Miles).  The lights shall be operated in unison with a flashing character according to Morse letter  U  (    ) and with a period of 15 seconds.  The vertical distribution of the projected beam shall be such that the light will be visible from the immediate vicinity of the structure to the maximum luminous range of the light.

The horizontal and vertical extremities of the structure shall be adequately marked in a manner determined by the Authority and in conformity with the requirements of air navigation regulations. Such marine lanterns are referred to as Subsidiary Lights.

Each structure shall, where practicable, display identification panels with black letters or numbers 1 m high on a yellow background visible in all directions.  These panels shall be easily visible in daylight as well as at night, either by the use of illumination or retro-reflecting material.

Each structure may carry one or more sound signals so constructed and fixed as to be audible upon approaching the structure from any direction.

The sound signals should be placed not less than 6 metres and not more than 30 metres above MHWS with a range of at least 2 nautical miles.  The character shall be rhythmic blasts corresponding to Morse letter  U  (    ) every 30 seconds.

The minimum duration of the short blast shall be 0.75 seconds.  The sound signals shall be operated when the meteorological visibility is two nautical miles or less.

Where there is a requirement to identify a particular structure, a radar beacon may be fitted.  The range and code shall be determined by the authority.  Any radar beacon on a temporary uncharted structure shall be coded  D  (     ). To discuss the difference between a Radar Beacon and Racon – mainly to avoid confusion.

Where a number of structures are situated so that the safety of navigation in the area may be secured without each of the structures being individually equipped with lights and sound signals, in accordance with these recommendations, or where the Authority considers that local conditions permit a relaxation of the requirements for the intensity of the light, the Authority shall determine what marking shall be applied.

Wherever deemed necessary by the authority, buoys or beacons shall be placed to mark the perimeter of a group of structures, or to mark channels through a group of structures, or to mark any fixed structure while being erected or dismantled.  The characteristics of such marks shall be determined by the Authority in accordance with the IALA Maritime Buoyage System (MBS).

Where underwater obstructions, such as submerged wells or pipelines, exist in depths of water so as to be a hazard to surface borne vessels, they should be adequately marked in accordance with the IALA MBS.

### Considerations During Construction / Decommissioning

During the construction / decommissioning of Offshore Structures in General, working areas should be established and marked in accordance with the IALA MBS.  National Authorities should also consider the use of guard ships in areas of high traffic density.

Notices to Mariners, Radio Navigational Warnings – NAVTEX and/or broadcast warnings must be promulgated in advance of and during any offshore wave and/or tidal energy extraction device construction.

Where individual offshore structures in general extend above the surface, careful consideration needs to be given to any additional temporary marking, e.g. floating AtoN or a Guard Ship, that may be required during the construction / decommissioning phase.

When decommissioning such devices, the Authority should ensure that the operator / contractor is obliged to remove all obstruction’s, so that the sea bed is returned to its original depth and topography.  In the event that any residue or obstruction remains that, in the opinion of the Aids to Navigation Authority, constitutes a danger to navigation, then the residue or obstruction shall be marked according to the authority’s requirements.

### Additional marking

Authorities may, in special cases due to extenuating navigational circumstances requirements, demand require that additional marking’s equipment are needed, such as for example:

A high intensity light with specific character

Maritime radio beacon with specially determined range, frequency and character.

Radar Beacon or Racon – to discuss.

Buoys in accordance with the IALA MBS

AIS AtoN

## MARKING OF OFFSHORE AQUACULTURE FARMS

2.2.1 General

When aquaculture farms are considered to represent a danger to navigation, it should be marked in accordance with IALA MBS, using Special Marks, Lateral Marks, or Cardinal Marks – or a combination thereof.

The use of radioAtoN, such as Racons or AIS AtoN may also be considered.

Consider Terminology. (radio is better than electronic?)

Aquaculture farms can be of various regular and irregular shapes and sizes and are usually either Rectangular or Circular in layout.

The farm, or group of farms, should be marked depending on their size, extent and location. In some cases it may be sufficient to mark only part of the perimeter, or the centre, only.

Administrations should bear in mind that the marking recommendations herein may be adjusted in consideration of traffic density, proximity to ports, proximity to dangers, tidal considerations and other factors. (Define between Administrations and National Authority in one paragraph).

- Aquaculture farms are normally marked by Special Marks;

- A Pictogram may be considred to be attached on Special Marks. (refer to IALA MBS)

(for example)

- If there is a requirement for vessel traffic between aquaculture farms, then such channels are normally marked by Lateral Marks;

- If the prevailing situation warrants, Cardinal Marking alone may be used to direct vessel traffic away from the aquaculture farm(s);

- To improve the effectiveness of the lighting, and taking into account background lighting, synchronisation should be considered.

- The National Authority should be satisfied that the selected lighting has a suitable Minimum Effective Range and sufficient autonomy with the ability to over-winter – especially in higher latitudes.

- To improve the radar target and radar visibility, radar reflectors and radar reflective material should be considered.

2.2.2.	Marking of Aquaculture farms ()

For the consistency of the paper. I would suggest to add numbering the contents and use the title of Marking of aquaculture farms.

Examples can be found in the following table which illustrates the minimum recommended marking arrangement with Special Marks.

Rectangular Aquaculture Farms

Should be marked according to the length of their sides.

Circular Aquaculture Farms

### Should be marked according to their diameter.

### 2.2.2.1 AtoN Layout for Rectangular Aquaculture

### 2.2.2.2 AtoN Layout for Circular Aquaculture

### ** Since the other contents such as Windfarms or Offshore structures has international system of units based on nautical mile, so we would also consider the units for aquaculture farm contents based on not meter but nautical mile.

### ** Instead of using the below new symbols, the orginal symbols are easily understood by the National Authority. We recommend the orginal symbols to be used without change.

### 2.2.3	Considerations During Provision / Decommissioning

### During the provision / decommissioning of an aquaculture farms, working areas should be established and marked in accordance with the IALA MBS. National Authorities should also consider the use of gurad ships in areas of high traffic density.

### Notice to Mariners, Radio Navigatinal Warnings – NAVTEX and/or broadcast warnings must be promulgated in advance of and during any aquaculture farm provision.

### When decommissioning such facilities, the Authority should ensure that the operator / contractor are obliged to remove all obstruction. In the event that any residue or obstruction remains that, in the opinion of the Aids to Navigation Authority, constitute a danger to navigation, the the residue or obstruction shall be marked according to the authority´s requirements.

## MARKING OF OFFSHORE WINDFARMS

### General

When mentioning Offshore Wind Farms, the following are included: Meteorological Mast, Wind Turbine Generator and Offshore Sub Station.

Consultation between the stakeholders such as Developers, National Administrations, Lighthouse Authorities, Aviation Authorities, AtoN providers, Competent Authorities and Wind Farm contractors and developers should take place at an early stage.  In general, development of offshore energy structures or OWF should not prejudice the safe use of Traffic Separation Schemes, Inshore Traffic Zones, recognised sea lanes and safe access to anchorages, harbours and places of refuge.  On a case-by-case basis, National Authorities may consider establishing Exclusion or Safety Zones, which would prohibit or restrict vessels from entering OWF.  Such information should be shown on the navigation chart, as appropriate.

In order to avoid confusion from a proliferation of Aids to Navigation in a high-density wind farm, full consideration should be given to the use of synchronised lighting, different light characters and varied light ranges.

Some IALA members have carried out trials on wind farms to identify whether interference to radar, radio navigation and radio communications is experienced.  Trials indicate that wind farm structures affect shipborne and shore based radar systems.  This interference returned radar responses strong enough to produce interfering side lobe, multiple and reflected echoes.  Bearing discrimination was also reduced by the magnitude of the response.  It has been determined that passage close to a wind farm boundary, or within the wind farm itself, could affect the vessel’s ability to fully comply with the International Regulations for the Prevention of Collisions at Sea.  Administrations / developers should keep this information in mind when designing wind farms, and they may wish to carry out individual trials to verify the impact of the wind farm on navigation.

There has been some evidence that scouring at the bases of WTG in areas of strong tides or currents has resulted in significant deposits of material in other locations.  Some authorities have insisted on fitting depth monitoring devices to wind generators to measure scour.  This may need to be considered when approving wind farm proposals/locations.

### Marking of Isolated WTG and Meteorological Mast

The tower of every structure should be painted yellow all round from the level of Highest Astronomical Tide (HAT) to 15 metres or the height of the AtoN, if fitted, whichever is greater.

Alternative marking may include horizontal yellow bands of not less than 2 metres in height and separation.

Consideration may be given to the use of additional retro reflective material.

Due to the increased danger posed by an isolated structure, it should be lighted with a white light flashing Morse code  U  (    ) with a period of 15 seconds, and with a minimum effective intensity of 1400 candelas. (approximately 10 Nautical Miles)

#### Aids to Navigation for marking Individual Structures

The AtoN on the structure of a WTG should be mounted below the lowest point of the arc of the rotor blades.  They should be exhibited at a height of at least 6 metres above the level of the HAT.  AtoN  on WTG should comply with IALA Recommendations and have an availability of not less than 99.0% (IALA Category 2 – see IALA NAVGUIDE).

HAT

TT

15 metres

HAT

15 metres

Height of AtoN,

if fitted

Sample marking of an individual wind turbine

### Marking of Groups of Structures (Offshore Wind Farms)

A Significant Peripheral Structure (SPS) is the ‘corner’ or other significant point on the periphery of the OWF.  Each individual SPS should be fitted with lights visible from all directions in the horizontal plane.  These lights should be synchronized to display an IALA Special Mark characteristic, flashing yellow, with a Minimum Effective Intensity of 75 candelas (approximately 5 Nautical Miles) nauticalmiles

As a minimum, lights on individual SPSs should exhibit synchronised flashing characteristics, however Administrations should consider the synchronisation of all SPSs.  In the case of a large or extended wind farm, the distance between SPSs should not normally exceed three (3) nautical miles.

Selected Intermediate structures on the periphery of an OWF other than the SPSs, should be marked with flashing yellow lights which are visible to the mariner from all directions in the horizontal plane.  The flash character of these lights should be distinctly different from those displayed on the SPSs, with Minimum Effective Intensity of 5 candelas  (approximately 2 Nautical Miles) 2nauticalmiles.  The lateral distance between such lit structures or the nearest SPS should not exceed two 2 Nnautical Miles.

#### Aids to Navigation for marking Wind Farms

In addition to the use of lights for marking the SPSs and selected Intermediate peripheral structures of a wind farm, further consideration should be given to the use of:

Lighting all peripheral structures;

Lighting all structures within the OWF;

Racons;

Radar Reflectors and Radar Target Enhancers; and/or

AIS AtoN

It is important that these AtoN be used with care to mark the grouping of WTG;  AIS AtoN may be used to generate synthetic targets.

Consideration may also be given to the provision of sound signals where appropriate, taking into account the prevailing visibility, topography and vessel traffic conditions. The typical range of such a sound signal should not be less than two 2 Nnautical miles.

SPS

SPS - lights visible from all directions in the horizontal plane.  These lights should be synchronized to display an IALA Special Mark’ characteristic, flashing yellow, with a range of not less than five (5) nautical miles

I

Intermediate structures on the periphery of a wind farm other than the SPSs - marked with flashing yellow lights which are visible to the mariner from all directions in the horizontal plane with a flash character distinctly different from those displayed on the SPSs and with a range of not less than 2 nautical miles

< 2 nm

SPS

SPS

SPS

SPS

SPS

I

I

< 3 nm

Sample marking of a wind farm

### Considerations During Construction

During the construction of an OWF, working areas should be established and marked in accordance with the IALA MBS.  National Authorities should also consider the use of Guard Ships in areas of high traffic density.

Notices to Mariners, Radio Navigational Warnings and Notices to Airmen must be promulgated in advance of and during any OWF construction.

Power cables between WTG, between WTG and the Offshore Sub Station, and between the Offshore Sub Station and the shore should be sufficiently trenched to avoid exposure from scouring / sand migration or trawling activities.

### Additional Considerations

Depending on the marking, lighting and lateral separation of the peripheral structures, the additional marking of the individual structures within a wind farm may be considered as follows:

Lighting of each structure;

Individual structures unlighted with retro-reflective areas;

Individual structures illuminated with down-lights on ladders and access platforms;

Use of flashing yellow lights with a Minimum Effective Intensity of 5 candelas  (approximately 2 Nautical Miles) 2nauticalmiles

Identifying numbers on each individual structure, whether lit or unlit.

An Offshore Sub Station or a Meteorological Mast, if considered to be a composite part of the OWF, should be included as part of the overall wind farm marking.  If not considered to be within the OWF block it should be marked as an offshore structure (i.e. a white light flashing Morse code  U  (    ) with a period of 15 seconds, and with a minimum effective intensity of 1400 candelas.

As far as practicable, Aeronautical obstruction warning lights fitted to the tops of wind generators should not be visible below the horizontal plane of these lights.  Aviation Authorities should be consulted regarding the specification of such lights.

## MARKING OF OFFSHORE WAVE AND TIDAL ENERGY DEVICES

### General

Wave and Tidal Energy Devices include: Tidal Generator, Tidal Generator field, Wave Generator, Wave Generator field, as defined in Appendix 1.

In general, any risk assessment of offshore wave and tidal extraction devices will likely determine that, when compared to gas and oil structures, there is a lower risk of either pollution or loss of life should a vessel foul such an installation.  Consequently, the marking requirements can be mitigated.  It should be borne in mind that many wave and tidal devices are low freeboard floating structures that are moored to the seabed.  They may be moored in deep or shallow water and some may be located on the seabed or just below the surface.  Surface piercing and subsurface elements may extend laterally beyond the surface elements.  This could include shared moorings and mid-water connections between units which may also carry electricity, control signals, hydraulics or pneumatics associated with the units.  It should also be noted that many tidal concepts have fast-moving sub-surface elements such as whirling blades, and these should be taken into account when identifying the marking requirements.

Consultation between the stakeholders such as Developers, National Administrations, AtoN Authorities, Competent Authorities and wave and tidal contractors should take place at an early stage.  In general, development of offshore energy structures should not prejudice the safe use of Traffic Separation Schemes, Inshore Traffic Zones, recognised sea lanes and safe access to anchorages, harbours and places of refuge.  On a case-by-case basis, National Authorities may consider establishing Exclusion or Safety Zones, which would prohibit or restrict vessels from entering Offshore Wave and Tidal Energy fields.  Such information should be shown on the navigation chart, as appropriate.

In order to avoid confusion from a proliferation of AtoN in a high-density wave and tidal energy extraction field, full consideration should be given to the use of synchronised lighting, different light characters and varied light ranges.

There has been some evidence that sea-bed scouring at the bases of offshore renewable energy installations in areas of strong tides or currents has resulted in significant deposits of material in other locations.  Some authorities have insisted on fitting depth monitoring devices to such installations to measure scour.  This may need to be considered when approving wave and tidal energy extraction proposals/locations.

### Marking

Wave and Tidal energy extraction devices should be marked as a single unit or as a block or field as follows:

- When structures are fixed to the seabed and extend above the surface, they should be marked in accordance with the recommendations contained in Section 2.3, Marking of Offshore Wind Farms.

Areas containing surface or sub-surface energy extraction devices (wave and/or tidal) should be marked by appropriate navigation buoys in accordance with the IALA MBS, fitted with the corresponding topmarks and lights.  In addition, active or passive radar reflectors, retro reflecting material, racons and/or AIS transponders should be fitted as the level of traffic and degree of risk requires.

The boundaries of the wave and tidal energy extraction field should be marked by lighted navigational buoys, so as to be visible to the Mariner from all relevant directions in the horizontal plane, by day and by night.  Taking the results of a risk assessment into account, lights should have a nominal range of at least 5 (five) nautical miles.  The Northerly, Easterly, Southerly and Westerly boundaries should normally be marked with the appropriate IALA Cardinal Mark.  However, depending on the shape and size of the field, there may be a need to deploy lateral or special marks.

In the case of a large or extended energy extraction field, the distance between navigation buoys that mark the boundary should not normally exceed 3 (three) nautical miles.

Taking into account environmental considerations, individual wave and tidal energy devices within a field which extend above the surface should be painted yellow above the waterline.  Depending on the boundary marking, individual devices within the field need not be marked.  However, if marked, they should have flashing yellow lights so as to be visible to the mariner from all relevant directions in the horizontal plane.  The flash character of such lights should be sufficiently different from those displayed on the boundary lights with a range of not less than 2 nautical miles.

Consideration should be given to the provision of AIS as an Aid to Navigation on selected peripheral wave and/or tidal energy devices.

A single wave and/or tidal energy extraction structure, standing alone, that extends above the surface should be painted black, with red horizontal bands, and should be marked as an Isolated Danger as described in the IALA MBS.

In the case of a single wave and/or tidal energy device which is not visible above the surface but is considered to be a hazard to surface navigation, it should be marked by an IALA special mark yellow buoy with flashing yellow light with a range of not less than 5 nautical miles, in accordance with the IALA MBS.  It should also be noted that many tidal concepts have fast-moving sub-surface elements such as whirling blades.

The Aids to Navigation described herein should comply with IALA Recommendations and have an appropriate availability, normally not less than 99.0% (IALA Category 2 – see IALA NAVGUIDE).

The relevant Hydrographic Office should be informed of the establishment of an energy extraction device or field, to permit appropriate charting of same.

Notices to Mariners should be issued to publicise the establishment of a wave and/or tidal energy device or field.  The Notice to Mariners should include the marking, location and extent of such devices/fields.

### Considerations During Construction / Decommissioning

During the construction / decommissioning of an offshore wave and/or tidal energy extraction device or field, working areas should be established and marked in accordance with the IALA MBS.  National Authorities should also consider the use of guard ships in areas of high traffic density.

Notices to Mariners, Radio Navigational Warnings – NAVTEX and/or broadcast warnings must be promulgated in advance of and during any offshore wave and/or tidal energy extraction device construction.

During construction, power cables between wave and tidal generators, between such generators and the transformer station, and between the transformer station and the shore should be sufficiently trenched to avoid exposure from scouring / sand migration or trawling activities.

Where individual wave and/or tidal energy devices extend above the surface careful consideration needs to be given to any additional temporary marking that may be required during the construction / decommissioning phase.

When decommissioning such devices, the Authority should ensure that the operator / contractor is obliged to remove all obstruction so the sea bed is returned to its original depth and topography. In the event that any residue or obstruction remains that, in the opinion of the Aids to Navigation Authority, constitutes a danger to navigation, then the residue or obstruction shall be marked according to the authority’s requirements.

SUGGESTIONS FOR ADDITIONAL

- Monitoring of the area by local traffic control.

- Current working areas will be marked in accordance with IALA MBS (Figure Z).

- Power cables protected by burial or special cable protection.

- Power cables are placed perpendicular to the bottom in relation to existing traffic routes and fairways.

- Power cables illustrated in current chart.

- Hydrographic surveying in standard IHO S-44 is recommended after recovery area.

### Contingency Plans

- Operators of wave and/or tidal energy extraction devices or fields should develop contingency plans and emergency response plans which address the possibility of individual devices breaking loose and becoming floating hazards. Automatic location and tracking devices should be considered.

Developers and/or operators should have a reliable maintenance and casualty response regime in place to ensure the required availability targets are met.  This will include having the necessary AtoN spares on hand, with provision made at the design stage, where necessary, to ensure safe access.

SUGGESTIONS FOR ADDITIONAL

- Contingency plans drawn up in consultation with the relevant authorities.

- Maintenance resources adapted to the current level of service.

REMARKS:		Marine Spatial Planning

Recommendation regarding marine spatial planning should always be carried out with the support of risk management tools (e.g. IALA IWRAP Mk II, PAWSA and simulation).

### Additional Considerations

Depending on the marking, lighting and lateral separation of the field boundary, the additional marking of the individual structures within an energy extraction field, visible above the surface of the sea, may be considered as follows:

Lighting of each structure.

Individual structures unlighted with retro-reflective areas.

Individual structures illuminated with down-lights on ladders and access platforms.

Use of flashing yellow lights with a Minimum Effective Intensity of 5 candelas  (approximately 2 Nautical Miles) 2nauticalmiles

Identifying numbers on each individual structure, either lit or unlit.

An electrical Offshore Sub Station or other structure, if considered to be a composite part of the energy extraction field, should be included as part of the overall marking.  If not considered to be within the boundaries of the field, it should be marked as a single stand alone device as described in Section 2.4.2 (paragraph 7 refers).

# FUTURE REQUIREMENTS

IALA is aware that there is an increasing number of energy devices and structures already in place with many more planned which may affect shipping.

It is therefore recommended that Authorities should continuously monitor these Recommendations to ensure that any navigational problems caused by offshore structures are solved in a satisfactory manner.

Authorities facing problems in this field are invited to bring them to the attention of IALA to obtain advice based on current practice.

IALA foresees the need to re-examine these Recommendations from time to time, and if necessary to update them.

# glossary

National Authority: the competent National Authority for determining the marking of offshore structures – hereafter referred to as the Authority.

Effective intensity  –  the effective intensity of rhythmic lights shall be calculated in accordance with the IALA Recommendations for the calculation of the effective intensity of a rhythmic light, November 1980, using the Schmidt-Clausen method of calculation, where this is applicable for the apparatus concerned.

Energy Extraction Device (EED)  -  a wave or tidal generator.

Highest Astronomical Tide (HAT)  -  is the highest level that can be expected to occur under average meteorological conditions and under any combination of astronomical conditions.  HAT is not an extreme level, as certain meteorological conditions can cause a higher.  The level under these circumstances is known as a 'storm surge'.  HAT is determined by inspecting predicted sea levels over a number of years.

Mean High Water Springs (MHWS).  The height of mean high water springs is the average throughout the year (when the average maximum declination of the moon is 23.5) of two successive high waters during those periods of 24 hours when the range of the tide is at its greatest.

Nautical mile – 1852m.

Significant Peripheral Structure (SPS)  -  the ‘corner’ wind generator on a rectangular wind farm or other significant point on the periphery of a wind farm.

Transformer Station (hub)  -  a special structure within or outside the wave and/or tidal energy extraction field and/or wind farm to which the individual generators are connected via a power cable.  Power is transferred ashore from the transformer station by submarine cable.  A ‘hub’ may be a separate fixed or floating platform, a unit very similar to the generators but carrying additional power conversion equipment.

Usual range  –  the usual range of the fog signal shall be calculated in accordance with IALA Recommendation E-109, on the calculation of the range of a sound signal, dated May, 1998.

INVENTORY OF OFFSHORE STRUCTURES

# oFFSHORE STRUCTURES: definitions

This Appendix states definitions and gives examples of current offshore structures.

## Aquaculture Farms

Aquaculture is the cultivation of fresh-water and marine species, including molluscs, crustaceans and aquatic plants.  Unlike fishing, aquaculture, also known as aquafarming, implies the cultivation of aquatic populations under controlled conditions.

Particular kinds of aquaculture include algaculture (the production of kelp/seaweed and other algae); fish farming; shrimp farming, shellfish farming, and the growing of cultured pearls.

The worldwide practice of aquaculture runs the gamut from low-technology extensive methods to highly intensive systems.

Aquaculture farms take on a variety of forms including huge tanks, freshwater ponds, and shallow- or deep-water marine environments.  This document will relate to farms in marine environments.

## Articulated Loading Platform (ALP) (Oil & Gas Permanent Installations)

A diagram of an Articulated Platform

An ALP is a metal lattice tower, buoyant at one end and attached at the other by a universal joint to a concrete filled base on the sea bed.  The platform may be fitted with a helicopter platform, emergency accommodation and hawser/hose retrieval.

## Catenary Anchor Leg Mooring (CALM) (Oil & Gas Permanent Installations - Loading / Discharge Buoys)

A Catenary Anchor Leg Mooring buoy

CALM systems, are named so for the characteristic curve of the anchor legs that hold the buoy in position.  These buoys are also often referred to as a single buoy mooring, monobuoy or loading buoy.

CALM buoys can be designed to berth any size tanker up to and including Ultra Large Crude Carriers (ULCCs).

The main applications of a CALM system are:

Short term mooring: for import and export of fluids between onshore or offshore facilities and a tanker;

Permanent Mooring: for production and storage systems;

Semi-permanent Mooring: permanent mooring with easy disconnect capability to evacuate the facility in case of severe weather conditions.

## Floating Production Storage Offloader (FPSOs) (Loading / Discharge Buoys)

Floating Production Storage Offloader

These are vessels which are usually self powered and make their own way from the builder yard to the oil fields where they are permanently anchored, and act as floating tanks.

They may be purpose built or can be ships (VLCC, for example) that have passed their active trading life and were refitted into a FPSO.

## Flotel (Oil & Gas Temporary Structures)

An example of a Flotel

This type of structure is a platform, that is used as an R&R (rest and recuperation) location for workers; it is not an active rig.

## LNG Offloading Points

An LNG Offloading Point

These Offloading Points, which are used for loading / unloading LNG, are linked or fixed to the seabed and include many types of transference structures.

## Meteorological Masts

Meteorological Masts

Any individual surface structure, usually consisting of an embedded mast or tower with meteorological measuring instruments.

## Minimum Facility Platform (MFP)

An example of a Minimum Facility Platform

These surface facilities have completed a variety of modules including power generation, compression, drilling and accommodation.  They are designed for simultaneous drilling and production and have undertaken the construction of many jackets and subsea structures.

## Offshore docks / Loading Islands

Offshore docks / Loading Islands

Floating structures of various types and sizes, moored to the seabed and used for berthing and loading / unloading cargo.

## Pipes

Underwater or subsea pipes are worldwide used.  They are usually made of steel, have a concrete coating, and depending on the conditions, can be placed by barges with divers assistance.

Typical underwater piping

Underwater pipe maintenance

## Production Platform / Drilling Rigs

An example of a Production Platform

Production Platforms or Drilling Rigs are large structures used to house workers and machinery needed to drill and/or extract oil and natural gas through wells in the ocean bed.  The platform may be attached to the ocean floor, consist of an artificial island, or be floating.

Many platforms also have remote wellheads attached by umbilical connections, these may be single wells or a manifold centre for multiple wells.

## Seaplane berth

An example of a Seaplane berth

A Seaplane is a fixed-wing aircraft designed to take off and land on water.  A seaplane berth is a structure that has berthing facilities for these kind of planes, which are generally used for connecting islands to the mainland.

## Seawater Intakes / Sewage Outfalls

A Seawater Intake / Sewage Outfall

A seawater supply system can be found in many projects, and it includes Seawater Intakes and a discharge system.  An outlet to a drainage network or a wastewater treatment is the Sewage Outfall.

## Single Point Mooring (SPM) (Loading / Discharge Buoys)

A Single Point Mooring buoy

Loading Buoys are anchored offshore and serve as a mooring point for tankers to (off)load gas or fluid products.  They are the link between the geostatic subsea manifold connections and the tanker.

The main purpose of the buoy is to transfer fluids between onshore or offshore facilities and the moored tanker.

These Buoys are generally referred to as Single Point Mooring systems or ‘SPMs’.  They are also often referred to as Single Buoy Moorings or ‘SBMs’.

## Tension Leg Platform (TLP) (Oil & Gas Permanent Installations)

A diagram of a Tension Leg Platform

A Tension Leg Platform (TLP) is a buoyant platform held in place by a mooring system.  The TLP’s are similar to conventional fixed platforms except that the platform is maintained on location through the use of moorings held in tension by the buoyancy of the hull.

The topside facilities (processing facilities, pipelines, and surface trees) of the TLP and most of the daily operations are the same as for a conventional platform.

## Tidal Generator

Tidal Generators

Any individual surface or sub-surface structure incorporating a generator, fixed or moored to the seabed and connected to an electrical terminal via cable(s).

## Tidal Generator Field

A Tidal Generator Field

A group of individual tidal generators, which are located in one block and are considered to be a unit, fixed or moored to the seabed and/or each other and connected to an electrical terminal via cable(s).

## Underwater manifolds / obstructions

An example of an underwater obstruction

This group of structures comprises manifolds and various types of man-made obstructions placed on the seabed.

## Water Injection Platforms or booster station (WIPs)

A Water Injection Platform

Water injection platforms usually comprise a fixed steel platform, linked to the a wellhead platform.  The integrated topside has water injection facilities, water treatment facilities and power generation.  This platform can also lodge a drilling rig.

## Wave Generator

Production models will be coloured yellow, in accordance with the Recommendation.

A Wave Generator

Any individual surface or sub-surface structure incorporating a generator, moored to the seabed and connected to an electrical terminal.

## Wave Generator Field

Production models will be coloured yellow, in accordance with the Recommendation.

A Wave Generator Field

A group of individual wave generators, which are located in one block and are considered to be a unit, moored to the seabed and/or each other and connected to electrical hub.

A Wave Generator Field

Wave Power Buoys

## Wind Generator

A Wind Generator

Any individual surface structure, usually consisting of an embedded mast or tower with rotating blades and incorporating a generator.

## Wind Farm

A Wind Farm

A group of individual wind generators, which are located in one block and are considered to be a unit.

## Floating Wind Turbine Generator

## Figure XX

## Floating OWF and WED

Figure XX