AISM Association Internationale de Signalisation Maritime      IALA

International Association of Marine Aids to Navigation and Lighthouse Authorities

IALA Guideline No. ####

On

User Requirements and Practical Applications of e-Navigation

from Berth to Berth, for AtoN Authorities

Edition 1

[Date issued]

10, rue des Gaudines

78100 Saint Germain en Laye, France

Telephone: +33 1 34 51 70 01  Fax:  +33 1 34 51 82 05

e-mail:  contact@iala-aism.org       Internet:  www.iala-aism.org

This document has been formatted in accordance with the current IALA template

Document Revisions (Title style)

Revisions to the IALA Document are to be noted in the table prior to the issue of a revised document.

| Date | Page / Section Revised | Requirement for Revision |
| --- | --- | --- |

Table of Contents

To update, right click anywhere on the table, then update field / update entire table.

Document Revisions (Title style)	2

Table of Contents (Title style)	3

Index of Tables	3

Index of Figures	3

Title of document (Title style)	3

1	Heading 1 [Introduction]	3

1.1	Heading 2	3

1.1.1	Heading 3	3

1.1.2	Heading 3	3

2	Heading 1 again [Background, as required]	3

2.1	Heading 2 again	3

2.1.1	Heading 3 again	3

3	Heading 1 again [Scope / Purpose (may be called Objectives)]	3

4	Heading 1 again [Definitions / Acronyms, as required]	3

5	Heading 1 again [as many as required]	3

6	Heading 1 again [Conclusions]	3

ANNEX 1	Annex Title	3

APPENDIX 1	Appendix title	3

1	Heading 1	3

Index of Tables

Table 1	Title required	3

Index of Figures

Figure 1	Title required	3

Title of document

# Scope and Purpose

The purpose of this document is to provide guidance for AtoN authorities on user requirements and applications of e-Navigation from berth to berth.

e-Navigation is currently defined as: the harmonised collection, integration, exchange, presentation and analysis of maritime information onboard and ashore by electronic means to enhance berth to berth navigation and related services, for safety and security at sea and protection of the marine environment’.

e-Navigation is intended to benefit all mariners and associated marine stakeholders. Shipborne carriage requirements and navigational suport infrastructure requirements will need to be such as to ensure a consistent international level of service.

User requirements arefor simplified navigation and communication support systems and reduce the risk of human error. Information must be reliable, and displayed in a clear and consistent manner. Therefore visible and aural AtoNs will remain an important element of passage planning and implementation.

The system should be capable of automatically using the most appropriate data available and alert the user to any data integrity issues as they arise.

Automated exchange and synchronisation of data can provide the basis for a risk analysis model that will inform and support safe decision making and reduce human error.

The core dataset at the heart of e-Navigation should be controlled by a competent Government authority. However, the user interface needs to be sufficiently open to encourage innovative applications and commercial competition. Control of user functionality and presentation needs to be balanced against the need to take advantage of rapidly developing technology and user needs.

A key factor of e-Navigation is the human element. Therefore applications should distinguish between the different levels of user knowledge and experience and present appropriate levels of functionality and assistance to the user. Training will be critical to successful implementation of e-Navigation. Training should be structured in such a way that all types of users experience the same approach to the use and interpretation of the shared data.

- Title required

Note.  Titles of Tables go above them.  This example has font size 10 with 3 pt above and below text and with the text centred vertically

# General Requirements

Among others requirements, e-Navigation requires:

- the provision of better integration of ship and shore-based systems with greater standardisation of interfaces,

- enhanced navigation system resilience;

- more standardised and automated reporting facilities;

- automated supporting tools and alert/alarm management.

- improved reliability, better indication of reliability and greater quality assurance;

- improved target detection;

- more effective use of area limits (e.g. guard zones, restricted areas, danger zones);

- reduction of administrative burden;

- better data collection for marine domain awareness and more automated updating of essential information

Integration and exchange of data between shore and ship can provide a simplified clear shared dataset that will enhance operational understanding and reduce the risk of human error.

A ‘Maritime Service Portfolio’ (MSP) defines and describes the set of operational and technical services or products and their level of service provided by a  Competent Authority (and other stakeholders) in a given sea area, waterway, or port, as appropriate. Thus, the MSP concept should allow for a selectable and scalable service based on the type of user and phase of the voyage.

- Data requirements

Source

Data must only be sourced from authorised sources and subjected to validation before incorporation in the dataset.

Provision and Management

The data must be maintained and managed by the data provider within agreed procedures and parameters depending on the type and purpose of the data.

Data provision and MSP content will differ for different users and areas. It is envisaged that a wide range of data providers will be involved.

Format

Data format must be in accordance with existing standards and specifications, approved by competent agencies.

Security / protection

Some data, depending on its purpose may be subject to additional protection measures, including the use of encrypted data.

Integrity and reliability

Each data source will require validation for reliability and integrity before onward dissemination to the user. Data validation processes should also be performed on reception.

Automatic data synchronisation should then provide identical reliable data for all users including static and dynamic data. Such data requirements may include:

•	Traffic density

•	Hydrographic data

•	CCTV and imagery data

•	METOC data

•	Vessel information (Position, Course/Speed, Voyage Plan, Cargo, etc.)

•	VTMS Data

•	Maritime Safety Information (MSI)

•	Domain awareness

•	Charts and publications

•	Communications channels

•	PNT data

requirements

Users of shipboard and shore based information systems and associated decision support tools demands for the utilization of:

Data mining techniques

Risk assessments models (real time or not) identification and mitigation

Enhanced

alarm and alert management.

Integrity enhancement, both system components and data quality and monitoring

Means to engage the user in different processes, namely the mariner;

Automatic e

vent recording and electronic log books

COMUNICATION AND NAVIGATION Requirements for

MSP

COMSAR 15 and NAV 57 identified 5 different areas in which Maritime Service Portfolios may be developed. These are:

- Harbour operations;

- Operations in coastal and confined or restricted waters;

- Transocean voyages;

- Offshore operations; and

- Operations in Arctic, Antarctic and remote areas.

The future e-Navigation system with automatic selection of data and channels should not require any user intervention to identify stage of voyage or optimum operating mode.

Making new vessels e-Nav ready

# Reliable communications at an appropriate bandwidth are critical to e-Navigation. The level of communications required will depend on the type and area of operations and will be set out in a Maritime Service Portfolio (MSP).

Communications requirements and capability will differ in each of these areas.

Harbour Operations.

Harbour operations will require the highest level of communications in terms of reliability, integrity, availability, bandwidth and variety of means.

It is envisaged that in harbour areas considerable volumes of data will automatically be synchronised between ship and shore, this data may include:

- Tidal and current data;

- Chart updates;

- Pilotage plan;

- DGNSS corrections;

- AtoN status;

- Port security information;

- Traffic reports

- Administrative information;

Potential communications channels include

- VHF (voice, AIS, DSC, data)

- GSM / 3G / 4G

- WiMax

- Proprietary local systems (may be various bands and protocols)

- Satellite

- HF / LF systems (including eLoran)

- WIFI

- National broadcasting services (e.g. information, time signals)

Any compatible system or mix of systems may be used provided they meet the reliability and integrity criteria.

Coastal, Confined & Restricted waters

Operations in coastal, confined and restricted waters will require a high level of communications. In the transition area from Harbour to Coastal the mix of systems will be greater but the system used should be capable of switching to the most suitable means without loss of data.

Potential communications channels include

- VHF (voice, AIS, DSC, data)

- GSM / 3G / 4G (Near coastal only)

- Satellite

- HF / LF systems (including eLoran)

- Proprietary regional systems (may be various bands and protocols)

- WiMax

- National broadcasting services (e.g. information, time signals)

Transocean voyages

Transocean voyages will require longer range communications systems but will also have reduced data needs.

Potential communications channels include

- Satellite

- LF systems (including eLoran)

- MF

- VHF (voice, AIS, DSC, data)

- National broadcasting services (e.g. information, time signals)

Offshore operations

Among the communications required for the respective GMDSS area, offshore operations will most probably need specific communication channels and may also require channels for large volume of data.

For this operation, it is envisaged that the communication channels that would possibly be used at the three previous stage of voyage could also be applicable, depending on the location and the type of operation.

Arctic, Antarctic and remote areas.

The potential communication channels will depend on the regions and the proximity to shore infrastructures. Therefore the following channels may be deployed:

- GSM / 3G / 4G

- WiMax

- Proprietary local systems (may be various bands and protocols)

- Satellite

- HF / LF systems (including eLoran)

- WIFI

- National broadcasting services (e.g. information, time signals)

# aton requirements

Aids to Navigation will be one of the key data sources for the e-Navigation dataset.

Figure 1 – Example of an  schematic representation of  e-Navigation information workflow

AtoN elements may include:

- Radio Navigation Systems (GNSS / Terrestrial backup)

  - GNSS for providing time reference for synchronized lights

  - E-Racons as terrestrial RADAR ranging mode

  - AIS ranging mode

  - E-Loran

- DGNSS beacons

  - For provision of GNSS corrections

- Lighthouses, Buoys and Beacons

  - Visual marks

  - Racons

- AIS data

  - Real,Virtual and synthetic AtoN

- Database matching (imagery, bathymetric, radar, CCTV, etc.)

- e-Pelorus (from e-NAV9/output/19)

  - •	The data captured by triggering the ePelorus (time, position, bearing) should be viewable on a navigation display, and allow the user to allocate a target name and information.

  - •	The electronic bearing line should be selectable for viewing on other displays, including ECDIS and radar.  For navigational objects e.g. AtoN, the user should have the capability to move the (non-shipboard end of the) displayed line in ECDIS and allocate it to a charted object, to provide a cross check of position.  This is the same concept as radar ‘tie point tagging’ as per the input paper.

  - •	The onboard database could also be used to indicate cues of known visual targets onto a visual display.  Taking this idea further would allow the provision of enhanced reality e.g. using an input from the vector chart to display a warning of shallow areas on the visual display.

  - •	Rangefinder functionality using active laser or passive (e.g. image processing) means could also be incorporated.

  - •	Visual images from the ePelorus may be able to be enhanced with low-light or infrared technology. This is considered to have particular benefit in the SAR situation. Reference to the onboard database could provide help in identifying targets or geographic features to the watchkeeper in low visibility conditions.

  - •	Automatic recognition of AtoN daymark characteristics or light character, based on the onboard database.

  - •	Voice-recognition functionality could be used to allocate a target name (e.g. “Lighthouse" or “Fishing boat”) whilst pressing the trigger.

  - •	The plotting of lines of position or relative bearing to other ships also creates a needed auditable record of human intervention that is required under the ISM Code, and by vetting inspectors such as PSC and SIRE.

# training

Body text

# Annex Title

Guidelines should have Annexes.  Appendices are attached to Annexes.

ANNEX HEAD1

Body Text

To restart the Annex Heading numbering at 1:

- Office 2003, go to Format / Bullets and Numbering / Restart numbering (lower left in the box)

- Office 2007, go to down arrow next to Numbering icon and select Set Numbering Value

Annex Heading 2

Body text

Annex Heading 3

Body Text Indent

Annex Heading 4

Body Text Indent 2

- Appendix title

- APPENDIX Heading 1

Followed by Body Text

  - Appendix Heading 2

Followed by Body Text

    - Appendix Heading 3

Followed by Body Text Indent