# ECDIS –Challenges and Concerns

Guy Platten

Northern Lighthouse Board

Scotland United Kingdom

## Introduction

- The development of Electronic Chart Display Information Systems (ECDIS), particularly when part of a modern integrated bridge system, has undoubtedly been a major step forward in marine safety in recent years.  Shipboard personnel are now able to access information quickly and efficiently and so are helped in the overall decision-making process and general management of the vessel.

- With a background in Marine Safety namely search and Rescue, Salvage and now Aids to Navigation provision, the author has attended a number of marine accidents including collision and groundings that were nearly always a result of human error.  A system, such as ECDIS, which helps reduce the workload of mariners by providing real time positional information, and allows them more time to concentrate on safety matters, is obviously therefore to be welcomed.

- However, whilst ECDIS has many advantages, particularly in information presentation and integration with other sensors such as ARPA and Automatic Identification System (AIS), there remain a number of concerns that need to be addressed before its potential can truly be realised.  These concerns include differing chart datums, lack of modern surveys in some parts of the world, vulnerability of GPS and loss of spatial awareness amongst mariners.

## Chart Datums

- ECDIS systems largely rely on GNSS or DGNSS input signals to obtain positional information.  GPS derived positions are expressed in WGS 84 datums and whilst Hydrographic offices are attempting to refer as many charts as possible to WGS 84, there still remain large areas of the world that are charted in a multitude of other datums.  Most manufacturers of GPS receivers now incorporate software within receivers that can convert WGS 84 positions into other chart datums however there must be a conscious effort on the part of the mariner to effect the conversion and this inherently relies on him/her having knowledge of the chart datum in use, not always obvious at first glance with Electronic Charts. In addition there remain many cases where a single conversion will not be accurate for a large regional datum The introduction of ENCs will alleviate the problem however it will be some time before many areas of the world are covered by them.

- When all positional information was derived from reference to visual bearings and radar ranges, or a more localised terrestrial based navigation system, the effect of different chart datums was negligible as such positional information was therefore related to the geodetic datum in use.  The use of ECDIS therefore exacerbates the problem and must be addressed. The rapid introduction of AIS onboard ships could also cause problems and it is conceivable that incorrect positions of vessels may be displayed if different datums are in use.

## Surveys

- The high levels of accuracy of GPS (in the order of 10 metres or less) has meant that mariners now have a higher expectancy of navigational accuracy than can be often supported by survey data found on standard, raster and ENC Charts. Hydrographic surveys are invariably conducted using the best position fixing technology available at the time however this was limited to visual fixing up until the development of terrestrial based and now satellite navigation systems.  Much of the world (particularly in the waters of those countries that are not members of the International Hydrographic Organisation (IHO)) is surveyed to very inaccurate standards reflecting the technology and surveying techniques in existence at the time of survey.  Even around the coast of the United Kingdom there remain areas that have still to be surveyed for the first time.  One such area lies within a few miles of a recognised IMO tanker route.

- The consequence is that, although a modern vessel fitted with ECDIS and GPS may know its position to an accuracy of better than 10 metres, the positions of objects on the seabed may only be known to an accuracy of 20 metres or worse, depending on the age of the latest survey.  Whilst this is true also for vessels navigating with conventional paper charts along with GPS, at least the survey information including dates is readily identifiable allowing the mariner to make a judgement as to how much reliance should be placed on chart information.

## GPS Vulnerability

- Global Positioning System signals are subject to a number of different types of interference, which can seriously degrade or even cause signal loss or false positional information being calculated. This could have potentially very dangerous consequences for ECDIS that relies heavily on the system and could affect the safety of the vessel and mariner.

- Sources include failure of shipboard equipment, although it is accepted that back up systems are now fitted in accordance with the latest requirements.  More worrying is interference from television broadcasts and microwave fixed links, which can be especially serious in harbour approaches, harbours or in a waterway where all vessels can be affected. There have been incidents where all GPS receivers in entire harbours have been blocked and no amount of back up antennas or receivers will be of any use.   Jamming of GNSS signals can also now be achieved quite easily using low cost equipment and there exists equipment capable of jamming on several frequencies at once. There are a number of Internet sites giving details of how to achieve this and it is thought that malicious jamming may start to be used by people in the same way as computer hacking.  Spoofing is also becoming possible and, although at present difficult to achieve, the effects could be more damaging as if the false signals are indistinguishable from the real ones and give a position close enough to be believable then the mariner could easily be led into danger.

- Overall awareness of the problem and developments in future system design, greater transmission power, more diverse operating frequencies and increasing sophistication in receiver design will help to reduce the vulnerability of GPS.  However the vulnerability, particularly deliberate and malicious degradation of service, can not be totally eliminated and it is therefore vital that an effective, compatible backup system to GNSS is developed in order to support GPS dependant systems such as ECDIS.

## Loss of Navigation Skills

11.	Traditional navigational skills appear to sometimes be superseded by new technological advances and whilst ECDIS and Integrated Bridge Systems ease the workload of mariners it can be all too easy to focus on the computer screens and rely on Radionaviagtion inputs to update the vessels position.  It is imperative that mariners do not rely on one sole means of ascertaining position however anecdotal evidence seems to suggest that these skills are in decline (on the vessels the author has been on recently there has been little evidence of alternative means of position verification including by radar and visual means).  The importance of effective Dead Reckoning and Estimating Position cannot be underestimated.  By using the external environment for position verification the mariner is encouraged to develop spatial awareness, vital in an emergency and collision situation.

- It is clear that lighthouses, buoys and beacons will continue to play a important role in a balanced AtoN system, not only to help in situational awareness but also as a backstop navigation system in the event of radionavigation failure.

## Challenges

- This paper is not meant to be negative but merely to highlight ongoing concerns with the use of ECDIS.  Positive steps, however,can be taken to enhance the value of the system and it is hoped that this conference can provide broad support in order to:-

- Encourage and apply pressure on authorities worldwide to continue to move towards a common datum.

- Provide help and support to progress modern surveys around the world.

- Support the back up and maintenance of adequate alternative navigation systems such as LORAN C and Gallileo.

- Ensure that over reliance is not placed on the electronic systems and that mariners are encouraged to navigate by visual means as well (perhaps like Airline Pilots proof of a number of visual as opposed to instrumental navigation should be mandatory for mariners to revalidate Certificates of Competency).  There must be continuing focus, not just on training to use ECDIS but also in retention of basic navigation and seamanship skills.

## Conclusion

14.	There is a very positive future for ECDIS and the development is undoubtedly to be welcomed; however it is vital that these areas of concern are adequately addressed to ensure the safety of the mariner both now and in the future.

Guy Platten

Director of Operations and Navigational Requirements

Northern Lighthouse Board

Scotland

United Kingdom

## References

- IMO Resolution A.572(14), 1985, General Provisions of Ship’ Routeing

- IMO SN/Circ 213, 2000, Guidance on Chart Datums and the accuracy of positions on charts.

- Ward, N Dr, 2003, GNSS Vulnerability and Mitigation Measures.

- John A. Volpe National Transportation Systems Center, 2001, Vulnerability assessment of the Transportation Infrastructure relying on the Global Positioning System.