Blue Lights as aids to navigation in Canada

In Canada, our database shows that there are 27 blue lights in operation. The Canadian Coast Guard currently operates only two blue lights out of a total of 17,300 aids to navigation. Others blue lights are provided by private organizations or other government departments. They are used as wharf lights, shore lights and pier lights.

The two blue lights provided by the Canadian Coast Guard have just been recently installed (April 2003) on a set of ranges near Montréal. This small waterway is used by pleasure boaters and cruise lines. Replacing two green Tideland FA-240 lanterns, blue stripe lights from Vega (VLL-43) were selected and have been in operation for 5 months without any complaints from mariners. The front range light operates at ISO 2s (1.0 sec. Flash, 1.0 sec. Eclipse) as the rear range is a steady light. They both provide a nominal range of 6 nm. The navigation portion of the waterways is 450m to 1100m from the front range. The following table summarises the reasons why blue lights were preferred for this particular location.

|  | Reasons for change | Advantage of blue lights |
| --- | --- | --- |
| Background lighting | As you can see on Figure 1, there are a lot of streetlights at this particular location making the range lights hard to differentiate from the background lightning. | Blue light is easily detected at first glance and therefore, can be differentiated from the ordinary yellow or white background lights. |
| Tower height | Since the towers of the ranges are not very high, the range lights are almost at the focal height as the streetlights. |  |
| Color of the light | Even though green range lights were used before, they appeared as yellow lights from the waterway, the same color as the street lights Furthermore, two streetlights installed under the bridge behind the ranges are amber and are located in a way that they can be perceived as range lights. |  |
| Shape | Ordinary range lights and streetlights all have a round shape. | The lantern used from Vega has a line shape. From the waterway, the light has an ellipsoidal shape but even with that, it is easier to distinguish from the street or the bridge lights. |
| Effective range of lights (Visibility) | Range and streetlights had a similar effective range. | At this particular location, blue lights appear to offer a longer effective range to the mariners. |

Figure 1 - Blue Lights installed on a set of ranges

# Figure 2 – Closer look at blue lights