Members were asked to present their experiences or their views on alternative service delivery such as contracting in/and or out and provide input papers to ANM3

CONTRACTING OUT – ARGENTINE EXPERIENCE AND GENERAL RECOMMENDATIONS

# GENERAL

We are going to analyze the case in which a Public Administration plans to contract a private firm to take care of the entire job related to the Aids to Navigation (AtoN) at a specific area. This is not an easy solution to adopt and has a lot of resistance mainly from the people actually involved in the job. With careful planning and taking into consideration the interest of all parties involved a solution can be find that improves the service and reduces costs. In this document in the first part it is described the experience done in Argentina on this subject and in the second part general comments on the best way of conducting such an experience are given.

# ARGENTINE´S EXPERIENCE

After several years of studies, consultation and discussions with different organizations related with shipping activities, in 1993 the Undersecretary of Ports and Waterways made an international tender for selecting a contractor to take care of the dredging and Aids to Navigation of an area in Argentina.   A copy of the relevant part of Terms of Reference (in Spanish) is available to those who could be interested.

The opening of bids was done on January 1994, the contract was signed in February 1995 and the works began in May 1995. The duration of the contract was originally of ten years and was extended afterwards for eight years more.

In the following points a short description about the requirements included in the TOR is made

## Object

The object of the contract was to modernize existing AtoN already installed in the system, to reinstall missing AtoN, to complete the system with necessary AtoN for safe navigation and to do the maintenance of the AtoN system during the period of the concession.

## Area

The area in consideration is indicated in the figure. Adding up the different channels, it comprises a total length of approximately 1.000 km

## Basic system

Appendix 6 of Terms of Reference included a list of minimum AtoN to install but with the concept that it should be installed as many AtoN as necessary by navigation requirements.

The total quantity of signals to have in the system according to Annex 6 was 718 of which only 268 existed at the beginning of the contract (May 1995).

The condition of the existing AtoN was not good. Of the 268 existing signals 80 were considered in fair condition, 161 in regular condition and 27 in bad condition

The required quantity of AtoN as per May 2003 is 758

Therefore the work to be done included new 490 signals between buoys and beacons, 7 Racons, 24 water level measuring and transmitting stations and the refurbishing of 268 existing signals

## Stages

The work was to be done in different stages. For stages 0, 1 and 2 a total time of 15 months was awarded. In this time all the system has to be installed. The rest of the time till completing the 18-year period was for maintenance.

Taking in account the numbers of new buoys and beacons to install it gives an average of 33  AtoN per month.

## Wrecks

All wrecks in the area should be located and signalized. All new danger should be signalized with one or two lighted buoys in less than a 12-hour period.  In case of impossibility to do so, a ship should remain in place. Number of wrecks in the area was 84. Some of them had a known position. Others were to be search by different means.

## Controls

Once a week HIDROVIA SA must make a survey of all AtoN informing to the Client all news related.

## Buoy tenders

The Client lent for their use to HIDROVIA SA the buoy tenders that he had used in the area.

## Personnel

In the TOR it was promoted to take personnel from the public administration to work in the concession. It was established an optimum number of people according to the Client that should be taken by the Concessionary. In case of not taking this number of people there were a discount in money in the payments of the Client.

## Technical specifications

Technical specifications for the AtoN

were given in excessive detail by the Client including focal height, type of buoys, luminous intensity. IALA recommendations were to be followed.

## End of the concession

At the end of the concession all AtoN should be delivered to the Client in goods conditions established in the TOR. For example, all buoy hulls should have been cleaned and repainted in the yard in the previous two years; all mooring systems should have been changed in the previous two years; etc

## Penalization system

In case of having AtoN not fulfilling the optimal conditions a penalization system was established.

- The contract establishes a formula to control that the aids to navigation system fulfill a quality criteria. Each main marker characteristic has been given a value and when this characteristic is not in order, it is called “an abnormality” for this aid to navigation. The Client makes daily and nightly inspections to determine the abnormalities of the system. Each abnormality has a value. This value is adjusted depending on the function of the aid to navigation. For example, “isolated danger” has a factor of 0,75 and “lateral” has a factor of 0,25. The addition of the adjusted abnormalities divided by the number of aids to navigation ought to be less than 3% for each section of the channel. Otherwise, there are severe penalties for the firm.

- Abnormalities and their values as defined by the Terms of Reference are:

- - Complete buoy missing				3

- - Light off						2

- - Out of position					2,5

- - Weak light						1,5

- - Wrong flashing					1 to 3

- - Discolored						1,5

- - Radar reflector missing				2

- - Tide lantern missing				1,5

- - RACON missing					2

- - RACON off					2

- - Topmark missing					1,5

- - Legends missing					1

- - Out of vertical					2

- - Half-submerged					2,

- - Without superstructure				2,5

- - Lantern missing					2

- - Aids to navigation not visible due to vegetation	2

- This formula was not applied before when the Government managed the system and therefore it was not previously tested. In practice, it results in a very exigent condition.

- The time to put the aids to navigation system in order once exceeded the allowed 3% is 48 hours.

## Payments

At the beginning of the contract the payment for the works was established as a fixed amount from the Client plus a toll to the passing ships according to a formula that considers the NRT of the ships.

After 7 years of contract all the payment passed to the toll of passing ships.

In some way it could be considered a lump sum contract where all responsibilities are in the side of the Contractor.

The toll is variable in function of the distance traveled by the ships. There are four sections defined from the Ocean to the upper end of the river.

At this moment the amount to be paid for a ship of given Net Register Tonnage (NRT) is

Toll = 0,0837 x 1,65  x NRT in American dollars

Where

T  	= toll to be paid

0,0837 = percentage to be applied for the section from the Ocean till Rosario. It means approximately 600 km of waterway

1,65 u$s/NRT	= approved tariff from the Client

As an example, for a ship of 12.000 NRT, the formula gives a value of u$s 1657. It means 2.76 u$s/km.

# COMMENTS

Based on the experience of HIDROVIA SA general comments are made on the aspects that should be taken in consideration in the process of passing the responsibility of a public AtoN service to a private company.

## Object of the contract

The object of the contract is to be very well specified and should consist on operational targets only. All future daily work planning should remain in the Contractor side.

## Experience

It is not easy to have firms with experience in maintaining AtoN services. At least it should be required that expert people should be in charge of the operations.

## Extension of the work

This is a very important point. The number of AtoN and the extension of the area to be attended by the new Company should have a minimum size. This is related to the number of buoy tenders that should have the Company. Minimum number is one buoy tender but to cope with time for repairs, classification and other non-working time it is very convenient that the work could justify two buoy tenders. When the work does not require one full buoy tender it is not easy to find additional work for this ship resulting in higher costs for the service. The same applies to the number of AtoN. There is a minimum number that justifies having enough spare parts of everything and competent people in the yard.

## Duration of the contract

There are many decisions that depend on the duration of the contract. It concerns people, ships and material. The duration of the contract should justify the investment in good quality elements for the AtoNs. It means at least a 5-year contract, being a 10-year period to be worth considering. Also there is the influence of the environmental factors. For the evaluation of the maintenance costs it should be taken into consideration that meteorological conditions are not the same year after year. The evaluation for shorter periods implies that worse conditions are considered always implying higher costs. With longer periods the bad years are somewhat compensated with the good ones.

## Type of contract

A lump sum contract with a strict control from the Client avoids a lot of discussions. If the objectives are clear and well defined the Contractor has no excuses to fulfill the objectives.

It is very difficult to control all aspects related to the good maintenance of an AtoN. And it requires a lot of people from the side of the Client and this is just the contrary of the basic idea of contracting a third Company to do the job. Therefore the best way of having good results and not having permanent discussions is that all responsibility for the AtoN is on the side of the Contractor and the money paid in any way for the job is a fixed amount. In this case the control activities can be very specific and can be done with few very capable people. On the other side, the Client knows in advance how much money is going to spend.

## Control

Control activities from the Client should be enough to guaranty that the works are done according to specifications but no exceeding the necessary frequencies. In general the Client tends to make a control much stricter of what he has done with himself during the previous years, also asking for aspects not previously considered in the AtoN service. The control system has to do with the penalization system adopted. .

## Buoy tenders

It is very interesting and convenient to find a formula for letting the Contractor use the buoy tenders of the Client if possible. The Contractor should have the obligation of keeping the buoy tenders in good condition and under class and the Client should have a good control schedule of the conditions of the ships. If not used the buoy tenders should return to the Client. The buoy tenders should not be used in other activities out of the AtoN service without the written permission of the Client. This solution saves a lot of money from the Contractor and from the Client. From the Contractor because is not easy to find appropriate ships for this type of job. For the Client because he has not any more work for the ship and in this way preserves the possibility of returning later to the previous system.

## Personnel

To solve adequately the problem of personnel is a condition for the successful result of the pass from the public administration to the private sector. One adequate way of doing this is to promote the incorporation of personnel from the administration in the new firm.

We can divide the personnel in four sectors: a) mariners and technical staff on buoy tenders, b) technical people at the yard, c) technical people at office and d) managerial sectors.  In general it is possible to include as condition in the TOR the inclusion of an important quantity of people from sectors a) and b) and a minimum from sector c). People from sectors c) and d) are necessary for the control of the contract

## Technical specifications

In normal circumstances the AtoN system is already installed in reasonably good state of maintenance. It is natural for the people working in that system to consider it the best solution achievable. Nevertheless, there are always fields where new technologies are emerging that could be worth use, trying to improve the quality or the economic equation of the system.

The Client should specify clearly the objectives to be obtained with the AtoN system but leaving the implementation of the same, with the adequate discussions and approvals, to the Contractor. In a 10-year contract it is reasonable to leave open the possibility of introducing changes to the established way of doing things.

## End of the concession

The technical conditions at which the AtoN should be returned should be clearly specified in the TOR and they should be reasonable. For example, the buoys of the system are taken out of the water to receive full clean up, reparation and painting each a number of years. If it is established in the TOR that at the time of returning the system  all buoys should have received such treatment in a period not exceeding one or two years of the date of ending of the Concession it puts a very difficult to fulfill obligation on the Contractor.

## Penalization system

The Administration is willingly to ask for quality conditions better than it has obtained during the years that the system was under his responsibility.

Percentages established in Navguide should be demanded

As IALA´s recommendation establishes a minimum period of two year to determine the percentages and these percentages are also a function of the time used for the calculation it should be studied carefully a way of estimating such percentages in the first two years.

It is also convenient that the Administration makes some calculations of the actual percentages that the AtoN system has had in the last ten years. Therefore it can be established the required level of the system according to the real needs of the mariners. Higher levels of service are always obtainable but that involves always higher costs.

IALA has not a recommendation as for the time to repair an AtoN out of service. Time to repair should be according to local circumstances but always reasonable. It should take into consideration environmental factors as well.

## Payments – Income

It is worth analyzing the possibility of putting part or total cost of the work on passing ships through a toll. It depends on each country legislation and uses.

When passing ships are involved in the cost of maintaining the AtoN there is an additional control on the quality of the system from the passing ships and pilots.

## Technical Committee

It is a very good practice to establish a forum where the different parties involved can meet regularly. This place should serve to discuss the actual condition of the AtoN system, future plans and related issues. To these meetings it should assist representatives of the Client, the Contractor, Coast Guard, Pilots, and other bodies with interest in navigation. The meetings allow making minor corrections to some AtoN, like position, flashing characteristics or others. Sometimes important aspects arise like marking a recent wreck. Usually pilots have an opinion on these topics and it is very helpful to give them a place where to express their ideas. It has to be established in the TOR as an obligation for both parties of the contract and establishing which institutions are going to form part of this body. One of the important objectives of this Technical Committee is to establish a good relationship between the different people involved and to develop mechanisms to affront difficult situations.

# FINAL REMARK

The change of system from a public administration to a private one in the subject of AtoN is not easy and involves technical, economic, social and political decisions. This process is usually long and should be done very carefully to avoid unnecessary drawbacks.

The subject is very much dependent on case by case due to local geographical conditions, applicable laws, general political strategy, and sometimes to a lot of non technical issues.

The ideas given previously reflect the experience gained in a 10 year successful process on this subject.