| From: AIS (AIS13) | ## AIS13/output/09 |
| --- | --- |
| #### To: RNAV (RNAV19); EEP (EEP3) and ANM (ANM3) | ## Sept. 04, 2003 |

Liaison Note

from the IALA AIS Committee to the RNAV, EEP and ANM Committee

Marking of Wind Farms

Using Message #21 for wind farms/ wind turbines

# Introduction

The AIS Committee considered the issue In response to a request from the ANM Committee to use AIS message #21 to identify wind farm areas.

At AIS13 the Committee concluded that the AIS message 21 should not be used to mark areas which, in the case of wind farms, would contain more than one object (wind turbine). However, AIS message #21 may be used to mark an individual wind turbine as an “offshore structure”. Therefore, the use of AIS message #21 is hereby clarified in accordance with this conclusion.

There are a number of reasons that led to this decision:

- The operational purpose of the AIS is essentially to identify objects, not areas. Therefore, an individual wind turbine may be identified as an “offshore structure” by using message #21.

- The AIS does not, and should not, provide a replacement for navigational charts and radar. Both of which would show each individual turbine within a wind farm or an area. The scope of the AIS should not be over-extended.

- Message#21 is not capable of designating an area shape other than a circle or rectangle. This would not be appropriate for most wind farms already established or being planned. Message #21 cannot relay data that describes a polygon.

There is concern that radio frequency interference may occur in the vicinity of operational wind turbines, due to reflected signals or electro-magnetic effects, resulting in reduced performance of AIS or preventing its operation completely.

# Action Requested

Based on the forgoing it is recommended that, if deemed appropriate and found free of interference, AIS may be used to mark only the most significant individual wind turbines of a wind farm, e.g. those wind turbines at a corner position(s), or at the change of direction of a line (or row) of wind turbines, by utilising AIS message #21.

It is also recommended that planning guidance be drafted by the AIS Committee to be included in the appropriate chapter on AIS for Aids to Navigation in Volume. 1, part 1 of the IALA Guidelines on AIS, and that the ANM Committee include the recommendation contained in this liaison note in a future revision of the IALA Recommendation 0-117 on the Marking of Offshore Wind Farms.

DRAFT FOR INCLUSION IN THE IALA GUIDELINES ON AUTOMATIC IDENTIFICATION SYSTEMS    Volume 1 Part 1

Section 4.1.5.1 Use of AIS as an Aid to Navigation (AtoN).

Add paragraph:

Applications of AIS as an AtoN include the marking of dangerous wrecks and offshore structures. In the case of areas containing multiple structures, such as wind farms, only the extremities would be marked using AIS.