Recommandation pour la signalisation des

AISM Association Internationale de Signalisation Maritime      IALA International Association of Marine Aids to Navigation and Lighthouse Authorities

champs d’éoliennes

Recommandation AISM O-117

Mai 2000

(traduction française en cours)

# Recommendation for the marking of

Offshore Wind Farms

## IALA Recommendation O-117

May 2000

### DRAFT REVISION – OCT 2003

20ter, rue Schnapper – 78100 Saint Germain en Laye – France

Telephone : +33 1 34 51 70 01  Telefax : +33 1 34 51 82 05

E-mail : aismiala@easynet.fr              Internet : http://www.iala-aism.org

THE COUNCIL

RECOGNISNG the increase in the number of areas with multiple wind generators (wind farms) being established and the danger posed to shipping [water born craft],

RECOGNISNG ALSO that it is a matter for a National Authority to decide on whether and/or how wind farm need to be marked, depending on the risk involved,

RECOGNISNG FURTHER that marking is to improve the safety of navigation and protect the wind generators themselves,

HAVING CONSIDERED the proposals by the IALA ANM Committee,

RECOMMENDS

Offshore Wind Farms should be marked so as to be conspicuous by day and night with consideration given to prevailing conditions of visibility and vessel traffic. The Council recognises that there are already sufficient recommendations contained within the IALA Maritime Buoyage System to fully cover the marking of wind generators[turbines] and wind farms.  It is usual that multiple wind generators are established in close proximity. These should be contained within a defined block in order for the wind farm to be marked as a unit as follows:

- The corners and other points where there is a change of direction of the boundary of the windpark should be marked by sufficient flashing yellow lights so as to be visible from all directions in the horizontal plane and with a nominal range of at least five (5) miles with a character as for a "special mark."   In extended blocks, a flashing yellow light of different character, and which may have a different range, should be used to mark other wind generators along the perimeter or within the interior of a block of wind generators. The lateral intervals for such wind generators should not exceed two (2) miles.

- Depending on[if] whether an Exclusion Zone or a Safety Zone is declared and depending on the separation of individual turbines, Administrations may require each individual turbine to be lit.

- The lights described herein should have an availability of better than 99.0 %.

- The lights shall be mounted at a height of 15 metres above the level of the Highest Astronomical Tide (HAT), but below the lowest point of the arc of the rotor [turbine] blade.

- The lights on a tower must cover 360 degrees in the horizontal plane which will require disposition of 3 separate lights. These lights must be syncronised.

- The tower of every wind generator shall be painted yellow all round from HAT to at least the height of the lights.

- Consideration should be given to the use of retro-reflective materials on the towers.

- Consideration should be given to the use of Racons, Radar Reflectors, Radar and/or AIS Target Enhancers in recognition of the poor radar conspicuity of some vertical cylindrical structures.

- Consideration may be given to the provision of sound signals when appropriate,  given prevailing conditions of visibility, topography and traffic.

- If a single (stand alone) wind generator is constructed, its daymarking should be in accordance with the above recommendation. However, due to the increased danger posed by such an isolated structure it should be lighted as any other offshore structure (i.e. a white light with flashing morse code "U").

- Electrical transformer stations and metrological or wind measuring masts if considered to be within the windpark block, may be marked as part of the overall windpark marking. Otherwise it should be marked as an isolated structure. (white light, Mo Code“U”)

- Aero lights fitted to the top of wind turbines should not be visible below the horizontal plane.

- Where multiple wind turbines are located in a wind farm, consideration should be given to the synchronization of lights.

Annex 1

Notes for National authorities:

      - During the construction of a wind farm, working areas should be established and marked in accordance with the IALA Maritime Bouyage System (MBS).

      - The towers of windfarms should not be utilised as  individual IALA marks.

      - Notice to Mariners and or Local Radio Navigational Warnings must be promulgated in advance of and during any offshore wind generator construction.

      - Mariner should be warned on the navigational chart that wind generators and seabed electric cables may cause propagation problems and electro-magnetic interference in onboard navigation equipment.

      - Ideally, power cables from wind generators should be trenched.

      - National Authorities may consider establishing safety zones or prohibiting, excluding or restricting vessels and craft from entering wind farms blocks on a case-by-case basis.  If necessary, such areas should be marked in accordance with the IALA Maritime Buoyage System and shown on the navigation chart as appropriate.

.

- Red/white aero light must not show below the horizontal plane of the light. Fixed/ Flashing?  Germany has reduced the aero candella from 2000 to 150 why?

- Illustrate document with ideal daylight marking.

- Is there any issue with sea surface access to tower and /or helicopter access to nacelle?

# IALA Recommendation on the marking of offshore wind farms

Lights to be mounted at a height of 15 meters above the level of Highest Astronomical Tide (HAT)

Lights to be visible 360 degrees and synchronised (minimum of 3 lights)