| From: EEP | EEP3 / output/ 03 |
| --- | --- |
| To: ANM | Date: Oct. 10, 2003 |

Liaison Note

THE COLOURS OF SIGNALS FOR AIDS TO NAVIGATION

- Introduction

The ANM Committee was asked to review Recommendation E-108, Surface Colours used as Visual Signals on Aids to Navigation.  In the process of review it was noted that the Recommendation requires updating with CIE publications.  This resulted in a liaison note to the EEP Committee (ANM1/output/04).  The EEP Committee reviewed the output and provided further information to the ANM Committee in liaison note EEP2/output/09).  The ANM Committee is continuing to work with the EEP Committee on this review through comments to the EEP Committee on the Colour of Signals for Aids to Navigation, with specific reference to the use of Blue Lights.

- Background

It is recognized that some authorities find it useful to use blue lights in circumstances where the conspicuity of the blue light is advantageous, e.g. when there may be non-navigational background lights that use regular IALA colours.

Should IALA decide to include blue in navigational light colours, then the Class A chromaticity area specified by CIE should be used, while taking into consideration the note in CIE S004/E-2001 Section 4.7 on blue light signal colours:

“Blue is not a suitable colour for long distance visual signals because:

- The eye is relatively insensitive to blue light

- Blue and green are confused by normal observers for signals that have small angular subtense or a low illuminance

- Blue is preferentially scattered by the atmosphere.”

This indicates that the use of Blue Lights is suitable only for short range AtoN use.  The EEP Committee also notes that the light characteristic should be selected to ensure there is no confusion with the lights of emergency vessels and vehicles.

- Action Requested

The ANM Committee is asked to take the comments on Blue Lights into account in future discussions on the use of Blue Lights.