ANM3 / 7 / info1

AIS13 / output / 01

DRAFT

IALA RECOMMENDATION

ON

AIS for AIDS TO NAVIGATION

[IALA Recommendation A-126]

Edition 1.0

03-09-2003

Table of Contents

1	Introduction	2

2	Background	3

3	Application of AIS to Aids to Navigation	4

4	Implementation methods	5

4.1	AtoN AIS	5

4.2	Synthetic AIS	5

4.3	Virtual AtoN AIS	6

4.4	MMSI numbers for Synthetic and Virtual AtoN AIS	6

5	AIS Messages for Use with Aids to Navigation	6

5.1	Message 21, Aids to Navigation Report	7

5.2	Message 12, Safety Related Addressed Message	9

5.3	Message 8, Binary Broadcast Message	9

5.4	Message 6, Addressed Binary Message	9

6	AIS on Floating Aids to Navigation	9

7	AIS on Fixed Aids to Navigation	10

8	AIS on Offshore structures	10

9	Radar Reference Target	10

10	The AtoN AIS station	10

10.1	Achieving the functionality of an AtoN AIS station	10

10.2	Autonomous, continuous system	10

10.3	Autonomous, non-continuous system	12

10.4	Assigned time slot system	13

11	References	15

- Introduction

The International Association of Marine Aids to Navigation and Lighthouse Authorities (IALA) define an Aid to Navigation (AtoN) as a device or system external to vessels that is designed and operated to enhance the safe and efficient navigation of vessels and/or vessel traffic.

A special type of an Automatic Identification System (AIS) station fitted to an Aid to Navigation (AtoN AIS station) could provide a positive identification of the aid In addition, this equipment can provide information and data that would, amongst other things:

- Complement an existing aid to navigation, providing identity, and additional information such as real tidal height and local weather to surrounding ships or back to a shore authority;

- Provide the position of floating aids (mainly buoys) by transmitting an accurate position (corrected by DGNSS) to monitor if they are on station;

- Provide real-time information for performance monitoring, including state of ‘health’.

The International Telecommunications Union (ITU) has recognized the potential of the AIS also for areas of shore-based application, other than ship reporting and VTS, namely maritime, safety-related information services, Aids-to-Navigation and Search and Rescue.

As a result provision has been made within AIS for an Aids to Navigation Report (Message 21). AtoN AIS will enable aids to navigation providers to broadcast information on the:

- Type of aid to navigation,

- Name of the aid to navigation;

- Position;

- Position accuracy indicator;

- Type of position fixing device;

- Time stamp;

- Dimension of the aid to navigation and reference positions;

- Bits reserved for use by the regional/local aids to navigation providers (can include the technical status of the aid to navigation);

- Virtual aid to navigation flag.

When a floating aid is out of position or malfunctioning, navigational warnings must be given. Therefore, an AtoN AIS station, which transmits Message 21, could also transmit Safety Related Addressed Message (Message 12) upon detecting that the floating Aid-to-Navigation has gone out of position or is malfunctioning,

In some cases, for practical or economic reasons it may not be appropriate to fit an AIS unit to an AtoN and the synthetic-AIS approach may be taken. A ‘monitored’ synthetic- AtoN AIS consists of a GNSS receiver, processor, and a non-AIS data-link, for example packet radio or GSM.  The data is received ashore where it is converted into AIS format and re-broadcast by AIS shore infrastructure or by an AIS AtoN station as if it came from an AIS unit on the AtoN.  In the case that this method is applied to fixed aids, the monitoring may be omitted and the AIS AtoN would then be called ‘predicted’ synthetic AIS AtoN.

In other cases it may be appropriate to create an apparent (‘virtual’) AtoN on a display, where none exists. In this case the symbol would appear on the display for a certain location, even though there is no physical AtoN there. The associated information would clearly identify this as a virtual AtoN and safeguards would be needed against false representations. There are some cases where virtual AtoN could be very useful, for example marking a new wreck until a buoy can be established.

Where AIS is provided from a physical AtoN, or a synthetic AIS approach is taken, sensors for environmental data may be provided, if required.

- Background

Automatic Identification System (AIS) is an autonomous and continuous broadcast system, operating in the VHF maritime mobile band.  It exchanges information such as vessel identification, position, course, speed, etc. between participating parties and shore.  It can handle multiple reports at rapid update rates and uses Time Division Multiple Access (TDMA) technology to meet these high broadcast rates, ensuring reliable and robust operation.

Chapter V of the 1974 SOLAS Convention (as amended) requires mandatory carriage of Automatic Identification System (AIS) equipment on all vessels constructed on or after 01 July 2002.  Implementation for other types and sizes of SOLAS Convention vessels is continuing and will be completed not later than 31 December 2004.

SOLAS Chapter V, Regulation 19, section 2.4.5 states that:

"AIS shall

- provide automatically to appropriately equipped shore stations, other ships and aircraft information, including ship's identity, type, position, course, speed, navigational status and other safety-related information;

- receive automatically such information from similarly fitted ships;

- monitor and track ships; and

- exchange data with shore-based facilities."

In addition, the IMO Performance Standards for AIS state:

"1.2	The AIS should improve the safety of navigation by assisting in the efficient navigation of ships, protection of the environment, and operation of Vessel Traffic Services (VTS), by satisfying the following functional requirements:

.1	in a ship-to-ship mode for collision avoidance;

.2	as a means for littoral States to obtain information about a ship and its cargo; and

.3	as a VTS tool, i. e. ship-to-shore (traffic management).

  - The AIS should be capable of providing to ships and to competent authorities, information from the ship, automatically and with the required accuracy and frequency, to facilitate accurate tracking. Transmission of the data should be with the minimum involvement of ship's personnel and with a high level of availability.”

AIS can also be applied to Aids to Navigation (AtoN) to further improve and enhance services provided to mariners.  The purpose of this document is to provide recommendations and guidance for the use of AIS in this field.

- Application of AIS to Aids to Navigation

The application of AIS to AtoNs is a significant use of the AIS, and ITU has defined AIS Message 21 for exclusive use with AtoNs. Other messages may be used with AtoNs and may be referenced in ITU-R M.1371-1.

The primary purpose of applying AIS to AtoNs is to promote and enhance navigation safety and efficiency by one or more of the following:

- Providing a positive and all-weather means of identifying an AtoN on AIS and ships’ radar displays

- Complementing existing signals from AtoNs

- Transmitting accurate positions of floating aids (possibly corrected by DGNSS)

- Indicating if a floating AtoN is off station

- Providing reference points for a ship’s radar

- A complement to, racons

- Providing virtual AtoNs

- Marking or delineating Tracks, Routes, Areas, and Limits (for example Areas To Be Avoided and Traffic Separation Schemes)

- Marking offshore structures (for example wind turbines, oil and gas platforms)

- Providing weather, tidal, and sea state data.

A further set of benefits includes the following:

- Monitoring of AtoNs status

- Tracking of a drifting floating AtoN

- Identifying ships involved in collisions with AtoNs

- Gathering real-time information for condition monitoring

- Remotely controlling changes in AtoN parameters.

AIS may be applied to both floating and fixed AtoNs, and more than one AIS message format may be transmitted as noted above. The service provider has an obligation to verify the broadcast information and the correct operation of the AIS for AtoN.

An authority planning the application of AIS to AtoN, should bear in mind the message capacity of the VDL, and avoid unnecessary load, for example by considering selection of data to be disseminated

- Implementation methods

AIS messages for an AtoN may be generated from information derived from the aid itself, and broadcast directly from the aid, or may be broadcast from an AIS unit not located at the AtoN.

These should be referred to by the following terms:

Physical Aid to Navigation

- AtoN AIS

  - Where the aid is equipped with an AIS Station designed to generate the appropriate AIS messages using local data from the aid

- Synthetic AIS

  - Where the AIS message for the AtoN is transmitted from another location and the AtoN is physically located at the position given in the AIS message.

Non-Physical Aid to Navigation

- Virtual AtoN AIS

  - Where the AIS message is an AtoN message but no real aid exists at the location indicated in the AIS message.

  - AtoN AIS

A “Real AIS AtoN” is transmitted as a message 21 from an AIS AtoN Station that is physically located on the AtoN”,

  - Synthetic AIS

For practical or economic reasons it may not be appropriate to fit an AIS unit to an AtoN and the Synthetic AIS approach may be taken.  There are 2 types of Synthetic AIS AtoN, “Monitored Synthetic AIS AtoN”  and “Predicted Synthetic AIS AtoN”.

“Monitored Synthetic AIS AtoN”

A “Monitored Synthetic AIS AtoN” is transmitted as a message 21 from an AIS Station that is located remotely from the AtoN.  The AtoN physically exists and there is a communication link between the AIS Station and the AtoN.  The communication between the Aid and AIS shall confirm the location and status of the AtoN.

“Predicted Synthetic AIS AtoN”

A “Predicted Synthetic AIS AtoN”  is transmitted as a message 21 from an AIS Station that is located remotely from the AtoN.  The AtoN physically exists but the AtoN is not monitored to confirm its location or status.

Note:  Only Monitored Synthetic AIS AtoN ensures the integrity of the floating AtoN, hence Predicted Synthetic AIS AtoN is not recommended for this application i.e. floating. Predicted Synthetic AIS AtoN broadcast for fixed AtoNs is acceptable as the location will not change but the status of the AtoN is not verified.

  - Virtual AtoN AIS

A “Virtual AIS AtoN” is transmitted as a message 21 for an AtoN that does not physically exist.

It may be appropriate to create a virtual AtoN on a display, where none exists. In this case the symbol would appear on the display for a certain location, even though there is no AtoN there.  This message could be broadcast by a nearby Base Station or AtoN Station. The flag in Message 21 would clearly identify this as a Virtual AtoN. There are some cases where Virtual AtoNs could be useful, for example marking hazards to navigation on a temporary basis, until permanent aids can be established.

As with all AtoN AIS, a virtual AIS should only be established, or approved, by the competent authority for that area, taking account the needs of non-AIS-equipped craft.

  - MMSI numbers for Synthetic and Virtual AtoN AIS

Message 21 must include an MMSI number if it is broadcast by an AtoN Station. For Synthetic or Virtual AtoN targets the MMSI field should contain the MMSI of the AtoN Station or Base Station that is generating the target.

When generating synthetic or virtual AtoN targets the MMSI should be the transmitting station’s MMSI. In order to discriminate between the individual synthetic or virtual AtoN targets, the name field should be used as the unique alpha-numeric identifier. The receiving application should be able to assemble synthetic or virtual AtoN targets’ identities by combining the MMSI and the name fields. For Real AtoN targets only the MMSI should be sufficient.

- AIS Messages for Use with Aids to Navigation

The following AIS messages, as defined by ITU, Technical Characteristics for a Universal Automatic Identification System Using Time Division Multiple Access in the VHF Maritime Mobile Band, ITU-R M.1371-1 (Reference 1) may be applied to AIS for AtoN.

- Message 21, AtoNs Message

  - Sent for use by vessels within range and by VTS centres

- Message 12, Addressed Safety Related Message.

- Where a Message is to be sent from the AtoN, the service provider must ensure that the Message is only sent when the relevant condition has been met [e.g. buoy off station] and it must be an addressed message sent only to vessels in the affected area and only sent until acknowledged. This is best achieved using Message 12. Broadcast Message 14 should not be used.

- Message 8, Binary broadcast message, as approved for international, or regional use, for example

  - Weather, wave, tide, sea data.

  - Tracks, Routes, Areas, and Limits (for example Areas To Be Avoided and Traffic Separation Schemes)

- Message 6, Binary addressed message

  - For example, local use for AtoN monitoring data

When connected to an AIS unit, external GPS/DGPS receivers should be configured to transmit co-ordinates in WGS84 datum. Failure to do so can result in misleading information being transmitted to the mariner.

  - Message 21, Aids to Navigation Report

Reference 1 paragraph 3.3.8.2.17 states that “This message should be transmitted autonomously at a reporting rate of once every three (3) minutes or it may be assigned by an Assigned Mode Command (Message 16) via the VHF data link, or by an external command.”

The default interval between transmissions of Message 21 is 3 minutes, but this may be increased or decreased at the discretion of the service provider, bearing in mind the capacity of the VDL and perhaps power consumption at the AtoN. The main contents of Message 21, which is defined elsewhere, are:

- Type of AtoN;

- Name of the AtoN;

- Position;

- Position accuracy indicator;

- RAIM indicator;

- Type of position fixing device;

- Time stamp;

- Off position indicator

- Dimension of the AtoN and reference positions;

- 8 bits reserved for use by the regional/local authority (can include the technical status of the aid to navigation);

- Virtual AtoN target flag.

The Details of the composition of Message 21 are defined by Reference 1, and clarified by “IALA Technical Clarifications on Recommendation ITU-R M.1371-1” (Reference 2). Refer to Annex.

    - Message 21, Configuration

- The position derived from an EPFS (Electronic Position Fixing System) can be used in conjunction with a reference position and a ‘guard ring’ to monitor the position of the floating AtoNs and to generate an ‘off station’ condition. This would be indicated in the AtoN AIS message and also be transmitted as a navigational warning message in the form of a Safety Related text message (message 12).

- When assigning the “Name of AtoN” field, the following priority should be used:

  - Charted name

  - National or international identification number

  - Description of special characteristics. (This content is up to the service provider, but examples are light flash character, range in nautical miles, elevation in metres.)

If more than one of a, b, c are used, they should be separated by comma(s).

- All positions should be given in WGS84 coordinates.

- The “Dimension/reference for position AtoN field”

    - This field should indicate the “dimension/reference for position” parameter of the AtoN object and not the dimensions of the area in which a floating aid can move (guard zone) or dimensions of a ”dangerous zone” around the AtoN.

    - For fixed AtoN a numeric value should be used as noted in the table below. The orientations established by the dimensions A, B, C & D should face true north, south, west & east respectively.  By setting A and C to zero, the reference point becomes the north-west corner.

    - For floating aids larger than 2 m x 2 m the dimensions of the AtoN should always be given as a circle, i.e. the dimensions should always be as follows: A=B=C=D>1. (This is due to the fact that an orientation of the floating aid is not transmitted.)

    - For floating objects smaller than or equal to 2 m x 2 m the values of the fields should be set to A=B=C=D=1.

    - When transmitting virtual AtoN information, i.e. the virtual AtoN flag is set to one (1), the dimension should be set to A=B=C=D=0 (=default). This should also be the case, when Type of AtoN is set to ”reference point”.

    - Off shore structures that are not fixed, such as rigs, should be considered as Code 31 type from table 34 AtoNs from ITU-R M.1371-1.  These structures shall have their “Dimension/reference for position” parameter as determined above in note iii.

    - Fixed off shore structures, Code 3 type from table 34, shall have their “Dimension/reference for position” parameter as determined above in note iii. Hence, all off shore AtoNs and structures have the dimension determined in the same manner and the actual dimensions are contained in message 21.

| Dimension/reference for position, for a fixed AtoN |  |
| --- | --- |
|  | Numeric |
| A | 0 |
| B | 2 |
| C | 0 |
| D | 1 |

| Dimensions for a floating AtoN and both fixed and floating off shore structures (table 34bis Codes 3 and 31) |  |
| --- | --- |
| A+B ≤ 2m | A=B=C=D=1 |
| A+B > 2m | A=B=C=D>1 |

    - Message 21, Data bits reserved for regional or local application

The eight data bits “Reserved for regional or local application” are defined at the regional or local level. If these bits are used for AtoN system monitoring, they could be used to address the following parameters in the manner indicated. The contents of Table 1 are offered as an example and the exact use is to be determined  by the Aids to Navigation  Authority

Table 1

| Bit number | Meaning |
| --- | --- |
| 0&1 | 00 = Lantern off (This is Day normal condition) 01 = Lantern on (This is Night normal condition) 10 = Lantern off (Failed condition) 11 = Lantern on (Any faulty condition i.e. reduced range, wrong character) |
| 2 | 0=battery voltage OK, 1 = low battery voltage |
| 3 | 0=racon OK, 1=racon alarm |
| 4 |  |
| 5 |  |
| 6 |  |
| 7 |  |

  - Message 12, Safety Related Addressed Message

An AtoN AIS station, which transmits Message 21, could also transmit at the competent authority's discretion Safety Related Addressed Message (Message 12, text message), for example, on detecting that a floating Aid-to-Navigation has gone out of position or is malfunctioning.

  - Message 8, Binary Broadcast Message

An AtoN AIS could broadcast a Message 8, Broadcast binary message, carrying information about conditions in the vicinity of the AtoN.

Application specific messages intended for general use and with world-wide application should be transmitted in messages in the international branch of the application specific messages.  An example of this type of information is meteorological and hydrological information.

Messages intended for general use but in a limited area can preferably be transmitted in messages in the regional branch of the application specific messages.  An example of this is the ‘lock order message’ used by the St Lawrence Seaway. The interpretation of these messages must then be implemented in the display systems used by the ship in the region.

  - Message 6, Addressed Binary Message

An AtoN AIS station may use Message 6 to carry information for AtoN monitoring purposes, addressed to the AtoN service provider. An acknowledgement re-transmission procedure following transmission of Message 6 is optional in this case.

- AIS on Floating Aids to Navigation

The position derived from an internal EPFS (Electronic Position Fixing System) can be used together with a reference position and a ‘guard zone’ to monitor the position of a buoy and to generate an ‘off station’ condition. This would be indicated in the AtoN AIS message 21 by setting the off-station flag.

IALA defines an Aid-to-Navigation as: "a device or system external to vessels designed and operated to enhance safe and efficient navigation of vessels and/or vessel traffic." The IALA Navguide stipulates: "A floating aid to navigation, which is out of position, adrift or during the night is unlighted, may itself become a danger to navigation. When a floating aid is out of position or malfunctioning, navigational warnings must be given." Therefore, a station, which transmits Message 21, should also transmit Safety Related Addressed Message (Message 12) upon detecting that the floating Aid-to-Navigation has gone out of position or is malfunctioning, at the competent authority's discretion.

- AIS on Fixed Aids to Navigation

AIS on a fixed Aid to Navigation such as a lighthouse could

- Broadcast its own messages, and optional local hydrological and meteorological data

- Forward messages from other AIS-equipped AtoNs for monitoring purposes

- Broadcast Synthetic AIS or Virtual AtoN messages.

- AIS on Offshore structures

The position of offshore structures is very important for safety and navigational purposes because they are seen as hazardous objects. Fixed offshore structures include offshore wind turbines and fixed offshore structures e.g. fixed drilling platforms. Message 21, ‘Aids-to-Navigation Report’ may be the most appropriate message for reporting position of an offshore structure. Other messages, such as Message 8, indicating, for example, an area to be avoided, could be broadcast.

- Radar Reference Target

With the ever-increasing requirement for integrated display systems, the problems of aligning two or more systems on one display screen are increased. If two (preferably three) AIS stations can be installed on prominent, fixed radar targets within an area of special interest, such as a harbour or harbour approach, the AIS EPFS positions, radar echoes and chart symbols of each of these targets can be used to align the three displays. This can result in reduced ambiguity and a less cluttered display. AIS for AtoNs can be a suitable technique for this purpose.

- The AtoN AIS station

The AtoN AIS station needs to perform in accordance with the documents specified in References 1, 2 and 3 to maintain harmony with the AIS system requirements.

The primary considerations for selecting the functional design of an AtoN AIS unit are power consumption and complexity, the latter being a direct influence on unit cost.

Most floating AtoN sites and many fixed sites have limited power availability due to physical constraints, and/or have a high cost of energy. An AtoN AIS unit derived directly from a ship AIS unit will likely have a power requirement beyond the practical or economic power available at the intended location site.

Besides the cost of energy, the cost of the AtoN unit itself is an important consideration for the service provider.

Other factors which influence the system selected to achieve AtoN AIS functionality are the AtoN type (where fixed AtoNs may for example require more functionality than smaller floating AtoNs), MTBF considerations, and the effect on the VDL.

  - Achieving the functionality of an AtoN AIS station

Three of the ways in which the AtoN AIS station functionality can be achieved are as follows.

- Autonomous, continuous system

- Autonomous non- continuous system

- Assigned time slot system.

  - Autonomous, continuous system

    - Functional block diagram

This system is realised in functional form by the method shown in Figure 1.

Figure 1: Functional block diagram of an autonomous continuous AtoN AIS station

Controller

Power Supply

BIIT

AIS RX

AIS RX

AIS TX

## Autonomous continuous

AtoN Interface

e.g. IEC 61162

Internal Sync Source

DSC

Configuration

EPFS

    - Description of functional elements

The following functional elements of the autonomous continuous AtoN are required (compulsory) in the minimum configuration of an autonomous continuous AtoN :

- Receiver: The minimum configuration may consist of one receiver for one AIS channel operation.

- TDMA transmitter

- Controlling unit: The controller unit is the interface to the AtoN and allows for input and supervision of the AtoN parameters such as battery voltage, light status etc.
The controller unit also checks the AtoN position against the preset position and allowed swinging radius. The controller unit composes the output messages to the VDL.

- Internal sync source: The internal sync. source may be included in the EPFS

- EPFS: Electronic Position Fixing Source to determine the AtoN position.

- Built-In-Integrity-Test unit (BIIT)

- Power supply

The following functional elements are optional:

- Additional receiver(s).

- DSC functionality

    - Characteristics

The characteristics of this system are:

- Full functionality and fully compliant to ITU-R-M1371-1.

- Ability to work independently using RATDMA access scheme.

- Can utilize  FATDMA slot allocations(reserved by Base station)

- Includes input and output interfaces for transmitting AtoN messages and control/configuration of the aid.

    - Applications

- Critical AtoN, no power limitation, good communications

- High capability/flexibility, range of messages

- Monitoring capability,

- Typically an onshore lighthouse with mains power, landline

  - Autonomous, non-continuous system

    - Functional block diagram

This system is realised in functional form by the method shown in Figure 2.

Figure 2: Functional block diagram of an autonomous non-continuous AIS base station

Controller

Power Supply

BIIT

AIS RX

AIS RX

AIS TX

Autonomous non-continuous

AtoN Interface

e.g. IEC 61162

Internal Sync Source

DSC

Configuration

EPFS

Timing

device

    - Description of functional elements

The following functional elements of the autonomous non-continuous AtoN are required (compulsory) in the minimum configuration of an autonomous non-continuous AtoN :

- Receiver: The minimum configuration may consist of one receiver for one AIS channel operation.

- TDMA transmitter

- Controlling unit: The controller unit is the interface to the AtoN and allows for input and supervision of the AtoN parameters such as battery voltage, light status etc.
The controller unit also checks the AtoN position against the preset position and allowed swinging radius. The controller unit composes the output messages to the VDL.

- Timing device: The timing device is responsible of scheduling the active/idle operation periods according to the setting made by configuration.

- Internal sync source: The internal sync. source may be included in the EPFS

- EPFS: Electronic Position Fixing Source to determine the AtoN position.

- Built-In-Integrity-Test unit (BIIT)

- Power supply

The following functional elements are optional:

- Additional receiver(s).

- DSC functionality

    - Characteristics

The characteristics of this system are:

- Ability to work independently using RATDMA access scheme.

- Can utilize  FATDMA slot allocations(reserved by Base station)

- Includes input and output interfaces for transmitting AtoN messages and control/configuration of the aid.

- Ability to be non-continuous e.g. “sleep mode capabilities”

- Limited assignment capabilities

- Limited interrogation capabilities

- Allows for intelligent AtoN control “transmit only when there is AIS traffic” e.g. for power saving purposes

    - Applications

- Limited power, limited messages

- Communications limited

- Typically isolated minor lights, buoys outside port area

  - Assigned time slot system

    - Functional block diagram

This system is realised in functional form by the method shown in Figure 3.

Figure 3: Functional block diagram of an AIS base station

Controller

Power Supply

BIIT

AIS TX

Assigned time slot

AtoN Interface

e.g. IEC 61162

Internal Sync Source

Configuration

EPFS

Timing

device

    - Description of functional elements

The following functional elements of the assigned timeslot AtoN are required (compulsory) in the minimum configuration of an autonomous continuous AtoN :

- TDMA transmitter

- Controlling unit: The controller unit is the interface to the AtoN and allows for input and supervision of the AtoN parameters such as battery voltage, light status etc.
The controller unit also checks the AtoN position against the preset position and allowed swinging radius. The controller unit composes the output messages to the VDL.

- Timing device: The timing device is responsible of scheduling the active/idle operation periods according to the setting made by configuration.

- Internal sync source: The internal sync. source may be included in the EPFS

- EPFS: Electronic Position Fixing Source to determine the AtoN position.

- Built-In-Integrity-Test unit (BIIT)

- Power supply

    - Characteristics

The characteristics of this system are:

- Limited functionality

- No receive capability e.g. Cannot be assigned via the VDL, dependent on FATDMA slot allocation from base station in the vicinity

- Includes input interfaces for transmitting AtoN messages

- No interrogation capabilities

    - Applications

- Significantly reduced power requirement, cost & complexity

- Restricted application (need base station, dependent on base station for reliable operation)

- Limited message capability, cannot be controlled via VDL

- High density AtoN coverage

- Typically buoys in port area

- References

Reference 1

ITU, Technical Characteristics for a Universal Automatic Identification System Using Time Division Multiple Access in the VHF Maritime Mobile Band, ITU-R M.1371-1

Reference 2

IALA, Technical Clarifications on Recommendations ITU-R M.1371-1, Edition 1.3 or later. (The most recent edition should be used.)

Reference 3

IEC document to be developed (may become part of the IEC Class B Standard)

Informative Annex, Message 21

(for the latest description of this message, refer to "Technical Clarifications on Recommendation ITU-R M.1371-1", latest edition)

The following text and tables are taken from “IALA Technical Clarifications on Recommendation ITU-R M.1371-1”, edition 1.3. The readers should ensure that they use the latest edition of this document.

This message should be used by an AtoN AIS station. This station may be mounted on an Aid to Navigation or this message may be transmitted by a fixed station when the functionality of an AtoNstation is integrated into the fixed station. This message should be transmitted autonomously at a Reporting Rate of once every three (3) minutes or it may be assigned by an Assigned Mode Command (Message 16) via the VHF data link, or by an external command. This message should not occupy more than two slots.

TABLE 34

| Parameter | Number of bits | Description |
| --- | --- | --- |
| Message ID | 6 | Identifier for this message 21 |
| Repeat Indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. 0 - 3; default = 0; 3 = do not repeat any more. |
| ID | 30 | MMSI number (1) |
| Type of Aids-to-Navigation | 5 | 0 = not available = default; refer to appropriate definition set up by IALA; refer to Table 34bis. |
| Name of Aids-to-Navigation | 120 | Maximum 20 characters 6 bit ASCII, "@@@@@@@@@@@@@@@@@@@@" = not available = default. The name of the Aid-to-Navigation may be extended by the parameter "Name of Aid-to-Navigation Extension" below. (1) |
| Position accuracy | 1 | 1  high ( 10 m; Differential Mode of e.g. DGNSS receiver) 0  low ( 10 m; Autonomous Mode of e.g. GNSS receiver or of other Electronic Position Fixing Device) ; Default = 0 |
| Longitude | 28 | Longitude in 1/10 000 min of position of Aids-to-Navigation (180 degrees, East  positive, West  negative. 181 degrees (6791AC0 hex) = not available = default) |
| Latitude | 27 | Latitude in 1/10 000 min of an Aids-to-Navigation (90 degrees, North  positive, South  negative, 91 degrees (3412140 hex) = not available = default) |
| Dimension/Reference for Position | 30 | Reference point for reported position; also indicates the dimension of an Aid to Navigation in metres (see 4.1.1.4 above).[New figure and table to be created in the clarifications document.] |
| Type of Electronic Position Fixing Device | 4 | 0 = Undefined (default); 1 = GPS, 2 = GLONASS, 3 = Combined GPS/GLONASS, 4 = Loran-C, 5 = Chayka, 6 = Integrated Navigation System, 7 = surveyed.For fixed AtoNs and virtual/pseudo AtoNs, the surveyed position should be used. The accurate position enhances its function as a radar reference target. 8 – 15 = not used. |
| Time Stamp | 6 | UTC second when the report was generated by the EPFS (0 –59, or 60 if time stamp is not available, which should also be the default value, or 61 if positioning system is in manual input mode, or 62 if Electronic Position Fixing System operates in estimated (dead reckoning) mode, or 63 if the positioning system is inoperative) |
| Off-Position Indicator | 1 | For floating Aids-to-Navigation, only: 0 = on position; 1 = off position; NOTE – This flag should only be considered valid by receiving station, if the Aid-to-Navigation is a floating aid, and if Time Stamp is equal to or below 59. For floating AtoN the guard zone parameters should be set on installation. |
| Reserved for regional or local application | 8 | Reserved for definition by a competent regional or local authority. Should be set to zero, if not used for any regional or local application. Regional applications should not use zero. |
| RAIM-Flag | 1 | RAIM (Receiver Autonomous Integrity Monitoring) flag of Electronic Position Fixing Device; 0 = RAIM not in use = default; 1 = RAIM in use) |
| Virtual AtoN Flag | 1 | 0 = default = real A to N at indicated position; 1 = virtual AtoN, does not physically exist, may only be transmitted from an AIS station nearby under the direction of a competent authority. (See 4.1.1.4.V above) |
| Assigned Mode Flag | 1 | 0 = Station operating in autonomous and continuous mode =default 1 = Station operating in assigned mode |
| Spare | 3 1 | Spare. Not used. Should be set to zero. |
| Name of Aid-to-Navigation Extension | 0, 6, 12, 18, 24, 30, 36, ... 84 | This parameter of up to 14 additional 6-bit-ASCII characters for a 2-slot message may be combined with the parameter "Name of Aid-to-Navigation" at the end of that parameter, when more than 20 characters are needed for the Name of the Aid-to-Navigation. This parameter should be omitted when no more than 20 characters for the name of the A-to-N are needed in total. Only the required number of characters should be transmitted, i. e. no @-character should be used. |
| Spare | 0, 2, 4, or 6 | Spare. Used only when parameter "Name of Aid-to-Navigation Extension" is used. Should be set to zero. The number of spare bits should be adjusted in order to observe byte boundaries. |
| Number of bits | 272 – 360 | Occupies two slots. |

Footnotes:

This message should be transmitted immediately after any parameter value has been changed.

(1) When generating synthetic or virtual AtoN targets the MMSI should be the transmitting station’s MMSI. In order to discriminate between the individual synthetic or virtual AtoN targets, the name field should be used as an unique alpha-numeric identifier. The receiving application should be able to assemble synthetic or virtual AtoN targets’ identities by combining the MMSI and the name fields. For Real AtoN targets only the MMSI should be sufficient.

Note on Aids-to-Navigation within AIS:

The competent international body for Aids-to-Navigation, IALA, defines an Aid to Navigation as: "a device or system external to vessels designed and operated to enhance safe and efficient navigation of vessels and/or vessel traffic." (IALA Navguide, Edition 1997, Chapter 7).

The IALA Navguide stipulates: "A floating aid to navigation, which is out of position, adrift or during the night is unlighted, may itself become a danger to navigation. When a floating aid is out of position or malfunctioning, navigational warnings must be given." Therefore, a station, which transmits Message 23 21, could also transmit Safety Related Addressed Message (Message 12) upon detecting that the floating Aid-to-Navigation has gone out of position or is malfunctioning, at the competent authority's discretion.  This message 12 would be addressed to targets within a predetermined but configurable range from the AtoN.

Table 34bis  Type of A to N

The nature and type of AtoN can be indicated with 32 different codes, as shown below

|  | Code | Definition |
| --- | --- | --- |
|  | 0 | Default, Type of A to N not specified |
|  | 1 | Reference point |
|  | 2 | RACON |
|  | 3 | Structure off shore, such as oil platforms, wind turbines,. (Note: This code should identify an obstruction that is fitted with an Aid-to-Navigation AIS station.) |
|  | 4 | Spare |
| Fixed A to N | 5 | Light, without sectors |
|  | 6 | Light, with sectors |
|  | 7 | Leading Light Front |
|  | 8 | Leading Light Rear |
|  | 9 | Beacon, Cardinal N |
|  | 10 | Beacon, Cardinal E |
|  | 11 | Beacon, Cardinal S |
|  | 12 | Beacon, Cardinal W |
|  | 13 | Beacon, Port hand |
|  | 14 | Beacon, Starboard hand |
|  | 15 | Beacon, Preferred Channel port hand |
|  | 16 | Beacon, Preferred Channel starboard hand |
|  | 17 | Beacon, Isolated danger |
|  | 18 | Beacon, Safe water |
|  | 19 | Beacon, Special mark |
| Floating A to N | 20 | Cardinal Mark N |
|  | 21 | Cardinal Mark E |
|  | 22 | Cardinal Mark S |
|  | 23 | Cardinal Mark W |
|  | 24 | Port hand Mark |
|  | 25 | Starboard hand Mark |
|  | 26 | Preferred Channel Port hand |
|  | 27 | Preferred Channel Starboard hand |
|  | 28 | Isolated danger |
|  | 29 | Safe Water |
|  | 30 | Special Mark |
|  | 31 | Light Vessel / LANBY/ Rigs |

Note:

(1) The types of AtoNs listed above are based on the IALA Maritime Buoyage System, where applicable.

(2) There is potential for confusion when deciding whether an aid is lighted or unlighted. Competent authorities may wish to use the regional/local section of the message to indicate this.

Rationale for Clarification:

1.) Rationale for introduction of parameter "Name of A-to-N Extension": IALA recognized that there are many cases, where longer designations for A-to-Ns are required (for details refer to the IALA AIS Guidelines). For a two-slot message, the maximum number of additional characters is 14. The rationale for not allowing more than 2 slots is to optimise the load of the VDL.

2.) Rationale for introduction of parameter "Virtual- A-to-N flag": In some situations it might be useful to transmit information on a physically non-existing AtoN. A method is required to differentiate this type of virtual AtoN target from real AtoN objects. It is proposed that one of the spare bits in Message #21 be used as a "Virtual AtoN flag".

3.) Rationale for introduction of the Type of AtoN table: Recommendation ITU-R M.1371-1 requires so.

4.) Rationale for the introduction of the Assigned mode flag: Since the A-to-N AIS Station may be assigned a different reporting rate, the entering into assigned mode needs to be indicated.