August 14th, 2003

MARKING OF NEW DANGERS

in

THE NETHERLANDS

# Introduction

At the 2nd Session of the IALA ANM Committee (April 14-17, 2003), the Committee noted the comments of the PAP on Emergency Wreck Marking.

In light of recent incidents the Committee discussed the requirements for timely and conspicuous marking of wrecks and also expressed their concern about the awareness level amongst mariners of the marking process for wrecks, the make up of the bridge watch team and keeping a proper look out.

It was decided that IALA documentation on the marking of wrecks should be reviewed to see if there are any gaps in the information presented and/or how the information could be better promulgated.

In that scope, for input in ANM3, members of the Committee were asked to provide examples on how, and how quickly new dangerous wrecks are marked by their administration.

# The North Sea

The North Sea is one of the busiest shipping areas in the world. The Dutch part of the North Sea, also known as the Netherlands Continental Shelf (NCS), represents only a tenth of the whole North Sea area. Nevertheless, a quarter of all shipping traffic can be found sailing here. Each year the NCS area sees around 260,000 ship journeys (‘route-bound’ and ‘non-route-bound’ traffic), representing a total of 27 million miles.

Related to safe shipping, an important characteristic of the NCS is that in particular the coastal zone is pretty shallow; the maximum water depth in the total NCS area does not exceed 50 m. If an accident occurs (e.g. sunken ship), often the wreck causes a danger for shipping.

# Marking process of New Dangers

Criteria:

In the early eighties of the past century, the Netherlands authorities felt it necessary that certain uniform criteria on wreck marking for the whole NCS should be developed.

The reason was, that until that time along the Netherlands coast different depth criteria on wreck marking were in use. Only with the Federal Republic of Germany arrangements were made for the Terschelling-German Bight TSS and Inshore Traffic Zones.

In the pragmatic approach to the problem, it was necessary that a distinction was made between obstructions of natural and artificial nature. Most of the existing shipping lanes and routes on the Netherlands part of the North Sea are situated outside the sandbanks, grounds and other natural dangers. These dangers are shown in the nautical charts and when necessary marked by fixed or floating AtoN’s.

Therefore it was concluded that only (depth) criteria for artificial dangers, such as wrecks, should be formulated. (All offshore installations and remnants thereof (conductor pipes, etc.), which protrude above the sea bottom must be marked according to the Offshore Mining Act to protect shipping, fishing vessels and the environment. Therefore these dangers are excluded.)

To set up criteria a lot of source material has been used, such as:

- Handbook on Ships’ Routeing (IMO Publication);

- Traffic data collected by maritime authorities and institutes;

- Admittance policy on maximum draught for the various ports;

- The forthcoming alternations of TSS ….

The result was a chart of the NCS divided in various areas, with for each area a certain “depth to be maintained”. (During the years this chart has been amended when necessary.)

The so called “Wreck marking depth standards chart”, shows for each selected area a minimum water depth reduced to MLLWS.

Depending on the importance of each area related to shipping, these (minimum) depth standards are: 10 m, 14 m, 16 m, 18 m, 23 m and 29 m.

In the event of a new danger/wreck above which the minimum water depth is less than the standard water depth required for that area, the danger/wreck will have to be marked properly until the danger has either been removed, or the minimum required water depth has been re-established.

First action(s):

In the Netherlands it is almost an “unwritten law” that the marking of a new danger for shipping  always has the highest priority. How quickly the new danger can be marked depends on many factors. The most important ones are: timely and exact information, weather conditions and the distance of the new danger from the nearest buoy tender station/depot.

In a situation where it can be foreseen that a ship will sink within a matter of time, it is possible that the  buoy tender is already standby on location. After the ship has sunk, the wreck can be marked directly.

In case of an “unexpected” new danger (e.g. because of a collision), the moment of marking after the incident has occurred mainly depends on the above mentioned factors (ranging from 24 hours to a couple of days and sometimes even more ….!). In that situation it is very important that during that time shipping is informed as adequate as possible with all means, such as frequent navigation warnings by radio and Navtex. Depending on the local circumstances, e.g. size of the wreck in relation to the water depth and traffic density, the competent maritime authority can also decide to establish a temporarily VTS, either by a shore station or - when available - by a (coast guard) vessel on location.

To mark a new wreck properly, the exact position has to be known. However in reality in some cases, just after an incident has occurred, only an approximate position is available. If the exact position is not known, additional (visual) information can be obtained from debris and/or if trails of oil leaking from the wreck come to the surface. Based on this information it can be decided to mark the wreck position “temporarily”. Frequent and adequate navigation warnings to the mariner are essential and should mention the position as being “approximate”.

After the wreck has been surveyed (exact position, heading and minimum water depth above wreck), the wreck can be marked properly according to requirements of the competent maritime authority and taking shipping pattern on location into account.

Positions of the number and types of marks will be broadcasted on NAVTEX and published by means of the NtM’s.

The marking:

Without any exception, in the Netherlands all new dangers will be marked according to the IALA MBS, rules for a New Danger. Although these rules allows the use of cardinal as well as lateral (lighted) marks, in the Netherlands for that purpose only cardinal marks are used. Cardinal marks are more conspicuous than lateral marks and give extra specific information for a safe shipping.

In case of a new danger/wreck of some importance, irrespective of the depth standard of the particular area, in principal the wreck will be marked with at least two identical cardinal buoys.

In the relatively narrow traffic lanes (of a TSS) it is not always possible to mark a new wreck according this rule. The configuration of marking in such a situation depends on the position (and size) of the wreck in the lane. In the scope of safe shipping the main goal is to “create” the maximum free space as possible for passing. Which buoy will be fitted with racon depends on the traffic pattern. As a supplement to the marking, temporarily VTS on location can be a realistic and practical option.

Although the IALA MBS, in case of duplicate marks, does not recommend which one of the two buoys may be fitted out with a racon (code D), in the Netherlands it is common that this is the outer one.

The main reason is to hold off shipping from the danger at a wide as possible berth. Under bad weather conditions (reduced visibility) in particular, quite often only the racon signal can be observed and recognized.

Question: In the scope of the safety of shipping, is it useful and realistic/feasible that IALA recommends only one category of marks for the Marking of New Dangers? (Note: For the discussion, it should be very interesting to know why some Authorities “never” make use of cardinal marks!)

Question: In the scope of the safety of shipping, in case of duplicate marks, is it useful that IALA recommends to fit the racon (if needed) either on the outer or on the inner mark?

In most cases the duplicate mark will be removed when the information concerning the new danger has been sufficiently promulgated. It is realistic to assume that by means of the NtM’s, after a period of six to eight weeks, the new danger is known to the mariners world wide.

By removing the duplicate mark, in the procedures used by the Netherlands, it implicates the removal of the racon too, unless the competent authority decides to maintain a racon coverage of the danger/wreck. In that situation the duplicate marks have to change position. Depending on the nautical importance the racon (with code D) can be maintained till the danger/wreck has been removed, sometimes after a period of many months!

Question: Racon code D is specific for a New Danger. After what period a New Danger is still a New Danger?

The extend of the hazard for shipping highly influences the required number of marks to cover the wreck properly. In highly hazardous situations, “properly marked” means that the danger has to be covered more or less all around, sometimes even by five cardinal buoys (like the TRICOLOR and ASSI EURO LINK).

As already mentioned, on the Netherlands part of the North Sea continental shelf, every new danger/wreck has to be marked. This also means that in case the water depth above a new wreck is more than the minimum standard water depth of that particular area, the wreck will be marked as described above, although for that situation there is no need to mark the wreck.

However, it can happen that a new wreck, at first lying at his side on the sea bottom, after a certain time (because of e.g. scouring) will raise to a more upright position. If that occurs, the water depth above the wreck changes substantially, possibly even to a depth less than the standard water depth of the particular area. Practice is that after at least a year of settlement, the wreck will be dragged in order to assess whether the mark can be removed or not. If the depth is less than the required depth it means in principle that the wreck has to  be removed, either partly or completely. However, practice is that this will not happen always, in that case the wreck requires adequate marking permanently.

The distance of the marks to the danger/wreck depends on the water depth on location, the size of the wreck, its position and heading of the wreck. In general the closest distance of the mark varies between 150 and 250 m. In case of duplicate marking, the distance between the outer and inner mark is approximately 150 m.

For cardinal marks the choice of the rhythm of light is strictly limited to the VQ or Q ones (four types cardinal marks). For the purpose of conspicuously marking the use of VQ is preferable to Q. However on the NCS a lot of lighted cardinal marks are used for the marking of banks and grounds along the coast; in general west and north cardinal marks. For the recognition of the individual marks both rhythms of light are in use (e.g. two or more consecutive west cardinal marks!).

Because of the route structure along the coast of the Netherlands, in case of a new danger mostly a west or north cardinal mark has to be used. If the new danger location, to be marked west cardinal, is close to a west cardinal mark (with a VQ(9) 10s light) covering a bank or ground, the rhythm of light of the cardinal wreck mark will be Q(9) 15s.

Question: To prevent confusion/misinterpretation, what distance between the same marks should be the minimum distance to allow the same rhythm of light?

# An example

General information:

On Saturday January 25th , 2003 the Dutch Roro carrier ASSI EURO LINK sunk, shortly after a collision at 07.25 h local time with the SEAWHEEL RHINE, in Friesland Junction (junction of the “Deep Water” (DW) route and the “West Friesland” TSS). Friesland Junction is, because of crossing traffic, a precautionary area with an average number of 50 ship movements a day.

The main dimensions of the ASSI EURO LINK are: length overall: 163.5 m; breath extreme: 21.16 m. The position of the wreck is approximately 45 nm NW of the island Terschelling (buoy tender station / depot).

At the time of the accident the weather conditions were: SW-W 5-6 Bf increasing to 7.

The Deep Water route (partly mandatory) is specifically for deep draught vessels and vessels with dangerous cargo. The standard water depth to be maintained of the DW route, including Friesland Junction, is 29 m MLLWS. The water depth on the position of the accident is approximately 40 m MLLWS.

First actions:

Shortly after the Netherlands Coast Guard was informed, by DSC and radio contact with the SEAWHEEL RHINE, the following actions started almost simultaneously:

- navigation warnings by NAVTEX and radio (extremely dangerous wreck at position ….. , wide berth requested, etc.);

- informing lifeboat service;

- informing buoy tender TERSCHELLING at nearest by station (Terschelling);

- informing coast guard vessel/tug WAKER at station Den Helder;

- informing the North Sea Directorate (urgent need for acoustic survey).

On the 25th at 15.30 h the WAKER was on location to start up active VTS, followed by the TERSCHELLING at 17.00 h. On board two cardinal buoys, a west cardinal with racon D and an east cardinal one.

Because of debris, including ropes etc. (thrusters!) and lack of the precise position, it was decided to mark the wreck after the survey. For the time being the main task of the TERSCHELLING was to cover the wreck position and inform/warn passing ships (VTS).

An acoustic survey on January 26th showed a minimum water depth above the wreck of 18.8 m LLWS.

The ASSI EURO LINK lies in two parts on his port side, with a general heading of 253.

At 13.45 h the wreck was marked: the west cardinal buoy with racon D 150 m west of the bow and the east cardinal one 150 m east of the stern of the ASSI EURO LINK.

It was already decided to mark the wreck all around. Because of bad weather conditions the supplementing buoyage - a (duplicate) west, north and south cardinal buoy - were positioned five days later. During that operation, also because of the bad weather, it was not possible to change the west cardinal racon buoy to the outer position.

Up to now the wreck is marked with five cardinal buoys:

- two duplicate west cardinal buoys, the inner with racon D, both VQ(9) 10 s;

- one north cardinal buoy, VQ;

- one east cardinal buoy, Q(3) 10 s;

- one south cardinal buoy, VQ (6) + LFl 10s.

Active VTS was carried out from the date of the accident till February 14th , either by the WAKER (on board two North Sea pilots with VTS qualifications) or the buoy tender TERSCHELLING. After that date the WAKER stayed near the wreck location for another two weeks (passive VTS), observing passing ships and when necessary supplying VTS.

Up to now, shipping is still informed of the presence of the danger by NAVTEX.

Because of the danger the wreck ASSI EURO LINK is creating, especially for deep draught ships, the wreck should be removed. However to re-establish the standard water depth for the location, there are now plans to achieve this by dredging activities after the bunker oil has been removed.

# Future

The continuous development, improvement and implementation of advanced electronic systems (e.g. ECDIS and AIS), offers the mariner an increasingly amount of real time information. In the scope of timely and adequate marking of a new danger/wreck, it should be useful that IALA examine the possibilities of virtual marking, as long as the new danger/wreck has not been really marked.

Adrian Verbaan

Annex 1: Route structure on the Dutch part of the North Sea (NCS). Position of ASSI EURO LINK.