IALA

Aids to Navigation Management (ANM) Committee

Task 11

Monitor the Work of the IHO on chart datums and accuracies

Rapporteur:  Guy Platten (Scotland)

# Aim

- To provide the IALA ANM Committee with a progress report on Task 11.

# Background

- At the inaugural IALA ANM it was agreed to appoint a Rapporteur to monitor the work of the International Hydrographic Office on chart datums and accuracies.

- The introduction of precision navigation systems (e.g. DGPS) and widespread adoption of Integrated Bridge Systems has highlighted problems with the current status quo i.e.

- GPS positions are expressed in WGS84 datum whereas large parts of the World are charted in a multitude of other datums.  Whilst many charts contain information as to how to modify GPS positions to agree with the chart the user must actively seek this information out and make the necessary corrections, not always easy when using electronic charts.

- The high levels of accuracy of GPS (GNSS) (in the order of 13 metres or less) has meant that seafarers have a higher expectancy of navigational accuracy than can be often be supported by survey data of many existing charts.  Much of the world (particularly in the waters of those countries that are not members of the IHO) is surveyed to very inaccurate standards reflecting the technology and surveying techniques in existence at the time of survey.  Even around the United Kingdom coastline there remains areas which have not been surveyed in over a century.

- The International Maritime Organisation (IMO) Maritime Safety Committee issued a circular (SN/Circ. 213) in May 2000 on ‘Guidance on Chart Datums and the Accuracy of Positions on Charts’.  This remains the best official explanation of the present situation.

# Progress since ANM1

- The IHO have now responded to the concerns raised about the present situation at the last ANM Meeting.  The IHO acknowledges that there are many areas of the world not surveyed to modern standards. The resources required to carry out such surveys are extensive and consequently each nation must necessarily priotise the areas in which surveys are conducted.  Chapter 5 of SOLAS (which came into force in July 2002) places clear requirements on Contracting Governments regarding hydrographic surveys and IHO has instigated a revision of IMO Resolution A532(13) concerning these obligations. IHO have also clarified that all approved Electronic Navigational Charts (ENCs) must be referred to the WGS 84 Horizontal datum although acknowledging that many existing paper and Raster charts still use a variety of datums. IHO have welcomed the oportunity of disucssing IALA concerns further.

6.	A paper was  presented (copy attached) at the 2nd International ECDIS conference although the author was speaking on behalf of the Northern Lighthouse Board and not IALA.

# Proposed Work Package during next 6 months

- Subject to ANM Committee approval it is intended that the Rapporteur carries out the following actions during the interval to ANM4 in April 2004.

- To meet with IHO to discuss further the relevant issues.

- Continue to monitor the role of IMO and IHO in this important area, contacting the relevant personnel within these Organisations in order to maintain discrete pressure.

c.	To continue raising concerns within his own National Government regarding the over reliance on Electronic Charts, particularly in light of developments in Integrated Bridge Management Systems.

## Guy Platten

Northern Lighthouse Board

Rapporteur Task 11