# REVIEW THE NEED FOR A BACK UP SYSTEM TO GLOBAL NAVIGATION SATELLITE SYSTEM (Task 9, in consultation with RNAV)

## Introduction

Following a request from Council the ANM Committee considered the need for a back up system to Global Navigation Satellite System (GNSS).

It was accepted by the Committee that there was a growing dependency on GNSS not just in the maritime field but in many other areas. In the new era of security concerns, there is a realisation that the vulnerability of this system poses a significant threat to worldwide applications, including critical safety of life systems. This dependancy has risen sharply with the ending of GPS Selective Availability (S/A) on 1st May 2000.

GNSS is a key input to Electronic Chart Display Information Systems (ECDIS) as well as other Electronic Charting Systems (ECS), Automatic Identification System (AIS) and Radar displays.  This has  resulted in a tendency to rely heavily on electronic systems - ‘heads down’ navigation - with the Mariner increasingly reluctant to use alternative means for position verification, as recommended by the International Maritime Organisation (IMO).

Therefore, back up systems must be carefully identified.

## Existing Terrestrial Radionavigation Systems

A number of alternatives currently exist to GNSS, including LORAN-C and Mobile Phone Technology.  However, the take up and use of  these systems is uncertain as the geographic areas covered are limited and the future of these radionavigation alternatives are by no means guaranteed. In addition the use of these systems as a viable electronic backup has still to be proven and, as of yet, few effective dual receivers are available in the market place.

## Conventional Aids to Navigation

Visual and Radar Aids to Navigation continue to provide an essential alternative  to GNSS although it is accepted that they are rarely used as the primary means of position verification any more. However the ANM Committee felt that this only provides part of their usefulness and that they should not be seen purely in this context. They form an essential safety function, in not only phyiscally marking hazards, but also in allowing the mariner to develop critical spatial and environmental awareness.

It is accepted, however, that if the onboard solutions were sufficiently reliable and had good integrity, then there would be a reduced need for certain types of traditional AtoN.

## Need for a Backup System

After discussion, the Committee came to the view that a backup system is essential for GNSS, however the type and level of backup must obviously be determined by the level of risk (discuss in plenary – such as level of service) and GPS dependent systems (such as AIS).  For example in the middle of the Pacific Ocean a suitable backup could consist purely of celestial navigation, dead reckoning and estimating position.  Alternatively, the backup system required in critical areas such as the Dover Straits or Singapore Straits needs to be significantly more robust as many vessels will be using integrated navigational systems such that a failure of GNSS could prove catastrophic.

In evaluating the need for a backup, the safety and acceptable performance risk areas were considered worthy of elaboration.

Safety - The transition arrangements from GNSS to alternative systems must also be considered from a practical and watchkeeping point of view. Visual AtoN provide a ‘reality check’, however integrating visual position verification into existing electronic systems is not always straightforward and will inevitably rely upon good continuation training and situation appreciation by mariners.  There may therefore be a need for additional Radio Navigation systems, to a common IMO approved standard, in safety critical areas to allow this progressive transition to take place smoothly.

Performance - It must be recognised that a loss of key operational capabilities, currently available with GNSS, is acceptable providing the vessel remains safe.

## Conclusions

The Committee endorses and supports many of the findings in the paper and the advice to Council produced by the RNAV Committee.  Whilst there needs to be a backup to GNSS, this does not necessarily require a radionavigation solution.  Whatever systems or procedures are deployed must be looked at in terms of the various stages of a voyage i.e. Ocean, Coastal, Harbour Approaches and Restricted Waters.

## Recommendations

The Committee advises that the draft IALA Recommendation on GNSS Vulnerability and Mitigation Measures be revised to take into account the requirements following a risk assessment.

Further, the Committee recommends that the Council directs the ANM Committee to produce detailed operational requirements for backup systems and thereafter refer the matter back to the RNAV committee for more detailed technical analysis.